# 3GPP TSG-RAN WG4 Meeting # 110				                                             R4-2400116

# Athens, Greece, February 26 – March 1, 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | 4256 | rev | - | Current version: | 17.12.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | (NR_pos_enh-Core) CR on positioning core requirements in RRC_INACTIVE state in R17 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_pos_enh-Core |  |  |  |  |  | Date: |  |  | 2024-02-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - The scaling factor for TEG in UE Rx-Tx measurement requirements is not correct. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Correct the scaling factor for TEG in UE Rx-Tx measurement requirements. - Other corrections e.g., signalling name, typos. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The positioning measurement requirements in RRC_INACTIVE are incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 37.571 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change 1>

5.6	NR measurements for positioning

5.6.1	Introduction

This clause contains requirements for UE capable of performing NR positioning measurements defined in TS 38.215 [4], including RSTD, PRS-RSRP, UE Rx-Tx time difference and PRS-RSRPP, in RRC_INACTIVE state.

The requirements in clauses 5.6.2, 5.6.3, 5.6.4 and 5.6.5 are applicable to PRS resources that do not collide with other DL signals/channels which include SSB, SIB1, CORESET0, MSG2/MSGB, paging and DL SDT. In addition, a UE is not expected to receive PRS resources that collide with a time interval starting at symbol m and ending at symbol m + N2, where symbol m is the last symbol in which the UE is configured to receive PDCCH and N2 is defined in clause 6.4 of [26, TS 38.214] for the subcarrier spacing μ of the DL PRS.

If a PRS resource is outside or partially overlapped with the intitial DL BWP, a PRS resource instance collides with another DL signal/channel if any portion of the other DL signal/channel overlaps with the time interval starting X symbols before the PRS instance and ending X symbols after the PRS instance, taking into account nr-DL- PRS-ExpectedRSTD-Uncertainty and nr-DL-PRS-ExpectedRSTD, where X is defined in Table 5.6.1-1.

Table 5.6.1-1: Value of X number of symbols

| FR |  | NR Slot length (ms) | X symbols |
| --- | --- | --- | --- |
| FR1 | 0 | 1 | 7 |
|  | 1 | 0.5 | 14 |
|  | 2 | 0.25 | 28 |
| FR2 | 2 | 0.25 | 14 |
|  | 3 | 0.125 | 28 |
| Note 1: The FR1 value applies if one or both of the serving cell and the positioning frequency layer are in FR1. FR2 value applies if both of the serving cell and the positioning frequency layer are in FR2. |  |  |  |

All measurement requirements specified in clauses 5.6.2, 5.6.3, 5.6.4 and 5.6.5 shall apply for any DRX configuration specified in TS 38.331 [2].

The requirements in clauses 5.6.2, 5.6.3, 5.6.4 and 5.6.5 are applicable provided that the cell selection procedure for the selected PLMN defined in TS 38.304 [1] is not triggered during PRS measurement period.

The requirements in clauses 5.6.2, 5.6.3, 5.6.4 and 5.6.5 apply provided that all PRS resources within a PFL are within up to 2 separate windows within TPRS,i for each positioning frequency layer i as defined in clauses 5.6.2, 5.6.3, 5.6.4 and 5.6.5, where each window is up to 10ms.

The UE is not required to perform additional SSB measurement for the SSB configured as QCL source of PRS resources.

When the UE is configured with measurement for more than one positioning requests, the measurement period for each request may be longer than measurement period when UE is configured with measurement for single positioning request.

5.6.2	RSTD measurements

5.6.2.1	Introduction

The requirements in clause 5.6.2 shall apply provided the UE has received NR-DL-TDOA-RequestLocationInformation message from the LMF via LPP [34] requesting the UE to measure and report DL RSTD measurements defined in TS 38.215 [4].

5.6.2.2	Requirements Applicability

The requirements in clause 5.6.2 apply for periodic and triggered RSTD measurements, provided:

-	PRS-RSTD related side conditions given in clause 10.1.23.2 for FR1 and FR2 are fulfilled, for a corresponding Band.

5.6.2.3	Measurement Capability

The UE PRS RSTD measurement capability in RRC_INACTIVE state is as indicated by the UE in NR-DL-TDOA-ProvideCapabilities, according to TS 37.355 [34].

#### 5.6.2.4	Measurement Reporting Requirements

The measurement reporting delay is defined as the time between the moment when the periodic measurement report is triggered and the moment when the UE is ready to transmit the measurement report over the air interface. If the UE supports reporting of NR positioning measurements via SDT, the UE may be able to report the measurements while it remains in RRC_INACTIVE state; otherwise, the UE will transition to RRC_CONNECTED state prior to transmitting the measurement report.

For RSTD measurements performed by the UE in RRC_INACTIVE state, the measurement reporting delay excludes all of the following:

-	additional delay caused by other LPP signalling on the DCCH,

-	delay uncertainty introduced when inserting the measurement report in the TTI of the uplink DCCH, equal to 2 x TTIDCCH where TTIDCCH is the duration of subframe or slot or subslot when the measurement report is transmitted on the PUSCH with subframe or slot or subslot duration,

-	any delay caused by unavailability of UL resources to transmit the measurement report,

-	any transmission delay needed by SDT,

-	the time needed to transition to RRC_CONNECTED state to report the measurements.

The reported RSTD measurement values contained in measurement reports shall be based on the measurement report mapping requirements specified in clause 10.1.23.3.

The RSTD measurements performed and reported according to this section shall meet the RSTD measurement accuracy requirements in clause 10.1.23.2, for each measured DL PRS resource.

5.6.2.5	Measurements Period Requirements

After receiving both NR-DL-TDOA-ProvideAssistanceData message and NR-DL-TDOA-RequestLocationInformation message from the LMF via LPP [34], the UE shall be able to measure multiple (up to the UE capability specified in Clause 5.6.2.3) DL RSTD measurements, defined in TS 38.215 [4], during the measurement period TRSTD,Total defined as:

TRSTD,Total=i=1LTRSTD,i+ L-1*maxTeffect,i

Where:

-	i is the index of positioning frequency layer,

-	L is total number of positioning frequency layers, and

-	Teffect,i is the periodicity of the PRS RSTD measurement in positioning frequency layer i

TRSTD,i is the measurement period for PRS RSTD measurement in positioning frequency layer i as specified below:

TRSTD,i= Kcarrier_PRS* NRx,TEG,i*NRxBeam,i*NPRS,islotN'*Lavailable_PRS,iN*Nsample-1*Teffect,i+Tlast,i ,

Where:

-	NRxBeam,i is the UE Rx beam sweeping factor:

-	NRxBeam,i = 1 if positioning frequency layer i is in FR1, and if positioning frequency layer i is in FR2

-		NRxBeam,i equals to the value as UE reported in supportedLowerRxBeamSweepingFactor-FR2 if the capability is reported by the UE for the band containing positioning frequency layer i, and LMF indicates lowerRxBeamSweepingFactor-FR2 in NR-DL-TDOA-RequestLocationInformation.

-		NRxBeam,i equals to 8, otherwise.

-	Kcarrier_PRS is a scaling factor for PRS-based NR positioning measurements in RRC_INACTIVE. If the UE supports parallelPRS-MeasRRC-Inactive-r17, Kcarrier_PRS = 1; otherwise,

-	If Srxlev ≤ SnonIntraSearchP or Squal ≤ SnonIntraSearchQ, Kcarrier_PRS=Kcarrier+1, where Kcarrier is defined in clause 4.2.2.4

-	If Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ, Kcarrier_PRS=Nlayers+1, where Nlayers is defined in clause 4.2.2.7.

-	NRx,TEG,i is the Rx TEG specific scaling factor:

-	NRx,TEG,i =1 if the UE is not configured by the LMF to measure a PRS resource with multiple Rx TEGs via measureSameDL-PRS-ResourceWithDifferentRxTEGs-r17 [34].

-	NRx,TEG,i is defined as follows if the UE is configured by the LMF with measureSameDL-PRS-ResourceWithDifferentRxTEGs-r17 [34] to perform measurement on same DL PRS resource of a TRP using different Rx TEGs in NR-DL-TDOA-RequestLocationInformation [34]:

-	NRx,TEG,i = P, if the UE is not capable of measuring same DL PRS resource simultaneously from multiple Rx TEGs, where P is the number of UE Rx TEGs that the UE is requested by LMF to measure the same DL-PRS Resource of a TRP indicated by measureSameDL-PRS-ResourceWithDifferentRxTEGs-r17 in [34], and in case ‘n0’ is indicated, P is the maximum number of Rx TEGs with which UE can support to measure the same PRS resource as reported in NR-UE-TEG-Capability.

-	NRx,TEG,i = PQ , if the UE is capable of measuring the same DL PRS resource simultaneously from multiple Rx TEGs, where Q is the number of UE Rx TEGs for measuring the same DL-PRS Resource simultaneously indicated by measureSameDL-PRS-ResourceWithDifferentRxTEGsSimul-r17 in [34].

-	NPRS,islot is the maximum number of DL PRS resources in positioning frequency layer i configured in a slot.

-	Lavailable_PRS,i is the time duration of available PRS in positioning frequency layer i to be measured within TPRS,i, and is calculated in the same way as PRS duration K defined in clause 5.1.6.5 of TS 38.214 [26]. For calculation of Lavailable_PRS,i, only unmuted PRS resources that are not fully overlapped with other higher-priority DL signals/channels are considered.

-	Nsample is the number of PRS RSTD samples, where

-	Nsample= 1 if the UE supports supportedDL-PRS-ProcessingSamples-RRC-Inactive [34], and the LMF requests the UE to perform positioning measurements with reduced number of samples, and meets the following conditions:

-	PRS bandwidth is within the initial BWP and

-	Magnitude of difference between the serving cell’s SS-RSRP and the neighbor cell’s PRS-RSRP is within 6 dB.

-	Nsample= 2 if the UE supports supportedDL-PRS-ProcessingSamples-RRC-Inactive [34], and the LMF requests the UE to perform positioning measurements with reduced number of samples, and does not meet the following conditions:

-	PRS bandwidth is within the initial BWP and

-	Magnitude of difference between the serving cell’s SS-RSRP and the neighbor cell’s PRS-RSRP is within 6 dB.

-	Nsample= 4 otherwise.

-	Tlast,i is the measurement duration for the last PRS RSTD sample in positioning frequency layer i, including the sampling time and processing time, Tlast,i = Ti + Tavailable_PRS,i ,

-	Teffect,i is the periodicity of the PRS RSTD measurement in positioning frequency layer i defined as:

Teffect,i = TiTavailable_PRS,i*Tavailable_PRS,i

Where:

-	Ti	corresponds to durationOfPRS-ProcessingSymbolsInEveryTms-r17 in TS 37.355 [34],

-	Tavailable_PRS,i=LCMTPRS,i,TDRX, the least common multiple between TPRS,i and the DRX cycle length TDRX

-	TPRS,i is the periodicity of DL PRS resource with muting on positioning frequency layer i.

If more than one PRS periodicities are configured in positioning frequency layer i, the least common multiple of PRS periodicities TperPRS with muting among all DL PRS resource sets in the positioning frequency layer is used to derive TPRS,i, where,

-	TperPRS with muting=Nmuting*TperPRS, is the PRS periodicity with muting per PRS resource,

-	TperPRS is the periodicity of PRS resource sets given by the higher-layer parameter DL-PRS-Periodicity.

-	Nmuting is the scaling factor considering PRS resource muting. Nmuting=TmutingPRS*Lmuting, where

-	TmutingPRS is the muting repetition factor given by the higher-layer parameter DL-PRS-MutingBitRepetitionFactor, and Lmuting is the size of the bitmap b1.

-	{N,T} is the UE capability combination per band for RRC_INACTIVE state where N is a duration of DL PRS symbols in ms corresponding to durationOfPRS-ProcessingSymbols-r17 in TS 37.355 [34],  T (ms) corresponds to durationOfPRS-ProcessingSymbolsInEveryTms-r17 in TS 37.355 [34], [ and T-N (>0) is the time required to process duration N of DL PRS symbols already buffered in memory], for a given maximum bandwidth supported by UE corresponding to supportedBandwidthPRS in TS 37.355 [34],

-	N’ is UE capability for number of DL PRS resources that it can process in a slot [in RRC_INACTIVE state as indicated by maxNumOfDL-PRS-ResProcessedPerSlot-RRC-Inactive-r17 specified in TS 37.355 [34].

The time TRSTD,Total starts from the first DRX cycle containing the DL PRS resource(s) in the assistance data after both the NR-DL-TDOA-ProvideAssistanceData message and NR-DL-TDOA-RequestLocationInformation message are delivered from LMF to the UE via LPP [34].

Note:	No per-positioning frequency layer requirement is applied in scenarios when multiple positioning frequency layers are configured.

If the DRX cycle is reconfigured during the RSTD measurement period, then the measurement period can be longer.

When PRS-RSRP is configured for DL-TDOA, RSTD and PRS-RSRP are performed over the same measurement period.

The measurement requirements do not apply to any PRS resource that always collides with other higher-priority DL signals/channels, as specified in clause 5.6.1.

Longer RSTD measurement period is expected when there are collisions between PRS resources and other higher-priority DL signals/channels.

If Kcarrier_PRS changes for any PFL during the measurement period, the measurement period could be longer.

The measurement requirements do not apply for a PRS resource, if the PRS resource is across two sampling duration of N within duration Lavailable_PRS,i.

The measurement requirements do not apply for a PRS resource, if time span of the PRS resource instance (including at least the minimum number of repetitions specified in the accuracy requirements) is greater than UE reported capability N.

The requirements in clause 5.6.2 do not apply if the PRS configuration given by higher layer paramters NR-DL-PRS-AssistanceData exceeds any of the UE measurement capabilities given by NR-DL-PRS-ResourcesCapability in NR-DL-TDOA-ProvideCapabilities, and it is up to UE implementation which PRS resources are measured, subject to UE measurement capabilities.

If cell re-selection occurs while RSTD measurements are being performed, then the UE shall continue and complete the on-going RSTD measurements after the cell selection is completed. The RSTD measurement period can be longer.

If the RRC state transition occurs from RRC_INACTIVE to RRC_CONNECTED state during the RSTD measurement period then the UE shall continue the RSTD measurement in the RRC_CONNECTED state. The RSTD measurement period can be longer.

5.6.3	PRS-RSRP measurements

5.6.3.1	Introduction

The requirements in clause 5.6.3 shall apply provided the UE has received a message from LMF via LPP [34] requesting the UE to measure and report PRS-RSRP measurements defined in TS 38.215 [4] and the UE is capable of supporting the PRS-RSRP measurement in RRC INACTIVE state.

5.6.3.2	Requirements applicability

The requirements in clause 5.6.3 apply for periodic and triggered PRS-RSRP measurements, provided:

-	PRS-RSRP related side conditions given in clause 10.1.24.2 are met for a corresponding Band.

5.6.3.3	Measurement Capability

UE PRS-RSRP measurement capability is as indicated by the UE in NR-DL-AoD-ProvideCapabilities according to TS 37.355 [34].

5.6.3.4	Measurement Reporting Requirements

The measurement reporting delay is defined as the time between the moment when the periodic measurement report is triggered and the moment when the UE is ready to transmit the measurement report over the air interface. If the UE supports reporting of NR positioning measurements via SDT, the UE may be able to report the measurements while it remains in RRC_INACTIVE state; otherwise, the UE will transition to RRC_CONNECTED state prior to transmitting the measurement report.

For PRS-RSRP measurements performed by the UE in RRC_INACTIVE state, the measurement reporting delay excludes all of the following:

-	any delay caused by other LPP signalling on the DCCH,

-	delay uncertainty introduced when inserting the measurement report in the TTI of the uplink DCCH which is equal to 2 x TTIDCCH where TTIDCCH is the duration of subframe or slot or subslot when the measurement report is transmitted on the PUSCH with subframe or slot or subslot duration,

-	any delay caused by no UL resources for UE to send the measurement report,

-	any transmission delay needed by SDT,

-	the time needed to transition to RRC_CONNECTED state to report the measurements.

The reported PRS-RSRP measurement values contained in measurement reports shall be based on the measurement report mapping requirements specified in clauses 10.1.24.3.

The PRS-RSRP measurement accuracy for all measured PRS resources shall be fulfilled according to the accuracy requriements specified in the clauses 10.1.24.2.

5.6.3.5	Measurement Period Requirements

When the physical layer receives NR-DL-AoD-ProvideAssistanceData message and NR-DL-AoD-RequestLocationInformation message from LMF via LPP [34], the UE shall be able to measure multiple (up to the UE capability specified in Clause 5.6.3.3) PRS-RSRP measurements, defined in TS 38.215 [4], from configured PRS resources for configured TRPs on configured positioning frequency layers, within TPRS-RSRP,total ms.

TPRS-RSRP, total=i=1LTPRS-RSRP,i+L-1*maxTeffect,i

Where:

-	i is the index of positioning frequency layer,

-	L is total number of positioning frequency layers,

-	Teffect,i is the periodicity of the PRS-RSRP measurement in positioning frequency layer i.

TPRS-RSRP,i=Kcarrier_PRSi*NRxBeam,i*NPRS,islotN'Lavailable_PRS,iN*Nsample-1*Teffect,i+Tlast,iWhere:

-	Kcarrier_PRS is a scaling factor for PRS-based NR positioning measurements in RRC_INACTIVE. If the UE supports parallelPRS-MeasRRC-Inactive-r17, Kcarrier_PRS= 1. Otherwise,

-	If Srxlev ≤ SnonIntraSearchP or Squal ≤ SnonIntraSearchQ, Kcarrier_PRSequals to the sum of Kcarrier in 4.2.2.4 and one positioning layer.

-	If Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ, Kcarrier_PRS equals to the sum of Nlayer in 4.2.2.7 and one positioning layer.

-	NRxBeam,i is the scaling factor for Rx beam sweeping:

- NRxBeam,i=1 if positioning frequency layer i is in FR1, and if positioning frequency layer i is in FR2

-		NRxBeam,i equals to the value as UE reported in supportedLowerRxBeamSweepingFactor-FR2 if the capability is reported by the UE for the band containing positioning frequency layer i, and LMF indicates lowerRxBeamSweepingFactor-FR2 in NR-DL-AoD-RequestLocationInformation.

-		NRxBeam,i equals to 8, otherwise.

-	Lavailable_PRS,i is the time duration of available PRS to be measured in the positioning frequency layer i to be measured during TPRS,i, and is calculated in the same way as PRS duration K defined in clause 5.1.6.5 of TS 38.214 [26]. For calculation of Lavailable_PRS,i, only unmuted PRS resources that are not fully overlapped with other higher-priority DL signals/channels are considered.

-	NPRS,islot is the maximum number of DL PRS resources of positioning frequency layer i configured in a slot,

-	{N,T} is UE capability combination per band where N is a duration of DL PRS symbols in ms corresponding to durationOfPRS-ProcessingSymbols-r17 in TS 37.355 [34] processed every T ms corresponding to durationOfPRS-ProcessingSymbolsInEveryTms-r17in TS 37.355 [34] for a given maximum bandwidth supported by UE corresponding to supportedBandwidthPRS in TS 37.355 [34],

-	N’ is UE capability for number of DL PRS resources that it can process in a slot as indicated by maxNumOfDL-PRS-ResProcessedPerSlot-RRC-Inactive-r17 in clause 6.4.3 of TS 37.355 [34],

-	Nsample is the number of PRS-RSRP measurement samples and

-	Nsample= 1, if UE supports supportedDL-PRS-ProcessingSamples-RRC-Inactive [34], and the LMF indicates the UE to perform positioning measurements with reduced number of samples by reducedDL-PRS-ProcessingSamples [34], and the following conditions are met:

-	PRS bandwidth is within the initial BWP and

-	Magnitude of difference between the serving cell’s SS-RSRP and the neighbor cell’s PRS-RSRP is within 6 dB.

-	Nsample= 2, if UE supports supportedDL-PRS-ProcessingSamples-RRC-Inactive [34], and the LMF indicates the UE to perform positioning measurements with reduced number of samples by reducedDL-PRS-ProcessingSamples [34], and the following conditions are not met:

-	PRS bandwidth is within the initial BWP and

-	Magnitude of difference between the serving cell’s SS-RSRP and the neighbor cell’s PRS-RSRP is within 6 dB.

-	Nsample= 4 otherwise

-	Tlast,i = Ti +Tavailable_PRS,i is the measurement duration for the last PRS-RSRP sample, including the sampling time and processing time,

-	Teffect,i= TiTavailable_PRS,i*Tavailable_PRS,i is the periodicity of PRS-RSRP measurement in positioning frequency layer i,

-	Ti	corresponds to durationOfPRS-ProcessingSymbolsInEveryTms-r17 in TS 37.355 [34],

-	Tavailable_PRS,i=LCMTPRS,i,TDRX is the least common multiple between TPRS,i and TDRX,

-	TPRS,i is the maximum PRS resource periodicity among all PRS resources in positioning frequency layer i,

-	TDRX is the DRX cycle length.

If positioning frequency layer i has more than one DL PRS resource set with different PRS periodicities with muting,  TperPRS with muting=Nmuting*TperPRS, the least common multiple of  TperPRS with muting among the DL PRS resource sets is used to derive TPRS,i, where:

-	TperPRS is the periodicity of PRS resource sets given by the higher-layer parameter DL-PRS-Periodicity.

-	Nmuting is the scaling factor considering PRS resource muting. Nmuting=TmutingPRS*Lmuting, where TmutingPRS is the muting repetition factor given by the higher-layer parameter DL-PRS-MutingBitRepetitionFactor, and Lmuting is the size of the bitmap b1.

When PRS-RSRP measurements are configured for DL-AoD, the time TPRS-RSRP,total starts from the first DRX cycle containing the DL PRS resources in the assistance data after both the NR-DL-AoD-RequestLocationInformation message and NR-DL-AoD-ProvideAssistanceData message from LMF via LPP [34] are delivered to the physical layer of UE.

Note:	No per-positioning frequency layer requirement is applied in scenarios when multiple positioning frequency layers are configured.

When the PRS-RSRP measurement is configured together with RSTD measurement then the PRS-RSRP measurement shall meet the RSTD measurement requirements defined in clause 5.6.2.

When the PRS-RSRP measurement is configured together with UE Rx-Tx time difference measurement then the PRS-RSRP measurement shall meet the UE Rx-Tx time difference measurement requirements defined in clause 5.6.4.

The measurement requirements do not apply for a PRS resource:

-	if the PRS resource is across two sampling duration of N within duration Lavailable_PRS,i or

-	if time span of the PRS resource instance (including at least the minimum number of repetitions specified in the accuracy requirements) is greater than UE reported capability N.

Longer PRS-RSRP measurement period is expected when there is collision/overlap between other DL signals/channels and PRS resources in RRC_INACTIVE state.

The requirements in clause 5.6.3 do not apply if the PRS configuration given by higher layer paramters NR-DL-PRS-AssistanceData exceeds any of the UE measurement capabilities given by NR-DL-PRS-ResourcesCapability in NR-DL-AoD-ProvideCapabilities, and it is up to UE implementation which PRS resources are measured, subject to UE measurement capabilities.

If the DRX cycle is reconfigured during the PRS-RSRP measurement period then the PRS-RSRP measurement period can be longer.

If cell reselection occurs while PRS-RSRP measurement is being performed, then the UE shall continue and complete the on-going PRS-RSRP measurement after the cell selection is completed. The PRS-RSRP measurement period can be longer.

If the UE’s RRC state changes from the RRC_INACTIVE to RRC_CONNECTED during the PRS-RSRP measurement period, then the UE shall continue the PRS-RSRP measurement in the RRC_CONNECTED state. The PRS-RSRP measurement period can be longer.

5.6.4	UE Rx-Tx time difference measurements

5.6.4.1	Introduction

The requirements in this clause shall apply, provided the UE has received nr-Multi-RTT-RequestLocationInformation message from LMF via LPP [34] requesting the UE to measure and report one or more UE Rx-Tx time difference measurements defined in TS 38.215 [4].

5.6.4.2	Requirements Applicability

The requirements in clause 5.6.4 apply for periodic and triggered UE Rx-Tx time difference measurements, provided:

-	UE Rx-Tx time difference measurement related side conditions given in clause 10.1.25.2 are met for a corresponding band.

-	SRS is configured on the PCell.

5.6.4.3	Measurement Capability

UE Rx-Tx time difference measurement capability is as indicated by the UE in NR-Multi-RTT-ProvideCapabilities, according to TS 37.355 [34].

5.6.4.4	Measurement Reporting Requirements

The measurement reporting delay is defined as the time between the moment when the measurement report is triggered and the moment when the UE starts to transmit the measurement report over the air interface.

This measurement reporting delay excludes the delay caused by any of the following:

-	delay caused by other LPP signalling on the DCCH.

-	delay uncertainty resulted when inserting the measurement report to the TTI of the uplink DCCH. The delay uncertainty is: 2 x TTIDCCH where TTIDCCH is the duration of subframe or slot or subslot when the measurement report is transmitted on the PUSCH with subframe or slot or subslot duration.

-	delay caused due to lack of UL resources for UE to send the measurement report.

-	delay required by SDT for reporting the measurement using SDT resouces.

-	delay required for transition to RRC_CONNECTED state for report the measurement in RRC_CONNECTED.

The UE Rx-Tx time difference measurement values contained in measurement reports shall be based on the measurement report mapping requirements specified in clause 10.1. 25.3.

The UE Rx-Tx time difference measurement accuracy for all measured DL PRS resources shall be fulfilled according to the accuracy requirements specified in clause 10.1. 25.2.

5.6.4.5	Measurement Period Requirements

When physical layer receives last of NR-Multi-RTT-ProvideAssistanceData message and NR-Multi-RTT-RequestLocationInformation message from LMF via LPP [34], UE shall be able to measure multiple (up to the UE capability specified in clause 5.6.4.3) UE Rx-Tx time difference measurements as defined in TS 38.215 [4] in configured positioning frequency layers within the measurement period TUERxTx,Total ms.

TUERxTx, Total=i=1LTUERxTx,i+L-1*maxTeffect,i

Where:

-	i is the index of positioning frequency layer,

-	TUERxTx,i is the measurement period for UE Rx-Tx time difference measurements in positioning frequency layer i as further defined in this clause,

-	L is total number of positioning frequency layers,

-	Teffect,i is the periodicity of the UE Rx-Tx time difference measurement in positioning frequency layer i as defined further in this clause.

TUERxTx,i=Kcarrier_PRS*NRxTx,TEG,i*NRxBeam,i*NPRS,islotN'Lavailable_PRS,iN*Nsample-1*Teffect,i+Tlast,i

Where:

-	Kcarrier_PRSKcarrierPRS=1 if the UE is capable of parallelPRS-MeasRRC-Inactive-r17 defined in [34].

-	Kcarrier_PRS=Nlayer+1 if the UE is not capable of parallelPRS-MeasRRC-Inactive-r17 defined in [34] and if Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ; where Nlayeris defined in clause 4.2.2.7.

-	Kcarrier_PRS=Kcarrier+1 if the UE is not capable of parallelPRS-MeasRRC-Inactive-r17 defined in [34] and if Srxlev ≤ SnonIntraSearchP or Squal ≤ SnonIntraSearchQ; where Kcarrieris defined in clause 4.2.2.4.

-	NRxBeam,i is the scaling factor for UE Rx beam sweeping:

-	NRxBeam,i=1 if positioning frequency layer i is in FR1, and if positioning frequency layer i is in FR2.

-		NRxBeam,i equals to the value as UE reported in supportedLowerRxBeamSweepingFactor-FR2 if the capability is reported by the UE for the band containing positioning frequency layer i, and LMF indicates lowerRxBeamSweepingFactor-FR2 in NR-Multi-RTT -RequestLocationInformation.

-		NRxBeam,i equals to 8, otherwise.

NRxTx,TEG,i is the Rx TEG specific scaling factor:

-	NRxTx,TEG,i = 1 if UE is not configured by LMF with measureSameDL-PRS-ResourceWithDifferentRxTxTEGs-r17 or measureSameDL-PRS-ResourceWithDifferentRxTEGs-r17 [34].

Otherwise,

-	NRxTx,TEG,i = NTEG,i, if UE is not capable of measuring same DL PRS resource simultaneously from multiple Rx TEGs, where NTEG,i is the number of Rx TEGs or RxTx TEGs with which UE is requested to measure a PRS resource indicated via measureSameDL-PRS-ResourceWithDifferentRxTxTEGs-r17 or measureSameDL-PRS-ResourceWithDifferentRxTEGs-r17 [34], and in case ‘n0’ is indicated, NTEG,i is the maximum number of Rx TEGs with which UE can support to measure the same PRS resource as reported in NR-UE-TEG-Capability.

-	NRxTx,TEG,i = NTEG,ikTEG,simul,i, if UE is capable of measuring same DL PRS resource simultaneously from multiple Rx TEGs, where kTEG,simul,i is the number of Rx TEGs UE can measure simultaneously which is reported via measureSameDL-PRS-ResourceWithDifferentRxTEGsSimul.

-	Lavailable_PRS,i is the time duration of available PRS resources in the positioning frequency layer i, to be measured during TPRS,i, and is calculated in the same way as PRS duration K defined in clause 5.1.6.5 of TS 38.214 [26]. For calculation of Lavailable_PRS,i, only unmuted PRS resources that are not fully overlapped with other higher-priority DL signals/channels are considered.

-	NPRS,islot is the maximum number of DL PRS resources of positioning frequency layer i configured in a slot,

-	{N,T} is UE capability combination per band where N is a duration of DL PRS symbols in ms corresponding to durationOfPRS-ProcessingSymbols-r17 in TS 37.355 [34] processed every T ms corresponding to durationOfPRS-ProcessingSymbolsInEveryTms-r17 in TS 37.355 [34] for a given maximum bandwidth supported by UE corresponding to supportedBandwidthPRS in clause 4.2.7.2 of TS 37.355 [34],

-	N’ is UE capability for number of DL PRS resources that it can process in a slot corresponding to maxNumOfDL-PRS-ResProcessedPerSlot-RRC-Inactive-r17 as specified in clause 6.4.3 of TS 37.355 [34],

-	Nsample is the number of UE Rx-Tx time difference measurement samples:

-	Nsample= 4 if the UE is not capable of supportedDL-PRS-ProcessingSamples-RRC-Inactive defined in [34] or not configured to perform positioning measurements with reduced number of samples by reducedDL-PRS-ProcessingSamples-r17 [34].

-	Nsample= 1 if the UE is capable of supportedDL-PRS-ProcessingSamples-RRC-Inactive defined in [34] and LMF requests the UE to perform positioning measurements with reduced number of samples by reducedDL-PRS-ProcessingSamples-r17 [34] and the following conditions are met:

-	PRS bandwidth is within the initial BWP and

-	Magnitude of difference between the serving cell’s SS-RSRP and the neighbor cell’s PRS-RSRP is within 6 dB.

-	Nsample=  2 if the UE is capable of supportedDL-PRS-ProcessingSamples-RRC-Inactive defined in [34] and the LMF requests the UE to perform positioning measurements with reduced number of samples by reducedDL-PRS-ProcessingSamples-r17 [34] but the following conditions are not met:

-	PRS bandwidth is within the initial BWP and

-	Magnitude of difference between the serving cell’s SS-RSRP and the neighbor cell’s PRS-RSRP is within 6 dB.

-	Tlast,i is the measurement duration for the last UE Rx-Tx time difference measurement sample in the positioning layer i, including the sampling time and processing time, Tlast,i = Ti + Tavailable_PRS,i ,

-	Teffect,i is periodicity of UE Rx-Tx time difference measurement in positioning frequency layer i:

Teffect,i= TiTavailable_PRS,i*Tavailable_PRS,i

Where:

-	Ti corresponds to durationOfPRS-ProcessingSymbolsInEveryTms-r17 in TS 37.355 [34],

-	Tavailable_PRS,i= LCMTPRS,i,TDRX, the least common multiple between TPRS,i and TDRX.

-	TDRX is the DRX cycle of the UE in the serving cell.

-	TPRS,i is the PRS resource periodicity in positioning frequency layer i. If the positioning frequency layer i has more than one DL PRS resource sets with different PRS periodicities with muting,  TperPRS with muting=Nmuting*TperPRS, the least common multiple of TperPRS with muting among DL PRS resource sets is used to derive TPRS,i, where:

-	TperPRS is the periodicity of PRS resource sets given by the higher-layer parameter DL-PRS-Periodicity.

-	Nmuting is the scaling factor considering PRS resource muting. Nmuting=TmutingPRS*Lmuting, where TmutingPRS is the muting repetition factor given by the higher-layer parameter DL-PRS-MutingBitRepetitionFactor, and Lmuting is the size of the bitmap b1

The time TUERxTx,Total starts from the first DRX cycle containing the DL PRS resources in the assistance data after both the NR-Multi-RTT-RequestLocationInformation message and NR-Multi-RTT-ProvideAssistanceData message from LMF via LPP [34] are delivered to the physical layer of UE.

Note: No per-positioning frequency layer requirement is applied in scenarios when multiple positioning frequency layers are configured.

If the RRC state transion occurs from RRC_INACTIVE to RRC_CONNECTED state during the UE Rx-Tx time difference measurement period then the UE shall restart the UE Rx-Tx time difference measurement after it obtains SRS configuration and Timing Advance command from the serving cell.

If cell reselection occurs during the UE Rx-Tx time difference measurement period then the UE shall restart the UE Rx-Tx time difference measurement after it obtains SRS configuration and Timing Advance command from the new serving cell.

The measurement requirements do not apply for a PRS resource:

-	if the PRS resource is across two sampling duration of N within duration Lavailable_PRS,i or

-	if time span of the PRS resource instance (including at least the minimum number of repetitions specified in the accuracy requirements) is greater than UE reported capability N.

If the DRX cycle is reconfigured during the UE Rx-Tx time difference measurement period then the UE Rx-Tx time difference measurement period can be longer.

If during UE Rx-Tx time difference measurement period PRS resources overlap with other DL signals/channels then the UE Rx-Tx time difference measurement period can be longer.

When PRS-RSRP is configured for multi-RTT, the UE Rx-Tx time difference measurements and PRS-RSRP measurements are performed over the same measurement period.

The requirements in clause 5.6.4 do not apply if the PRS configuration given by higher layer paramters NR-DL-PRS-AssistanceData exceeds any of the UE measurement capabilities given by NR-DL-PRS-ResourcesCapability in NR-Multi-RTT-ProvideCapabilities, and it is up to UE implementation which PRS resources are measured, subject to UE measurement capabilities.

If UE uplink transmission timing changes due to the network-configured Timing Advance command during the UE Rx-Tx measurement period, then the UE Rx-Tx time difference measurement period is restarted after uplink transmission timing changes, and the UE Rx-Tx time difference measurement period requirements in this clause shall not apply.

If UE uplink transmission timing changes due to the change in the NTA_offset defined in Table 7.1.2-2 during the UE Rx-Tx measurement period, then the UE Rx-Tx time difference measurement period is restarted after uplink transmission timing changes, and the UE Rx-Tx time difference measurement period requirements in this clause shall not apply.

5.6.5	PRS-RSRPP measurements

5.6.5.1	Introduction

The requirements in clause 5.6.5 shall apply provided the UE has received a message from LMF via LPP requesting the UE to measure and report PRS-RSRPP measurements defined in TS 38.215 [4]. And the UE is capable of supporting the PRS-RSRPP measurement in RRC INACTIVE state.

5.6.5.2	Requirements Applicability

The requirements in clause 5.6.5 apply for periodic and triggered PRS-RSRPP measurements, provided:

-	PRS-RSRPP related side conditions given in clause 10.1.38 are met for a corresponding Band.

5.6.5.3	Measurement Capability

UE PRS-RSRPP measurement capability is as indicated by the UE in NR-DL-AoD-ProvideCapabilities according to TS 37.355 [34].

5.6.5.4	Measurement Reporting Requirements

The measurement reporting delay is defined as the time between the moment when the periodic measurement report is triggered and the moment when the UE is ready to transmit the measurement report over the air interface. If the UE supports reporting of NR positioning measurements via SDT, the UE may be able to report the measurements while it remains in RRC_INACTIVE state; otherwise, the UE will transition to RRC_CONNECTED state prior to transmitting the measurement report.

For PRS-RSRPP measurements performed by the UE in RRC_INACTIVE state, the measurement reporting delay excludes all of the following:

-	any delay caused by other LPP signalling on the DCCH,

-	delay uncertainty introduced when inserting the measurement report in the TTI of the uplink DCCH which is equal to 2 x TTIDCCH where TTIDCCH is the duration of subframe or slot or subslot when the measurement report is transmitted on the PUSCH with subframe or slot or subslot duration,

-	any delay caused by no UL resources for UE to send the measurement report,

-	any transmission delay needed by SDT,

-	the time needed to transition to RRC_CONNECTED state to report the measurements.

The reported PRS-RSRPP measurement values contained in measurement reports shall be based on the measurement report mapping requirements specified in clause 10.1.38.3.

The PRS-RSRPP measurement accuracy for all measured PRS resources shall be fulfilled according to the accuracy requriements specified in the clause 10.1.38.2.

5.6.5.5	Measurement Period Requirements

When the physical layer receives NR-DL-AoD-ProvideAssistanceData message and NR-DL-AoD-RequestLocationInformation message in RRC_INACTIVE state, measurement period requirements for PRS-RSRP defined in 5.6.3 is re-used for PRS-RSRPP measurement.

<End of Change 1>