3GPP TSG-RAN4 Meeting #110	R4-2400285

# Greece, Athens, February 16th – March 1st, 2024

Agenda item:			8.18.5.1

Source:	Samsung

Title:	Discussions on NTN UE Tx RF requirements

Document for:	Approval

Introduction

In the previous RAN4 #109 meeting, most Tx and Rx requirements for NTN-enh UE were discussed and determined, but the minimum EIRP requirements were still open.

In our previous paper [1], we provided our calculation and proposal for the meeting to determine the minimum EIRP requirements for NTN UE. And in the WF of #109 meeting [2], there were some high level agreements achieved to further discuss this requirement.

Discussions on minimum peak EIRP requirements

Agreement in #109 [2]:

Issue 1-1 NTN UE power class/types definition Agreement: - Define the requirements based on the following UE types UE class; UE type; Type description. Fixed VSAT; 1; Fixed VSAT supporting GSO and LEO with mechanical steering antenna.. 2; Fixed VSAT supporting GSO and LEO with electronical steering antenna.. 3; Fixed VSAT supporting LEO only with electronical steering antenna.. Mobile VSAT; 4; Mobile VSAT supporting GSO with mechanical steering antenna.. 5; Mobile VSAT supporting GSO with electronical steering antenna.. Note: Assuming that UE has single beam towards one single satellite at a given time.. Issue 1-2: Mapping between NF and NTN UE power class/types Agreement: Fixed VSAT: Electronic steering Class 1: 2.5 dB NF (LEO and GSO) Class 2: 6 dB NF (LEO only) Mechanical steering Class 1: 2.5 dB NF (LEO and GSO) Mobile VSAT: Electronic steering Class 1: 2.5 dB NF (GSO) Mechanical steering Class 1: 2.5 dB NF (GSO) Issue 2-2: Transmit power assumption for each UE types Agreement: - for each UE type, single value for minimum EIRP limit for certain elevation angle and FFS for the exact value; - If UE is capable of communicating with both GSO and LEO, to follow the minimum EIRP for worse case.

From the above agreements, we understand the minimum EIRP requirements were agreed to be specified with following aspects:

- Antenna steering types;

- Noise figure class;

- Elevation angle;

- Orbit altitudes.

For the ‘elevation angle’ consideration,

- the VSAT UE would require higher Tx power to compensate the increased pathloss due to increased communication distance and higher atmospheric loss under lower elevation angle assumptions.

- Also the VSAT UE is more likely to be in the cell edge area in lower elevation angle, which caused the decreased Rx antenna gain from SAN side.

With these, we find the minimum elevation angle would be the worst case for a VSAT UE to be specified for a minimum EIRP requirement for deployment.

Observation 1: The worst case of minimum EIRP requirements for VSAT UE would be specified under minimum elevation angle required (or claimed) in its network deployment.

For the ‘orbit altitudes’ consideration, if a VSAT UE can communicate with both GSO and LEO, we use following method to determine which should be the worst case for a VSAT UE.

Table 2.1-1 VSAT UE supporting both LEO and GSO (with NF as 2.5)

| Orbit | LEO 600 |  | LEO 1200 |  | GSO |  |
| --- | --- | --- | --- | --- | --- | --- |
| Elevation angle (VSAT) | 30 deg | 90 deg | 30 deg | 90 deg | 30 deg | 90 deg |
| Distance (VSAT-SAN) | 1075 km | 600 km | 1999 km | 1200 km | 38612 km | 35786 km |
| Free space path loss ITU-R P.525 | 181.7 dB | 176.6 dB | 187.1 dB | 182.7 dB | 212.8 dB | 212.1 dB |
| Other propagation losses (atmospheric and others) | 2 dB |  |  |  |  |  |
| Noise Figure | 2.5 dB (supporting both LEO and GSO) |  |  |  |  |  |
| BW | 200 MHz |  |  |  |  |  |
| Target SNR | -1 dB |  |  |  |  |  |
| SAN Rx gain (-3dB at cell edge) | 35.5 dBi |  | 35.5 dBi |  | 55.5 dBi |  |
| Required minimum EIRP | 58.7 dBm | 53.6 dBm | 64.1 dBm | 59.7 dBm | 69.8 dBm | 69.2 dBm |

Table 2.1-2 VSAT UE supporting LEO only (with NF as 6)

| Orbit | LEO 600 |  | LEO 1200 |  |
| --- | --- | --- | --- | --- |
| Elevation angle (VSAT) | 30 deg | 90 deg | 30 deg | 90 deg |
| Distance (VSAT-SAN) | 1075 km | 600 km | 1999 km | 1200 km |
| Free space path loss ITU-R P.525 | 181.7 dB | 176.6 dB | 187.1 dB | 182.7 dB |
| Other propagation losses (atmospheric and others) | 2 dB |  |  |  |
| Noise Figure | 6 dB |  |  |  |
| BW | 200 MHz |  |  |  |
| Target SNR | -1 |  |  |  |
| SAN Rx gain (-3dB at cell edge) | 35.5 dBi |  | 35.5 dBi |  |
| Required minimum EIRP | 62.2 dBm | 57.1 dBm | 67.6 dBm | 63.2 dBm |

From the calculation above, we propose that a VSAT UE supporting both GSO and LEO should have a minimum peak EIRP level of 69.8 dBm regardless of whether it’s a mechanical steering or electronic steering UE. The EIRP level at its wanted direction (i.e. to the SAN direction) should be higher than this minimum requirement in order to reach its SAN under the assumption of minimum elevation angle as 30 degree.

From the calculation above, we propose that a VSAT UE supporting LEO only should have a minimum peak EIRP leel of 67.6 dBm at its wanted direction regardless of whether it’s a mechanical steering or electronic steering UE.

Proposal 1: The minimum peak EIRP for fixed VSAT UE supporting both GSO and LEO (UE Type 1 and 2) can be specified as [69.8] dBm plus implementation loss. And it should be noted that such requirement is derived based on minimum elevation angle as 30 degree.

Proposal 2: The minimum peak EIRP for fixed VSAT UE supporting LEO only (UE Type 3) can be specified as [67.6] dBm plus implementation loss. And it should be noted that such requirement is derived based on minimum elevation angle as 30 degree.

Proposal 3: The minimum peak EIRP for mobile VSAT UE supporting GSO (UE Type 4 and 5) can be specified as [69.8] dBm plus implementation loss. And it should be noted that such requirement is derived based on minimum elevation angle as 30 degree.

With the above, the proposal 1~3 can be transferred to the following table:

Table 2.1-3 VSAT UE minimum peak EIRP

| UE type | Type description | Minimum peak EIRP requirements |
| --- | --- | --- |
| 1 | Fixed VSAT supporting GSO and LEO with mechanical steering antenna. | [69.8] dBm + Implement loss |
| 2 | Fixed VSAT supporting GSO and LEO with electronical steering antenna. | [69.8] dBm + Implement loss |
| 3 | Fixed VSAT supporting LEO only with electronical steering antenna. | [67.6] dBm + Implement loss |
| 4 | Mobile VSAT supporting GSO with mechanical steering antenna. | [69.8] dBm + Implement loss |
| 5 | Mobile VSAT supporting GSO with electronical steering antenna. | [69.8] dBm + Implement loss |

Conclusion

The following observations and proposals were submitted in this document for the meeting to discuss:

Observation 1: The worst case of minimum EIRP requirements for VSAT UE would be specified under minimum

Proposal 1: The minimum peak EIRP for fixed VSAT UE supporting both GSO and LEO (UE Type 1 and 2) can be specified as [69.8] dBm plus implementation loss. And it should be noted that such requirement is derived based on minimum elevation angle as 30 degree.

Proposal 2: The minimum peak EIRP for fixed VSAT UE supporting LEO only (UE Type 3) can be specified as [67.6] dBm plus implementation loss. And it should be noted that such requirement is derived based on minimum elevation angle as 30 degree.

Proposal 3: The minimum peak EIRP for mobile VSAT UE supporting GSO (UE Type 4 and 5) can be specified as [69.8] dBm plus implementation loss. And it should be noted that such requirement is derived based on minimum elevation angle as 30 degree.

Reference

[1] R4-2319179, “Discussions on NTN UE RF”, Samsung

[2] R4-2321974, “WF on NTN UE RF requirements”, ZTE, Samsung