3GPP TSG-RAN4 Meeting #110	R4-2400413

Athens, Greece, 26th Feb – 1st Mar 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-1 | CR | 2008 | rev | - | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Modification on power imbalance requirements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Apple |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NonCol_intraB_ENDC_NR_CA-Core |  |  |  |  |  | Date: |  |  | 2024-01-29 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Power imbalance is used in the description of power imbalance requirement. This is not aligned with previous collocated case for which 6dB is power spectral density imbalance. It is also unclear when the channel banswidths are uneaual between 2 CCs. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Power spectral density imbalance is used instead of power imbalance. - Note x is changed to note 5. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Requriement is not clear. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.10A |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.521-1 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of change>

## 7.10A	Power imbalance for CA

### 7.10A.1	General

Power imbalance requirement is a measure of the receiver’s ability to receive a wanted signal in the presence of another signal with a power spectral density (PSD) imbalance and a specific frequency offset from the wanted signal.

Power imbalance requirement in this subclause is only applicable for a UE indicating [intraBandNR-CA-non-collocated-r18] and if [nonCollocatedTypeNR-CA-r18] is not provided and UE is configured with maxMIMO-Layers with value less than or equal to 2.

### 7.10A.2	Minimum requirement

For the test parameters in table 7.10A.2-1, the throughput shall be ≥ 95 % of the maximum throughput of the reference measurement channels as specified in Annexes A.2.2, A.3.2, and A.3.3 (with one sided dynamic OCNG Pattern OP.1 TDD for the DL-signal as described in Annex A.5.2.1).

Table 7.10A.2-1: Power imbalance parameters for intra-band non-contiguous CA

| Test configurations | Carriers | Rx Power in transmission bandwidth configuration (dBm) | channel bandwidth | Center of BWanother Relative to edge of BWwanted |
| --- | --- | --- | --- | --- |
| 1 | Wanted carrier | REFSENS NOTE 4 + 1 | BWwanted ≤ BWanother | < max (5/2* BWanother, 50MHz |
|  | Another wanted carrier | PSD of wanted carrier + 25 |  |  |
| 2 | Wanted carrier | REFSENS NOTE 4 + 1 | BWwanted > BWanother |  |
|  | Another wanted carrier | PSD of wanted carrier + 25 – 10*log10(BWwanted /BWanother) |  |  |
| 3 | Wanted carrier | REFSENS NOTE 4 + 1 | NA | ≥ max (5/2* BWanother, 50MHz) |
|  | Another wanted carrier | PSD of wanted carrier + 25 |  |  |
| NOTE 1: The transmitter shall be set to 24dB below PCMAX_L,f,c at the minimum uplink configuration specified in Table 7.3.2-3 with PCMAX_L,f,c as defined in clause 6.2A.4. NOTE 2: BWwanted is the channel bandwidth of wanted carrier. BWanother is the channel bandwidth of another wanted carrier with 25 dB power imbalance. NOTE 3: It’s allowed to use one of test configurations to verify the RX power imbalance requirement for type 2 UE. NOTE 4: REFSENS is the reference sensitivity level for two antenna port in Table 7.3.2-1b. NOTE 5: The minimum requirements only apply for non simultaneous Rx/Tx between all carriers for TDD intra-band non-contiguous CA band combinations in Table 7.10A.2-2 |  |  |  |  |

For a UE indicating [intraBandNR-CA-non-collocated-r18] for the following CA band combinations in Table 7.10A.2-2, the Rx requirements for two Rx ports are applicable for each component carrier if [nonCollocatedTypeNR-CA-r18] is not provided and UE is configured with maxMIMO-Layers with value less than or equal to 2.

Table 7.10A.2-2: NR CA combinations

CA combination

CA_n77(2A)

CA_n78(2A)

<End of change>