3GPP TSG-RAN WG4 Meeting #110		R4-2400439

Athens, GR, 26 Feb – 01 Mar, 2024

Agenda item:	4.4

Source:	Apple

Title:   	(NR_newRAT-Core) On active TCI state list update delay

Document for:	Discussion

# 1	Introduction

At RAN4 meeting#109, we submitted a CR to clarify the active TCI state list update delay requirement in 38.133 [1]. During the meeting, valuable comments were received. In this contribution, we share our views on how to fix the issue.

# 2	Discussion

Currently in TS38.133, the active TCI state list update delay is requirement is specified as follows:

### 8.10.6	Active TCI state list update delay

If the target TCI state is known, upon receiving PDSCH carrying MAC-CE active TCI state list update at slot n, UE shall be able to receive PDCCH to schedule PDSCH with the new target TCI state at the first slot that is after n+ THARQ +3Nslotsubframe,µ +TOk*(Tfirst-SSB + TSSB-proc) / NR slot length. Where THARQ, Tfirst-SSB, TSSB-proc and TOk are defined in clause 8.10.3.

There are some ambiguity/inaccuracy in the above text:

- The delay is from the time UE receives PDSCH carrying MAC-CE active TCI state list update to the time UE receives PDCCH scheduling PDSCH with the new target TCI state, not the PDCCH with the new target TCI state. The current wording “with the new target TCI state” could be misinterpreted as PDCCH with the new target TCI state.

- The delay is the time needed for UE to decode the MAC-CE and perform T/F synchronization for the known TCI state(s) to be added to the active TCI state list. Using the time instant to receive PDCCH as the endpoint of the active TCI state list update is not precise, because PDCCH is up to network scheduling and the network could transmit PDCCH before the UE completes active TCI state list update.

In addition, it is known that when a UE receives DCI for TCI state switching, it is allowed a time duration denoted by “timeDurationForQCL” before it is ready to receive the first symbol of PDSCH with the target TCI state.

The timing relation is shown in Fig. 1.

To address the above ambiguity/inaccuracy and avoid all the complication regarding the arrival time of PDCCH, we can use the time instant when the UE is ready to receive PDSCH with the new target TCI state, if scheduled, as the endpoint. However, in unified TCI requirements, Clause 8.15.5, active downlink TCI state list update delay and Clause 8.16.5, active uplink TCI state list update delay, after the list update is completed, PDCCH/PDSCH or PUCCH/PUSCH/SRS can be received/transmitted after some beam application time. Therefore, it is better to use the endpoint of when the list update is completed. At the same time, to make sure the requirement is testable, we can further specify when PDSCH with new target TCI state.

UE completes decoding of the MAC-CE and T/F sync. required for the newly activated TCI state(s)

UE receives PDSCH carrying MAC-CE active TCI state list update

Possible PDCCH arriving instant at the UE

Figure 1. Timing relation

Considering all the above, the specification text can be revised as follows:

“If the target TCI state is known, upon receiving PDSCH carrying MAC-CE active TCI state list update at slot n, UE shall have completed the active TCI state list update by the end of slot n+ THARQ +3Nslotsubframe,µ +TOk*(Tfirst-SSB + TSSB-proc) / NR slot length and be able to receive scheduled PDSCH with the new target TCI state at the first slot that is after n+ THARQ +3Nslotsubframe,µ +TOk*(Tfirst-SSB + TSSB-proc) / NR slot length + ceiling(timeDurationForQCL/14). Where THARQ, Tfirst-SSB, TSSB-proc and TOk are defined in clause 8.10.3, ceiling(x) is the smallest integer such that ceiling(x)  x, timeDurationForQCL is the time required by the UE to perform PDCCH reception and applying spatial QCL information received in DCI for PDSCH processing as described in TS 38.214 [26], and the value of timeDurationForQCL is defined in TS 38.331 [2].”

Proposal 1: It is proposed to specify the active TCI state list update delay as follows:

“If the target TCI state is known, upon receiving PDSCH carrying MAC-CE active TCI state list update at slot n, UE shall have completed the active TCI state list update by the end of slot n+ THARQ +3Nslotsubframe,µ +TOk*(Tfirst-SSB + TSSB-proc) / NR slot length and be able to receive scheduled PDSCH with the new target TCI state at the first slot that is after n+ THARQ +3Nslotsubframe,µ +TOk*(Tfirst-SSB + TSSB-proc) / NR slot length + ceiling(timeDurationForQCL/14). Where THARQ, Tfirst-SSB, TSSB-proc and TOk are defined in clause 8.10.3, ceiling(x) is the smallest integer such that ceiling(x)  x, timeDurationForQCL is the time required by the UE to perform PDCCH reception and applying spatial QCL information received in DCI for PDSCH processing as described in TS 38.214 [26], and the value of timeDurationForQCL is defined in TS 38.331 [2].”

# 3	Conclusions

In this contribution, we make the following proposals.

Proposal 1: It is proposed to specify the active TCI state list update delay as follows:

“If the target TCI state is known, upon receiving PDSCH carrying MAC-CE active TCI state list update at slot n, UE shall have completed the active TCI state list update by the end of slot n+ THARQ +3Nslotsubframe,µ +TOk*(Tfirst-SSB + TSSB-proc) / NR slot length and be able to receive scheduled PDSCH with the new target TCI state at the first slot that is after n+ THARQ +3Nslotsubframe,µ +TOk*(Tfirst-SSB + TSSB-proc) / NR slot length + ceiling(timeDurationForQCL/14). Where THARQ, Tfirst-SSB, TSSB-proc and TOk are defined in clause 8.10.3, ceiling(x) is the smallest integer such that ceiling(x)  x, timeDurationForQCL is the time required by the UE to perform PDCCH reception and applying spatial QCL information received in DCI for PDSCH processing as described in TS 38.214 [26], and the value of timeDurationForQCL is defined in TS 38.331 [2].”

# 4	References

R4-2318699, "CR on active TCI state list update delay - R15," Apple