3GPP TSG-RAN WG4 Meeting # 110	R4-2400454

Athens, GR, 26 Feb – 01 Mar, 2024

Agenda item:	8.3.4.2

Source:	Apple

Title:	Simulation results for PDSCH with multi-RX in FR2

Release:	Rel-18

Document for:	Discussion

# 1. Introduction

In RAN4#109 demodulation requirements for multi-RX in FR2 were discussed and way forward [1] was agreed.  In this contribution we present simulation results for defining demod with multi-RX reception on the DL in FR2.

# 2. Simulation Results

In RAN4#109 we agreed on the simulation assumptions for multi-RX demod requirements. We present simulation results for the agreed test cases below:

Table 1: Simulation results for mDCI

| MCS | Layer per TRP/ scheme | TO/FO | UE Processing | r | SNR @ 70% Max TP |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Alignment | Impairment |
| MCS13 | 2L-Non overlapping | Freq offset 0Hz Time offset 0.25us | Separate Processing | -12 | 13.8 | 16.8 |
| MCS17 | 1L-Overlapping | Freq offset 600Hz Time offset -0.0625us | Separate Processing | -12 | 12.3 | 15.3 |

Table 1: Simulation results for sDCI

| MCS | Layer per TRP | TO/FO | UE Processing | r | SNR @ 70% Max TP |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Alignment | Impairment |
| MCS17 | 1L | Freq offset 600Hz Time offset -0.0625us | Separate Processing | -12 | 12.2 | 15.2 |

# 3. Conclusion

In this paper, we provide simulation results for defining demod requirements with multi-RX reception on the DL in FR2.

# Reference

- R4-2321140, “WF on [109][318] NR_FR2_multiRX_DL_Demod”, Qualcomm.