3GPP TSG-RAN WG4 Meeting # 110	R4-2400468

Athens, GR, 26 Feb – 01 Mar, 2024

Agenda item:	8.27.2

Source:	Apple

Title:	UE demodulation performance requirements for NR dynamic spectrum sharing

Release:	Rel-18

Document for:	Discussion

# 1. Introduction

In RAN4#109 requirements for enhanced DSS were discussed and way forward [1] were agreed.  In this contribution we present our views on demodulation requirements for eDSS.

# 2. Discussion

In RAN4#109 the demodulation requirements for eDSS were discussed and the following agreements were reached [1]:

CRS rate matching pattern assumption Agreement: - Single LTE CRS rate matching pattern UE receiver assumption Agreement: - Leave the receiver design to implementation for evaluation purposes. - Further check submitted results. If there is a large variation in results, then discuss receiver assumptions. Common parameters Agreement: - DCI format: 1_0 - CCE to REG mapping: Non-interleaved - REG bundle size: 6 PRB - Precoder: Random precoder with precoder cycling per REG bundle - LTE CRS: 4 Ports - TX assumption for overlapping REs: Puncture PDCCH and DMRS REs overlapping with LTE CRS - LTE PDCCH, PDSCH: No transmission - Slots with LTE PBCH/PSS/SSS: No PDCCH transmission - Channel estimation: PDCCH CE only on clean symbols Payload bits Agreement: - 39 bits Channel bandwidth Agreement: - For FDD, consider 10MHz/15kHz Aggregation level Agreement: - 4, 8 (for evaluation purpose) Channel model Agreement: - TDLA30-10 and TDLC300-100 - Discuss whether to down-select in the next meeting Power ratio of LTE-CRS RE/NR PDCCH-DMRS RE Agreement: - 0dB

Based on the simulation assumptions and evaluation cases agreed in [1] we provide simulation results for performance evaluation.

Table 1: PDCCH performance evaluation results in eDSS

| SCS(KHz)/CBW (MHz) | CORESET RB | Payload | AL | Propagation condition | Antenna configuration | SNR @ 1% Pm-dsg |
| --- | --- | --- | --- | --- | --- | --- |
| 15/10 | 48 | 39 | 8 | TDLA30-10 | 2x2 Low | -0.7 |
| 15/10 | 48 | 39 | 4 | TDLA30-10 | 2x2 Low | 2.8 |
| 15/10 | 48 | 39 | 8 | TDLC300-100 | 2x2 Low | -2.4 |
| 15/10 | 48 | 39 | 4 | TDLC300-100 | 2x2 Low | 1.6 |
| 15/10 | 48 | 39 | 8 | TDLA30-10 | 4x2 Low | -2.2 |
| 15/10 | 48 | 39 | 4 | TDLA30-10 | 4x2 Low | 1.5 |
| 15/10 | 48 | 39 | 8 | TDLC300-100 | 4x2 Low | -3.0 |
| 15/10 | 48 | 39 | 4 | TDLC300-100 | 4x2 Low | 1.0 |
| 15/10 | 48 | 39 | 8 | TDLA30-10 | 2x4 Low | -3.9 |
| 15/10 | 48 | 39 | 4 | TDLA30-10 | 2x4 Low | -0.6 |
| 15/10 | 48 | 39 | 8 | TDLC300-100 | 2x4 Low | -4.8 |
| 15/10 | 48 | 39 | 4 | TDLC300-100 | 2x4 Low | -1.2 |
| 15/10 | 48 | 39 | 8 | TDLA30-10 | 4x4 Low | -4.7 |
| 15/10 | 48 | 39 | 4 | TDLA30-10 | 4x4 Low | -1.3 |
| 15/10 | 48 | 39 | 8 | TDLC300-100 | 4x4 Low | -5.2 |
| 15/10 | 48 | 39 | 4 | TDLC300-100 | 4x4 Low | -1.5 |

Based on the simulation results, we provide our views on the open issues

Antenna configuration

Way forward:

- Option 1: 2x2, 2x4, 4x2, 4x4

- Option 2: 4x2, 4x4 only

Currently RAN4 requirements for PDCCH are with 1TX and 2TX.  We have an agreement that 4 CRS ports are configured for LTE CRS in symbol 0,1. If 4 ports are configured for LTE CRS, then a minimum of 4 TX antenna should be configured for NR as well. We don’t see how a test configuration with 4 CRS ports and 2 TX antenna in DSS is possible.

If 4 CRS ports are configured, we need to configure 4TX antenna in DSS.

- Define PDCCH requirements for eDSS with 4x2 and 4x4.

Channel bandwidth

Agreement:

- For FDD, consider 10MHz/15kHz

Way forward:

- For TDD,

  - Option 1: 10MHz/15kHz

  - Option 2: 20MHz/15kHz

For requirements in TDD the same channel bandwidth as that for FDD can be used, since the same SCS is used as FDD. The same CORESET bandwidth can be reused for TDD and FDD.

- Define requirements in TDD with 10MHz/ 15KHz.

Aggregation level

Agreement:

- 4, 8 (for evaluation purpose)

Channel model

Agreement:

- TDLA30-10 and TDLC300-100

- Discuss whether to down-select in the next meeting

The aggregation level and channel model for defining requirements are still to be confirmed. Although we evaluated both AL 4 and AL 8, we think it is sufficient to define requirements for 1 case. Also, we need not define requirements for both TDLA and TDLC channel model. In order to minimize the total number of test cases, we propose to define different test cases for TDD and FDD. For FDD define requirements with AL 4 and TDLA and for TDD with TDLC and AL 8.

- Define PDCCH requirements for eDSS with AL 4 and TDLA30-10 for FDD; AL 8 and TDLC300-100 for TDD.

# 3. Conclusion

In this paper, we provide our views on the scope for UE demodulation requirements for DSS enhancements. Our observations and proposals are captured below:

- If 4 CRS ports are configured, we need to configure 4TX antenna in DSS.

- Define PDCCH requirements for eDSS with 4x2 and 4x4.

- Define requirements in TDD with 10MHz/ 15KHz.

- Define PDCCH requirements for eDSS with AL 4 and TDLA30-10 for FDD; AL 8 and TDLC300-100 for TDD.

# Reference

[1].  R4-2321139, “WF on [109][331] NR_DSS_enh”, Ericsson.