3GPP TSG-RAN4 Meeting #110	R4-2400519

# Athens, Greece, 26th Feb 2024 - 1st Mar 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-1 | CR | 2011 | rev | - | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to TS38.101-1 Rel-18 CAT-A: On corrections for NR-U R16 A-MPR requirements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Apple |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_unlic-Core |  |  |  |  |  | Date: |  |  | 2024-02-15 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The CRs R4-2313003, R4-2313004 and R4-2313005 introduced specific reference on PSD requirements to NS_59 in Table 6.2F.3.1-1. The reason is that those requirements contribute to A-MPR. While the PSD requirements are referenced to in clause 6.2F.3.1 the idea is that the reqirements shall be explicitly referenced together with all the other emission requirements. To obtain a unified handling of requirements, this appraoch needs to be applied to all NR-U network signalling labels which have PSD limits. This CR adds the references on PSD limits in Table 6.2F.3.1-1. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added the clause 6.2F.1 for requirement reference for NS_28, NS_29, NS_30, NS_31, NS_53 and NS_54. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The PSD requirements for several network signalling labels are not correctly specified. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2F.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS38.521-1 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<start of change>>

#### 6.2F.3.1	General

Additional emission requirements can be signalled by the network. Each additional emission requirement is associated with a unique network signalling (NS) value indicated in RRC signalling by an NR frequency band number of the applicable operating band and an associated value in the field additionalSpectrumEmission. Throughout this specification, the notion of indication or signalling of an NS value refers to the corresponding indication of an NR frequency band number of the applicable operating band, the IE field freqBandIndicatorNR and an associated value of additionalSpectrumEmission in the relevant RRC information elements [7].

To meet the additional requirements, additional maximum power reduction (A-MPR) is allowed for the maximum output power as specified in Table 6.2F.1-1. Unless stated otherwise, the total reduction to UE maximum output power is max(MPR, A-MPR) where MPR is defined in clause 6.2F.2.

Table 6.2F.3.1-1 specifies the additional requirements with their associated network signalling values and the allowed A-MPR and applicable operating band(s) for each NS value. The mapping of NR frequency band numbers and values of the additionalSpectrumEmission to network signalling labels is specified in Table 6.2F.3.1-1A and table 6.2F.3.1-1B.

Table 6.2F.3.1-1: Additional maximum power reduction (A-MPR)

| Network signalling label | Requirements (clause) | NR Band | Channel bandwidth (MHz) | Resources blocks (NRB) | A-MPR (clause) |
| --- | --- | --- | --- | --- | --- |
| NS_01 |  | n46, n96 | 20, 40, 60, 80 |  | N/A |
| NS_28 | 6.5F.3.3.1, 6.2F.1 | n46 | 20, 40, 60, 80 |  | 6.2F.3.2 |
| NS_29 | 6.5F.3.3.2, 6.2F.1 | n46 | 20, 40, 60, 80 |  | 6.2F.3.3 |
| NS_30 | 6.5F.3.3.3, 6.2F.1 | n46 | 20, 40, 60, 80 |  | 6.2F.3.4 |
| NS_31 | 6.5F.3.3.4, 6.2F.1 | n46 | 20, 40, 60, 80 |  | 6.2F.3.5 |
| NS_53 | 6.5F.3.3.5, 6.2F.1 | n96 | 20, 40, 60, 80, 100 |  | 6.2F.3.6 |
| NS_54 | 6.5F.3.3.5, 6.2F.1 | n96 | 20, 40, 60, 80 |  | 6.2F.3.7 |
| NS_58 | 6.5F.3.3.6 | n102 | 20, 40, 60, 80 |  | 6.2F.3.8 |
| NS_59 | 6.2F.1 | n96 | 20, 40, 60, 80, 100 |  | 6.2F.3.9 |
| NS_60 | 6.5F.3.3.5 | n96 | 20, 40, 60, 80, 100 |  | 6.2F.3.10 |
| NS_61 | 6.5F.3.3.7 | n96 | 20, 40, 60, 80 |  | 6.2F.3.11 |
| NS_63 | 6.5F.3.3.8 | n102 | 20, 40, 80 |  | 6.2F.3.12 |
| NS_64 | 6.5F.3.3.9 | n102 | 20, 40, 60, 80, 100 |  | 6.2F.3.13 |
| NS_65 | 6.2F.1 | n102 | 20, 40, 60, 80, 100 |  | 6.2F.3.14 |
| NS_66 | 6.5F.3.3.5 | n96 | 20, 40, 60, 80, 100 |  | 6.2F.3.15 |
| NS_67 | 6.5F.3.3.5 | n96 | 20, 40, 60, 80, 100 |  | 6.2F.3.16 |
| NS_68 | 6.2F.1 | n102 | 20, 40, 60, 80, 100 |  | 6.2F.3.17 |
| NS_69 | 6.5F.3.3.8 | n102 | 20, 40, 80 |  | 6.2F.3.18 |
| NOTE 1: The A-MPR shall apply to all active 20 MHz sub-bands contiguously allocated in the channel. |  |  |  |  |  |

[The NS_01 label with the field additionalPmax [7] absent is default for all NR bands.]

Table 6.2F.3.1-1A: Mapping of network signaling label

| NR band | Value of additionalSpectrumEmission |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| n46 | NS_01 | NS_28 | NS_29 | NS_30 | NS_31 |  |  | Reserved |
| n96 | NS_01 | NS_53 | NS_54 | NS_59 | NS_60 | NS_61 |  | Reserved |
| n102 | NS_01 | NS_58 | NS_63 |  |  |  |  | Reserved |
| NOTE: additionalSpectrumEmission corresponds to an information element of the same name defined in clause 6.3.2 of TS 38.331 [7]. |  |  |  |  |  |  |  |  |

Table 6.2F.3.1-1B: Mapping of extended network signaling label

| NR band | Value of extendedAdditionalSpectrumEmission |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 |
| n96 | NS_66 | NS_67 |  |  |  |  |  |  |
| n102 | NS_64 | NS_65 | NS_68 | NS_69 |  |  |  |  |
| NOTE: extendedAdditionalSpectrumEmission corresponds to an information element of the name [EXTENDED_ additionalSpectrumEmission] defined in clause 6.3.2 of TS 38.331 [7]. |  |  |  |  |  |  |  |  |

<<end of change>>