3GPP TSG-RAN4 Meeting #110	R4-2400609

# Athens, Greece, 26th Feb 2024 - 1st Mar 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.307 | CR | 4498 | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR for 36.307 General enhancement for future proofing R18 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, CHTTL, Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI18 |  |  |  |  |  | Date: |  |  | 2024-02-16 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Explicit release references are replaced with generic reference to latest release. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Explicit release references are replaced with REL-P |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Specification is susceptable to errors on applicaple release |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3, Annexes A-F |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

****************************** Start of changes ***************************************

# 3	Definitions and abbreviations

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

release independent: applicable to some frozen releases, starting from a certain release Rel-M

NOTE 1:	Normally, a feature is introduced only in the latest open release Rel-N and future releases are based on the previous one so that future releases inherit the requirements of this feature. Introducing a feature "in a release independent way from Rel-M onwards" (M<N) means it was decided by TSG RAN that this feature would be also beneficial in previous, already frozen releases starting with Rel-M until Rel-(N-1). In order to avoid touching TS 36.101 [2] or TS 36.133 [3] of these frozen releases, the corresponding requirements are captured in TS 36.307 via pointers to [2] or [3] of the release in which the feature was introduced.

NOTE 2:	Release independent does not mean applicable to all releases.

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

4Rx	4 UE receiver antenna ports

CA	Carrier Aggregation

CRS	Cell-specific Reference Signal

CSI	Channel State Indicator

FDD	Frequency Division Duplex

LAA	License-Assisted Access

RRC	Radio Resource Control

RRM	Radio Resource Management

SDR	Sustained Data Rate

TDD	Time Division Duplex

UE	User Equipment

## 3.3	Symbols

For the purposes of the present document, the following symbols apply:

N	Release in which a feature is introduced into TS 36.101 [2] or TS 36.133 [3]

M	Release from which onwards (including release M) a feature is release independent

P    	Represent the present release of this specification

# 3A	Release independent features

## 3A.0	General

TSG-RAN has agreed for certain features (see the following clauses) to introduce them in a "release independent way".

This means for each feature:

-	it is "introduced" in a release N, i.e. TS 36.101 [2] and TS 36.133 [3] of release N define certain UE requirements for this feature; the feature is indicated in the tables of the following clauses;

-	it is "release independent" starting from a release M (M<N); M for the given feature is provided in the tables of the following clauses;

-	UEs supporting this feature have to fulfil additional requirements in release M or higher which are specified in one or more Annexes of TS 36.307 of release N; the applicable Annexes for a given feature are provided in the tables of the following clauses.

The applicable UE Categories are specified in TS 36.306 [4] according to the release to which the UE conforms.

## 3A.1	Additional E-UTRA operating bands

Requirements for a Rel-P UE for additional E-UTRA operating bands compared to TS 36.101 Rel-P [2] are introduced via this clause.

Table 3A.1-1: E-UTRA operating bands and UE power class

| Feature | Duplex-mode | Release independent from | Requirements to be fulfilled (see TS 36.307 of the release in which the band was introduced) |
| --- | --- | --- | --- |
| Operating bands, band number <= 64, Power Class 3 | FDD, TDD | Rel-8 | Table B.2.1-1, Table B.4.1-1 |
| Operating bands, band number > 64, Power Class 3 | FDD, TDD | Rel-9 | Table B.2.1-1, Table B.4.1-1 |
| Operating bands, NS-value > 32 | FDD, TDD | Rel-10 | Table B.2.1-1, Table B.4.1-1 |
| Asymmetric operating bands, Power Class 3 | FDD | Rel-10 | Table B.2.1-1, Table B.4.1-1 |
| Operating bands, band number <= 64, Power Class 1 | FDD | Rel-10 | Table B.2.1-1, Table B.4.1-1 |
| Operating bands, Power Class 2 | TDD | Rel-10 | Table B.2.1-1, Table B.4.1-1 |

For example, Band 19 was introduced in the Release 9 specifications. In order to implement a UE conforming to Release 8 but supporting Band 19, it is necessary for the UE to additionally conform to some parts of the Release 9 specifications (see corresponding Annexes of TS 36.307 Rel-9 which will point to the requirements in the Rel-9 of TS 36.101 [2] or TS 36.133 [3] to be fulfilled), such as the radio frequency and radio resource management requirements for the Band 19.

## 3A.2	Additional E-UTRA CA configurations

Requirements for a Rel-P UE for additional E-UTRA CA configurations compared to TS 36.101 Rel-P [2] are introduced via this clause.

Table 3A.2-1: Intra-band contiguous CA configurations and UE CA power class

| Feature | DL/UL | CA BW Class | Duplex-mode | Release independent from | requirements to be fulfilled (see 36.307 of the REL in which the CA configuration and the power class were introduced) |
| --- | --- | --- | --- | --- | --- |
| Intra-band contiguous CA configurations, power class 3 | DL | B | FDD | Rel-10 | Table B.2.2-1, Table B.3.2-1, Table B.4.2-1 |
|  |  | C | FDD, TDD | Rel-10 | Table B.2.2-1, Table B.3.2-1, Table B.4.2-1 |
|  |  | D | TDD | Rel-10 | Table B.2.2-1, Table B.3.2-1, Table B.4.2-1 |
|  |  | E | TDD | Rel-11 | Table B.2.2-1, Table B.3.2-1, Table B.4.2-1 |
|  |  | F | TDD | Rel-12 | Table B.2.2-1, Table B.3.2-1, Table B.4.2-1 |
|  | UL | B | FDD | Rel-10 | Table B.2.2-1, Table B.3.2-1, Table B.4.2-1 |
|  |  | C, D | FDD, TDD | Rel-10 | Table B.2.2-1, Table B.3.2-1, Table B.4.2-1 |
| Intra-band contiguous CA configurations, power class 2 | UL | C | TDD | Rel-10 | Table B.2.2-1, Table B.3.2-1, Table B.4.2-1 |
| NOTE: The duplex mode "FDD, TDD" refers to a CA configuration composed by only FDD bands or only TDD bands, respectively. |  |  |  |  |  |

Table 3A.2-2: Inter-band CA configurations

| Feature | DL/UL | number of bands | number of CCs | CA BW Classes | Duplex-mode | Release independent from | requirements to be fulfilled (see 36.307 of the REL in which the CA configuration was introduced) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Inter-band CA configurations | DL | 2 | 2-4 | A, B, C | FDD, TDD | Rel-10 | Table B.2.4-1, Table B.3.2-1, Table B.4.3-1 or Table B.4.4-1 |
|  |  |  | 2-5 | D, E | FDD, TDD | Rel-11 | Table B.2.4-1, Table B.3.2-1, Table B.4.3-1 or Table B.4.4-1 |
|  |  |  | 2-5 | A, B, C, D, E | FDD and TDD | Rel-12 | Table B.2.4-1, Table B.3.2-1, Table B.4.3-1 or Table B.4.4-1 |
|  |  |  | 6-7 | A, C, D, E, F | FDD, TDD | Rel-14 | Table B.2.4-1, Table B.3.2-1, Table B.4.3-1 or Table B.4.4-1 |
|  |  |  | 6-7 |  | FDD and TDD | Rel-14 | Table B.2.4-1, Table B.3.2-1, Table B.4.3-1 or Table B.4.4-1 |
|  |  | 3 | 3 | A | FDD, TDD | Rel-10 | Table B.2.4-1, Table B.3.2-1, Table B.4.3-1 or Table B.4.4-1 |
|  |  |  | 3-5 | B, C, D | FDD, TDD | Rel-11 | Table B.2.4-1, Table B.3.2-1, Table B.4.3-1 or Table B.4.4-1 |
|  |  |  | 3 | A | FDD and TDD | Rel-12 | Table B.2.4-1, Table B.3.2-1, Table B.4.3-1 or Table B.4.4-1 |
|  |  |  | 6-7 | A, C, D, E, F | FDD, TDD | Rel-14 | Table B.2.4-1, Table B.3.2-1, Table B.4.3-1 or Table B.4.4-1 |
|  |  |  | 6-7 |  | FDD and TDD | Rel-14 | Table B.2.4-1, Table B.3.2-1, Table B.4.3-1 or Table B.4.4-1 |
|  |  | 4 | 4-5 | A, C | FDD, TDD | Rel-11 | Table B.2.4-1, Table B.3.2-1, Table B.4.3-1 or Table B.4.4-1 |
|  |  |  | 4-5 |  | FDD and TDD | Rel-12 | Table B.2.4-1, Table B.3.2-1, Table B.4.3-1 or Table B.4.4-1 |
|  |  |  | 6-7 | A, C, D, E | FDD, TDD | Rel-14 | Table B.2.4-1, Table B.3.2-1, Table B.4.3-1 or Table B.4.4-1 |
|  |  |  | 6-7 |  | FDD and TDD | Rel-14 | Table B.2.4-1, Table B.3.2-1, Table B.4.3-1 or Table B.4.4-1 |
|  |  | 5 | 5 | A | FDD, TDD | Rel-12 | Table B.2.4-1, Table B.3.2-1, Table B.4.3-1 or Table B.4.4-1 |
|  |  |  | 5 |  | FDD and TDD | Rel-12 | Table B.2.4-1, Table B.3.2-1, Table B.4.3-1 or Table B.4.4-1 |
|  |  |  | 6-7 | A, C, D | FDD, TDD | Rel-14 | Table B.2.4-1, Table B.3.2-1, Table B.4.3-1 or Table B.4.4-1 |
|  |  |  | 6-7 |  | FDD and TDD | Rel-14 | Table B.2.4-1, Table B.3.2-1, Table B.4.3-1 or Table B.4.4-1 |
|  |  | 6 | 6 | A | FDD | Rel-14 | Table B.2.4-1, Table B.3.2-1, Table B.4.3-1 or Table B.4.4-1 |
|  | UL | 2 | 2-4 | A, C | FDD, TDD | Rel-11 | Table B.2.4-1, Table B.3.2-1, Table B.4.3-1 or Table B.4.4-1 |
|  |  |  | 2-3 | A, C | FDD and TDD | Rel-12 | Table B.2.4-1, Table B.3.2-1, Table B.4.3-1 or Table B.4.4-1 |
| NOTE1: The duplex mode "FDD, TDD" refers to a CA configuration composed by only FDD bands or only TDD bands, respectively. The duplex mode "FDD and TDD" refers to a CA configuration including both FDD and TDD bands. NOTE2: CA configurations involving downlink only operation in Band 46 are release independent from Rel-13 onwards (LAA was introduced in Rel-13). The 10 MHz channel bandwidth for Band 46 was introduced in TS 36.101 Rel-14 [2] and can be implemented in a release independent way from Rel-13. |  |  |  |  |  |  |  |

For example, CA configuration CA_1A-19A was introduced in the Release 11 specifications. In order to implement a UE conforming to Release 10 but supporting the CA configuration CA_1A-19A, it is necessary for the UE to additionally conform to some parts of the Release 11 specifications (see corresponding Annexes of TS 36.307 Rel-11 which will point to the requirements in the Rel-11 of TS 36.101 [2] or TS 36.133 [3] to be fulfilled), such as the radio frequency and radio resource management requirements for the CA configuration CA_1A-19A.

Table 3A.2-3: Intra-band non-contiguous CA configurations

| Feature | DL/UL | number of sub-blocks | number of CCs | CA BW Classes | Duplex-mode | Release independent from | requirements to be fulfilled (see 36.307 of the REL in which the CA configuration was introduced) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Intra-band non-contiguous CA configurations | DL | 2 | 2-5 | A, C, D | FDD, TDD | Rel-11 | Table B.2.3-1, Table B.3.2-1, Table B.4.5-1 |
|  |  | 3 | 3-5 | A, C | FDD, TDD | Rel-11 | Table B.2.3-1, Table B.3.2-1, Table B.4.5-1 |
|  | UL | 2 | 2 | A | FDD | Rel-11 | Table B.2.3-1, Table B.3.2-1, Table B.4.5-1 |
| NOTE1: The duplex mode "FDD, TDD" refers to a CA configuration composed by only FDD bands or only TDD bands, respectively. |  |  |  |  |  |  |  |

## 3A.3	Additional operating bands and/or CA configurations for specific features

For a specific feature introduced in an earlier release, it may be decided in a later release to apply this specific feature in a release independent way for additional operating bands and/or CA configurations. For a Rel-P UE corresponding requirements are then introduced via this clause.

Table 3A.3-1: Operating bands for specific features

| Feature | Release independent from | Requirements to be fulfilled (see 36.307 of the REL when the feature was introduced) | Further information |
| --- | --- | --- | --- |
| Operating bands for UE category 0 | Rel-12 | Table B.2.9-1, Table B.3.5-1, Table B.4.10-1 | Rel-14 WI LC_MTC_LTE_cat0_B25_B26-Core introduced RF, RRM, demodulation and CSI requirements for bands 25 and 26, see Table B.2.9-1, Table B.3.5-1, Table B.4.10-1 |
| Operating bands for UE category M1 | Rel-13 | Table B.2.10-1, Table B.3.6-1, Table B.4.11-1 | Rel-14 WI LTE_MTCe2_L1_cat1_B25_B40-Core introduced RF, RRM, demodulation and CSI requirements for bands 25 and 40, see Table B.2.10-1, Table B.3.6-1, Table B.4.11-1. Rel-15 WI LTE_bands_R15_M1_NB1-Core introduced RF, RRM, demodulation and CSI requirements for bands 14 and 71, see Table B.2.10-1, Table B.3.6-1, Table B.4.11-1. Rel-17 WI LTE_bands_R17_M1_M2_NB1_NB2 introduced RF, RRM, demodulation for band 24 and band 48, see Table B.2.10-1, Table B.3.6-1, Table B.4.11-1. Rel-18 WI LTE_bands_R18_M1_M2_NB1_NB2 introduced RF, RRM, demodulation for band 54, see Table B.2.10-1, Table B.3.6-1, Table B.4.11-1. |
| Operating bands for HD-FDD UE category M1, Power class 2 | Rel-13 | Table B.2.10-1, Table B.3.6-1, Table B.4.11-1 | Rel-16 WI LTE_PC2_B31_B72 introduced power class 2 feature for bands 31 and 72. |
| Operating bands for UE category M2 | Rel-14 | Table B.2.11-1, Table B.3.6-1, Table B.4.11-1 | Rel-15 WI LTE_bands_R15_M2_NB2-Core introduced RF and RRM requirements for bands 14 and 71, see Table B.2.11-1, Table B.4.11-1. Rel-17 WI LTE_bands_R17_M1_M2_NB1_NB2 introduced RF, RRM, demodulation for band 24 and band 48, see Table B.2.11-1, Table B.3.6-1, Table B.4.11-1. Rel-18 WI LTE_bands_R18_M1_M2_NB1_NB2 introduced RF, RRM, demodulation for band 54, see Table B.2.11-1, Table B.3.6-1, Table B.4.11-1 |
| Operating bands for HD-FDD UE category M2, Power class 2 | Rel-14 | Table B.2.11-1, Table B.4.11-1 | Rel-16 WI LTE_PC2_B31_B72 introduced power class 2 feature for bands 31 and 72. |
| Operating bands for (FDD) UE category NB1 | Rel-13 | Table B.2.8-1, Table B.3.7-1, Table B.4.9-1 | Rel-14 WI NB_IOT_R14_bands introduced RF, RRM and demodulation requirements for bands 11, 21, 25, 31, 70, see Table B.2.8-1, Table B.3.7-1, Table B.4.9-1. Rel-15 WI LTE_bands_R15_M1_NB1-Core introduced RF, RRM and demodulation for bands 4, 14 and 71 see Table B.2.8-1, Table B.3.7-1, Table B.4.9-1. Rel-16 WI LTE_bands_R16_M1_NB1 introduced RF, RRM, demodulation for band 65, see Table B.2.8-1, Table B.3.7-1, Table B.4.9-1. Rel-17 WI LTE_bands_R17_M1_M2_NB1_NB2 introduced RF, RRM, demodulation for band 24, see Table B.2.8-1, Table B.3.7-1, Table B.4.9-1. |
| Operating bands (FDD) for UE category NB2 | Rel-14 | Table B.2.12-1, Table 3.7-1, Table B.4.9-1 | Rel-15 WI LTE_bands_R15_M2_NB2-Core introduced RF, RRM and demodulation requirements for bands 4, 14 and 71, see Table B.2.12-1, Table 3.7-1, Table B.4.9-1. Rel-16 WI LTE_bands_R16_M2_NB2 introduced RF, RRM, demodulation for band 65 , see Table B.2.12-1, Table B.3.7-1, Table B.4.9-1. Rel-17 WI LTE_bands_R17_M1_M2_NB1_NB2 introduced RF, RRM, demodulation for band 24, see Table B.2.8-1, Table B.3.7-1, Table B.4.9-1. |
| Operating bands (TDD) for UE category NB1 and NB2 | Rel-15 | Table B.2.12-1, Table 3.7-1, Table B.4.9-1 | Rel-16 WI LTE_bands_R16_M1_NB1 and Rel-16 WI LTE_bands_R16_M2_NB2 introduced RF, RRM, demodulation for band 42 and band 43, see Table B.2.12-1, Table B.3.7-1, Table B.4.9-1. Rel-17 WI LTE_bands_R17_M1_M2_NB1_NB2 introduced RF, RRM, demodulation for band 48, see Table B.2.12-1, Table B.3.7-1, Table B.4.9-1. Rel-18 WI LTE_bands_R18_M1_M2_NB1_NB2 introduced RF, RRM, demodulation for band 54, see Table B.2.12-1, Table B.3.7-1, Table B.4.9-1. |
| Operating bands for UE category 1bis | Rel-13 | Table B.2.15-1, Table B.3.8-1, Table B.4.14-1 | Rel-16 RF, RRM, demodulation and CSI requirements for band 34 and band 40, see Table B.2.15-1, Table B.3.8-1, Table B.4.14-1. |

Table 3A.3-2: CA configurations for specific features

| Feature | Release independent from | Requirements to be fulfilled (see 36.307 of the REL when the feature was introduced) | Further information |
| --- | --- | --- | --- |
| Operating bands for V2X communication with con-current operation | Rel-14 | Table B.2.13-1, Table B.4.12-1 | Rel-15 WI V2X new band combinations (V2X_5A-47A, V2X_20A-47A, V2X_34A-47A, V2X_28A-47A, V2X_71A-47A) introduced and should be satisfied for the RF and RRM requirements in Table B.2.13-1, Table B.4.12-1 |
| Operating band for V2X communication with multi-carrier at Band 47 | Rel-14 | Table B.2.13-1, Table B.4.12-1 | In Rel-15 WI for eV2X, introduce intra-band multi-carrier V2X_47C and V2X_47C1 and should be satisfied for the RF and RRM requirements in Table B.2.13-1, Table B.4.12-1 |

## 3A.4	Other release independent features

This clause covers requirements for a Rel-P UE coming from all other release independent features that are not covered under clause 3A.1, 3A.2 and 3A.3, e.g. generic baseband requirements or requirements that are not band/CA configuration specific.

Table 3A.4-1: Additional requirements of other release independent features

| Feature | Release independent from | Requirements to be fulfilled (see 36.307 of the REL when the feature was introduced) | Further information |
| --- | --- | --- | --- |
| RF and performance requirements for 4Rx UEs | Rel-10 | Table C.1-1, Table C.2-1 for single carrier and Table C.1-2, Table C.2-2 for CA | Rel-13 WI LTE_4Rx_AP_DL introduced: - single carrier RF requirements for bands 1, 2, 3, 7, 20, 39, 41, 42: see Table C.1-1 - CA RF requirements for CA_3A-42A and other 1UL CA configurations (see TS 36.101 REL-13 [2] Table 7.3.1A-0a NOTE 20): see Table C.1-2 - single carrier performance requirements for demodulation and CSI: see Table C.2-1 REL-14 WI LTE_4Rx_AP_DL_bands introduced: - single carrier RF requirements for band 35, 40: see Table C.1-1 - CA RF requirements for some further 1UL CA configurations (see TS 36.101 REL-14 [2]): see Table C.1-2 REL-14 WI LTE_4Rx_AP_DL_CA introduced: - CA RF requirements for some 2DL/2UL CA configurations (see TS 36.101 REL-14 [2]): see Table C.1-2 - CA performance requirements for demodulation/SDR and CSI: see Table C2-2 REL-15 WI LTE_4Rx_AP_DL_bands_R15 introduced: - single carrier RF requirements for band 4, 34, 43, 66: see Table C.1-1 - CA RF requirements for some further 1UL CA configurations (see TS 36.101 REL-15 [2]): see Table C.1-2 |
| RF and performance requirements for 8Rx UEs | Rel-13 | Table E.1-1, Table E.2-1 for single carrier and Table E.1-2, Table E.2-2 for CA | Rel-15 WI LTE_8Rx_AP_DL introduced: - single carrier RF requirements for band 41, 42,43: see Table E.1-1 - CA RF requirements for CA_41C, CA_42C and CA_41A-42A CA configurations (see TS 36.101 Rel-15 [2]): see Table E.1-2 - single carrier performance requirements for demodulation and CSI: see Table E.2-1 - CA performance requirements for demodulation/SDR: see Table E.2-2 |
| RRM and demodulation requirements for high speed scenario | Rel-13 (NOTE 1) | Table D.1-1, Table D.2-1 | Rel-14 WI LTE_high_speed introduced band independent RRM and demodulation requirements. see Table D.1-1, Table D.2-1 |
| RF, RRM and demodulation requirements for NB-IoT standalone operation over NTN | Rel-17 | Table F.1-1 for UE RF requirements, Table F.2-1 for RRM requirements Table F.3-1 for UE demodulation requirements | Rel-18 WI LTE_NBIoT_eMTC_NTN_req introduced RF, RRM and demodulation requirements for NB-IoT standalone operation over NTN. See tables F.1-1, F.2-1, F.3-1. |
| RF, RRM and demodulation requirements for eMTC operation over NTN | Rel-17 | Table F.1-2 for UE RF requirements, Table F.2-2 for RRM requirements Table F.3-2 for UE demodulation requirements | Rel-18 WI LTE_NBIoT_eMTC_NTN_req introduced RF, RRM and demodulation requirements for eMTC operation over NTN. See tables F.1-2, F.2-2, F.3-2. |
| NOTE: Rel-13 UEs supporting the high speed scenario requirements are assumed to read the Rel-14 high speed scenario information, which is broadcast to all UEs. |  |  |  |

# 4 – 292	Void

Annex A (informative) :
Frequency arrangement for overlapping operating bands

The following information is provided in order to assist a UE derive the DL EARFCN and UL EARFCN in a multi-band environment, in which multiple overlapping operating bands may be indicated in the fields freqBandIndicator and multiBandInfoList of SIB1.

The overlapping bands, independent of release, which may be indicated in a cell are shown in Table A-1 for applicable E-UTRA bands. The DL EARFCN and UL EARFCN are derived according to TS 36.101 Rel-P [2].

Table A-1: Overlapping bands (multi-band environments) for each E-UTRA band

| E-UTRA Operating Band | Overlapping E-UTRA operating bands | Duplex Mode |
| --- | --- | --- |
| 2 | 25 | FDD |
| 3 | 9 | FDD |
| 4 | 10, 66 | FDD |
| 5 | 18, 19, 26 | FDD |
| 8 | 106 | FDD |
| 9 | 3 | FDD |
| 10 | 4, 66 | FDD |
| 12 | 17 | FDD |
| 17 | 12 | FDD |
| 18 | 5, 26, 27 | FDD |
| 19 | 5, 26 | FDD |
| 25 | 2 | FDD |
| 26 | 5, 18, 19, 27 | FDD |
| 27 | 18, 26 | FDD |
| 33 | 39 | TDD |
| 38 | 41 | TDD |
| 39 | 33 | TDD |
| 41 | 38 | TDD |
| 66 | 4, 10 | FDD |
| 106 | 8 | FDD |

Annex B (normative):
Common Requirements for bands or CA

# B.1	Purpose of annex

The purpose of Annex B is to group the requirements that are common for several bands or CA configurations in this specification and use the common tables as references.

# B.2	Common RRM requirements

## B.2.1	Common RRM requirements for a release independent band

The requirements and test cases listed in Table B.2.1-1 are specified in TS 36.133 Rel-P [3].

Table B.2.1-1: Common RRM requirements for a release independent band

| Clause | Description |
| --- | --- |
| 4 Note 1 | E-UTRAN RRC_IDLE state mobility |
| 5 | E-UTRAN RRC_CONNECTED state mobility |
| 6 Note 2 | RRC Connection Mobility Control |
| 7 Note 3 | Timing and signalling characteristics |
| 8 Note 4 | UE Measurements Procedures in RRC_CONNECTED State |
| 9 Note 5 | Measurements performance requirements for UE |
| A.4 Note 1 | E-UTRAN RRC_IDLE state |
| A.5 | E-UTRAN RRC CONNECTED Mode Mobility |
| A.6 Note 2 | RRC Connection Control |
| A.7 Note 3 | Timing and Signalling Characteristics |
| A.8 Note 4 | UE Measurements Procedures |
| A.9 Note 5 | Measurement Performance Requirements |
| NOTE 1: All requirements and the corresponding test cases shall apply, except: - for supporting the corresponding band in Rel-9 and below: clause 4.3 (Minimization of Drive Tests). NOTE 2: All requirements and the corresponding test cases shall apply, except: - for supporting the corresponding band in Rel-8: clauses 6.3 (RRC Connection Release with Redirection), 6.4 (CSG Proximity Indication for E-UTRAN and UTRAN). NOTE 3: All requirements and corresponding test cases shall apply, except those defined in clauses 7.4 and 7.5. NOTE 4: All requirements and corresponding test cases shall apply, except: - for supporting the corresponding band in Rel-8: clauses 8.1.2.5 (E-UTRAN OTDOA Intra-Frequency RSTD Measurements), 8.1.2.6 (E-UTRAN Inter-Frequency OTDOA Measurements), 8.1.2.7 (E-UTRAN E-CID Measurements). NOTE 5: All requirements and corresponding test cases shall apply, except: - for supporting the corresponding band in Rel-8: clauses 9.1.9 (UE Rx–Tx time difference), 9.1.10 (Reference Signal Time Difference). - for supporting the corresponding band in Rel-11 or below: the RSRP absolute accuracy requirement under normal conditions in table 9.1.2.1-1, 9.1.2.3-1, 9.1.2.5-1 and 9.1.3.1-1 when Io≤-70dBm is ±6dB. - for supporting the corresponding band in Rel-11 or below: the interfrequency RSRP relative accuracy requirement under normal conditions in table 9.1.3.2-1 is ±6dB. NOTE 6: In addition to the exceptions above, all requirements and test cases in this table shall apply, except those defined for: - carrier aggregation; - for supporting the corresponding band in Rel-9 or below: measurements under time-domain measurement resource restriction without CRS assistance information; - for supporting the corresponding band in Rel-10 or below: measurements under time-domain measurement resource restriction with CRS assistance information; - for supporting the corresponding band in Rel-11 or below: requirements introduced in Rel-12. |  |

## B.2.2	Common RRM requirements for an intra-band contiguous CA configuration

The requirements and test cases listed in Table B.2.2-1 are specified in TS 36.133 Rel-P [3].

Table B.2.2-1: Common RRM requirements for a release independent single-band CA configuration

| Clause | Description |
| --- | --- |
| 7.1 | UE transmit timing |
| 7.7 | SCell Activation and Deactivation Delay for E-UTRA Carrier Aggregation |
| 7.8 | Interruptions with Carrier Aggregation |
| 8.2 | Capabilities for Support of Event Triggering and Reporting Criteria |
| 8.3 | Measurements for E-UTRA carrier aggregation |
| 8.4 | OTDOA RSTD Measurements for E-UTRAN carrier aggregation |
| 9.1.11 Note 3 | Carrier aggregation measurement accuracy |
| 9.1.12 | Reference Signal Time Difference (RSTD) Measurement Accuracy Requirements for Carrier Aggregation |
| A.7 | Timing and Signalling Characteristics |
| A.8 | UE Measurements Procedures |
| A.9 Note 3 | Measurement Performance Requirements |
| NOTE 1: Only requirements and test cases defined for intra-band contiguous carrier aggregation shall apply. NOTE 2: In addition to the exceptions above, all requirements and test cases in this table shall apply, except: - for supporting the corresponding band in Rel-11 or below: requirements introduced in Rel-12. NOTE 3: - For supporting the corresponding band in Rel-11 or below: the RSRP absolute accuracy requirement under normal conditions in table 9.1.2.1-1, 9.1.2.3-1, 9.1.2.5-1 and 9.1.3.1-1 when Io≤-70dBm is ±6dB. - For supporting the corresponding band in Rel-11 or below: the interfrequency RSRP relative accuracy requirement under normal conditions in table 9.1.3.2-1 is ±6dB. |  |

## B.2.3	Common RRM requirements for an intra-band non-contiguous CA with single uplink configuration

The requirements and test cases listed in Table B.2.3-1 are specified in TS 36.133 Rel-P [3].

Table B.2.3-1: Common RRM requirements for a release independent single-band CA configuration

| Clause | Description |
| --- | --- |
| 7.1 | UE transmit timing |
| 7.7 | SCell Activation and Deactivation Delay for E-UTRA Carrier Aggregation |
| 7.8 | Interruptions with Carrier Aggregation |
| 8.2 | Capabilities for Support of Event Triggering and Reporting Criteria |
| 8.3 | Measurements for E-UTRA carrier aggregation |
| 8.4 | OTDOA RSTD Measurements for E-UTRAN carrier aggregation |
| 9.1.11 Note 3 | Carrier aggregation measurement accuracy |
| 9.1.12 | Reference Signal Time Difference (RSTD) Measurement Accuracy Requirements for Carrier Aggregation |
| A.7 | Timing and Signalling Characteristics |
| A.8 | UE Measurements Procedures |
| A.9 Note 3 | Measurement Performance Requirements |
| NOTE 1: Only requirements and test cases defined for intra-band non-contiguous carrier aggregation with single uplink shall apply. NOTE 2: In addition to the exceptions above, all requirements and test cases in this table shall apply, except: - for supporting the corresponding band in Rel-11 or below: requirements introduced in Rel-12. NOTE 3: - For supporting the corresponding band in Rel-11 or below: the RSRP absolute accuracy requirement under normal conditions in table 9.1.2.1-1, 9.1.2.3-1, 9.1.2.5-1 and 9.1.3.1-1 when Io≤-70dBm is ±6dB - for supporting the corresponding band in Rel-11 or below: the interfrequency RSRP relative accuracy requirement under normal conditions in table 9.1.3.2-1 is ±6dB. |  |

## B.2.4	Common RRM requirements for an inter-band CA with single uplink configuration

The requirements and test cases listed in Table B.2.4-1 are specified in TS 36.133 Rel-P [3].

Table B.2.4-1: Common RRM requirements for a release independent band-combination CA configuration

| Clause | Description |
| --- | --- |
| 7.1 | UE transmit timing |
| 7.7 | SCell Activation and Deactivation Delay for E-UTRA Carrier Aggregation |
| 7.8 | Interruptions with Carrier Aggregation |
| 8.2 | Capabilities for Support of Event Triggering and Reporting Criteria |
| 8.3 | Measurements for E-UTRA carrier aggregation |
| 8.4 | OTDOA RSTD Measurements for E-UTRAN carrier aggregation |
| 9.1.11 Note 3 | Carrier aggregation measurement accuracy |
| 9.1.12 | Reference Signal Time Difference (RSTD) Measurement Accuracy Requirements for Carrier Aggregation |
| A.7 | Timing and Signalling Characteristics |
| A.8 | UE Measurements Procedures |
| A.9 Note 3 | Measurement Performance Requirements |
| NOTE 1: Only requirements and test cases defined for inter-band with single uplink carrier aggregation shall apply. NOTE 2: In addition to the exceptions above, all requirements and test cases in this table shall apply, except: - for supporting the corresponding band in Rel-11 or below: requirements introduced in Rel-12. NOTE 3: - For supporting the corresponding band in Rel-11 or below: the RSRP absolute accuracy requirement under normal conditions in table 9.1.2.1-1, 9.1.2.3-1, 9.1.2.5-1 and 9.1.3.1-1 when Io≤-70dBm is ±6dB. - for supporting the corresponding band in Rel-11 or below: the interfrequency RSRP relative accuracy requirement under normal conditions in table 9.1.3.2-1 is ±6dB. |  |

## B.2.5	Common RRM requirements for an inter-band CA with dual uplink configuration

The requirements and test cases listed in Table B.2.5-1 are specified in TS 36.133 Rel-P [3].

Table B.2.5-1: Common RRM requirements for a release independent band-combination CA configuration with dual uplink

| Clause | Description |
| --- | --- |
| 7.1 | UE transmit timing |
| 7.7 | SCell Activation and Deactivation Delay for E-UTRA Carrier Aggregation |
| 7.8 | Interruptions with Carrier Aggregation |
| 7.17 | Maximum Transmission Timing Difference in Dual Connectivity |
| 8.2 | Capabilities for Support of Event Triggering and Reporting Criteria |
| 8.3 | Measurements for E-UTRA carrier aggregation |
| 8.4 | OTDOA RSTD Measurements for E-UTRAN carrier aggregation |
| 9.1.11 Note 3 | Carrier aggregation measurement accuracy |
| 9.1.12 | Reference Signal Time Difference (RSTD) Measurement Accuracy Requirements for Carrier Aggregation |
| A.7 | Timing and Signalling Characteristics |
| A.8 | UE Measurements Procedures |
| A.9 Note 3 | Measurement Performance Requirements |
| NOTE 1: Only requirements and test cases defined for inter-band with dual uplink carrier aggregation shall apply. NOTE 2: In addition to the exceptions above, all requirements and test cases in this table shall apply, except: - for supporting the corresponding band in Rel-11 or below: requirements introduced in Rel-12. NOTE 3: - For supporting the corresponding band in Rel-11 or below: the RSRP absolute accuracy requirement under normal conditions in table 9.1.2.1-1, 9.1.2.3-1, 9.1.2.5-1 and 9.1.3.1-1 when Io≤-70dBm is ±6dB. - for supporting the corresponding band in Rel-11 or below: the interfrequency RSRP relative accuracy requirement under normal conditions in table 9.1.3.2-1 is ±6dB. |  |

## B.2.6	Common RRM requirements for an intra-band non-contiguous CA with dual uplink configuration

The requirements and test cases listed in Table B.2.6-1 are specified in TS 36.133 Rel-P [3].

Table B.2.6-1: Common RRM requirements for a release independent single-band CA configuration with dual uplink

| Clause | Description |
| --- | --- |
| 7.1 | UE transmit timing |
| 7.7 | SCell Activation and Deactivation Delay for E-UTRA Carrier Aggregation |
| 7.8 | Interruptions with Carrier Aggregation |
| 7.17 | Maximum Transmission Timing Difference in Dual Connectivity |
| 8.2 | Capabilities for Support of Event Triggering and Reporting Criteria |
| 8.3 | Measurements for E-UTRA carrier aggregation |
| 8.4 | OTDOA RSTD Measurements for E-UTRAN carrier aggregation |
| 9.1.11 Note 3 | Carrier aggregation measurement accuracy |
| 9.1.12 | Reference Signal Time Difference (RSTD) Measurement Accuracy Requirements for Carrier Aggregation |
| A.7 | Timing and Signalling Characteristics |
| A.8 | UE Measurements Procedures |
| A.9 Note 3 | Measurement Performance Requirements |
| NOTE 1: Only requirements and test cases defined for intra-band non-contiguous carrier aggregation with dual uplinks shall apply. NOTE 2: In addition to the exceptions above, all requirements and test cases in this table shall apply, except: - for supporting the corresponding band in Rel-11 or below: requirements introduced in Rel-12. NOTE 3: - For supporting the corresponding band in Rel-11 or below: the RSRP absolute accuracy requirement under normal conditions in table 9.1.2.1-1, 9.1.2.3-1, 9.1.2.5-1 and 9.1.3.1-1 when Io≤-70dBm is ±6dB. - for supporting the corresponding band in Rel-11 or below: the interfrequency RSRP relative accuracy requirement under normal conditions in table 9.1.3.2-1 is ±6dB. |  |

## B.2.7	Common RRM requirements for an inter-band CA with three uplink configuration

The requirements and test cases listed in Table B.2.7-1 are specified in TS 36.133 Rel-P [3].

Table B.2.7-1: Common RRM requirements for a release independent band-combination CA configuration with three uplink

| Clause | Description |
| --- | --- |
| 7.1 | UE transmit timing |
| 7.7 | SCell Activation and Deactivation Delay for E-UTRA Carrier Aggregation |
| 7.8 | Interruptions with Carrier Aggregation |
| 7.17 | Maximum Transmission Timing Difference in Dual Connectivity |
| 8.2 | Capabilities for Support of Event Triggering and Reporting Criteria |
| 8.3 | Measurements for E-UTRA carrier aggregation |
| 8.4 | OTDOA RSTD Measurements for E-UTRAN carrier aggregation |
| 9.1.11 Note 3 | Carrier aggregation measurement accuracy |
| 9.1.12 | Reference Signal Time Difference (RSTD) Measurement Accuracy Requirements for Carrier Aggregation |
| NOTE 1: Only requirements defined for three uplink carrier aggregation shall apply. There are no test cases defined with a three uplink carrier aggregation configuration. NOTE 2: In addition to the exceptions above, all requirements and test cases in this table shall apply, except: - for supporting the corresponding band in Rel-11 or below: requirements introduced in Rel-12. NOTE 3: - For supporting the corresponding band in Rel-11 or below: the RSRP absolute accuracy requirement under normal conditions in table 9.1.2.1-1, 9.1.2.3-1, 9.1.2.5-1 and 9.1.3.1-1 when Io≤-70dBm is ±6dB. - for supporting the corresponding band in Rel-11 or below: the interfrequency RSRP relative accuracy requirement under normal conditions in table 9.1.3.2-1 is ±6dB. |  |

## B.2.8	Common RRM requirements for operating bands for UE category NB1

The requirements and test cases listed in Table B.2.8-1 are specified in TS 36.133 Rel-P [3].

Table B.2.8-1: Common RRM requirements for release independent
operating bands for UE category NB1

| Clause | Description |
| --- | --- |
| 4.6 | Cell Selection and Reselection Requirements for UE category NB1 |
| 6.6 | Random Access for UE category NB1 |
| 7.23 | Radio Link Monitoring for category NB1 UE |
| 8.14 | Measurements for UE category NB1 |
| 9.1.22 | Measurement accuracy for UE Category NB1 |
| 9.1.23 | Power Headroom for UE category NB1 |
| NOTE 1: Only requirements and test cases defined for UE category NB1 shall apply. |  |

## B.2.9	Common RRM requirements for operating bands for UE category 0

The requirements and test cases listed in Table B.2.9-1 are specified in TS 36.133 Rel-P [3].

Table B.2.9-1: Common RRM requirements for release independent
operating bands for a UE category 0

| Clause | Description |
| --- | --- |
| 7.11 | Radio Link Monitoring for UE category 0 |
| 8.5 | Measurements for UE category 0 |
| 9.1.13 | Measurement accuracy for UE category 0 |

## B.2.10	Common RRM requirements for operating bands for UE category M1

The requirements and test cases listed in Table B.2.10-1 are specified in TS 36.133 Rel-P [3].

Table B.2.10-1: Common RRM requirements for release independent
operating bands for a UE category M1

| Clause | Description |
| --- | --- |
| 4.7 | Cell Selection and Re-selection Requirements for UE category M1 |
| 5.5 | E-UTRAN Handover for cat.M1 UEs in CEModeA |
| 5.6 | E-UTRAN Handover for cat.M1 UEs in CEModeB |
| 6.2.3 | Random Access Requirements for cat.M1 UEs |
| 6.7 | RRC Re-establishment for cat.M1 UEs |
| 6.8 | RRC Connection Release with Redirection for Cat-M1 UEs |
| 7.19 | Radio Link Monitoring for UE Category M1 |
| 7.24 | UE transmit timing for category M1 |
| 7.27 | UE timer accuracy for category M1 |
| 7.28 | Timing Advance for Category M1 |
| 8.13 | Measurements for UE category M1 |
| 9.1.21 | Measurement accuracy for UE category M1 |

## B.2.11	Common RRM requirements for operating bands for UE category M2

The requirements and test cases listed in Table B.2.11-1 are specified in TS 36.133 Rel-P [3].

Table B.2.11-1: Common RRM requirements for release independent
operating bands for a UE category M2

| Clause | Description |
| --- | --- |
| 4.7 | Cell Selection and Re-selection Requirements |
| 5.5 | E-UTRAN Handover in CEModeA |
| 5.6 | E-UTRAN Handover in CEModeB |
| 6.2.3 | Random Access Requirements |
| 6.7 | RRC Re-establishment |
| 6.8 | RRC Connection Release with Redirection |
| 7.19 | Radio Link Monitoring |
| 7.26 | UE transmit timing for category M2 |
| 7.27 | UE timer accuracy |
| 7.28 | Timing Advance |
| 8.13.2.1 and 8.13.3.1 | E-UTRAN intra frequency measurement requirements |
| 8.13.2.6 and 8.13.3.5 | E-UTRAN inter frequency measurement requirements |
| 8.13.2.7 and 8.13.3.6 | UE measurement capability |
| 8.13.2.5.1, 8.13.2.5.2, 8.13.2.5.3, 8.13.2.5.4, 8.13.2.5.5, 8.13.2.5.6 and 8.13.3.4 | E-UTRAN E-CID measurements requirements |
| 8.16 | Measurements for UE Category M2 |
| 9.1.21 | Measurement accuracy |
| 9.1.25 | Measurement accuracy for UE category M2 |

## B.2.12	Common RRM requirements for operating bands for UE category NB2

The requirements and test cases listed in Table B.2.12-1 are specified in TS 36.133 Rel-P [3].

Table B.2.12-1: Common RRM requirements for release independent
operating bands for UE category NB2

| Clause | Description |
| --- | --- |
| 4.6.1 and 4.6.2 | Cell selection and re-selection requirements |
| 4.8 | UE Positioning measurement in idle state |
| 6.5 | RRC Re-establishment requirements |
| 6.6 | Random access requirements |
| 6.9 | RRC connection redirection to non-anchor carrier requirements |
| 7.20 | UE transmit timing requirements |
| 7.21 | UE timer accuracy requirements |
| 7.22 | Timing advance requirements |
| 7.23 | Radio link monitoring requirements |
| 8.14 | UE RRC_CONNECTED state measurement requirement |
| 9.1.22 | UE measurement accuracy requirements |
| 9.1.23 | Power headroom requirements |
| NOTE 1: Only requirements and test cases defined for UE category NB2 shall apply. |  |

## B.2.13	Common RRM requirements for operating bands for LTE-based V2X Communication

The requirements and test cases listed in Table B.2.13-1 are specified in TS 36.133 Rel-P [3].

Table B.2.13-1: Common RRM requirements for release independent operating bands for LTE-based V2X communication

| Clause | Description |
| --- | --- |
| 13.2 | UE Transmit Timing |
| 13.3 | Initiation/Cease of SLSS Transmissions |
| 13.4 | Selection / Reselection of V2X Synchronization Reference Source |
| 13.5 | Autonomous Resource Selection/Reselection measurements |
| 13.6 | Congestion Control measurements |
| 13.7 | Interruption |
| 13.8 | Reliability of GNSS signal |

## B.2.14	Common RRM requirements for an inter-band CA with four uplink configuration

The requirements and test cases listed in Table B.2.14-1 are specified in TS 36.133 Rel-P [3].

Table B.2.14-1: Common RRM requirements for a release independent band-combination CA configuration with four uplink

| Clause | Description |
| --- | --- |
| 7.1 | UE transmit timing |
| 7.7 | SCell Activation and Deactivation Delay for E-UTRA Carrier Aggregation |
| 7.8 | Interruptions with Carrier Aggregation |
| 7.17 | Maximum Transmission Timing Difference in Dual Connectivity |
| 8.2 | Capabilities for Support of Event Triggering and Reporting Criteria |
| 8.3 | Measurements for E-UTRA carrier aggregation |
| 8.4 | OTDOA RSTD Measurements for E-UTRAN carrier aggregation |
| 9.1.11 Note 3 | Carrier aggregation measurement accuracy |
| 9.1.12 | Reference Signal Time Difference (RSTD) Measurement Accuracy Requirements for Carrier Aggregation |
| NOTE 1: Only requirements defined for four uplink carrier aggregation shall apply. There are no test cases defined with a four uplink carrier aggregation configuration. NOTE 2: In addition to the exceptions above, all requirements and test cases in this table shall apply, except: - for supporting the corresponding band in Rel-11 or below: requirements introduced in Rel-12. NOTE 3: For supporting the corresponding band in Rel-11 or below: the RSRP absolute accuracy requirement under normal conditions in table 9.1.2.1-1, 9.1.2.3-1, 9.1.2.5-1 and 9.1.3.1-1 when Io≤-70dBm is ±6dB. For supporting the corresponding band in Rel-11 or below: the interfrequency RSRP relative accuracy requirement under normal conditions in table 9.1.3.2-1 is ±6dB. |  |

## B.2.15	Common RRM requirements for operating bands for UE category 1bis

The requirements and test cases listed in Table B.2.15-1 are specified in TS 36.133 Rel-P [3].

Table B.2.15-1: Common RRM requirements for release independent
operating bands for a UE category 1bis

| Clause | Description |
| --- | --- |
| 8.1.2.5.3, 8.1.2.5.4 | Intra-Frequency OTDOA Measurements for UE Category 1bis |
| 8.1.2.6.5, 8.1.2.6.6, 8.1.2.6.7, 8.1.2.6.8 | Inter-Frequency OTDOA Measurements for UE Category 1bis |
| 9.1.2.7, 9.1.2.8 | Intra-frequency RSRP Accuracy Requirements for UE Category 1bis |
| 9.1.3.1, 9.1.3.4 | Intra-frequency RSRP Accuracy Requirements for UE Category 1bis |
| 9.1.5.5 | Intra-frequency RSRQ Accuracy Requirements for UE Category 1bis |
| 9.1.6.5, 9.1.6.6 | Inter-frequency RSRQ Accuracy Requirements for UE Category 1bis |
| 9.1.10.5, 9.1.10.6 | Reference Signal Time Difference (RSTD) for UE Category 1bis |

# B.3	Common UE performance requirements

## B.3.1	Void

## B.3.2	Common UE performance requirements and tests for different CA configurations and combination sets

The requirements and test cases listed in Table B.3.2-1 are specified in TS 36.101 Rel-P [2].

Table B.3.2-1: Common UE performance requirements and tests for different CA configurations and combination sets

| Clause | Description |
| --- | --- |
| 8.2.1.1.1 | Single-antenna port performance (FDD) |
| 8.2.2.1.1 | Single-antenna port performance (TDD) |
| 8.2.3.1.1 | Single-antenna port performance (TDD-FDD CA) |
| 8.2.1.3.1 | Open-loop spatial multiplexing performance - Minimum Requirement 2 Tx Antenna Port (FDD) |
| 8.2.2.3.1 | Open-loop spatial multiplexing performance - Minimum Requirement 2 Tx Antenna Port (TDD) |
| 8.2.3.3.1 | Open-loop spatial multiplexing performance - Minimum Requirement 2 Tx Antenna Port (TDD-FDD CA) |
| 8.2.1.3.1A | Open-loop spatial multiplexing performance - Soft buffer management test (FDD) |
| 8.2.2.3.1A | Open-loop spatial multiplexing performance - Soft buffer management test (TDD) |
| 8.2.3.3.1A | Open-loop spatial multiplexing performance - Soft buffer management test (TDD-FDD CA) |
| 8.2.1.4.3 | Closed-loop spatial multiplexing performance - Minimum Requirement Multi-Layer Spatial Multiplexing 4 Tx Antenna Port (FDD) |
| 8.2.2.4.3 | Closed-loop spatial multiplexing performance - Minimum Requirement Multi-Layer Spatial Multiplexing 4 Tx Antenna Port (TDD) |
| 8.2.3.4.3 | Closed-loop spatial multiplexing performance - Minimum Requirement Multi-Layer Spatial Multiplexing 4 Tx Antenna Port (TDD-FDD CA) |
| 8.2.1.7 | Carrier aggregation with power imbalance (FDD) |
| 8.2.1.8 | Intra-band non-contiguous carrier aggregation with timing offset (FDD) |
| 8.2.2.7 | Carrier aggregation with power imbalance (TDD) |
| 8.7.1 | Sustained downlink data rate provided by lower layers (FDD) |
| 8.7.2 | Sustained downlink data rate provided by lower layers (TDD) |
| 8.7.5 | Sustained downlink data rate provided by lower layers (TDD-FDD CA) |
| 8.7.12.1 | Sustained downlink data rate provided by lower layers (FDD CA in licensed bands) |
| 8.7.12.2 | Sustained downlink data rate provided by lower layers (TDD CA in licensed bands) |
| 8.7.12.3 | Sustained downlink data rate provided by lower layers (TDD-FDD CA in licensed bands) |
| 9.6.1.1 | Additional requirements for carrier aggregation - Periodic reporting on multiple cells (Cell Specific Reference symbols) (FDD) |
| 9.6.1.2 | Additional requirements for carrier aggregation - Periodic reporting on multiple cells (Cell Specific Reference symbols) (TDD) |
| 9.6.1.3 | Additional requirements for carrier aggregation - Periodic reporting on multiple cells (Cell Specific Reference symbols) (TDD-FDD CA) |
| NOTE 1: The applicability of requirements for different CA configurations and bandwidth combination sets is specified in Clause 8.1.2.3 and 9.1.1.2. NOTE 2: The test coverage for different number of component carriers is defined in 8.1.2.4. |  |

## B.3.3	Void

## B.3.4	Void

## B.3.5	Common UE performance requirements and tests for operating bands for UE category 0

The requirements and test cases listed in Table B.3.5-1 are specified in TS 36.101 Rel-P [2].

Table B.3.5-1: Common UE performance requirements and tests for release independent operating bands for UE category 0

| Clause | Description |
| --- | --- |
| 8.9 | Demodulation (single receiver antenna) |
| 9.7 | CSI reporting (Single receiver antenna) |

## B.3.6	Common UE performance requirements and tests for operating bands for UE category M1 and M2

The requirements and test cases listed in Table B.3.6-1 are specified in TS 36.101 Rel-P [2].

Table B.3.6-1: Common UE performance requirements and tests for release independent operating bands for UE category M1 and M2

| Clause | Description |
| --- | --- |
| 8.11 | Demodulation (UE supporting coverage enhancement) |
| 9.8 | CSI reporting (UE supporting coverage enhancement) |

## B.3.7	Common UE performance requirements and tests for operating bands for UE category NB1 and NB2

The requirements and test cases listed in Table B.3.7-1 are specified in TS 36.101 Rel-P [2].

Table B.3.7-1: Common UE performance requirements and tests for release independent operating bands for UE category NB1 and NB2

| Clause | Description |
| --- | --- |
| 8.12 | Demodulation of Narrowband IoT |

## B.3.8	Common UE performance requirements and tests for operating bands for UE category 1bis

The requirements and test cases listed in Table B.3.8-1 are specified in TS 36.101 Rel-P [2].

Table B.3.8-1: Common UE performance requirements and tests for release independent operating bands for UE category 1bis

| Clause | Description |
| --- | --- |
| 9.7.1.3, 9.7.1.4, 9.7.2.3, 9.7.2.4 | CSI reporting (Single receiver antenna) |

# B.4	Common UE RF requirements

## B.4.1	Common UE RF requirements for a release independent band

The requirements and test cases listed in Table B.4.1-1 are specified in TS 36.101 Rel-P [2].

Table B.4.1-1: Common UE RF requirements for a release independent band

| Clause | Description |
| --- | --- |
| 5.5 | Operating bands |
| 5.6 | Channel bandwidth |
| 5.7 | Channel arrangement |
| 6.2 | Transmit power |
| 6.3 | Output power dynamics |
| 6.5 | Transmit signal quality |
| 6.6 | Output RF spectrum emissions |
| 6.7 | Transmit intermodulation |
| 7.3 | Reference sensitivity power level |
| 7.4 | Maximum input level |
| 7.5 | Adjacent Channel Selectivity (ACS) |
| 7.6 | Blocking characteristics |
| 7.7 | Spurious response |
| 7.8 | Intermodulation characteristics |
| 7.9 | RX spurious emissions |

## B.4.2	Common UE RF requirements for an intra-band contiguous CA configuration

The requirements and test cases listed in Table B.4.2-1 are specified in TS 36.101 Rel-P [2].

Table B.4.2-1: Common UE RF requirements for a release independent intra-band contiguous CA configuration

| Clause | Description |
| --- | --- |
| 5.5A | Operating bands for CA |
| 5.6A | Channel bandwidths per operating band for CA |
| 5.7.1A | Channel spacing for CA |
| 5.7.2A | Channel raster for CA |
| 5.7.4A | TX–RX frequency separation for CA |
| 6.2.2A | UE maximum output power for CA |
| 6.2.3A | UE maximum output power for modulation/channel bandwidth for CA |
| 6.2.4A | UE maximum output power with additional requirements for CA |
| 6.2.5A | Configured transmitted power for CA |
| 6.3.2A | UE Minimum output power for CA |
| 6.3.3A | UE Transmit OFF power for CA |
| 6.3.4A | ON/OFF time mask for CA |
| 6.3.5A | Power control for CA |
| 6.5.1A | Frequency error for CA |
| 6.5.2A | Transmit modulation quality for CA |
| 6.6.1A | Occupied bandwidth for CA |
| 6.6.2.1A | Spectrum emission mask for CA |
| 6.6.2.2A | Additional Spectrum Emission mask for CA |
| 6.6.2.3.2A | UTRA ACLR for CA |
| 6.6.2.3.3A | E-UTRA ACLR for CA |
| 6.6.3.1A | Minimum requirements for CA |
| 6.6.3.2A | Spurious emission band UE co-existence for CA |
| 6.6.3.3A | Additional spurious emissions for CA |
| 6.7.1A | Minimum requirement for CA |
| 7.3.1A | Reference sensitivity for CA |
| 7.4.1A | Maximum input level for CA |
| 7.5.1A | Adjacent Channel Selectivity (ACS) for CA |
| 7.6.1.1A | In-band blocking for CA |
| 7.6.2.1A | Out-of-band blocking for CA |
| 7.6.3.1A | Narrow band blocking for CA |
| 7.7.1A | Spurious response for CA |
| 7.8.1A | Wideband intermodulation for CA |
| 7.10.1A | Receiver response for CA |

## B.4.3	Common UE RF requirements for an single uplink inter-band CA configuration

The requirements and test cases listed in Table B.4.3-1 are specified in TS 36.101 Rel-P [2].

Table B.4.3-1: Common UE RF requirements for a release independent inter-band CA configuration

| Clause | Description |
| --- | --- |
| 5.5A | Operating bands for CA |
| 5.6A.1 | Channel bandwidths per operating band for CA |
| 5.7.2A | Channel raster for CA |
| 6.2.2A | UE maximum output power for CA |
| 6.2.3A | UE maximum output power for modulation/channel bandwidth for CA |
| 6.2.5 | Configured transmitted power |
| 7.3.1A | Reference sensitivity for CA |
| 7.4.1A | Maximum input level for CA |
| 7.5.1A | Adjacent Channel Selectivity (ACS) for CA |
| 7.6.1.1A | In-band blocking for CA |
| 7.6.2.1A | Out-of-band blocking for CA |
| 7.6.3.1A | Narrow band blocking for CA |
| 7.7.1A | Spurious response for CA |
| 7.8.1A | Wideband intermodulation for CA |

## B.4.4	Common UE RF requirements for an inter-band CA configuration including an operating band without uplink band

The requirements and test cases listed in Table B.4.4-1 are specified in TS 36.101 Rel-P [2].

Table B.4.4-1: Common UE RF requirements for a release independent inter-band CA configuration including an operating band without uplink band

| Clause | Description |
| --- | --- |
| 5.5 | Operating bands |
| 5.5A | Operating bands for CA |
| 5.6A.1 | Channel bandwidths per operating band for CA |
| 5.7 | Channel arrangement |
| 6.2.2A | UE maximum output power for CA |
| 6.2.3A | UE maximum output power for modulation/channel bandwidth for CA |
| 6.2.5 | Configured transmitted power |
| 7.3.1A | Reference sensitivity for CA |
| 7.4.1A | Maximum input level for CA |
| 7.5.1A | Adjacent Channel Selectivity (ACS) for CA |
| 7.6.1.1A | In-band blocking for CA |
| 7.6.2.1A | Out-of-band blocking for CA |
| 7.6.3.1A | Narrow band blocking for CA |
| 7.7.1A | Spurious response for CA |
| 7.8.1A | Wideband intermodulation for CA |

## B.4.5	Common UE RF requirements for a single uplink intra-band non-contiguous CA configuration

The requirements and test cases listed in Table B.4.5-1 are specified in TS 36.101 Rel-P [2].

Table B.4.5-1: Common UE RF requirements for a release independent single uplink intra-band non-contiguous CA configuration

| Clause | Description |
| --- | --- |
| 5.5A | Operating bands for CA |
| 5.6A1 | Channel bandwidths per operating band for CA |
| 5.7.2A | Channel raster for CA |
| 6.2.2A | UE maximum output power for CA |
| 6.2.3A | UE maximum output power for modulation/channel bandwidth for CA |
| 7.3.1A | Reference sensitivity for CA |
| 7.4.1A | Maximum input level for CA |
| 7.5.1A | Adjacent Channel Selectivity (ACS) for CA |
| 7.6.1.1A | In-band blocking for CA |
| 7.6.2.1A | Out-of-band blocking for CA |
| 7.6.3.1A | Narrow band blocking for CA |
| 7.7.1A | Spurious response for CA |
| 7.8.1A | Wideband intermodulation for CA |

## B.4.6	Common UE RF requirements for dual uplink inter-band CA configuration

The requirements and test cases listed in Table B.4.6-1 are specified in TS 36.101 Rel-P [2].

Table B.4.6-1: Common UE RF requirements for a release independent dual uplink inter-band CA configuration

| Clause | Description |
| --- | --- |
| 5.6A.1 | Channel bandwidths per operating band for CA |
| 6.2.2A | UE maximum output power for CA |
| 6.2.5A | Configured transmitted Power for CA |
| 6.3.2A | UE Minimum output power for CA |
| 6.3.3A | UE Transmit OFF power for CA |
| 6.3.4A | ON/OFF time mask for CA |
| 6.3.5A | Power control for CA |
| 6.5.1A | Frequency error for CA |
| 6.5.2A | Transmit modulation quality for CA |
| 6.6.1A | Occupied bandwidth for CA |
| 6.6.2.1A | Spectrum emission mask for CA |
| 6.6.2.3 | Adjacent Channel Leakage Ratio |
| 6.6.3.1A | Spurious Emission for CA |
| 6.6.3.2A | Spurious emission band UE co-existence for CA |
| 6.7.1A | Transmit intermodulation for CA |
| 7.3.1A | Reference sensitivity for CA |
| 7.6.2.1A | Out-of-band blocking for CA |
| 7.7.1A | Spurious response for CA |

## B.4.7	Common UE RF requirements for dual uplink intra-band non-contiguous CA configuration

The requirements and test cases listed in Table B.4.7-1 are specified in TS 36.101 Rel-P [2].

Table B.4.7-1: Common UE RF requirements for a release independent dual uplink intra-band non-contiguous CA configuration

| Clause | Description |
| --- | --- |
| 5.6A.1 | Channel bandwidths per operating band for CA |
| 6.2.2A | UE maximum output power for CA |
| 6.2.3A | UE Maximum Output power for modulation / channel bandwidth for CA |
| 6.2.5A | Configured transmitted Power for CA |
| 6.3.2A | UE Minimum output power for CA |
| 6.3.3A | UE Transmit OFF power for CA |
| 6.3.4A | ON/OFF time mask for CA |
| 6.3.5A | Power control for CA |
| 6.5.1A | Frequency error for CA |
| 6.5.2A | Transmit modulation quality for CA |
| 6.6.1A | Occupied bandwidth for CA |
| 6.6.2.1A | Spectrum emission mask for CA |
| 6.6.2.3 | Adjacent Channel Leakage Ratio |
| 6.6.3.1A | Spurious Emission for CA |
| 6.6.3.2A | Spurious emission band UE co-existence for CA |
| 7.3.1A | Reference sensitivity for CA |
| 7.6.2.1A | Out-of-band blocking for CA |
| 7.7.1A | Spurious response for CA |

## B.4.8	Common UE RF requirements for three uplink inter-band CA configuration

The requirements and test cases listed in Table B.4.8-1 are specified in TS 36.101 Rel-P [2].

Table B.4.8-1: Common UE RF requirements for a release independent three uplink inter-band CA configuration

| Clause | Description |
| --- | --- |
| 5.6A.1 | Channel bandwidths per operating band for CA |
| 6.2.2A | UE maximum output power for CA |
| 6.2.5A | Configured transmitted Power for CA |
| 6.3.2A | UE Minimum output power for CA |
| 6.3.3A | UE Transmit OFF power for CA |
| 6.3.4A | ON/OFF time mask for CA |
| 6.3.5A | Power control for CA |
| 6.5.1A | Frequency error for CA |
| 6.5.2A | Transmit modulation quality for CA |
| 6.6.1A | Occupied bandwidth for CA |
| 6.6.2.1A | Spectrum emission mask for CA |
| 6.6.2.3 | Adjacent Channel Leakage Ratio |
| 6.6.3.1A | Spurious Emission for CA |
| 6.6.3.2A | Spurious emission band UE co-existence for CA |
| 6.7.1A | Transmit intermodulation for CA |
| 7.3.1A | Reference sensitivity for CA |
| 7.6.2.1A | Out-of-band blocking for CA |
| 7.7.1A | Spurious response for CA |

## B.4.9	Common UE RF requirements for operating bands for UE category NB1 and NB2

The requirements and test cases listed in Table B.4.9-1 are specified in TS 36.101 Rel-P [2].

Table B.4.9-1: Common UE RF requirements for release independent operating bands for UE category NB1

| Clause | Description |
| --- | --- |
| 5.5F | Operating bands for category NB1 and NB2 |
| 5.6F | Channel bandwidth for category NB1 and NB2 |
| 5.7.1F | Channel spacing for category NB1 and NB2 |
| 5.7.2F | Channel raster for category NB1 and NB2 |
| 5.7.3F | Carrier frequency and EARFCN for category NB1 and NB2 |
| 5.7.4F | TX–RX frequency separation for category NB1 and NB2 |
| 6.2.2F | UE maximum output power for category NB1 and NB2 |
| 6.2.3F | UE maximum output power for category NB1 and NB2 |
| 6.2.5F | Configured transmitted Power for category NB1 and NB2 |
| 6.3.2F | UE Minimum output power for category NB1 and NB2 |
| 6.3.3F | Transmit OFF power for category NB1 and NB2 |
| 6.3.4F | ON/OFF time mask for category NB1 and NB2 |
| 6.3.5F | Power Control for category NB1 and NB2 |
| 6.5.1F | Frequency error for UE category NB1 and NB2 |
| 6.5.2F | Transmit modulation quality for Category NB1 and NB2 |
| 6.6.1F | Occupied bandwidth for category NB1 and NB2 |
| 6.6.2F | Out of band emission for category NB1 and NB2 |
| 6.6.3F | Spurious emission for category NB1 and NB2 |
| 6.7.1F | Transmission intermodulation for category NB1 and NB2 |
| 7.3.1F | Reference sensitivity for UE category NB1 and NB2 |
| 7.4.1F | Maximum input level for category NB1 and NB2 |
| 7.5.1F | Adjacent channel selectivity for category NB1 and NB2 |
| 7.6.1.1F | In-band blocking for category NB1 and NB2 |
| 7.6.2.1F | Out-of-band blocking for category NB1 and NB2 |
| 7.7.1F | Spurious response for category NB1 and NB2 |
| 7.8.1F | Intermodulation characteristics for category NB1 and NB2 |

## B.4.10	Common UE RF requirements for operating bands for UE category 0

The requirements and test cases listed in Table B.4.10-1 are specified in TS 36.101 Rel-P [2].

Table B.4.10-1: Common UE RF requirements for release independent operating bands for UE category 0

| Clause | Description |
| --- | --- |
| 5.5E | Operating bands for UE category 0 |
| 7.3.1E | Minimum requirements (QPSK) for UE category 0 |

## B.4.11	Common UE RF requirements for operating bands for UE category M1 and M2

The requirements and test cases listed in Table B.4.11-1 are specified in TS 36.101 Rel-P [2].

Table B.4.11-1: Common UE RF requirements for release independent operating bands for UE category M1 and M2

| Clause | Description |
| --- | --- |
| 5.5E | Operating bands for UE category 0, UE category M1 and M2 and UE category 1bis |
| 5.7.4E | TX–RX frequency separation for category M1 and M2 |
| 6.2.2E | UE maximum output power for Category M1 and M2 UE |
| 6.2.3E | UE maximum output power for modulation / channel bandwidth for category M1 and M2 |
| 6.2.4E | UE maximum output power with additional requirements for category M1 and M2 UE |
| 6.3.5E | Power control for category M1 and M2 |
| 6.5.1E | Frequency error for UE category M1 and M2 |
| 6.5.2E | Transmit modulation quality for category M1 and M2 |
| 6.6.3.2 | Spurious emission band UE co-existence |
| 7.3.1E | Minimum requirements (QPSK) for UE category 0, M1, M2 and 1bis |
| 7.5 | Adjacent Channel Selectivity (ACS) |
| 7.6.1 | In-band blocking |
| 7.6.2 | Out-of-band blocking |
| 7.6.3 | Narrow band blocking |
| 7.8.1 | Wide band intermodulation |

## B.4.12	Common UE RF requirements for operating bands for LTE-based V2X operation

The requirements and test cases listed in Table B.4.12-1 are specified in TS 36.101 Rel-P [2].

Table B.4.12-1: Common UE RF requirements for release independent operating bands for V2X operation

| Clause | Description |
| --- | --- |
| 5.5G | Operating bands for V2X Communication |
| 5.6G | Channel bandwidth for V2X Communication |
| 6.2.2G | UE maximum output power for V2X Communication |
| 6.2.3G | UE maximum output power for modulation / channel bandwidth for V2X Communication |
| 6.2.4G | UE maximum output power with additional requirements for V2X Communication |
| 6.2.5G | Configured transmitted power for V2X Communication |
| 6.3.2G | UE Minimum output power for V2X Communication |
| 6.3.3G | Transmit OFF power for V2X Communication |
| 6.3.4G | ON/OFF time mask for V2X Communication |
| 6.3.5G | Power Control for V2X Communication |
| 6.5.1G | Frequency error for V2X Communication |
| 6.5.2G | Transmit modulation quality for V2X Communication |
| 6.6.3G | Spurious emission for V2X Communication |
| 7.3.1G | REFSENS requirements (QPSK) for V2X communication |
| 7.4.1G | Maximum input level for V2X communication |
| 7.5.1G | Adjacent Channel Selectivity (ACS) for V2X communication |
| 7.6.1.1G | In-band blocking for V2X communication |
| 7.6.2.1G | Out-of-band blocking for V2X communication |
| 7.7.1G | Spurious response for V2X communication |
| 7.8.1G | Intermodulation characteristics for V2X communication |
| 7.10.1G | Receiver image for V2X communication. (It is only applicable for intra-band multi-carrier V2X operation) |

## B.4.13	Common UE RF requirements for four uplink inter-band CA configuration

The requirements and test cases listed in Table B.4.13-1 are specified in TS 36.101 Rel-P [2].

Table B.4.13-1: Common UE RF requirements for a release independent four uplink inter-band CA configuration

| Clause | Description |
| --- | --- |
| 5.6A.1 | Channel bandwidths per operating band for CA |
| 6.2.2A | UE maximum output power for CA |
| 6.2.5A | Configured transmitted Power for CA |
| 6.3.2A | UE Minimum output power for CA |
| 6.3.3A | UE Transmit OFF power for CA |
| 6.3.4A | ON/OFF time mask for CA |
| 6.3.5A | Power control for CA |
| 6.5.1A | Frequency error for CA |
| 6.5.2A | Transmit modulation quality for CA |
| 6.6.1A | Occupied bandwidth for CA |
| 6.6.2.1A | Spectrum emission mask for CA |
| 6.6.2.3 | Adjacent Channel Leakage Ratio |
| 6.6.3.1A | Spurious Emission for CA |
| 6.6.3.2A | Spurious emission band UE co-existence for CA |
| 6.7.1A | Transmit intermodulation for CA |
| 7.3.1A | Reference sensitivity for CA |
| 7.6.2.1A | Out-of-band blocking for CA |
| 7.7.1A | Spurious response for CA |

## B.4.14	Common UE RF requirements for operating bands for UE category 1bis

The requirements and test cases listed in Table B.4.14-1 are specified in TS 36.101 Rel-P [2].

Table B.4.14-1: Common UE RF requirements for release independent operating bands for UE category 1bis

| Clause | Description |
| --- | --- |
| 5.5E | Operating bands for UE category 1bis |
| 7.3.1E | Minimum requirements (QPSK) for UE category 1bis |

Annex C (normative):
Common Requirements for 4Rx

# C.1	Common UE RF requirements

The requirements and test cases listed in Table C.1-1 are specified in TS 36.101 Rel-P [2].

Table C.1-1: RF requirements for 4Rx for single band

| Clause | Description |
| --- | --- |
| 7.3 | Reference sensitivity power level |
| 7.4 | Maximum input level |
| 7.5 | Adjacent channel selectivity |
| 7.6 | Blocking characteristics |
| 7.7 | Spurious response |
| 7.8 | Intermodulation characteristics |
| 7.9 | Spurious emissions |

The requirements and test cases listed in Table C.1-2 are specified in TS 36.101 Rel-P [2].

Table C.1-2: RF requirements for 4Rx for CA

| Clause | Description |
| --- | --- |
| 6.2.5A | Configured maximum output power |
| 7.3.1A | Reference sensitivity for CA |
| 7.4.1A | Maximum input level for CA |
| 7.5.1A | Adjacent Channel Selectivity for CA |
| 7.6.1.1A | In-band blocking for CA |
| 7.6.2.1A | Out-of-band blocking for CA |
| 7.6.3.1A | Narrow band blocking for CA |
| 7.7.1A | Spurious response for CA |
| 7.8.1A | Wideband intermodulation for CA |

# C.2	Common UE demodulation and CSI requirements

The requirements and test cases listed in Table C.2-1 are specified in TS 36.101 Rel-P [2].

Table C.2-1: UE Demodulation and CSI requirements for 4Rx for single band

| Clause | Description |
| --- | --- |
| 8.10.1 (NOTE) | PDSCH |
| 8.10.2 | PDCCH/PCFICH |
| 8.10.3 | PHICH |
| 8.10.4 | ePDCCH |
| 9.9 | CSI reporting for 4Rx UE |

The requirements and test cases listed in Table C.2-2 are specified in TS 36.101 Rel-P [2].

Table C.2-2: UE Demodulation and CSI requirements for 4Rx CA/DC

| Clause | Description |
| --- | --- |
| 8.13 | Demodulation of PDSCH CA |
| 8.7.9 | SDR of FDD CA (4 layer MIMO) |
| 8.7.10 | SDR of TDD CA (4 layer MIMO) |
| 8.7.11 | SDR of TDD-FDD CA (4 layer MIMO) |
| 8.7.13 | SDR of FDD DC (4 layer MIMO) |
| 8.7.14 | SDR of TDD DC (4 layer MIMO) |
| 8.7.15 | SDR of TDD-FDD DC (4 layer MIMO) |
| 9.1.1.4.2 | CSI CA tests for 4Rx UE |

Annex D (normative):
Common Requirements for performance enhancements for high speed scenario

# D.1	Common RRM requirements for performance enhancements for high speed scenario

The requirements and test cases listed in Table D.1-1 are specified in TS 36.133 Rel-P [3].

Table D.1-1: RRM requirements for performance enhancements for high speed scenario

| Clause | Description |
| --- | --- |
| 4.2 | Cell Re-selection |
| 8.1.2.2 | E-UTRAN intra frequency measurements in RRC connected state |

# D.2	Common UE demodulation requirements for performance enhancements for high speed scenario

The requirements and test cases listed in Table D.2-1 are specified in TS 36.101 Rel-P [2].

Table D.2-1: UE Demodulation requirements for performance enhancements for high speed scenario

| Clause | Description |
| --- | --- |
| 8.2.1.9 | FDD PDSCH |
| 8.2.2.9 | TDD PDSCH |

Annex E (normative):
Common Requirements for 8Rx

# E.1 Common UE RF requirements

The requirements and test cases listed in Table E.1-1 are specified in TS 36.101 Rel-P [2].

Table E.1-1: RF requirements for 8Rx

| Clause | Description |
| --- | --- |
| 7.3 | Reference sensitivity power level |

The requirements and test cases listed in Table E.1-2 are specified in TS 36.101 Rel-P [2].

Table E.1-2: RF requirements for 8Rx for CA

| Clause | Description |
| --- | --- |
| 7.3.1A | Reference sensitivity for CA |

# E.2	Common UE demodulation and CSI requirements

The requirements and test cases listed in Table E.2-1 and Table E.2-2 are specified in TS 36.101 Rel-P [2].

Table E.2-1: UE Demodulation and CSI requirements for 8Rx for single band

| Clause | Description |
| --- | --- |
| 8.14.1 | PDSCH |
| 9.12 | CSI reporting for 8Rx UE |

Table E.2-2: UE Demodulation and CSI requirements for 8Rx CA/DC

| Clause | Description |
| --- | --- |
| 8.14.2 | Demodulation of PDSCH CA |
| 8.7.17 | SDR of TDD CA (8 layer MIMO) |

Annex F (normative):
Common requirements for NB-IoT or eMTC operation over NTN

# F.1	Common UE RF requirements

The requirements and test cases listed in Table F.1-1 are specified in TS 36.102 Rel-P [6].

Table F.1-1: RF requirements for NB-IoT operation over NTN

| Clause | Description |
| --- | --- |
| 5.2B | Operating bands |
| 5.3B | Channel bandwidth |
| 5.4B | Channel arrangement |
| 6.1 | General transmitter characteristics |
| 6.2B | Transmit power |
| 6.3B | Output power dynamics |
| 6.4B | Transmit signal quality |
| 6.5B | Output RF spectrum emissions |
| 6.6B | Transmit intermodulation |
| 7.1 | General receiver characteristics |
| 7.2 | Diversity characteristics |
| 7.3B | Reference sensitivity power level |
| 7.4B | Maximum input level |
| 7.5B | Adjacent Channel Selectivity (ACS) |
| 7.6B | Blocking characteristics |
| 7.7B | Spurious response |
| 7.8B | Intermodulation characteristics |
| 7.9 | RX spurious emissions |

The requirements and test cases listed in Table F.1-2 are specified in TS 36.102 Rel-P [6].

Table F.1-2: RF requirements for eMTC operation over NTN

| Clause | Description |
| --- | --- |
| 5.2A | Operating bands |
| 5.3A | Channel bandwidth |
| 5.4A | Channel arrangement |
| 6.1 | General transmitter characteristics |
| 6.2A | Transmit power |
| 6.3A | Output power dynamics |
| 6.4A | Transmit signal quality |
| 6.5A | Output RF spectrum emissions |
| 6.6A | Transmit intermodulation |
| 7.1 | General receiver characteristics |
| 7.2 | Diversity characteristics |
| 7.3A | Reference sensitivity power level |
| 7.4A | Maximum input level |
| 7.5A | Adjacent Channel Selectivity (ACS) |
| 7.6A | Blocking characteristics |
| 7.7A | Spurious response |
| 7.8A | Intermodulation characteristics |
| 7.9 | RX spurious emissions |

# F.2	Common RRM requirements

The requirements and test cases listed in Table F.2-1 are specified in 36.133 Rel-P [3]. Note the requirements apply to serving cell measurements and GEO intra-frequency measurements when no satellite assistance information is provided to the UE.

T Table F.2-1: RRM requirements for NB-IoT operation over NTN

| Clause | Description |
| --- | --- |
| 4.6A1,2 | Cell Selection and Re-selection Requirements for UE category NB-IoT for Satellite Access |
| 6.5A2 | RRC Re-establishment for NB-IoT UEs for Satellite Access |
| 6.6A | Random Access for UE category NB1 for Satellite Access |
| 6.9A2 | RRC Connection Redirection to Non-anchor Carrier in NB-IoT for Satellite Access |
| 7.20A | UE transmit timing for NB-IoT for Satellite Access |
| 7.21A | UE timer accuracy for NB-IoT for Satellite Access |
| 7.22A | Timing Advance for NB-IoT for Satellite Access |
| 7.23A | Radio Link Monitoring for Category NB-IoT UE for Satellite Access |
| 8.14A1 | Measurements for UE category NB-IoT for Satellite Access |
| NOTE 1: If no satellite assistance information is provided for neighbor cells, the requirements in this clause for the serving cell measurement are also applicable. NOTE 2: If no satellite assistance information is provided for neighbor cells, the intra-frequency requirements in this clause are also applicable for GEO operations. |  |

The requirements and test cases listed in Table F.2-2 are specified in 36.133 Rel-P [3]. Note the requirements apply to serving cell measurements and GEO intra-frequency measurements when no satellite assistance information is provided to the UE.

Table F.2-2: RRM requirements for eMTC operation over NTN

| Clause | Description |
| --- | --- |
| 4.7A1,2 | Cell Selection and Re-selection Requirements for UE category M1 |
| 5.5A | E-UTRAN Handover for Cat-M1 UEs for Satellite Access |
| 6.2.3A | Random Access Requirements for Cat-M1 UEs for Satellite Access |
| 6.7A2 | RRC Re-establishment for Cat-M1 UEs for Satellite Access |
| 6.8A2 | RRC Connection Release with Redirection for UE Category M1 for Satellite Access |
| 7.19A | Radio Link Monitoring for UE Category M1 for Satellite Access |
| 7.24A | UE transmit timing for Category M1 for Satellite Access |
| 7.27A | UE timer accuracy for category M1 for Satellite Access |
| 7.28A | Timing Advance for Category M1 for Satellite Access |
| 8.13A1,2 | Measurements for UE Category M1 for Satellite Access |
| NOTE 1: If no satellite assistance information is provided for neighbor cells, the requirements in this clause for the serving cell measurement are also applicable. NOTE 2: If no satellite assistance information is provided for neighbor cells, the intra-frequency requirements in this clause are also applicable for GEO operations. |  |

# F.3	Common UE Demodulation requirements

The requirements and test cases listed in Table F.3-1 are specified in TS 36.102 Rel-P [6].

Table F.3-1: Demodulation requirements for NB-IoT operation over NTN

| Clause | Description |
| --- | --- |
| 8.3 | Demodulation performance requirements for UE category NB1 and NB2 |

The requirements and test cases listed in Table F.3-2 are specified in TS 36.102 Rel-P [6].

Table F.3-2: Demodulation requirements for eMTC operation over NTN

| Clause | Description |
| --- | --- |
| 8.2 | Demodulation performance requirements for UE category M1 |

************************* End of changes ******************************************