3GPP TSG-RAN WG4 Meeting#110      	R4-2400618

Athens, 26th Feb – 1st March, 2024

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-1 | CR | <CR#> | rev | - | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | draftCR for 38.101-1 addition of PC1 operation for n41 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, T-Mobile USA |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_NR_HPUE_FWVM_R18-Core |  |  |  |  |  | Date: |  |  | 2024-19-02 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Addition of PC1 operation for n41 |  |  |  |  |  |  |  |  |
| Summary of change: |  | PC1 information added for n41 and n41 A-MPR updated |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | PC1 operation is not possible for n41 |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.1, 6.2.32 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

***************************** Start of changes ************************************

### 6.2.1	UE maximum output power

The following UE Power Classes define the maximum output power for any transmission bandwidth within the channel bandwidth of NR carrier unless otherwise stated. The period of measurement shall be at least one sub frame (1ms).

Table 6.2.1-1: UE Power Class

| NR band | Class 1 (dBm) | Tolerance (dB) | Class 1.5 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n1 |  |  |  |  | 26 | +2/-3 | 23 | ±2 |
| n2 |  |  |  |  |  |  | 23 | ±23 |
| n3 |  |  |  |  | 26 | +2/-33 | 23 | ±23 |
| n5 |  |  |  |  |  |  | 23 | ±2 |
| n7 |  |  |  |  |  |  | 23 | ±23 |
| n8 |  |  |  |  | 26 | +2/-33 | 23 | ±23 |
| n12 |  |  |  |  |  |  | 23 | ±23 |
| n13 |  |  |  |  |  |  | 23 | ±2 |
| n14 | 316 | +2/-3 |  |  |  |  | 23 | ±2 |
| n18 |  |  |  |  |  |  | 23 | ±2 |
| n20 |  |  |  |  |  |  | 23 | ±23 |
| n24 |  |  |  |  |  |  | 23 | +2/-33 |
| n25 |  |  |  |  | 26 | +2/-33 | 23 | ±23 |
| n26 |  |  |  |  |  |  | 23 | ±23 |
| n28 |  |  |  |  |  |  | 23 | +2/-2.5 |
| n30 |  |  |  |  |  |  | 23 | ±2 |
| n31 | 316 | +2/-3 |  |  |  |  | 23 | ±2 |
| n34 |  |  | 295 | +2/-3 | 26 | +2/-3 | 23 | ±2 |
| n38 |  |  |  |  |  |  | 23 | ±2 |
| n39 |  |  |  |  | 26 | +2/-3 | 23 | ±2 |
| n40 |  |  | 295 | +2/-3 | 26 | +2/-3 | 23 | ±2 |
| n41 | 316 | +2/-3 | 295 | +2/-33 | 26 | +2/-33 | 23 | ±23 |
| n47 |  |  |  |  |  |  | 23 | ±2 |
| n48 |  |  |  |  |  |  | 23 | +2/-3 |
| n50 |  |  |  |  |  |  | 23 | ±2 |
| n51 |  |  |  |  |  |  | 23 | ±2 |
| n53 |  |  |  |  |  |  | 23 | ±2 |
| n54 |  |  |  |  |  |  | 23 | ±2 |
| n65 |  |  |  |  |  |  | 23 | ±2 |
| n66 |  |  |  |  | 26 | +2/-3 | 23 | ±2 |
| n70 |  |  |  |  | 26 | +2/-3 | 23 | ±2 |
| n71 | 316 | +2/-3 |  |  | 26 | +2/-3 | 23 | +2/-2.5 |
| n72 | 316 | +2/-3 |  |  |  |  | 23 | ±2 |
| n74 |  |  |  |  |  |  | 23 | ±2 |
| n77 | 316 | +2/-3 | 295 | +2/-3 | 26 | +2/-3 | 23 | +2/-3 |
| n78 |  |  | 295 | +2/-3 | 26 | +2/-3 | 23 | +2/-3 |
| n79 |  |  | 295 | +2/-3 | 26 | +2/-3 | 23 | +2/-3 |
| n80 |  |  |  |  | 26 | +2/-33 | 23 | ±23 |
| n81 |  |  |  |  |  |  | 23 | ±2 |
| n82 |  |  |  |  |  |  | 23 | ±2 |
| n83 |  |  |  |  |  |  | 23 | +2/-2.5 |
| n84 |  |  |  |  | 26 | +2/-3 | 23 | ±2 |
| n85 | 316 | +2/-3 |  |  |  |  | 23 | ±23 |
| n86 |  |  |  |  |  |  | 23 | ±2 |
| n89 |  |  |  |  |  |  | 23 | ±2 |
| n91 |  |  |  |  |  |  | 23 | ±23, 4 |
| n92 |  |  |  |  |  |  | 23 | ±23, 4 |
| n93 |  |  |  |  |  |  | 23 | ±23, 4 |
| n94 |  |  |  |  |  |  | 23 | ±23, 4 |
| n95 |  |  |  |  | 26 | +2/-3 | 23 | ±2 |
| n97 |  |  |  |  | 26 | +2/-3 | 23 | ±2 |
| n98 |  |  |  |  | 26 | +2/-3 | 23 | ±2 |
| n99 |  |  |  |  |  |  | 23 | +2/-33 |
| n100 | 316 | +2/-3 |  |  |  |  | 23 | ±2 |
| n101 | 316 | +2/-3 |  |  |  |  | 23 | ±2 |
| n104 |  |  |  |  | 26 | +2/-3 | 23 | +2/-3 |
| n105 |  |  |  |  |  |  | 23 | +2/-2.5 |
| n106 |  |  |  |  |  |  | 23 | ±2 |
| n109 |  |  |  |  |  |  | 23 | ±23, 4 |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance NOTE 2: Power class 3 is default power class unless otherwise stated NOTE 3: Refers to the transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB. NOTE 4: The maximum output power requirement is relaxed by reducing the lower tolerance limit by 0.3 dB NOTE 5: Achieved via dual Tx NOTE 6: Generally, PC1 UE is not targeted for smartphone form factor. The UE power class 1 requirements for Band n14 are applicable for public safety scenario only. |  |  |  |  |  |  |  |  |

If a UE supports a different power class than the default UE power class for the band and the supported power class enables the higher maximum output power than that of the default power class:

-	if the field of UE capability maxUplinkDutyCycle-PC2-FR1 is absent and the field of UE capability maxUplinkDutyCycle-PC1dot5-MPE-FR1 is absent and the percentage of uplink symbols transmitted in a certain evaluation period is larger than 50% (The exact evaluation period is no less than one radio frame); or

-	if the field of UE capability maxUplinkDutyCycle-PC2-FR1 is not absent and the percentage of uplink symbols transmitted in a certain evaluation period is larger than maxUplinkDutyCycle-PC2-FR1 as defined in TS 38.306 (The exact evaluation period is no less than one radio frame); or

-	if the field of UE capability maxUplinkDutyCycle-PC1dot5-MPE-FR1 is not absent and half the percentage of uplink symbols transmitted in a certain evaluation period is larger than maxUplinkDutyCycle-PC1dot5-MPE-FR1 as defined in TS 38.306 (The exact evaluation period is no less than one radio frame); or

-	if the IE P-Max as defined in TS 38.331 [7] is provided and set to the maximum output power of the default power class or lower;

-	shall apply all requirements for the default power class to the supported power class and set the configured transmitted power as specified in clause 6.2.4;

-	else if the UE does not support a power class with higher maximum output power than PC2; or

-	if the field of UE capability maxUplinkDutyCycle-PC2-FR1 is absent and the field of UE capability maxUplinkDutyCycle-PC1dot5-MPE-FR1 is absent and the percentage of uplink symbols transmitted in a certain evaluation period is larger than 25% (The exact evaluation period is no less than one radio frame); or

-	if the field of UE capability maxUplinkDutyCycle-PC2-FR1 is not absent and the percentage of uplink symbols transmitted in a certain evaluation period is larger than 0.5*maxUplinkDutyCycle-PC2-FR1 (The exact evaluation period is no less than one radio frame); or

-	if the field of UE capability maxUplinkDutyCycle-PC1dot5-MPE-FR1 is not absent and the percentage of uplink symbols transmitted in a certain evaluation period is larger than maxUplinkDutyCycle-PC1dot5-MPE-FR1 as defined in TS 38.306 (The exact evaluation period is no less than one radio frame); or

-	if the IE P-Max as defined in TS 38.331 [7] is provided and set to the maximum output power of the power class 2 or lower;

-	shall apply all requirements for power class 2 to the supported power class and set the configured transmitted power as specified in clause 6.2.4;

-	else shall apply all requirements for the supported power class and set the configured transmitted power as specified in clause 6.2.4.

***************************** No changes ************************************

#### 6.2.3.2	A-MPR for NS_04

For NS_04, A-MPR is not added to MPR. Also, when NS_04 is signalled, MPR shall be set to zero in the PCMAX equations to avoid double counting MPR.

For power Class 1.5, 2 and 3, allowed maximum power reduction is defined as A-MPR = max(MPR, A-MPR'),

Note that A-MPR' = 0 dB means only MPR is applied,

where A-MPR' is defined as

if RBstart ≤ fstart,max,IMD3 / (12SCS) and LCRB ≤ AWmax,IMD3 / (12SCS) and FC - BWChannel/2 < FUL_low + offsetIMD3,
then

the A-MPR' is defined according to Table 6.2.3.2-2 PC3_A2 relative to 23 dBm for power class 3,  PC2_A4 relative to 26 dBm for power class 2, and PC1.5_A6 relative to 29 dBm for power class 1.5,

else,

if RBstart ≤ LCRB/2 + start / (12SCS) and LCRB ≤ AWmax,regrowth / (12SCS) and FC - BWChannel/2 < FUL_low + offsetregrowth,
then

the A-MPR' is defined according to Table 6.2.3.2-2 PC3_A1 relative to 23 dBm for power class 3,  PC2_A3 relative to 26 dBm for power class 2, , and PC1.5_A5 relative to 29 dBm for power class 1.5,

else

A-MPR' = 0 dB and apply MPR.

With the parameters defined in Table 6.2.3.2-1.

Table 6.2.3.2-1: Parameters for region edges and frequency offsets (Power Class 1.5, 2 and 3)

| Parameter | Symbol | Value |  | Related condition |
| --- | --- | --- | --- | --- |
|  |  | CP-OFDM | DFT-s-OFDM |  |
| Max allocation start in IMD3 region | fstart,max,IMD3 | 0.33 BWChannel |  | RBstart ≤ fstart,max,IMD3 / (12SCS) |
| Max allocation BW in IMD3 region | AWmax,IMD3 | 4 MHz |  | LCRB ≤ AWmax,IMD3 / (12SCS) |
| Freq. offset required to avoid A-MPR in IMD3 region | offsetIMD3 | BWChannel – 6 MHz |  | FC - BWChannel/2 ≥ FUL_low + offsetIMD3 |
| Right edge of regrowth region | start | 0.08 BWChannel |  | RBstart ≤ LCRB/2 + start / (12SCS) |
| Max allocation BW in regrowth region | AWmax,regrowth | 100 MHz |  | LCRB ≤ Min(LCRB,Max, AWmax,regrowth / (12SCS)) |
| Freq. offset required to avoid A-MPR in regrowth region | offsetregrowth | Max (10 MHz, 0.25* BWChannel MHz) | Max (10 MHz, 0.45* BWChannel MHz) | FC - BWChannel/2 ≥ FUL_low + offsetregrowth |

Table 6.2.3.2-2: A-MPR' values Access (Power Class1.5, 2 and 3)

| Modulation/Waveform |  | A-MPR' (dB) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | PC3_A1 | PC3_A2 | PC2_A3 | PC2_A4 | PC1.5_A51 | PC1.5_A61 |
| DFT-s-OFDM | Pi/2-BPSK | ≤ 3.5 | ≤ 3.5 | ≤ 3.5 | ≤ 5.5 | ≤ 5 | ≤ 7 |
|  | QPSK | ≤ 4 | ≤ 4 | ≤ 4.5 | ≤ 6 | ≤ 6 | ≤ 7.5 |
|  | 16 QAM | ≤ 4 | ≤ 4 | ≤ 5 | ≤ 6 | ≤ 6.5 | ≤ 7.5 |
|  | 64 QAM | ≤ 4 | ≤ 4.5 | ≤ 5 | ≤ 6.5 | ≤ 6.5 | ≤ 8 |
|  | 256 QAM | ≤ 4.5 | ≤ 6 | ≤ 6.5 | ≤ 8 | ≤ 8 | ≤ 9.5 |
| CP-OFDM | QPSK | ≤ 5.5 | ≤ 5.5 | ≤ 6.5 | ≤ 7.5 | ≤ 8 | ≤ 9 |
|  | 16 QAM | ≤ 5.5 | ≤ 5.5 | ≤ 6.5 | ≤ 7.5 | ≤ 8 | ≤ 9 |
|  | 64 QAM | ≤ 5.5 | ≤ 5.5 | ≤ 6.5 | ≤ 7.5 | ≤ 8 | ≤ 9 |
|  | 256 QAM | ≤ 6.5 | ≤ 8 | ≤ 7.5 | ≤ 10 | ≤ 9 | ≤ 11.5 |
| NOTE 1: PC1.5 assumes dual Tx. |  |  |  |  |  |  |  |

For Power Class 1, NS_04 A-MPR is defined as 
A-MPR = max(MPR, A-MPRregrowth, A-MPRIMD3, A-MPRCIM3, A-MPRedge).

A-MPRregrowth is obtained from Table 6.2.3.2-3 in terms of total guard bandwidth (TGBW). The TGBW is defined as the frequency distance between the RB allocation and the additional spurious emission limit defined in Table 6.5.3.3.1-1, i.e.,

TGBW=falloc,low-2496 MHz,

where

falloc,low=FC-BWChannel2+BWGB+RBstart∙12 SCS

is the lower edge frequency of the RB allocation, FC is the channel centre frequency, BWChannel is the channel bandwidth, and BWGB is the minimum guard bandwidth defined in Table 5.3.3-1.

Table 6.2.3.2-3: A-MPRregrowth for NS_04 (Power Class 1)

| TGBW range | A-MPRregrowth *(dB) |
| --- | --- |
| G0dB(BWalloc) TGBW | 0 |
| G1dB(BWalloc) TGBW < G0dB(BWalloc) | 1 |
| G2dB(BWalloc) TGBW < G1dB(BWalloc) | 2 |
| G3dB(BWalloc) TGBW < G2dB(BWalloc) | 3 |
| G4dB(BWalloc) TGBW < G3dB(BWalloc) | 4 |
| G5dB(BWalloc) TGBW < G4dB(BWalloc) | 5 |
| TGBW < G5dB(BWalloc) | 6 |

Each function GA(BWalloc) defines the required minimum total guard bandwidth for A-MPR value A and is defined as

GA(BWalloc)=max⁡(0,  C2BWalloc100MHz2+C1BWalloc100MHz+C0),

where BWalloc=LCRB∙12 SCS is the allocation bandwidth, and C2, C1, and C0 are obtained from Table 6.2.3.2-4 for each combination of waveform, modulation, and back-off value A.

Table 6.2.3.2-4: Polynomial coefficients for determining the required 
total guard bandwidth for each value of A-MPRregrowth (Power Class 1)

| Waveform | Modulation | Back-off value A [dB] / Polynomial coefficients C2, C1, C0 [MHz] |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 0 | 1 | 2 | 3 | 4 | 5 | 6 |
| DFT-s-OFDM | Pi/2-BPSK | -3, 40, 10 | -91, 98, 0 | -248, 143, -9 | -2148, 604, -40 | 0,0,0 | N/A | N/A |
|  | QPSK | -27, 71, 1 | -49, 73, 0 | -76, 76, -3 | -108, 62, -3 | 0,0,0 | N/A | N/A |
|  | 16-QAM | -34, 81, -1 (NOTE 1) | -57, 92, -5 | -69, 82, -5 | -119, 79, -5 | 0,0,0 | N/A | N/A |
|  | 64-QAM | -23, 74, -1 (NOTE 1) | -61, 91, -4 (NOTE 1) | -84, 92, -5 | -116, 80, -5 | 0,0,0 | N/A | N/A |
|  | 256-QAM | 0, 0, 0 (NOTE 2) | N/A | N/A | N/A | N/A | N/A | N/A |
| CP-OFDM | QPSK | -32, 81, -2 (NOTE 1) | -36, 77, -3 | -40, 70, -3 | -52, 67, -3 | -67, 60, -3 | -101, 56, -4 | 0,0,0 |
|  | 16-QAM | -31, 80, -2 (NOTE 1) | -32, 72, -2 (NOTE 1) | -36, 66, -2 | -48, 62, -2 | -66, 58, -2 | -108, 60, -4 | 0,0,0 |
|  | 64-QAM | -28, 78, -2 (NOTE 1) | -33, 73, -2 (NOTE 1) | -36, 66, -2 (NOTE 1) | -48, 62, -2 | -65, 58, -2 | -124, 67, -5 | 0,0,0 |
|  | 256-QAM | 0, 0, 0 (NOTE 2) | N/A | N/A | N/A | N/A | N/A | N/A |
| NOTE 1: Back-off value A is less than floor(MPR), thus this polynomial is irrelevant. NOTE 2: For 256-QAM, always A-MPRregrowth = 0. |  |  |  |  |  |  |  |  |

For both OFDM and DFT-S-OFDM, A-MPRIMD3 = max0,  min3,  4-2.77 BWalloc1 MHz dB 
if  3falloc,low-2FC  2490.5 MHz; otherwise, A-MPRIMD3 = 0 dB.

For OFDM, A-MPRCIM3 = max0,  min11,  12-2 BWalloc1 MHz dB if  4 FC-3 falloc,high   2490.5 MHz; 
otherwise, A-MPRCIM3 = 0.

For DFT-S-OFDM, A-MPRCIM3 = 3 dB if BWalloc 1.08 MHz and 4 FC-3 falloc,high   2490.5 MHz; 
otherwise, A-MPRCIM3 = 0.

Here, falloc,high=falloc,low+BWalloc is the upper edge frequency of the RB allocation.

For both OFDM and DFT-S-OFDM, if RBstart=0, LCRB=1,  and FC-BWChannel2-2496 MHz<360 kHz, A-MPRedge is defined in Table 6.2.3.2-5. Otherwise, A-MPRedge = 0 dB.

Table 6.2.3.2-5: A-MPRedge for NS_04  (Power Class 1)

| Waveform | BWChannel [MHz] | BWalloc [MHz] | A-MPR [dB] |
| --- | --- | --- | --- |
| DFT-s-OFDM | < 40 | N/A | 0 |
|  | 40…90 | -9.8CBW100MHz2+24.1CBW100MHz-6.3 | 3.5 |
|  | 100 |  | 11 |
| OFDM | < 90 | N/A | 0 |
|  | 90 | 6.5 | 3.5 |
|  | 100 | 8 | 11 |

***************************** End of changes ************************************