3GPP TSG-RAN WG4 Meeting #110	R4-2400694

Athens, Greece, 26th February – 1st March 2024

Source:	AT&T, FirstNet, Apple

Title:	TP for TR 36.770 A-MPR requirements

Agenda item:			9.3.2.1

Document for:	Approval

# 1. Introduction

A text proposal for TP for TR 36.770 to add content to the A-MPR requirements clause based on the analysis in [1]. It is also proposed to rename the main clause to “A-MPR” since there is an introduction of a sub-clause using “A-MPR requirements”.

# 2. Text Proposal

---Start of changes---

## 6.2	A-MPR

### 6.2.1	NS_06

The introduction of PC2 to band 14 requires A-MPR evaluation for NS_06.

#### 6.2.1.1	A-MPR simulation results from Apple

Simulations have been conducted for all channel bandwidth from 1.4MHz to 10MHz. To allow easy comparison the power back-off is provided for PC3 and PC2 and the scaling is the same for all plots.

The following assumptions and requirements are used for the simulations:

-	Power Class 3 & 2

-	Fixed Bias

-	LTE Waveform

-	Calibration: 1dB MPR: DFT-s-OFDM QPSK 20MHz, 100RB

-	Carrier Leakage: 25dBc

-	Image: 25dBc

-	CIM3: 60dBc

The NS_06 Additional SEM is specified in clause 6.6.2.2.3 of 3GPP 36.101 [XX]. Simulation results can be found in Table 6.2.1.1-1. Typically, the plots are quantised to 0.5dB steps. As this resolution is too coarse for a range of 0 to 1.5dB the granularity is improved by using 0.25dB steps.

Table 6.2.1.1-1: Power back-off for QPSK and NS_06

| CBW MHz | PC3 | PC2 |
| --- | --- | --- |
| 1.4 |  |  |
| 3 |  |  |
| 5 |  |  |
| 10 |  |  |

By comparing the power back-off between PC3 and PC2, it can be observed that there is roughly a 0.5dB difference between PC3 and PC2. Higher order modulations such as 64QAM and 256QAM are EVM limited and the back-off is not dominated by the requirements of NS_06. Therefore, it is proposed to introduce a 0.5dB relaxation for QPSK and 16QAM.

#### 6.2.1.2	A-MPR requirements

Based on the simulation results from previous section, the PC2 A-MPR requirements for NS_06 are proposed below in Table 6.2.1.2-1.

Table 6.2.1.2-1: Additional Maximum Power Reduction (A-MPR) for NS_06 for Power Class 2

| Channel bandwidth [MHz] | Parameters | Region A | Region B | Region C |
| --- | --- | --- | --- | --- |
| 1.4 | RBstart [RB] | ≥ 0 | ≥ 0 | ≥ 4 |
|  | LCRB [RB] | > 2 | ≤ 2 | ≤ 2 |
|  | A-MPR [dB] | 0.5 | 0.5 | 0.5 |
| 3 | RBstart [RB] | ≥ 0 | ≥ 0 | ≥ 13 |
|  | LCRB [RB] | > 4 | ≤ 2 | ≤ 2 |
|  | A-MPR [dB] | 0.5 | 0.5 | 0.5 |
| 5 | RBstart [RB] | ≥ 0 | ≥ 0 | ≥ 23 |
|  | LCRB [RB] | > 6 | ≤ 2 | ≤ 2 |
|  | A-MPR [dB] | 0.5 | 0.5 | 0.5 |
| 10 | RBstart [RB] | ≥ 0 | ≥ 0 | ≥ 48 |
|  | LCRB [RB] | > 10 | ≤ 2 | ≤ 2 |
|  | A-MPR [dB] | 0.5 | 0.5 | 0.5 |

Region A re-uses slightly extended MPR regions and allows for 0.5dB additional back-off. When comparing the limits from NS_06 to standard SEM, it can be found that the requirements are tighter inside the very first MHz outside the channel. Edge allocations are very sensitive to the deployed channel filter and WOLA setup. Therefore, it is proposed to introduce a mild relaxation of 0.5dB for those allocations. The approach is similar to Edge RB allocations in NR. Relaxation for the RBs directly bordering the channel edge is captured with regions B and C.

---End of changes---

# References

[1] R4-2400171: On A-MPR for FDD single band with PC2; Apple