3GPP TSG-RAN WG4 Meeting #110	R4-2400699

Athens, Greece, 26th February – 1st March 2024

Source:	AT&T, FirstNet

Title:	TP for TR 38.896 to add PC2 for n14

Agenda item:			7.20.2

Document for:	Approval

# 1. Introduction

A text proposal for TR 38.896 to add PC2 for n14 including RSD requirements based on the WF in [1]. The A-MPR text in 5.x.2 is proposed to “Re-use PC3 A-MPR.”. In the WF in [1], it was proposed to further check whether 0.5dB relaxation for outer allocations can be justified for PC2 for FDD HPUE NS_06 based on the A-MPR discussion paper in [2]. This TP will be updated with any outcome on A-MPR.

# 2. Text Proposal

---Start of changes---

## 5.x	NR band n14

### 5.x.1	UE maximum output power

Table 5.x.1-1: UE output power for PC2

| NR band | Class 2 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| n14 | 26 | +2/-33 |
| NOTE 3: Refers to the transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB. |  |  |

### 5.x.2	A-MPR requirements

Re-use PC3 A-MPR.

### 5.x.3	Reference sensitivity requirements

For PC2 single band n14, both 1Tx and 2Tx requirements are considered. Therefore, Reference Sensitivity Degradation requirements for not supporting Tx diversity (i.e., 1Tx) and for supporting Tx diversity (i.e., 2Tx) are needed to be defined.

For PC2 single band n14 not supporting Tx diversity (i.e., 1Tx), several companies shared the analysis, and we summarize the required sensitivity degradation levels from companies as shown in 5.x.3-1:

Table 5.x.3-1: Reference Sensitivity Degradation from PC3 to PC2 for FDD bands for UE not supporting Tx Diversity

| Operating Band | Source | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 30 MHz (dB) | 35 MHz (dB) | 40 MHz (dB) | 45 MHz (dB) | 50 MHz (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n14 | Skyworks (R4-2311152) | 0.4 | 0.8 |  |  |  |  |  |  |  |  |
|  | Apple (R4-2311251) | 0.3 | 0.6 |  |  |  |  |  |  |  |  |
|  | Huawei, HiSilicon(R4- 2316472) | 0.9 | 1.0 |  |  |  |  |  |  |  |  |
|  | Murata (R4-2318253) | 0.7 | 0.8 |  |  |  |  |  |  |  |  |
| Average |  | 0.6 | 0.8 |  |  |  |  |  |  |  |  |

For PC2 single band n14 supporting Tx diversity (i.e., 2Tx), several companies shared the analysis, and we summarize the required sensitivity degradation levels from companies as shown in 5.x.3-2:

Table 5.x.3-2: Reference Sensitivity Degradation from PC3 to PC2 for FDD bands for UE supporting Tx Diversity

| Operating Band | Source | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 30 MHz (dB) | 35 MHz (dB) | 40 MHz (dB) | 45 MHz (dB) | 50 MHz (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n14 | Skyworks (R4-2311152) | 0.8 | 1.3 |  |  |  |  |  |  |  |  |
|  | Apple (R4-2311251) | 0.8 | 0.8 |  |  |  |  |  |  |  |  |
|  | Huawei, HiSilicon(R4- 2316472) | 2.0 | 2.0 |  |  |  |  |  |  |  |  |
|  | Murata (R4-2318253) | 0.7 | 1.0 |  |  |  |  |  |  |  |  |
| Average |  | 1.1 | 1.3 |  |  |  |  |  |  |  |  |

---End of changes---

# References

[1] R4-2321715: WF on HPUE for FDD bands; China Unicom

[2] R4-2318749: A-MPR discussion for FDD HPUE NS_06; Apple