3GPP TSG-RAN WG4 Meeting #110	R4-2400881

Athens, Greece, Feb 26– Mar 1, 2024

Source:	Nokia, Nokia Shanghai Bell

Title:	On Advanced Receivers - Test parameters - Simulations

Agenda item:	8.13.2.2

Document for:	Discussion

Introduction

This document details Nokia’s current simulation results for alignment on the WID for advanced receivers for MU-MIMO scenario. Updated agreements were made in RAN4#109-bis [1] for facilitating companies to provide simulation results.

Simulation assumptions

Table 1 provides an overview of the already agreed simulation assumptions taken from the WF [1] and proposed test parameters:

Table 1: Agreed simulation assumptions for Phase I

| Reference receiver for phase II simulation | R-ML |
| --- | --- |
| Rank allocation for the target and co-scheduled UE, with 1 co-scheduled UE | 2Tx2Rx UE: 1+1 2Tx4Rx UE: 1+1 4Tx4Rx UE: 2+2 |
| DMRS port configurations for the target and co-scheduled UEs | - Use different CDM groups for: - rank 2 (DMRS port 0, 1) + 2 (DMRS port 2, 3) - Use the same CDM group for - rank 1 (DMRS port 0) + 1 (DMR port 1) |
| Antenna configuration | For rank 1+1: 2T2R, 2T4R For rank 2+2: 4Tx 4Rx |
| Channel model | Use TDLC300-100, TDLA30-10 when the rank of the target UE is 1 Use TDLA30-10, XP medium when the rank of the target UE is 2 |
| Antenna correlation | Rank 1+1: ULA medium and ULA low (for 2T2R,2T4R) Rank 2+2: ULA Low, XP medium |
| Precoder selection target and co-scheduled UEs | Single panel Type 1 For rank 1+1: Random and Orthogonal PMI selection For rank 2+2: Orthogonal PMI selection |
| QCL assumptions | Assume all scheduled DMRS ports have same QCL assumptions |
| Evaluation metric | Reuse the Rel-17 MMSE-IRC phase I evaluation assumptions captured in TR38.833 as a start point. |
| The number of co-scheduled UEs | Only consider 1 co-scheduled UE |
| DMRS sequence for the co-scheduled UE | Assume the scrambling ID for DMRS sequence is the same for the target UE the co-scheduled UE(s) |
| PDSCH resource allocation for the target UE | Maximum transmission bandwidth configuration.10 MHz 15KHz SCS Full OFDM symbol allocation. |
| TDRA of co-scheduled UE | Assume TDRA of target and co-scheduled UE is same |
| FDRA of co-scheduled UE | Full transmission bandwidth configuration., Partial transmission bandwidth configuration |

Simulation results

Simulations requested for RAN4#110

Based on the information in Table 1, we provide the simulation results on the cases of interest for this meeting, listed in Table 2 for cases without MO BD and for cases with MO BD in Table 3.

Table 2 Simulation results for cases with 1 Co-UE, Full FDRA bandwidth and without MO BD, DCI index 1 to 5 indicated.

| Alignment Case Number | Duplex Mode and SCS | CHBW | Rank for target + Co-UE | MIMO | Precoder selection | Channel Model | Antenna correlation | MCS for the target UE (MCS Table 1) | Modulation order for the co-scheduled UE | Nokia |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  |  | R-ML | IRC/E-IRC |
| 1 | FDD 15kHz SCS | 10MHz | 1+1 | 2T2R | Random | TDLC300-100 | ULA medium | MCS 13 | QPSK | 15.1 | 22.7 |
| 2 |  |  |  |  |  |  | ULA Low |  |  | 12.4 | 15.8 |
| 3 |  |  |  | 2T4R |  | TDLA30-10 | ULA Low |  |  | 6.9 | 8.9 |
| 4 |  |  |  |  |  | TDLC300-100 | ULA medium |  |  | 12.7 | 21.1 |
| 5 (optional) |  |  |  | 2T2R | Orthogonal | TDLC300-100 | ULA medium |  |  | 14.4 | 19.9 |
| 6 (optional) |  |  |  | 2T4R |  |  |  |  |  | 12.1 | 18.7 |
| 7 |  |  | 2+2 | 4T4R |  | TDLA30-10 | ULA Low |  |  | 10.7 | 13.9 |
| 8 |  |  |  |  |  |  | XP medium |  |  | 13.7 | 20.8 |
| 9 |  |  |  |  |  |  | ULA low | MCS 17 | 16QAM | 16.4 | 18 |
| 10 |  |  |  |  |  |  | XP medium |  |  | 21.1 | 25.4 |
|  |  |  | 1+1 | 2T2R | Random | TDLC300-100 | ULA medium | MCS17 | 16QAM | 22.9 | NA1 |
|  |  |  |  |  |  |  | ULA low |  |  | 18 | 19.9 |
|  |  |  |  | 2T4R |  | TDLA30-10 | ULA low |  |  | 11.3 | 12 |
|  |  |  |  |  |  | TDLC300-100 | ULA medium |  |  | 18.8 | 22.6 |
|  |  |  |  | 2T2R | Orthogonal | TDLC300-100 | ULA medium |  |  | 21.2 | 27.3 |
|  |  |  |  | 2T4R |  |  |  |  |  | 18.8 | 22.6 |
| NA Not achievable |  |  |  |  |  |  |  |  |  |  |  |

Table 3 Simulation results for the cases with 1 Co-UE, Full FDRA bandwidth and with MO BD, DCI index 6 indicated.

| Alignment Case Number | Duplex Mode and SCS | CHBW | Rank for target + Co-UE | MIMO | Precoder selection | Channel Model | Antenna correlation | MCS for the target UE (MCS Table 1) | Modulation order for the co-scheduled UE | Nokia |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  |  | R-ML | IRC/E-IRC |
| 21 | FDD 15kHz SCS | 10MHz | 1+1 | 2T2R | Random | TDLC300-100 | ULA medium | MCS 13 | QPSK | 15.2 | 22.7 |
| 22 |  |  |  |  |  |  | ULA Low |  |  | 12.4 | 15.8 |
| 23 |  |  |  | 2T4R |  | TDLA30-10 | ULA Low |  |  | 6.9 | 8.9 |
| 24 |  |  |  |  |  | TDLC300-100 | ULA medium |  |  | 12.8 | 21.1 |
| 25 |  |  |  | 2T2R |  | TDLC300-100 | ULA medium | MCS 17 | 16QAM | 23.6 | NA |
| 26 |  |  |  |  |  |  | ULA Low |  |  | 18.2 | 19.9 |
| 27 |  |  |  | 2T4R |  | TDLA30-10 | ULA Low |  |  | 11.4 | 12 |
| 28 |  |  |  |  |  | TDLC300-100 | ULA medium |  |  | 19.5 | 22.6 |
| 29 (Optional) |  |  |  | 2T2R | Orthogonal | TDLC300-100 | ULA medium | MCS 13 | QPSK | 14.5 | 19.9 |
| 30 (Optional) |  |  |  | 2T4R |  |  |  |  |  | 12.1 | 18.7 |
| 31 (Optional) |  |  |  | 2T2R |  |  |  | MCS 17 | 16QAM | 21.8 | 27.3 |
| 32 (Optional) |  |  |  | 2T4R |  |  |  |  |  | 19.5 | 22.6 |
| 33 |  |  | 2+2 | 4T4R |  | TDLA30-10 | ULA Low | MCS 13 | QPSK | TBA | 13.9 |
| 34 |  |  |  |  |  |  | XP medium |  |  | TBA | 20.8 |
| 35 |  |  |  |  |  |  | ULA Low | MCS 17 | 16QAM | TBA | 18 |
| 36 |  |  |  |  |  |  | XP medium |  |  | TBA | 25.4 |

Study cases with Partial FDRA allocation

Table 4 Simulation Overview – FDD study cases with blind detection of parameters for R-ML and E-IRC receiver, 1 co-UE with Partial FDRA allocation

| Co-UE FDRA | Rank of target and co- UE | MCS target UE | Modulation order co- UE | MIMO | Antenna correlation | Channel model | Precoder Co- UE | R-ML BD1 | R-ML Full BD2 | E-IRC BD1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Partial (0 to 25 PRB) | 1+1 | 13 | QPSK | 2T2R | ULA low | TDLC300-100 | random | 10.9 | 11 | 12 |  |
|  |  | 17 | 16QAM | 2T4R | ULA medium |  |  | 14.3 | 15.2 | 16.1 |  |
|  | 2+2 | 17 | 16QAM | 4T4R | XP medium | TDLA30-10 | orthogonal | 18.6 | TBA | 18.8 |  |

From the partial FDRA simulation results it is seen that significant gain (1dB) of R-ML receiver over E-IRC receiver is achievable for the cases shown in Table 4. Also comparing 2T4R case 28 from Table 3 and the corresponding partial FDRA case from Table 4 it is seen that partial FDRA case lowers the required SNR for R-ML receiver while still giving a performance gain over E-IRC receiver.

Partial FDRA interference cases show good gain of R-ML receiver over E-IRC (1dB or larger) receiver and provide testable SNR for UEs scheduled with high modulation order.

Conclusion

In this contribution we have provided Nokia's simulation results for Advanced receivers. This includes the cases of interest for this meeting as well as additional study cases with partial FDRA allocation. We have the following observations.

- Partial FDRA interference cases show good gain of R-ML receiver over E-IRC (1dB or larger) receiver and provide testable SNR for UEs scheduled with high modulation order.

- References

- R4-2321114 WF on advanced receiver for MU-MIMO scenario