3GPP TSG-RAN WG4 Meeting #110	R4-2400883

Athens, Greece, Feb 26 – Mar 1, 2024

Source:	Nokia, Nokia Shanghai Bell

Title:	On Lessthan5MHz UE demod perf and CSI requirements - Simulations

Agenda item:	8.10.6.1

Document for:	Discussion

Introduction

This contribution contains Nokia’s simulation results for Lessthan5MHz topic.

Simulation results

PDSCH

The following section contains Nokia’s simulation results for PDSCH.

PDSCH Non-HST conditions

Table 1 Simulation results for non-HST conditions with 15kHz SCS

| Reference channel | Modulation format and code rate | Propagation condition | Rank | Correlation matrix and antenna configuration | Reference value |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) 10MHz BW 52 PRB | SNR (dB) 3MHz BW 15 PRB |
| R.PDSCH.1-1.1 FDD | QPSK, 0.30 | TDLB100-400 | Rank 1 | 2x2, ULA Low | 70 | -3.0 | -2.3 |
|  |  |  |  | 2x4, ULA Low | 70 | -6.2 | -5.7 |
| R.PDSCH.1-2.1 FDD | 16QAM, 0.48 | TDLC300-100 | Rank 1 | 2x2, ULA Low | 70 | 5.5 | 5.0 |
|  |  |  |  | 2x4, ULA Low | 70 | 1.6 | 1.3 |

PDSCH HST Single-tap

Table 2 Simulation results for HST Single-tap conditions

| Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| R.PDSCH.1-8.2 FDD | 3 / 15 | 64QAM, 0.43 | [HST-417] | 1x2 | 70 | 6.5 |
| R.PDSCH.1-8.2 FDD | 3 / 15 | 64QAM, 0.43 | [HST-417] | 1x4 | 70 | 3.5 |

PDSCH HST-DPS

Table 3 Simulation results for HST Single Tap conditions

| Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | Propagation condition | Number of active PDSCH TCI states | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| R.PDSCH.1-8.4 FDD | 3 / 15 | 64QAM, 0.43 | [HST-DPS-417] | 1 | 2x2 | 70 | 6.1 |
| R.PDSCH.1-8.4 FDD | 3 / 15 | 64QAM, 0.43 | [HST-DPS-417] | 1 | 2x4 | 70 | 3.3 |

PDCCH

The following section contains Nokia’s simulation results for PDCCH.

Punctured PDCCH CORESET#0

Configuration used for the following results: (15PRBs, 3 symbols, non-interleaved, AL4, DCI 1_0 (35 bits for 15 PRBs); Use CCEs #4, #5, #6, and #7 to transmit PDCCH with DCI 1_0.)

Table 4 PDCCH Simulation results for CORESET#0 PDCCH (Punctured)

| Duplex | Bandwidth (MHz) | Coreset RB | Coreset duration | AL | PropCond | Ant Cfg | Interleaved | pm-dsg(%) | SNR(dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| FDD | 3 | 15 | 3 | 4 | TDLA30-10 | 1x2 Low | no | 1 | 3.3 |
| FDD | 3 | 15 | 3 | 4 | TDLC300-100 | 1x2 Low | no | 1 | 0.4 |
| FDD | 3 | 15 | 3 | 4 | TDLC300-100 | 2x2 Low | no | 1 | -0.9 |
| FDD | 3 | 15 | 3 | 4 | TDLA30-10 | 1x4 Low | no | 1 | -3.3 |
| FDD | 3 | 15 | 3 | 4 | TDLC300-100 | 2x4 Low | no | 1 | -5.2 |
| FDD | 3 | 15 | 3 | 4 | TDLA30-10 | 2x4 Low | no | 1 | -4.8 |
| FDD | 3 | 15 | 3 | 4 | [HST-DPS-417] | 1x2 Low | no | 1 | -5.8 |
| FDD | 3 | 15 | 3 | 4 | [HST-DPS-417] | 2x2 Low | no | 1 | -6.1 |
| FDD | 3 | 15 | 3 | 4 | [HST-DPS-417] | 1x4 Low | no | 1 | -8.4 |
| FDD | 3 | 15 | 3 | 4 | [HST-DPS-417] | 2x4 Low | no | 1 | -8.6 |

Non-punctured PDCCH

Table 5 PDCCH Simulation results for non-HST non-punctured

| Duplex | Bandwidth (MHz) | Coreset RB | Coreset duration | AL | PropCond | Ant Cfg | Interleaved | pm-dsg(%) | SNR(dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| FDD | 3 | 15 | 2 | 2 | TDLC300-100 | 1x2 Low | no | 1 | 4 |
| FDD | 3 | 15 | 2 | 2 | TDLC300-100 | 2x2 Low | no | 1 | 2.6 |
| FDD | 3 | 15 | 2 | 2 | TDLA30-10 | 1x4 Low | no | 1 | -0.5 |
| FDD | 3 | 15 | 2 | 2 | TDLA30-10 | 2x4 Low | no | 1 | -1.7 |
| FDD | 10 | 15 | 2 | 2 | TDLC300-100 | 1x2 Low | no | 1 | 6.2 |
| FDD | 10 | 15 | 2 | 2 | TDLA30-10 | 1x4 Low | no | 1 | -0.8 |

PDCCH HST Conditions non-punctured

Table 6 PDCCH Simulation results for HST non-punctured

| Duplex | Bandwidth (MHz) | Coreset RB | Coreset duration | AL | PropCond | Ant Cfg | Interleaved | pm-dsg(%) | SNR(dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| FDD | 3 | 15 | 2 | 2 | [HST-417] | 1x2 Low | no | 1 | -3.1 |
| FDD | 3 | 15 | 2 | 2 | [HST-417] | 2x2 Low | no | 1 | -3.1 |
| FDD | 3 | 15 | 2 | 2 | [HST-417] | 1x4 Low | no | 1 | -5.8 |
| FDD | 3 | 15 | 2 | 2 | [HST-417] | 2x4 Low | no | 1 | -5.7 |
| FDD | 3 | 15 | 2 | 2 | [HST-DPS-417] | 1x2 Low | no | 1 | -3.2 |
| FDD | 3 | 15 | 2 | 2 | [HST-DPS-417] | 2x2 Low | no | 1 | -3.1 |
| FDD | 3 | 15 | 2 | 2 | [HST-DPS-417] | 1x4 Low | no | 1 | -5.8 |
| FDD | 3 | 15 | 2 | 2 | [HST-DPS-417] | 2x4 Low | no | 1 | -5.7 |

PBCH

The simulation results below follow the previous RAN4 agreement:

- Define requirements for 12 PRB PBCH with 3 MHz CBW, 15kHz SCS, FDD, unknown SSB/PBCH index

Non-HST conditions

Table 7 PBCH simulation results for non-HST conditions

| Duplex | Bandwidth (MHz) / Subcarrier spacing (kHz) | SSB/PBCH index | Propagation condition | Antenna configuration and correlation matrix | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Pm-bch (%) | SNR (dB) |
| FDD | 3 / 15 | Unknown | TDLC300-100 | 1x2 Low | 1 | -3.6 |
| FDD | 3 / 15 | Unknown | TDLC300-100 | 1x4 low | 1 | -7.4 |

HST Single-tap

Table 8 PBCH simulation results for HST Single-tap

| Duplex | Bandwidth (MHz) / Subcarrier spacing (kHz) | SSB/PBCH index | Propagation condition | Antenna configuration and correlation matrix | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Pm-bch (%) | SNR (dB) |
| FDD | 3 / 15 | Unknown | [HST-417] | 1Tx/2Rx Low | 1 | -6.4 |
| FDD | 3 / 15 | Unknown | [HST-417] | 1Tx/4Rx Low | 1 | -8.9 |

HST-DPS

Table 9 PBCH simulation results for HST-DPS

| Duplex | Bandwidth (MHz) / Subcarrier spacing (kHz) | SSB/PBCH index | Propagation condition | Antenna configuration and correlation matrix | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Pm-bch (%) | SNR (dB) |
| FDD | 3 / 15 | Unknown | [HST-DPS-417] | 1Tx/2Rx Low | 1 | -6.5 |
| FDD | 3 / 15 | Unknown | [HST-DPS-417] | 1Tx/4Rx Low | 1 | -9 |

Conclusion

We have presented Nokia's simulation results for Lessthan5MHz UE demod performance and CSI requirements.

- References

- R4-2321064 - WF on [109][322] NR_FR1_lessthan_5MHz_BW_demod