3GPP TSG-RAN4 Meeting #110	R4-2400928

# Athens, Greece, 26th Feb 2024 – 1st Mar 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR |  | rev |  | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | DraftCR on Improvement for SCell/SCG setup delay |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_Mob_enh2-Core |  |  |  |  |  | Date: |  |  | 2024-02-12 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The feature is about what measurement results can be reported for UE supporting [existing measurement-based solution]. - There is no new measurement requirement. - UE measurement behavior is not changed. Since Clause 4.7 and 5.8 about reporting requirement, subclause for measurement requirement is not appropriate. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Delete measurement requirement subclause under 4.7 and 5.8 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Requirement is not defined properly. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.7.1, 4.7.2, 4.7.3. 5.8.1, 5.8.2, 5.8.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## << Start of 1st change >>

## 4.7	Measurement report for fast CA/DC setup

### 4.7.1	Introduction

A UE supporting [solution based on existing measurement] is able to report the idle mode measurement results on the inter-frequency CA and DC candidate frequencies/cells indicated by higher layers and meet the requirement specified in this clause. The UE is able to report idle mode measurements provided that the serving cell support early measurement and is within the validity area. The idle mode measurement requirements apply to a configured carrier frequency and the serving cell are among the supported band combination of the UE.

### 4.7.2	Measurement Report Requirements

For a UE which supports [solution based on existing measurement] the UE is able to report valid measurement results upon RRC setup complete. The measurement results are considered valid if the following conditions are met:

-	the measurements are performed within the last [X] seconds before msg1 transmission for RRC resume/setup request, where [X] is configured by [TBD], and

-	the measurement results satisfy measurement accuracy requirement at the measurement instance.

Otherwise, the measurement results are considered invalid. The UE shall not report invalid measurement results. If network doesn’t provide configuration of [X], UE is not required to perform validity check but the measurement results satisfy measurement accuracy requirement at the measurement instance.

Editor’s note, the section may be further updated, if needed

## << End of 1st change >>

## << Start of 2nd change >>

## 5.8	Measurement report for fast CA/DC setup

### 5.8.1	Introduction

A UE supporting [solution based on existing measurement] is able to report the inactive mode measurement on the inter-frequency CA and DC candidate frequencies/cells indicated by higher layers and meet the requirement specified in this clause. The UE is able to report inactive mode measurements provided that the serving cell support early measurement and is within the validity area. The inactive mode measurement requirements apply to a configured carrier frequency and the serving cell are among the supported band combination of the UE.

### 5.8.2	Measurement Report Requirements

The requirements in clause 4.7.2 shall apply.

## << End of 2nd change >>