3GPP TSG-RAN4 Meeting #110	R4-2400948

# Athens, Greece,  Feb. 26 - March 1, 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR |  | rev | - | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR for measurement accuracy for SL-U |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | LG Electronics |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SL_enh2-Perf |  |  |  |  |  | Date: |  |  | 2024-02-16 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | CR to introduce requirements for measurement accuracy for SL-U. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add measurement accuracy requirements such as PSBCH-RSRP, L1 SL-RSRP, and SL-RSSI for SL-U and condition for the requirements based on the measurement accuracy requirements for SL |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The performance requirements of measurement accuracy for SL-U operation are not completed. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | New 10.4.2A, 10.4.3A, 10.4.4A, B.4.5, B.4.6, B.4.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.533 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

-------------- Start of Change <1> --------------

10.4	V2X measurements

10.4.1	Introduction

The requirements in this section are applicable for a UE capable of V2X sidelink communication.

The accuracy requirements in this clause are:

-	applicable for AWGN radio propagation conditions,

-	assume independent interference (noise) at each receiver antenna port.

----- Ommit unchanged part -----

10.4.2A	Intra-frequency PSBCH-RSRP accuracy requirements for FR1 under CCA

10.4.2A.1	PSBCH-RSRP Absolute Accuracy

The requirements for absolute accuracy of PSBCH-RSRP in this clause apply to a sidelink synchronization source on the same frequency as that of the own sidelink UE performing the measurement in FR1 under CCA.

The accuracy requirements in Table 10.4.2A.1-1 are valid under the following conditions:

-	Demodulation reference signals are transmitted from one port.

-	Conditions defined in Clause 7.3E of TS38.101-1 [18] for reference sensitivity are fulfilled.

-	Conditions for PSBCH-RSRP measurements are fulfilled according to Annex B.4.2 for a corresponding Band for each relevant PSBCH-DMRS.

Table 10.4.2A.1-1: Intra-frequency PSBCH-RSRP absolute accuracy in FR1

| Accuracy |  | Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | Ês/Iot Note 3 | Io Note 1 range |  |  |  |  |
|  |  |  |  |  |  | NR sidelink operating band groups Note 2 |  | Minimum Io |  |  | Maximum Io |
| dB |  | dB |  | dB |  |  |  | dBm / SCSSL |  | dBm/BWChannel | dBm/BWChannel |
|  |  |  |  |  |  |  |  | SCSSL = 15 kHz | SCSSL = 30 kHz |  |  |
| 4.5 |  | 9 |  | -6 |  | NR_CCA_FR1_I |  | -117 | -114 | N/A | -70 |
|  |  |  |  |  |  | NR_CCA_FR1_J |  | -116.5 | -113.5 | N/A | -70 |
| 8 |  | 11 |  | -6 |  | NR_CCA_FR1_I, NR_CCA_FR1_J |  | N/A | N/A | -70 | -50 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: NR sidelink operating band groups in FR1 are as defined in clause 3.5.2. NOTE 3: Ês/Iot for a SyncRef UE is the Ês/Iot of PSBCH-DMRS. |  |  |  |  |  |  |  |

10.4.2A.2	PSBCH-RSRP Relative Accuracy

The relative accuracy of PSBCH-RSRP is defined as the PSBCH-RSRP measured from one sidelink synchronization source compared to the PSBCH-RSRP measured from another sidelink synchronization source on the same frequency in FR1 under CCA.

The accuracy requirements in Table 10.4.2A.2-1 are valid under the following conditions:

-	Demodulation reference signals are transmitted from one port.

-	Conditions defined in Clause 7.3E of TS38.101-1 [18] for reference sensitivity are fulfilled.

-	Conditions for PSBCH-RSRP accuracy measurements are fulfilled according to Annex B.4.2 for a corresponding Band for each relevant PSBCH-DMRS.

-	The same number of S-SSB repetitions on frequency domain is configured between two sidelink synchronization sources.

Table 10.4.2A.2-1: Intra-frequency PSBCH-RSRP relative accuracy in FR1

| Accuracy |  | Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | Ês/Iot Note 3 | Io Note 1 range |  |  |  |  |
|  |  |  |  |  |  | NR sidelink operating band groups Note 2 |  | Minimum Io |  |  | Maximum Io |
| dB |  | dB |  | dB |  |  |  | dBm / SCSSL |  | dBm/BWChannel | dBm/BWChannel |
|  |  |  |  |  |  |  |  | SCSSL = 15 kHz | SCSSL = 30 kHz |  |  |
| 2 |  | 3 |  | -3 |  | NR_CCA_FR1_I |  | -117 | -114 | N/A | -50 |
|  |  |  |  |  |  | NR_CCA_FR1_J |  | -116.5 | -113.5 | N/A | -50 |
| 3 |  | 3 |  | -6 |  | Note 4 |  | Note 4 | Note 4 | N/A | Note 4 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: NR sidelink operating band groups in FR1 are as defined in clause 3.5.2. NOTE 3: Ês/Iot for a SyncRef UE is the Ês/Iot of PSBCH-DMRS. NOTE 4: The same bands and the same Io conditions for each band apply for this requirement as for the corresponding highest accuracy requirement. |  |  |  |  |  |  |  |

10.4.3A	Intra-Frequency SL-RSSI Measurement Accuracy Requirements for FR1 under CCA

10.4.3A.1	Absolute SL-RSSI Accuracy

The intra-frequency SL-RSSI requirements are specified in Table 10.4.3A.1-1 under CCA. The requirements apply for measurement period of 1slot and for any configured measurement bandwidth larger than 10 RBs, provided that:

-	All symbols duing each RSSI measurement duration according to indication of first or second starting symbol within a slot are available for RSSI sampling within the same measurement interval.

Table 10.4.3A.1-1: Intra-frequency SL-RSSI absolute accuracy

| Accuracy |  | Conditions |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | Io Note 1 range |  |  |  |
|  |  |  |  | NR sidelink operating band groups Note 2 |  | Minimum Io |  |  | Maximum Io |
| dB |  | dB |  |  |  | dBm/SCSSL |  |  | dBm/BWChannel |
|  |  |  |  |  |  |  | SCSSL = 15kHz | SCSSL = 30kHz |  |
| 2.5 |  | 5.5 |  | NR_CCA_FR1_I |  |  | -117 | -114 | -50 |
|  |  |  |  | NR_CCA_FR1_J |  |  | -116.5 | -113.5 | -50 |
| 4.5 |  | 7.5 |  | Note 3 |  |  | Note 3 | Note 3 | Note 3 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: NR sidelink operating band groups are as defined in Section 3.5 for the corresponding NR operating bands. NOTE 3: The same bands and the same Io conditions for each band apply for this requirement as for the corresponding highest accuracy requirement. |  |  |  |  |  |

10.4.4A	Intra-Frequency L1 SL-RSRP Measurement Accuracy Requirements for FR1 under CCA

10.4.4A.1	Absolute L1 SL-RSRP Accuracy

The requirements for absolute accuracy of L1 SL-RSRP in this clause apply to a UE performing PSCCH-RSRP and/or PSSCH-RSRP measurements on the same frequency as used by operating sidelink communication under CCA.

The accuracy requirements in Table 10.4.4A.1-1 are valid under the following conditions:

-	Demodulation reference signals for PSCCH and/or PSSCH are transmitted from one port.

-	Conditions defined in clause 7.3E of TS38.101-1 [18] for reference sensitivity are fulfilled.

-	PSCCH-RSRP|dBm and/or PSSCH-RSRP|dBm according to Annex B.4.4 for a corresponding Band are fulfilled.

Table 10.4.4A.1-1: Intra-frequency L1 SL-RSRP absolute accuracy for UE capable of sidelink communication

| Accuracy |  | Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | Ês/Iot Note 3 | Io Note 1 range |  |  |  |  |
|  |  |  |  |  |  | NR sidelink operating band groups Note 2 |  | Minimum Io |  |  | Maximum Io |
| dB |  | dB |  | dB |  |  |  | dBm/SCS |  | dBm/BWChannel | dBm/BWChannel |
|  |  |  |  |  |  |  |  | SCS = 15kHz | SCS = 30kHz |  |  |
| 4.5 |  | 9 |  | 0 dB |  | NR_CCA_FR1_I |  | -117 | -114 | N/A | -70 |
|  |  |  |  |  |  | NR_CCA_FR1_J |  | -116.5 | -113.5 | N/A | -70 |
| 8 |  | 11 |  | 0 dB |  | NR_CCA_FR1_I NR_CCA_FR1_J |  | N/A | N/A | -70 | -50 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: NR sidelink operating band groups are as defined in Section 3.5 for the corresponding NR operating bands. NOTE 3: The parameter Ês/Iot is the Ês/Iot of PSCCH-DMRS and/or PSSCH-DMRS. |  |  |  |  |  |  |  |

-------------- End of Change <1> --------------

-------------- Start of Change <2> --------------

B.4.5	Conditions for PSBCH-RSRP Accuracy Requirements under CCA

This clause defines the following conditions for PSBCH-RSRP measurement accuracy requirements applicable for a corresponding operating band under CCA.

The conditions are defined in Table B.4.5-1 for FR1.

Table B.4.5-1: Conditions for PSBCH-RSRP measurements in FR1

| Parameter | NR sidelink operating band groups Note1 | Minimum S-SSB_RP |  | S-SSB Ês/Iot |
| --- | --- | --- | --- | --- |
|  |  | dBm/SCSS-SSB |  | dB |
|  |  |  |  | SCSS-SSB = 15kHz |  | SCSS-SSB = 30kHz |  |  |
|  |  | NR_CCA_FR1_I |  | -123 |  | -120 | -6 |  |
|  |  | NR_CCA_FR1_J |  | -122.5 |  | -119.5 |  |  |
| NOTE 1: NR sidelink operating band groups are as defined in Section 3.5 for the corresponding NR operating bands. |  |  |  |  |

B.4.6	Conditions for Selection/Reselection to Intra-frequency SyncRef UE under CCA

This clause defines the S-SSB_RP and S-SSB Ês/Iot applicable for a corresponding operating band under CCA.

The conditions for selection/reselection to intra-frequency SyncRef UE are defined in Table B.4.6-1 for FR1.

Table B.4.3-1: Sidelink synchronization measurements in FR1

| Parameter | NR sidelink operating band groups Note1 | Minimum S-SSB_RP |  | S-SSB Ês/Iot |
| --- | --- | --- | --- | --- |
|  |  | dBm/SCSS-SSB |  | dB |
|  |  |  |  | SCSS-SSB = 15kHz |  | SCSS-SSB = 30kHz |  |  |
|  |  | NR_CCA_FR1_I |  | -117 |  | -114 | 0 |  |
|  |  | NR_CCA_FR1_J |  | -116.5 |  | -113.5 | 0 |  |
| NOTE 1: NR sidelink operating band groups are as defined in Section 3.5 for the corresponding NR operating bands. NOTE 2: The SyncRef UE transmission frequency shall be accurate to within ±5 PPM compared to the absolute frequency. |  |  |  |  |

B.4.7	Conditions for L1 SL-RSRP Accuracy Requirements under CCA

This clause defines the following condtions for L1 SL-RSRP measurement accuracy requirements applicable for a corresponding operating band under CCA.

The conditions are defined in Table B.4.7-1 for FR1.

Table B.4.4-1: Conditions for L1 SL-RSRP measurements in FR1

| Parameter | NR sidelink operating band groups Note1 | Minimum L1 SL-RSRP |  | Ês/Iot |
| --- | --- | --- | --- | --- |
|  |  | dBm/SCS |  | dB |
|  |  |  |  | SCS= 15kHz |  | SCS= 30kHz |  |  |
|  |  | NR_CCA_FR1_I |  | -117 |  | -114 | 0 |  |
|  |  | NR_CCA_FR1_J |  | -116.5 |  | -113.5 |  |  |
| NOTE 1: NR sidelink operating band groups are as defined in Section 3.5 for the corresponding NR operating bands. NOTE 2: The parameter Ês/Iot is the Ês/Iot of PSCCH-DMRS and/or PSSCH-DMRS. NOTE 3: The SCS is for PSCCH and/or PSSCH. |  |  |  |  |

-------------- End of Change <2> --------------