# 3GPP TSG-RAN WG4 Meeting # 110	R4-2400964

# Athens, Greece, Feb 26 – Mar 1, 2024

Agenda Item:	8.17.1.2

Source: 	Ericsson

Title:	Remaining issues on collision between MUSIM

Document for:	Discussion

# Introduction

In last meetings, RAN4 made great progress on MUSIM WI[1]. Most of open issues were agreed[2]. In this contribution, we will discuss some remaining issues for MUSIM gaps.

# MUSIM colliding with HO/SCell activation

In RAN4 #107 and #108b meeting, RAN4 made the agreements that UE shall still meet handover and SCell activation, but these agreements haven’t captured in the spec. We think these agreements shall be captured in the spec. as applicability rules and as a compromise, no test case will be defined to verify these cases.

RAN4 #107 Agreements - When MUSIM gaps are configured, UE is still required to meet Scell activation RRM requirements for NW-A. FFS whether to capture this conclusion in the specifications. - No test case will be defined to verify this case RAN4 #108b Agreements - When MUSIM gaps are configured, UE is still required to meet handover RRM requirements for NW-A. FFS whether to capture this conclusion in the specifications. - No test case will be defined to verify this case

Proposal 1: RAN4 to capture the following agreements in the spec. applicability section.

- When MUSIM gaps are configured, UE is still required to meet Scell activation RRM requirements for NW-A.

- When MUSIM gaps are configured, UE is still required to meet handover RRM requirements for NW-A.

# NW-A’s requirement

One of the remaining issues in NW-A’s requirement is when UE performs a measurement without gap which is partially overlapping with the gap but fully overlapping with the union of the NW-A’s gap and MUSIM gaps. We suggest to follow the conclusion in Rel-17 concurrent gaps(agreed in last meeting R4-2321625). For example, in the figure below, UE had to perform the measurement within the MG1. The cell identification requirement will be max(600ms, 5* MGRP)* CSSFwithin_gap,i.

Figure 1. SMTCs are fully overlapping with the union of the gaps

Proposal 2: When UE performs a measurement without gap which is partially overlapping with the MG but fully overlapping with the union of the NW-A’s gap and MUSIM gaps, UE shall perform the measurement within MG. The requirement will be

Table 1: Time period for PSS/SSS detection, (Frequency range FR1)

| DRX cycle | TPSS/SSS_sync_intra |
| --- | --- |
| No DRX | max( 600ms, 5 x Kgap x max(MGRP, SMTC period)) x CSSFwithin_gap,i |
| DRX cycle≤ 320ms | max( 600ms, ceil(M2 x 5 x Kgap) x max(MGRP, SMTC period, DRX cycle)) x CSSFwithin_gap,i |
| DRX cycle>320ms | ceil(5 x Kgap) x DRX cycle x CSSFwithin_gap,i |

- Summary

In this contribution, we have discussed the MUSIM gaps requirements. Based on the discussions, we have made following proposals and observations:

Proposal 1: RAN4 to capture the following agreements in the spec. applicability section.

- When MUSIM gaps are configured, UE is still required to meet Scell activation RRM requirements for NW-A.

- When MUSIM gaps are configured, UE is still required to meet handover RRM requirements for NW-A.

Proposal 2: When UE performs a measurement without gap which is partially overlapping with the MG but fully overlapping with the union of the NW-A’s gap and MUSIM gaps, UE shall perform the measurement within MG. The requirement will be

Table 1: Time period for PSS/SSS detection, (Frequency range FR1)

| DRX cycle | TPSS/SSS_sync_intra |
| --- | --- |
| No DRX | max( 600ms, 5 x Kgap x max(MGRP, SMTC period)) x CSSFwithin_gap,i |
| DRX cycle≤ 320ms | max( 600ms, ceil(M2 x 5 x Kgap) x max(MGRP, SMTC period, DRX cycle)) x CSSFwithin_gap,i |
| DRX cycle>320ms | ceil(5 x Kgap) x DRX cycle x CSSFwithin_gap,i |

- References

- RP-220955, “Revised WID: Dual Transmission/Reception (Tx/Rx) Multi-SIM for NR”, RAN 95e

- R4-2321613, “WF on RRM requirements for Rel-17 MUSIM gaps”, Vivo

- R4-2321625, “CR on ConMGs”, Ericsson, ZTE, MediaTek, Intel