3GPP TSG-RAN WG4 # 110	R4-2401160

Athens, Greece, February 26 – March 1, 2024

Agenda item:	8.3.4.3

Source:	MediaTek Inc.

Title:	Simulation results of PMI requirements of FR2 Multi-Rx DL Demod

Document for:	Discussion

# Introduction

In RAN4#109, the Way Forward document was approved [1]. We provide some simulation results of PMI reporting requirements for simulation alignment for the Rel-18 work in this contribution.

# Simulation results

In this chapter we provide some simulation result of agreed PMI reporting requirement test for final result alignment.

We have simulated Single-DCI SDM transmission scheme by using separate processing receiver assumption as agreed.

Channel model is TDLA30-35 for 100 MHz/120 kHz with antenna correlation of XP medium. The correlation matrix model follows agreement from the previous meeting [1]. MCS13 is simulated as agreed. TX EVM is 6% as agreed. For phase noise modelling we are using Example 1 model from TR38.803 [2] assuming carrier frequency parameters for 29.55 GHz. Time and frequency offsets between TRPs are set to zero. For PT-RS configuration we have used single port PT-RS configuration because it seems to be feasible.

The simulated cross-talk power ratios ρ is -12dB as agreed. Here, ρ in [dB] is defined as 10·log10(ρ). In these simulations we have not assumed any additional scaling to TRPs as agreed in the previous meetings.

In Figure 1 is agreed test configuration using Multi-TRP Single-DCI SDM scheme with MCS13 Rank 2 using separate processing receiver assumption. Corresponding γ value is shown in Table 1.

Figure 1: Multi-TRP Single-DCI SDM MCS13 Rank 2, separate processing receiver assumption, ρ=-12dB

|  | ρ=-12dB |
| --- | --- |
| MCS13 / separate processing / 90% tput | 1.29 |

Table 1: Simulation result collection of Single-DCI SDM scenario with MCS13
showing (throughput ratio) γ in SNR of follow PMI in 90% of maximum throughput.

From simulation result in Table 1, we can do following observation:

Observation #1: In our simulation γ value without margins is 1.29.

# Conclusion

In this paper we provided simulation results on the PMI reporting requirements of FR2 multipanel RX downlink for performance alignment. The following observation is made:

Observation #1: γ value without margins is 1.29.

# Reference

- R4-2321140, “WF on UE demodulation and CSI requirements for FR2 multi-Rx DL reception”, Qualcomm

- 3GPP TR 38.803 V14.3.0 (2022-03), “Study on new radio access technology: Radio Frequency (RF) and co-existence aspects”