3GPP TSG-RAN WG4 Meeting #110	R4-2401225

Athens, Greece, Feb 26 – Mar 1, 2024

Agenda item:	8.14.3.3

Source: 	Qualcomm Incorporated

Title: 	On performance requirements for RedCap positioning

Document for:	Discussion

# Introduction

RAN4 started discussing performance requirements for Rel-18 RedCap positioning in RAN4#109. Agreements and open issues were captured in a WF [1].

In this brief paper we provide our views and proposals on measurement accuracy requirements.

# Discussion

## Side conditions for measurements without frequency hopping

RAN4 continued discussing the PRS Es/Iot side-conditions for 1Rx measurement accuracy requirements. Several proposals were captured in [2].

Issue 4-1-2: PRS measurement accuracy for 1Rx RedCap UE without FH

- Proposals

  - Option 1: HW

    - For RedCap with 1RX without FH, new accuracy requirements are defined based on the agreed Es/Iot side condition and related simulation results.

  - Option 2: E///, QC, CATT

    - Option 2A: E///, CATT

      - RSTD accuracy requirement for 1Rx RedCap UE under fading propagation condition is defined for SINR values (-6, -10, -10) dB.

      - UE Rx-Tx accuracy requirement for 1Rx RedCap UE under fading propagation condition is defined for SINR values (-3, -10, -10) dB.

      - PRS-RSRP accuracy requirement for 1Rx RedCap UE is defined for SINR values (-3, -10, -10) dB.

      - PRS-RSRPP accuracy requirement for 1Rx RedCap UE under two path propagation condition is defined for SINR values (-3, -10, -10) dB.

    - Option 2B: QC

      - For RSTD accuracy requirements for 1Rx RedCap UE under fading propagation condition, the reference and target SINR side-conditions are SINRref = -6 dB, SINRtarget = -10 dB.

      - For UE Rx-Tx accuracy requirements for 1Rx RedCap UE under fading propagation condition, the SINR side-conditions are (-3, -10) dB.

      - RAN4 to define a single set of PRS-RSRP accuracy requirements for 1Rx RedCap UEs under both AWGN and fading conditions, at SINR side-conditions (-3, -10) dB.

      - RAN4 to define PRS-RSRPP accuracy requirements for 1Rx RedCap UEs under the 2-tap channel model used in Rel-17, at SINR side-conditions (-3, -10) dB.

It should be clarified in the above agreement that the proposed side-conditions would apply for Nsamples = 4. For RSTD and UE Rx-Tx, RAN4 defined separate accuracy requirements for AWGN and fading conditions. In our view, the proposed side-conditions above can be confirmed for RSTD and UE Rx-Tx measurements with 1Rx and Nsamples = 4.

Proposal 1:

- For RSTD accuracy requirements for 1Rx RedCap UE under fading propagation condition with Nsamples = 4, the reference and target SINR side-conditions are SINRref = -6 dB, SINRtarget = -10 dB.

- For UE Rx-Tx accuracy requirements for 1Rx RedCap UE under fading propagation condition with Nsamples = 4, the SINR side-conditions are (-3, -10) dB.

For PRS-RSRP a single set of accuracy requirements and side-conditions were specified for AWGN and fading propagation conditions for 2Rx UEs in Rel-16. In our view, it would be reasonable to follow the same approach for 1Rx RedCap UEs.

Proposal 2: RAN4 to define a single set of PRS-RSRP accuracy requirements for 1Rx RedCap UEs under both AWGN and fading conditions with Nsamples = 4, at SINR side-conditions (-3, -10) dB.

For PRS-RSRPP, accuracy requirements were defined for the 2-tap channel model in Rel-17, and the SINR side-conditions for RSRP were reused. In our view, it would be reasonable to follow the same approach for 1Rx RedCap UEs.

Proposal 3: RAN4 to define PRS-RSRPP accuracy requirements for 1Rx RedCap UEs under the 2-tap channel model used in Rel-17 with Nsamples = 4, at SINR side-conditions (-3, -10) dB.

# Conclusions

Proposal 1:

- For RSTD accuracy requirements for 1Rx RedCap UE under fading propagation condition with Nsamples = 4, the reference and target SINR side-conditions are SINRref = -6 dB, SINRtarget = -10 dB.

- For UE Rx-Tx accuracy requirements for 1Rx RedCap UE under fading propagation condition with Nsamples = 4, the SINR side-conditions are (-3, -10) dB.

Proposal 2: RAN4 to define a single set of PRS-RSRP accuracy requirements for 1Rx RedCap UEs under both AWGN and fading conditions with Nsamples = 4, at SINR side-conditions (-3, -10) dB.

Proposal 3: RAN4 to define PRS-RSRPP accuracy requirements for 1Rx RedCap UEs under the 2-tap channel model used in Rel-17 with Nsamples = 4, at SINR side-conditions (-3, -10) dB.

# References

[1] R4-2321543, WF on R18 NR positioning – RedCap positioning and PRS/SRS BW aggregation, RAN4#109

[2] R4-2318175, Topic summary for [109][219] NR_pos_enh2_part1, RAN4#109