3GPP TSG-RAN WG4 Meeting #110	R4-2401229

Athens, Greece, Feb 26 – Mar 1, 2024

Agenda item:	8.14.2.6

Source: 	Qualcomm Incorporated

Title: 	On RRM core maintenance for carrier phase positioning

Document for:	Discussion

# Introduction

RAN4 completed RRM core requirements for carrier phase positioning (CPP) in RAN4#109. New agreements and open issues were captured in a WF [1].

In this paper we provide our views and proposals on core maintenance issues for CPP.

# Discussion

## Measurement period

RAN4 reached further agreements for the measurement period requirement for DL RSCP/RSCPD [1].

#### Issue 2-1-1: PRS measurement period requirements for DL RSCP/DL RSCPD:

Agreements:

- When LMF requests the UE to perform measurements on indicated DL PRS resource set(s) occurring within indicated time window(s) for a PFL, and UE supports FG 41-2-3, the Twindow needs to be considered in the measurement period:

  - For periodic time window,

    - Adopt the following updates to the existing measurement period requirements for the PFL

      - Tavailable is defined as LCM(Tprs, MGRP, Twindow), where Twindow is the maximum periodicity of the indicated time window(s)

      - When calculating Lprs and Tprs, only the PRS resources in the indicated resources sets and overlapped with both the MG and the indicated time window(s) are considered.

  - For one-shot time window case,

    - Lavailable_PRS,i only counts PRS resource instances within the indicated time window(s).

    - the start of the measurement period is the start of the time window.

#### ue 2-1-4: Requirements applicability regarding MG reconfiguration

Agreements:

- If during the measurement period, the MG pattern is reconfigured or time window for carrier phase measurement is reconfigured, the measurement period for RSCPD with RSTD and RSCP with UE Rx-Tx can be longer.

#### Issue 2-1-5: Impact of UE mobility

Agreements:

- Uphold the previous agreement at RAN4#108.

#### Issue 2-1-4: Requirements applicability regarding MG reconfiguration

Agreements:

- If during the measurement period, the MG pattern is reconfigured or time window for carrier phase measurement is reconfigured, the measurement period for RSCPD with RSTD and RSCP with UE Rx-Tx can be longer.

#### Issue 2-1-5: Impact of UE mobility

Agreements:

- Uphold the previous agreement at RAN4#108.

#### ue 2-1-4: Requirements applicability regarding MG reconfiguration

Agreements:

- If during the measurement period, the MG pattern is reconfigured or time window for carrier phase measurement is reconfigured, the measurement period for RSCPD with RSTD and RSCP with UE Rx-Tx can be longer.

#### Issue 2-1-5: Impact of UE mobility

Agreements:

- Uphold the previous agreement at RAN4#108.

Draft CRs were endorsed in RAN4#109, adding the requirements for CPP measurements to Rel-18 specifications. Nonetheless, companies were aware of some issues in the CRs that must be addressed during maintenance.

One significant issue is that there were discrepancies in the measurement period requirements captured in different sections of the specifications. In section 4.5.5.5, the measurement period requirement for DL RSCPD in RRC_IDLE covers scenarios in which multiple PFLs are configured in the assistance data, even though CPP measurements themselves are performed on indicated PRS resource sets contained in a single PFL. In contrast, the corresponding requirements in RRC_INACTIVE (section 5.6.7.5) and RRC_CONNECTED (section 9.9.7.5) do not address PRS configurations with multiple PFLs. CPP measurements are always reported with other positioning measurements and the LMF can always configure multiple PFLs for the other measurements if the UE supports it.

Proposal 1: The measurement period requirements for DL RSCPD (reported with DL RSTD) and DL RSCP (reported with UE Rx-Tx) will be defined for scenarios where multiple PFLs are configured in the PRS assistance data.

In our view, when multiple PFLs are configured the measurement delay for the primary (timing) measurement applies for all the other PFLs where CPP measurements are not performed. For the PFL where CPP measurements are performed, the measurement delay should take into account the indicated PRS resource sets and time windows.

Proposal 2: When DL RSCPD is reported with RSTD and DL RSCPD is measured in PFL j with periodic time windows, the measurement period is given by

TRSCPD with RSTD,Total=TRSCPD with RSTD,j+i=1,i≠jLTRSTD,i+ L-1*maxTeffect,i

- FFS how to adapt the requirement for aperiodic time window

Proposal 3: When DL RSCP is reported with UE Rx-Tx and DL RSCP is measured in PFL j with periodic time windows, the measurement period is given by

TRSCP with UERxTx, Total=TRSCP with UERxTx, j+i=1,i≠jLTUERxTx,i+ L-1*maxTeffect,i

- FFS how to adapt the requirement for aperiodic time window

The second issue is that RAN1 has introduced two optional UE capabilities to support performing RSTD and UE Rx-Tx measurements, respectively, inside the indicated time windows [2]. UEs that do not support these optional capabilities can measure RSTD or UE Rx-Tx using PRS resources in the indicated PRS resource sets outside the time windows. RAN4 should revise the measurement period requirements for CPP to accommodate such UEs.

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. |
| --- | --- | --- | --- | --- | --- | --- |
| 41. NR_pos_enh2 | 41-2-8 | Support to perform legacy measurements inside the indicated time window only for DL TDoA | Support to perform legacy measurements inside the indicated time window only | FFS | No | N/A |
| 41. NR_pos_enh2 | 41-2-9 | Support to perform legacy measurements inside the indicated time window only for multi-RTT | Support to perform legacy measurements inside the indicated time window only | FFS | No | N/A |

| Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- |
| The UE may use the indicated DL PRS resource set(s) occurring outside the indicated time window for legacy measurements in addition to the indicated DL PRS resource set(s) occurring inside the indicated time window | Per band | No | No | No | Need for location server to know if the feature is supported | Optional with capability signaling |
| The UE may use the indicated DL PRS resource set(s) occurring outside the indicated time window for legacy measurements in addition to the indicated DL PRS resource set(s) occurring inside the indicated time window | Per band | No | No | No | Need for location server to know if the feature is supported | Optional with capability signaling |

Proposal 4a: When DL RSCPD is reported with RSTD,

- If the UE supports FG 41-2-8 and the LMF configures measurement time windows, Lavailable_PRS,i  is calculated counting only PRS resources in the indicated PRS resource sets on the indicated PFL i  that overlap with the time windows.

- If the UE does not support FG 41-2-8 and the LMF configures measurement time windows, the time window(s) are not considered when calculating Lavailable_PRS,i. i.e. PRS resources in the indicated PRS resource sets on the indicated PFL i  that do not overlap with the time windows can be counted.

- If the LMF does not configure measurement time windows, Lavailable_PRS,i  is calculated counting all PRS resources in the indicated PRS resource sets on the indicated PFL i .

Proposal 4b: When DL RSCP is reported with UE Rx-Tx,

- If the UE supports FG 41-2-9 and the LMF configures measurement time windows, Lavailable_PRS,i  is calculated counting only PRS resources in the indicated PRS resource sets on the indicated PFL i  that overlap with the time windows.

- If the UE does not support FG 41-2-9 and the LMF configures measurement time windows, the time window(s) are not considered when calculating Lavailable_PRS,i. i.e. PRS resources in the indicated PRS resource sets on the indicated PFL i  that do not overlap with the time windows can be counted.

- If the LMF does not configure measurement time windows, Lavailable_PRS,i  is calculated counting all PRS resources in the indicated PRS resource sets on the indicated PFL i .

# Conclusions

Proposal 1: The measurement period requirements for DL RSCPD (reported with DL RSTD) and DL RSCP (reported with UE Rx-Tx) will be defined for scenarios where multiple PFLs are configured in the PRS assistance data.

Proposal 2: When DL RSCPD is reported with RSTD and DL RSCPD is measured in PFL j with periodic time windows, the measurement period is given by

TRSCPD with RSTD,Total=TRSCPD with RSTD,j+i=1,i≠jLTRSTD,i+ L-1*maxTeffect,i

- FFS how to adapt the requirement for aperiodic time window

Proposal 3: When DL RSCP is reported with UE Rx-Tx and DL RSCP is measured in PFL j with periodic time windows, the measurement period is given by

TRSCP with UERxTx, Total=TRSCP with UERxTx, j+i=1,i≠jLTUERxTx,i+ L-1*maxTeffect,i

- FFS how to adapt the requirement for aperiodic time window

Proposal 4a: When DL RSCPD is reported with RSTD,

- If the UE supports FG 41-2-8 and the LMF configures measurement time windows, Lavailable_PRS,i  is calculated counting only PRS resources in the indicated PRS resource sets on the indicated PFL i  that overlap with the time windows.

- If the UE does not support FG 41-2-8 and the LMF configures measurement time windows, the time window(s) are not considered when calculating Lavailable_PRS,i. i.e. PRS resources in the indicated PRS resource sets on the indicated PFL i  that do not overlap with the time windows can be counted.

- If the LMF does not configure measurement time windows, Lavailable_PRS,i  is calculated counting all PRS resources in the indicated PRS resource sets on the indicated PFL i .

Proposal 4b: When DL RSCP is reported with UE Rx-Tx,

- If the UE supports FG 41-2-9 and the LMF configures measurement time windows, Lavailable_PRS,i  is calculated counting only PRS resources in the indicated PRS resource sets on the indicated PFL i  that overlap with the time windows.

- If the UE does not support FG 41-2-9 and the LMF configures measurement time windows, the time window(s) are not considered when calculating Lavailable_PRS,i. i.e. PRS resources in the indicated PRS resource sets on the indicated PFL i  that do not overlap with the time windows can be counted.

- If the LMF does not configure measurement time windows, Lavailable_PRS,i  is calculated counting all PRS resources in the indicated PRS resource sets on the indicated PFL i .

# References

[1] R4-2321524, WF on R18 NR positioning – SL positioning and Carrier Phase Positioning, RAN4#109

[2] R1-2312572, Updated RAN1 UE features list for Rel-18 NR after RAN1#115, RAN1#115