# 3GPP TSG- RAN WG4 Meeting #110	R4-2401247

# Athens, Greece, 26th Feb–1st Mar, 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.307 | CR | 0155 | rev | - | Current version: | 17.11.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | (NR_CADC_R17_2BDL_xBUL-Core) CR for TS38.307: Update and correct the requirements for inter-band NR-DC configurations |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation, Samsung, CHTTL |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_CADC_R16_2BDL_xBUL-Core, NR_CADC_R16_3BDL_2BUL-Core, NR_CADC_R17_2BDL_xBUL-Core, NR_CADC_R17_3BDL_2BUL-Core, NR_CADC_R17_4BDL_2BUL-Core |  |  |  |  |  | Date: |  |  | 2024-02-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The requirements for the additional NR-DC configuration in Table 5.5.1-1 and Table 7.2-1 need to be updated after more inter-band NR-DC configurations and more bands being supported in Release 17. Besides, a new clause that lists the common UE RF requirements for a release independent Inter-band NR-DC configurations within FR1 should be provided. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update Table 5.5.1-1 and Table 7.2-1. Add Clause B.4.5a to illustrate the requirements for the Inter-band NR-DC configurations within FR1. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - The requirements for the additional NR-DC configuration are missing, and the information in the table cannot cover the inter-band NR DC configurations in later release. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.5, 7.2, B.4.5a(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## < START OF CHANGE>

5.5	Additional Inter-band NR-DC configurations for NR frequency range 1

Requirements for a Rel-16 UE for additional NR-DC configurations within FR1 compared to TS 38.101-1 of Rel-16 [2] are introduced via this clause.

Table 5.5.1-1: NR-DC within FR1

| Feature | DL/UL | Maximum number of bands | Maximum number of CCs | CA BW Classes | Duplex-mode | Release independent from | requirements to be fulfilled (see 38.307 of the REL in which the DC configuration was introduced) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| NR-DC configurations within NR FR1 | DL | 4 | 6 | A, B, C, D, E, N | FDD, TDD, FDD and TDD | Rel-16 | Table B.4.5a-1 |
|  | UL | 2 | 3 | A, B | FDD, TDD, FDD and TDD | Rel-16 |  |

< Omit section >

7.2	Additional Inter-band NR-DC configurations between frequency range 1 and frequency range 2

Requirements for a Rel-16 UE for additional Inter-band NR-DC configurations between FR1 and FR2 compared to TS 38.101-3 of Rel-16 [4] are introduced via this clause.

Table 7.2-1: Inter-band NR-DC between FR1 and FR2

| Feature | DL/UL | maximum number of bands | maximum number of CCs | CA BW Classes | Duplex-mode | Release independent from | requirements to be fulfilled (see 38.307 of the REL in which the CA configuration was introduced) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Inter-band DC configurations for NR interworking between FR1 and FR2 | DL FR1 | 3 | 4 | A, B, C | FDD, TDD, FDD and TDD | Rel-15 | Table B.4.5-1 |
|  | DL FR2 | 1 | 8 | A, B, C, D, E, F, G, H, I, J, K, L, M, O, P, Q | TDD | Rel-15 |  |
|  | UL FR1 | 1 | 2 | A, B | FDD, TDD | Rel-15 |  |
|  | UL FR2 | 1 | 8 | A, D, E, F, G, H, I, J, K, L,M | TDD | Rel-15 |  |

< Omit section >

B.4.5	Common UE RF requirements for Inter-band NR-DC configurations between frequency range 1 and frequency range 2

The requirements and test cases listed in Table B.4.5-1 are specified in in REL-16 version of TS 38.101-3 [4].

Table B.4.5-1: Common UE RF requirements for a release independent Inter-band NR-DC configurations between frequency range 1 and frequency range 2

| Clause | Description |
| --- | --- |
| 4.2 | Applicability of minimum requirements |
| 5.2B | Operating bands for DC |
| 6.2B.5 | Configured output power for NR-DC |

B.4.5a	Common UE RF requirements for Inter-band NR-DC configurations within frequency range 1

The requirements and test cases listed in Table B.4.5a-1 are specified in REL-17 version of TS 38.101-1 [2].

Table B.4.5a-1: Common UE RF requirements for release independent Inter-band NR-DC configurations within frequency range 1

| Clause | Description |
| --- | --- |
| 5.2B | Operating bands for DC |
| 5.5B | Configurations for DC |
| 6.2B | Transmitter power for NR-DC |
| 6.3B | Output power dynamics for NR-DC |
| 6.4B | Transmit signal quality for NR-DC |
| 6.5B | Output RF spectrum emissions for NR-DC |
| 7.3B | Reference sensitivity for NR-DC |
| 7.4B | Maximum input level for NR-DC |
| 7.5B | Adjacent channel selectivity for NR-DC |
| 7.6B | Blocking characteristics for NR-DC |
| 7.7B | Spurious response for NR-DC |
| 7.8B | Intermodulation characteristics for NR-DC |
| 7.9B | Spurious emissions for NR-DC |

## < END OF CHANGE>