3GPP TSG-RAN WG4 Meeting # 110		R4-2401326

# Athens, GR, 26 Feb – 01 Mar, 2024

Agenda item:	8.21.2.3

Source: 	Huawei, HiSilicon

Title: 	Discussion on RRM requirements for uTCI extension to mTRP for Rel-18 MIMO

Document for:	Discussion

# Introduction

The Rel-18 WI MIMO Evolution for Downlink and Uplink was approved in [1] and further revised in [2]. In previous RAN4 meeting, the impacts on RRM requirements were initially discussed with agreements captured in [3][4][5][6][7][8]. In last meeting, the core requirements were completed. In this contribution, we provide our views on maintenance issues for extension of unified TCI framework to MTRP.

# Discussion

During the discussion in last meeting, it is agreed that RAN4 to define requirements for FR1 in this WI and corresponding agreements are shown as follows:

Issue 3-1-11: L1-RSRP measurement when RTD>CP Agreement: - Define requirements for FR1 for intra/inter-cell. - For FR1, if UE supports RTD>CP, reuse Rel-17 L1-RSRP measurement period in 9.13 for a cell with different PCI from serving cell, remove the RTD is smaller than CP side condition. Reuse L1-RSRP measurement period in 9.5 for serving cell. - If UE does not support RTD>CP, for RTD is larger than CP, no requirements. - FFS on scheduling/measurement restriction for different SCS and TDD. Issue 3-1-9-a: For mDCI mTRP, how to specify DL TCI state switching requirements for eUTCI if UE supporting two TAs and supporting RTD>CP in FR1? Agreement: - Rel-17 unified TCI state switching requirements are applicable for each TCI state associated with coresetPoolIndex independently

One following question is how to reflect the agreement in the spec. It could be observed that MRTD for multi-TRPs are introduced as follows:

### 7.6.x Minimum Requirements for Multi-TRPs A UE supporting [FG 40-2-1 or FG 40-2-2] shall be capable of handling at least a relative receive timing difference between slot timing of different TRPs on the same carrier at the UE receiver as shown in Table 7.6.x-1 below. A UE supporting [FG 40-2-1 or FG 40-2-2] shall be capable of handling at least a relative receive timing difference between slot timing of different TRPs on the same carrier at the UE receiver as shown in Table 7.6.x-2 below, provided that the UE indicates that it is capable of [FG 40-2-6]. Table 7.6.x-1: Maximum receive timing difference requirement for multi-TRPs with two TAs for UE not capable of [FG 40-2-6] Frequency Range; Maximum receive timing difference. FR1; CP lengthnote 1. FR2-1; CP lengthnote 1. Note 1: CP length of the maximum SCS on the carrier.. Table 7.6.4-2: Maximum receive timing difference requirement for multi-TRPs with two TAs for UE capable of [FG 40-2-6] Frequency Range of the pair of carriers; Maximum receive timing difference (µs). FR1; 33. FR2-1; 8.

As agreed in last meeting, following requirement apply for UE support RTD larger than CP:

- Rel-17 L1-RSRP measurement period in 9.13 for a cell with different PCI from serving cell with the RTD is smaller than CP side condition removed

- L1-RSRP measurement period in 9.5 for serving cell.

- Rel-17 unified TCI state switching requirement.

From our understanding, the above three requirements are not the whole set that can be supported. For instance, if UE can support RTD larger than CP, UE shall also support RLM/BFD requirements when the RTD is within the MRTD.

Observation 1: L1-RSRP, L1-RSRP with different PCI and uTCI state switching are not the only requirements that UE can support when UE is capable of RTD larger than CP.

During the discussion in previous meetings, we met the similar situation about requirements for mTRP in previous release that the applicability about RTD is only stated in 9.13. But the common understanding among the group is that all legacy requirements apply with the sample condition (RTD less than CP), and it does not mean only requirements in 9.13 can apply to mTRP.

Another problem is that it is agreed that RAN4 only define requirements For FR1 for RTD larger than CP. Then how to capture such applicability rule shall be discussed.

Proposal 1: For UE which is capable of RTD larger than CP in FR1, legacy requirements (not only L1-RSRP, L1-RSRP with different PCI and uTCI state switching) apply when RTD is less than MRTD.

Proposal 2: RAN4 to discuss whether and how to capture proposal 1 in the spec.

Another issue about RTD larger than CP is the FFS point on scheduling/measurement restriction for different SCS and TDD. Take L1-RSRP requirements as example, for measurement requirements, the existing requirements are shown as follows:

#### 9.5.5.1 Measurement restriction for SSB based L1-RSRP For FR1, when the SSB for L1-RSRP measurement is in the same OFDM symbol as CSI-RS for RLM, BFD, CBD or L1-RSRP measurement, - If SSB and CSI-RS have same SCS, UE shall be able to measure the SSB for L1-RSRP measurement without any restriction; - If SSB and CSI-RS have different SCS, - If UE supports simultaneousRxDataSSB-DiffNumerology, UE shall be able to measure the SSB for L1-RSRP measurement without any restriction; - If UE does not support simultaneousRxDataSSB-DiffNumerology, UE is required to measure one of but not both SSB for L1-RSRP measurement and CSI-RS. Longer measurement period for SSB based L1-RSRP measurement is expected, and no requirements are defined. For FR2, when the SSB for L1-RSRP measurement on one CC is in the same OFDM symbol as CSI-RS for RLM, BFD, CBD or L1-RSRP measurement on the same CC or different CCs in the same band, UE is required to measure one of but not both SSB for L1-RSRP measurement and CSI-RS. Longer measurement period for SSB based L1-RSRP measurement is expected, and no requirements are defined. For FR2, if the network configures same or mixed numerology between SSB for L1-RSRP measurement on one FR2 band and CSI-RS for RLM, BFD, CBD, L1-RSRP or L1-SINR measurement on the other FR2 band, UE shall be able to perform the related SSB based measurements in one band without any measurement restrictions in the other band, provided that UE is capable of independent beam management on this FR2 band pair. #### 9.5.6.2 Scheduling availability of UE performing L1-RSRP measurement with a different subcarrier spacing than PDSCH/PDCCH on FR1 For UEs which support simultaneousRxDataSSB-DiffNumerology [14] there are no restrictions on scheduling availability due to L1-RSRP measurement based on SSB as RS for L1-RSRP measurement. For UEs which do not support simultaneousRxDataSSB-DiffNumerology [14] the following restrictions apply due to L1-RSRP measurement based on SSB configured for L1-RSRP measurement. - The UE is not expected to transmit PUCCH/PUSCH/SRS or receive PDCCH/PDSCH/CSI-RS for tracking/CSI-RS for CQI on symbols corresponding to the SSB indexes configured for L1-RSRP measurement. When intra-band carrier aggregation in FR1 is configured, the scheduling restrictions on serving cell where L1-RSRP measurement is performed apply to all serving cells in the same band on the symbols that fully or partially overlap with restricted symbols. When inter-band carrier aggregation within FR1 is configured, there are no scheduling restrictions on FR1 serving cell(s) configured in other bands than the bands in which the serving cell where L1-RSRP measurement is performed is configured.

It could be observed that for measurement and scheduling restriction in legacy requirements, since the RTD RS/physical channel from the same cell are negligible, the restrictions on defined on the same OFDM symbols. However, for UE supporting RTD larger than CP, according to the agreed the MRTD which is same as inter-band CA, the L1-RSRP measurement symbols and data transmission could be different by overlapped in time domain. Thus, for UE supporting RTD larger than CP in FR1, the measurement and scheduling restriction can be updated as:

#### 9.5.5.1 Measurement restriction for SSB based L1-RSRP For FR1, when the SSB for L1-RSRP measurement is in the same OFDM symbol CSI-RS for RLM, BFD, CBD or L1-RSRP measurement, or the SSB for L1-RSRP measurement is overlapped with the OFDM symbol as CSI-RS for RLM, BFD, CBD or L1-RSRP measurement for UE support [RTD larger than CP], - If SSB and CSI-RS have same SCS, UE shall be able to measure the SSB for L1-RSRP measurement without any restriction; - If SSB and CSI-RS have different SCS, - If UE supports simultaneousRxDataSSB-DiffNumerology, UE shall be able to measure the SSB for L1-RSRP measurement without any restriction; - If UE does not support simultaneousRxDataSSB-DiffNumerology, UE is required to measure one of but not both SSB for L1-RSRP measurement and CSI-RS. Longer measurement period for SSB based L1-RSRP measurement is expected, and no requirements are defined. For FR2, when the SSB for L1-RSRP measurement on one CC is in the same OFDM symbol as CSI-RS for RLM, BFD, CBD or L1-RSRP measurement on the same CC or different CCs in the same band, UE is required to measure one of but not both SSB for L1-RSRP measurement and CSI-RS. Longer measurement period for SSB based L1-RSRP measurement is expected, and no requirements are defined. For FR2, if the network configures same or mixed numerology between SSB for L1-RSRP measurement on one FR2 band and CSI-RS for RLM, BFD, CBD, L1-RSRP or L1-SINR measurement on the other FR2 band, UE shall be able to perform the related SSB based measurements in one band without any measurement restrictions in the other band, provided that UE is capable of independent beam management on this FR2 band pair. #### 9.5.6.2 Scheduling availability of UE performing L1-RSRP measurement with a different subcarrier spacing than PDSCH/PDCCH on FR1 For UEs which support simultaneousRxDataSSB-DiffNumerology [14] there are no restrictions on scheduling availability due to L1-RSRP measurement based on SSB as RS for L1-RSRP measurement. For UEs which do not support simultaneousRxDataSSB-DiffNumerology [14] the following restrictions apply due to L1-RSRP measurement based on SSB configured for L1-RSRP measurement. - The UE is not expected to transmit PUCCH/PUSCH/SRS or receive PDCCH/PDSCH/CSI-RS for tracking/CSI-RS for CQI on symbols corresponding to the SSB indexes configured for L1-RSRP measurement, or on symbols overlapped with symbols corresponding to the SSB indexes configured for L1-RSRP measurement for UE support [RTD larger than CP]. When intra-band carrier aggregation in FR1 is configured, the scheduling restrictions on serving cell where L1-RSRP measurement is performed apply to all serving cells in the same band on the symbols that fully or partially overlap with restricted symbols. When inter-band carrier aggregation within FR1 is configured, there are no scheduling restrictions on FR1 serving cell(s) configured in other bands than the bands in which the serving cell where L1-RSRP measurement is performed is configured.

Proposal 3: For UE support RTD larger than CP in FR1, the measurement and scheduling restrictions are updated to include the case when the symbols are overlapped in time domain.

Another issue is how to define UL TCI state switching for UE supporting 2 TA. The status are summarized as follows:

Issue 3-1-8-b: For mDCI mTRP, how to specify UL TCI state switching requirements for eUTCI if UE supporting two TAs and but not supporting RTD>CP? Way forward: - Option 1: - [Known case] THARQ + 3Nslotsubframe,µ + TOk-ref (Tfirst-SSB-DLRef + 2ms)+NM*( Tfirst-PL-RS + 4*Ttarget_PL-RS + 2ms) - [Unknown case]: THARQ + 3Nslotsubframe,µ + TL1-RSRP + TOuk-ref (Tfirst-SSB-DLRef + 2ms)+ Tfirst-PL-RS + 4*Ttarget_PL-RS + 2ms - Option 2: - Rel-17 unified TCI state switching requirements are applicable for each TCI state associated with coresetPoolIndex independently - [Known case] THARQ + 3Nslotsubframe,µ + NM*( Tfirst_target-PL-RS + 4*Ttarget_PL-RS + 2ms) - [Unknown case]: THARQ + 3Nslotsubframe,µ + TL1-RSRP + Tfirst_target-PL-RS + 4*Ttarget_PL-RS + 2ms Issue 3-1-9-b: For mDCI mTRP, how to specify UL TCI state switching requirements for eUTCI if UE supporting two TAs and supporting RTD>CP in FR1? Way forward: - Option 1: - [Known case] THARQ + 3Nslotsubframe,µ + TOk-ref (Tfirst-SSB-DLRef + 2ms)+NM*( Tfirst-PL-RS + 4*Ttarget_PL-RS + 2ms) - [Unknown case]: THARQ + 3Nslotsubframe,µ + TL1-RSRP + TOuk-ref (Tfirst-SSB-DLRef + 2ms)+ Tfirst-PL-RS + 4*Ttarget_PL-RS + 2ms - Option 2: - Rel-17 unified TCI state switching requirements are applicable for each TCI state associated with coresetPoolIndex independently - [Known case] THARQ + 3Nslotsubframe,µ + NM*( Tfirst_target-PL-RS + 4*Ttarget_PL-RS + 2ms) - [Unknown case]: THARQ + 3Nslotsubframe,µ + TL1-RSRP + Tfirst_target-PL-RS + 4*Ttarget_PL-RS + 2ms

The problem is whether to consider the measurement time for DL reference timing. Regarding how to determine the DL reference timing, the related RAN4 agreements are summarized as follows:

Issue 3-1-1: DL reference timing Agreement: - For multi-DCI based multi-TRP operation with two TAs, for each TAG, the uplink transmission timing takes place before the reception of the first detected path (in time) of the corresponding downlink frame of the reference signal associated with UL/joint TCI state.

Regarding the which reference signal is considered as the one associated with the UL/joint TCI state, and whether it ca n be assumed that the associated reference signal is always monitored by UE shall be further clarified before deciding whether measurement time is needed for UL TCI state switching.

Proposal 4: RAN4 shall discuss which reference signal is considered as the one associated with the UL/joint TCI state, and whether it can be assumed that the associated reference signal is always monitored by UE.

# Conclusions

Observation 1: L1-RSRP, L1-RSRP with different PCI and uTCI state switching are not the only requirements that UE can support when UE is capable of RTD larger than CP.

Proposal 1: For UE which is capable of RTD larger than CP in FR1, legacy requirements (not only L1-RSRP, L1-RSRP with different PCI and uTCI state switching) apply when RTD is less than MRTD.

Proposal 2: RAN4 to discuss whether and how to capture proposal 1 in the spec.

Proposal 3: For UE support RTD larger than CP in FR1, the measurement and scheduling restrictions are updated to include the case when the symbols are overlapped in time domain.

Proposal 4: RAN4 shall discuss which reference signal is considered as the one associated with the UL/joint TCI state, and whether it can be assumed that the associated reference signal is always monitored by UE.

# References

[1] RP-213598 New WID: MIMO Evolution for Downlink and Uplink

[2] RP-223276 WID Update: MIMO Evolution for Downlink and Uplink

[3] R4-2306362 WF on R18 NR MIMO RRM requirements

[4] R4-2217248 WF on TCI state switching in multi-Rx chain DL reception

[5] R4-2310178 WF on NR MIMO evolution RRM requirements

[6] R4-2314347 WF on R18 NR MIMO RRM requirement

[7] R4-2317371 WF on R18 NR MIMO RRM requirement

[8] R4-2321563 WF on NR_MIMO_evo_DL_UL WI