3GPP TSG-RAN WG4 Meeting #110                                                       R4-2401548

Athens, Greece, 26th Feb – 1st Mar 2024

Agenda Item:	8.9.7.2

Source:	Ericsson

Title:	Simulation results for ATG PDSCH demodulation requirements

Document for:	Information

# 1	Introduction

In this contribution, we submitted our simulation results for all the proposed test cases for 1024QAM listed in our companion contribution [2].

# 2	Discussion

## 2.1 Assumptions

In this section, we summarized the simulation assumptions that we used for our evaluations.

- AWGN + Doppler for the channel model

  - Channel matrix using:

H = [1 j; 1 -j]

- koffset value = 2ms

- rank = 1

- DMRS: 1 front loaded + 1 additional DMRS

- TDD pattern:

  - For test 5-6, new TDD pattern (30D4S6U) is used

  - For test 3-4, legacy TDD pattern (7D1S2U) is used

## 2.2 Simulation result summary

Here please find our simulation results for 1024QAM:

Table 2.2-1 PDSCH results for FDD and TDD

| Test number | Duplex mode, SCS and CBW | Propagation condition | Antenna configuration | MCS | Ideal (dB) |
| --- | --- | --- | --- | --- | --- |
| 1 | FDD 15kHz/10MHz | AWGN+220Hz Doppler | 2T2R | 1024QAM MCS23 0.79 | 23.51 |
| 2 |  |  | 2T4R |  | 20.46 |
| 3 | TDD 30kHz/40MHz 7DS2U | AWGN+500Hz Doppler | 2T2R |  | 23.55 |
| 4 |  |  | 2T4R |  | 20.46 |
| 5 | TDD 30kHz/40MHz 30D4S6U | AWGN+500Hz Doppler | 2T2R |  | 23.54 |
| 6 |  |  | 2T4R |  | 20.46 |

Figure 2.2-1 FDD simulation results

Figure 2.2-2 TDD with legacy pattern simulation results

Figure 2.2-3 TDD with new pattern simulation results

# 3	Summary

In this contribution, we submitted our simulation results for all the proposed test cases for 1024QAM listed above.

# References

[1]	R4-2315167: Summary of simulation results for ATG UE and BS demodulation requirements, CMCC, RAN4 #108bis, Xiamen, China

[2]	R4-2401547: On PDSCH requirement for ATG network, Ericsson, RAN4 #110, Athens, Greece