3GPP TSG-RAN WG4 Meeting #110                                                       R4-2401551

Athens, Greece, 26th Feb. – 1st Mar. 2024

Agenda Item:	8.27.2

Source:	Ericsson

Title:	Simulation results for Rel-18 DSS enhancement demodulation

Document for:	Discussion

# 1	Introduction

In the last RAN4#109 meeting, several simulation parameters have been agreed for evaluation purpose. In this paper, we provided our simulation results based on the agreed simulation assumption listed in the WF [1].

# 2	Discussion

## 2.1 Simulation assumptions

Table 2.1-1 Simulation parameters

| Parameters | Values |
| --- | --- |
| Duplex mode | FDD |
| SCS | 15 kHz |
| Bandwidth | 10 MHz |
| Channel model | TDLA30-10 and TDLC300-100 |
| Antenna configuration | 2x2, 2x4, 4x2, 4x4 |
| CRS | 4 port CRS |
| DCI payload (excluding CRC) | 39 bits |
| Interleaving | Non-interleaved |
| Precoding | Precoder cycling per REG bundle |
| REG bundle size | 6 |
| Shift index | 0 |
| CORSET duration | 2 (2nd symbol and 3rd symbol) |
| CORSET RB | 48 |
| AL | 4, 8 |
| - Note 1: The bandwidth of NR CORESET is within the bandwidth of LTE. - Note 2: gNB punctures the PDCCH data and DMRS REs overlapped with LTE CRS. Note 3: PDCCH channel estimation is assumed to use only the clean PDCCH symbol. |  |

## 2.2 Simulation results

Table 2.2-1 Simulation results

| Test case | SCS(KHz)/CBW (MHz) | CORESET RB | Payload | AL | Propagation condition | Antenna configuration | SNR(dB) @1%BLER |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 15/10 | 48 | 39 | 4 | TDLA30-10 | 2x2 Low | 1.92 |
| 2 | 15/10 | 48 | 39 | 8 | TDLC300-100 | 2x2 Low | -2.79 |
| 3 | 15/10 | 48 | 39 | 4 | TDLA30-10 | 2x4 Low | -2.85 |
| 4 | 15/10 | 48 | 39 | 8 | TDLC300-100 | 2x4 Low | -5.93 |
| 5 | 15/10 | 48 | 39 | 4 | TDLA30-10 | 4x2 Low | 0.44 |
| 6 | 15/10 | 48 | 39 | 8 | TDLC300-100 | 4x2 Low | -3.21 |
| 7 | 15/10 | 48 | 39 | 4 | TDLA30-10 | 4x4 Low | -3.16 |
| 8 | 15/10 | 48 | 39 | 8 | TDLC300-100 | 4x4 Low | -6.03 |

# 3	Summary

In this contribution, we provided our simulation results based on the agreed simulation assumption listed in the WF [1].

# References

[1]	R4-2321187: WF for Enhancement of Dynamic Spectrum Sharing demodulation requirements, RAN4 #109, Chicago, US.