3GPP TSG-RAN WG4 Meeting #110                                                       R4-2401557

Athens, Greece, 26th Feb. – 1st Mar. 2024

Agenda Item:	8.18.8.2

Source:	Ericsson

Title:	Simulation results for NR NTN enhancement UE demodulation requirements

Document for:	Information

# 1	Introduction

By the end of last RAN4#109 meeting, companies were encouraged to bring initial evaluations on different Doppler shift values. In this contribution, we provided our initial simulation results configuring different Doppler-shift values. In addition, we provided the simulation results for studying the k-factor and phase noise impact.

# 2	Discussion

## 2.1 Simulation assumptions

Table 2.1-1 Simulation parameters for NR NTN PDSCH on Ka band

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Center Frequency |  | GHz | 20 |
| Bandwidth |  | MHz | 100 |
| Subcarrier spacing |  | kHz | 120 |
| Duplex Mode |  |  | FDD |
| TDD Slot Configuration |  |  | N/A |
| Channel model |  |  | NTN-TDLC5-600/1200/2000 |
| MCS/Rank |  |  | Rank 1 QPSK 0.3 16QAM 0.48 64QAM 0.46 |
| Antenna configuration |  |  | 1T1R |
| Transform precoding |  |  | CP-OFDM |
| HARQ | Maximum number of HARQ transmissions |  | 16/32 and disabled |
|  | RV sequence |  | 0, 2, 3, 1 |
| DM-RS | DM-RS configuration type |  | 1 |
|  | DM-RS duration |  | single-symbol DM-RS |
|  | Additional DM-RS position |  | pos 1 |
|  | Number of DM-RS CDM group(s) without data |  | 2 |
|  | Ratio of PUSCH EPRE to DM-RS EPRE |  | -3 dB |
|  | DM-RS port |  | {0} |
|  | DM-RS sequence generation |  | NID0=0, nSCID =0 |
| Time domain resource assignment | PDSCH mapping type |  | A |
|  | Start symbol |  | 2 |
|  | Allocation length |  | 12 |
| Frequency domain resource assignment | RB assignment |  | 100MHz for 120kHz SCS |
| Test metric |  |  | 70% of max throughput at target SNR. |

## 2.2 Simulation results

### 2.2.1 Study of k-factor

In this section, we provided our evaluations on the study of different k-factor values. According to TR38.811 Table 6.7.2-7b and together considering the elevation angle equals to 30o, the corresponding average K-factor = 8.4dB. In Rel-17, however, we already defined NTN-TDLC3.5-200 channel model with k-factor 8.04. To avoid defining new channel model profile with the only difference k-factor, we have studied the performance difference with two k-factor values.

Corresponding simulation results have been summarized in Table 2.2.1-1 and 2.2.1-2.

Table 2.2.1-1 Simulation results NR NTN PDSCH on Ka band with k-factor=8.4

| Bandwidth (MHz) / Subcarrier spacing (kHz) | Propagation condition | k-factor(dB) | Antenna configuration / Rank |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Fraction of maximum throughput (%) |  | DM-RS Additional position | HARQ | MCS | SNR (dB) |
| 100MHz/120kHz | NTN-TDLC5- 2000Hz | 8,4 | 1x1, Rank 1 | 70 |  | Pos1 | disabled | 4 | -0.10 |  |
|  |  |  |  |  |  |  | enabled | 4 | -0.92 |  |
|  |  |  |  |  |  |  | disabled | 13 | 7.82 |  |
|  |  |  |  |  |  |  | enabled | 13 | 6.81 |  |
|  |  |  |  |  |  |  | disabled | 18 | 11.47 |  |
|  |  |  |  |  |  |  | enabled | 18 | 10.38 |  |
| 100MHz/120kHz | NTN-TDLC5- 600Hz(doppler) | 8,4 | 1x1, Rank 1 | 70 |  | Pos1 | disabled | 4 | 0.19 |  |
|  |  |  |  |  |  |  | enabled | 4 | -1.25 |  |
|  |  |  |  |  |  |  | disabled | 13 | 8.06 |  |
|  |  |  |  |  |  |  | enabled | 13 | 6.45 |  |
|  |  |  |  |  |  |  | disabled | 18 | 11.67 |  |
|  |  |  |  |  |  |  | enabled | 18 | 9.91 |  |
| 100MHz/120kHz | NTN-TDLC5- 1200Hz(doppler) | 8,4 | 1x1, Rank 1 | 70 |  | Pos1 | disabled | 4 | -0.03 |  |
|  |  |  |  |  |  |  | enabled | 4 | -1.14 |  |
|  |  |  |  |  |  |  | disabled | 13 | 7.89 |  |
|  |  |  |  |  |  |  | enabled | 13 | 6.60 |  |
|  |  |  |  |  |  |  | disabled | 18 | 11.51 |  |
|  |  |  |  |  |  |  | enabled | 18 | 10.13 |  |

Table 2.2.1-2 Simulation results NR NTN PDSCH on Ka band with k-factor=8.05

| Bandwidth (MHz) / Subcarrier spacing (kHz) | Propagation condition | k-factor(dB) | Antenna configuration / Rank |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Fraction of maximum throughput (%) |  | DM-RS Additional position | HARQ | MCS | SNR (dB) |
| 100MHz/120kHz | NTN-TDLC5- 2000Hz | 8,05 | 1x1, Rank 1 | 70 |  | Pos1 | disabled | 4 | -0.04 |  |
|  |  |  |  |  |  |  | enabled | 4 | -0.90 |  |
|  |  |  |  |  |  |  | disabled | 13 | 7.88 |  |
|  |  |  |  |  |  |  | enabled | 13 | 6.86 |  |
|  |  |  |  |  |  |  | disabled | 18 | 11.53 |  |
|  |  |  |  |  |  |  | enabled | 18 | 10.39 |  |
| 100MHz/120kHz | NTN-TDLC5- 600Hz(doppler) | 8,05 | 1x1, Rank 1 | 70 |  | Pos1 | disabled | 4 | 0.26 |  |
|  |  |  |  |  |  |  | enabled | 4 | -1.22 |  |
|  |  |  |  |  |  |  | disabled | 13 | 8.12 |  |
|  |  |  |  |  |  |  | enabled | 13 | 6.46 |  |
|  |  |  |  |  |  |  | disabled | 18 | 11.74 |  |
|  |  |  |  |  |  |  | enabled | 18 | 9.93 |  |
| 100MHz/120kHz | NTN-TDLC5- 1200Hz(doppler) | 8,05 | 1x1, Rank 1 | 70 |  | Pos1 | disabled | 4 | 0.03 |  |
|  |  |  |  |  |  |  | enabled | 4 | -1.11 |  |
|  |  |  |  |  |  |  | disabled | 13 | 7.93 |  |
|  |  |  |  |  |  |  | enabled | 13 | 6.64 |  |
|  |  |  |  |  |  |  | disabled | 18 | 11.57 |  |
|  |  |  |  |  |  |  | enabled | 18 | 10.15 |  |

Observation 1:	The SNR values at 70% throughput are nearly the same for both k-factors 8.4 and 8.05.

Observation 2:  The NTN-TDLC with 5ns and 2000Hz Doppler can be feasible for 120kHz SCS with DM-RS 1+1 configuration.

### 2.2.2 Study of phase noise impact

In this section we provided our evaluations for the purpose of studying the phase noise impact.

Two phase noise models have been considered. Basic assumptions are as follows:

- Bandwidth (MHz) / Subcarrier spacing (kHz): 100MHz/120Hz

- Antenna configuration / Rank: 1x1, rank 1

- Fraction of maximum throughput (%): 70

- DMRS additional position: Pos1

Table 2.2.2-1 Simulation results with phase noise model 1 [2]

| Propagation condition | k-factor (dB) | HARQ | MCS | SNR (dB) No PN (50k Slots) | SNR (dB) PN Model 1 with No Compensation (50k Slots) | Gap |
| --- | --- | --- | --- | --- | --- | --- |
| NTN-TDLC5- 2000Hz | 8,09 | disabled | 4 | 0.02 | 0.16 | 0.14 |
|  |  | enabled | 4 | -0.73 | -0.58 | 0.15 |
|  |  | disabled | 13 | 7.95 | 8.26 | 0.31 |
|  |  | enabled | 13 | 7.08 | 7.36 | 0.28 |
|  |  | disabled | 18 | 11.55 | 11.89 | 0.34 |
|  |  | enabled | 18 | 10.64 | 10.99 | 0.35 |
| NTN-TDLC5- 600Hz(doppler) | 8,09 | disabled | 4 | -0.15 | -0.03 | 0.12 |
|  |  | enabled | 4 | -1.36 | -1.26 | 0.1 |
|  |  | disabled | 13 | 7.75 | 7.9 | 0.15 |
|  |  | enabled | 13 | 6.35 | 6.61 | 0.26 |
|  |  | disabled | 18 | 11.35 | 11.65 | 0.3 |
|  |  | enabled | 18 | 9.85 | 10.21 | 0.36 |
| NTN-TDLC5- 1200Hz(doppler) | 8,09 | disabled | 4 | 0.02 | 0.16 | 0.14 |
|  |  | enabled | 4 | -1.08 | -0.94 | 0.14 |
|  |  | disabled | 13 | 7.9 | 8.23 | 0.33 |
|  |  | enabled | 13 | 6.65 | 6.91 | 0.26 |
|  |  | disabled | 18 | 11.5 | 11.85 | 0.35 |
|  |  | enabled | 18 | 10.2 | 10.53 | 0.33 |

Table 2.2.2-2 Simulation results with phase noise model 2 [2]

| Propagation condition | k-factor (dB) | HARQ | MCS | SNR (dB) No PN (50k Slots) | SNR (dB) PN Model 2 with No Compensation (50k Slots) | Gap |
| --- | --- | --- | --- | --- | --- | --- |
| NTN-TDLC5- 2000Hz | 8,09 | disabled | 4 | 0.02 | 0.18 | 0.16 |
|  |  | enabled | 4 | -0.73 | -0.58 | 0.15 |
|  |  | disabled | 13 | 7.95 | 8.26 | 0.31 |
|  |  | enabled | 13 | 7.08 | 7.37 | 0.29 |
|  |  | disabled | 18 | 11.55 | 11.88 | 0.33 |
|  |  | enabled | 18 | 10.64 | 10.98 | 0.34 |
| NTN-TDLC5- 600Hz(doppler) | 8,09 | disabled | 4 | -0.15 | -0.03 | 0.12 |
|  |  | enabled | 4 | -1.36 | -1.25 | 0.11 |
|  |  | disabled | 13 | 7.75 | 7.99 | 0.24 |
|  |  | enabled | 13 | 6.35 | 6.62 | 0.27 |
|  |  | disabled | 18 | 11.35 | 11.68 | 0.33 |
|  |  | enabled | 18 | 9.85 | 10.21 | 0.36 |
| NTN-TDLC5- 1200Hz(doppler) | 8,09 | disabled | 4 | 0.02 | 0.17 | 0.15 |
|  |  | enabled | 4 | -1.08 | -0.93 | 0.15 |
|  |  | disabled | 13 | 7.9 | 8.21 | 0.31 |
|  |  | enabled | 13 | 6.65 | 6.91 | 0.26 |
|  |  | disabled | 18 | 11.5 | 11.85 | 0.35 |
|  |  | enabled | 18 | 10.2 | 10.52 | 0.32 |

Observation 3: The maximum gap between the SNR no phase noise and SNR with phase noise is 0.36dB

Observation 4: There is only minor impact to PDSCH performance from phase noise under the frequency of 20GHz

# 3	Summary

In this contribution, we provided our simulation results for NR NTN enhancement PDSCH demodulation initial studies.

We summarized our observations as follows:

Observation 1:	The SNR values at 70% throughput are nearly the same for both k-factors 8.4 and 8.05.

Observation 2:  The NTN-TDLC with 5ns and 2000Hz Doppler can be feasible for 120kHz SCS with DM-RS 1+1 configuration.

Observation 3: The maximum gap between the SNR no phase noise and SNR with phase noise is 0.36dB

Observation 4: There is only minor impact to PDSCH performance from phase noise under the frequency of 20GHz

# References

[1]	R4-2321187: WF on NR_NTN_enh_SAN_UE_demod, Qualcomm, RAN4 #109, Chicago, US.

[2]	TS38.821