3GPP TSG-RAN WG4 Meeting # 110	R4-2401586

Athens, GR, Feb 26 – Mar 01, 2024

Agenda Item: 8.3.2.1

Source: MediaTek Inc.

Title: Discussion on UE feature list for multi-Rx UEs

Document for: Discussion

# Introduction

RAN2 already implemented the corresponding UE capability for UE to indicate the preference of FR2 multi-Rx. However, this capability was not reflected in the latest RAN4 feature list. In this paper, we suggest adding this feature group the keep the complete information in RAN4 feature list.

# Discussion

In last meeting, the UE feature list was agreed as Table 1 below.

Agreement (RAN4 meeting #109)

However, there’s a missing part. From RAN2, it has already introduced a UE capability for the preference of multi-RX fallback indication. We captured some related UE capability from RAN2 spec as below.

UE-NR-Capability-v1750 ::= SEQUENCE { crossCarrierSchedulingConfigurationRelease-r17 ENUMERATED {supported} OPTIONAL, nonCriticalExtension UE-NR-Capability-v18xy OPTIONAL } -- Regular non-critical Rel-18 extensions: UE-NR-Capability-v18xy ::= SEQUENCE { multiRx-FR2-Preference-r18 ENUMERATED {supported} OPTIONAL,, nonCriticalExtension SEQUENCE{} OPTIONAL }

In our understanding, if UE encounter some problem (E.g., overheating), it could indicate multi-RX fallback in UE assistance information. Therefore, the following proposal is suggested:

Proposal 1: Add a feature group (30-3) to indicate whether the UE supports providing multi-Rx operation preference for FR2, as already captured in RAN2.

Table 2 Updated UE feature list.

| Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to V2X | Consequence if the feature is not supported by the UE | Type | Need of FDD/TDD dif | Need of FR1/FR2 dif | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 30-1 | Support of requirements for multiRx_DL | - Supports simultaneous reception of PDCCH/PDSCH and RS for L1 measurement with different QCL Type-D on overlapping OFDM symbols, and - Supports simultaneous measurement of 2 RS for L1 measurements different QCL Type-D on overlapping OFDM symbol(s), and - Supports requirements for dual TCI state switch in TS38.133 Note: Above 3 bullets belong to one single UE capability | 16-2c | Yes | No | UE does not follow the requirements | [Per band] | TDD only | FR2-1 only |  |  | Optional with capability signalling |
| 30-2 | Fast beam sweeping for L1 measurement | - Supports beam sweeping factor reduction for SSB-based layer 1 measurement. |  | [No] | N/A | UE’s L1 measurement delay is not reduced | Per band | TDD only | FR2-1 only |  | Candidate values: {2,4,6} | [Optional without capability signalling] |
| 30-3 | multiRx-FR2-Preference-r18 | - Indicates whether the UE supports providing UAI for multi-Rx operation preference (i.e., not supporting simultaneous reception with different QCL-typeD) for FR2 |  | Yes | N/A |  | [Per UE] | TDD only | FR2-1 only |  | [It can be supported for all power classes excepted FR2 PC6] | Optional with capability signalling |

# Summary

In this paper, the discussion of R18 multi-Rx chains is provided. We have the following proposal:

Proposal 1: Add a feature group (30-3) to indicate whether the UE supports providing multi-Rx operation preference for FR2, as already captured in RAN2.

# Reference

[1] R4-2321602 WF on core maintenance and performance part, Vivo, Ericsson RAN4 #109