3GPP TSG-RAN WG4 Meeting # 110	R4-2401679

# Athens, Greece, Feb.26th – March.1st, 2024

Title: 	Discussions on demodulation and CSI requirements for NES

Source: 	Huawei, HiSilicon

Agenda item:	8.26.4

Document for:	Discussion

Background

In last meeting, a WF on demodulation and CSI requirements for NES was agreed. In this contribution we provide our views on the open issues.

Discussions

SSB less Scell requirements

Whether to introduce SSB less Scell requirements for inter-band CA

In RAN4 #109 bis meeting, an open issue is whether to introduce SSB less Scell requirements for inter-band CA. The options are listed as follows:

- Way forward - Option 1: RAN4 shall evaluate the performance of SSB-less SCells CA performance with Rel-15 compliant demodulator implementation (without TRS based TOE) and decide, if the observed performance constitutes a practical operating point of the feature, or if new requirements capturing improved demodulator implementations are needed, or if SSB-less SCell compliant UE shall be tested with normal CA requirements. - Option 2: Define the SSB less Scell requirements for inter-band CA by reusing the existing minimum PDSCH CA demodulation requirements with RTD=[0]us - Apply the same requirements as the minimum CA demodulation requirements, TS38.101-4 5.2A.2.1 for 2Rx UE and 5.2A.3.1 for 4Rx UE. - Add a new test parameter table for SCell where SSB is not configured: Table 5.2A.x.y-1: Additional test parameters for SCell Parameter; Unit; Value. Common serving cell parameters; Physical Cell ID; Same as PSell. SSB position in burst; Not configured. SSB periodicity; ms; N/A.. - Applicable only for inter-band CA in FR1. - Option 3: Not to introduce the requirements

Regarding Option1, Rel-15 TOE algorithm depends on UE implementation, furthermore, TRS is specified for time offset and frequency offset estimation, so it’s unreasonable to conclude that without TRS based TOE was assumed for Rel-15 CA requirements definition. There may be some performance degradation when RTD is very large causing some paths outside the FFT window, however, the performance loss could be ignored because Network always control the RTD to guarantee the performance. RRM session also reached an agreement that RTD should be less than CP to avoid significant performance loss, which means RTD is not bottleneck of PDSCH requirements, so we don’t think it is necessary to evaluate the Scell PDSCH performance with RTD enabled. So we prefer Option3. I.e. Not to introduce the requirements.

Proposal 1: Not to introduce the requirements for SSB less for SCell.

  - CSI requirements

Our understanding is that the biggest baseband impact of this feature is multiple CSI measurements should be performed. From RAN4 side, Only CSI requirements with single CSI channel measurement has been introduced so far. It’s necessary to introduce CSI performance with multiple CSI measurements, just like did in LTE CRI test.

Power domain adaption

We propose to use following simulation assumption:

Table 2-1: Proposed simulation assumption for Power domain adaption test

| Propagation channel |  |  | AWGN |
| --- | --- | --- | --- |
| Antenna configuration |  |  | 2×2 with static channel specified in Annex B.1 |
| CSI-ReportSubConfig | Sub-config1 |  | PowerOffset: 0dB nzp-CSI-RS-ResourceList-r18: Not configured portSubsetIndicator-r18: Not configured |
|  | Sub-config2 |  | PowerOffset: [3]dB nzp-CSI-RS-ResourceList-r18: Not configured portSubsetIndicator-r18: Not configured |
| Others | Reuse Rel-15 CQI test under AWGN conditions |  |  |

The general proposed test setup is defined as follows:

Set two PDSCH to CSI-RS power offsets, each of which corresponds to one sub-configuration (denoted as offset1 and offset2).

For case with N=1, use A-CSI reporting, TE triggers two sub-configurations alternatively with SNR(defined as ratio of  CSI-RS power to noise) and collects reported CQI (denoted as CQI1 and CQI2) and calculates median CQI for each sub-configuration respectively (denoted as median CQI1 and median CQI2).

For case with N=2, use P-CSI reporting or A-CSI reporting. TE triggers two sub-configurations simultaneously with SNR(defined as ratio of  CSI-RS power to noise) and collects reported CQI (denoted as CQI1 and CQI2) and calculate median CQI respectively for sub-configuration (denoted as median CQI1 and median CQI2).

TE transmits PDSCH with two different SNR: SNR+offset1 and SNR+offset2, following test metrics are defined:

a) The reported CQI1 value according to the reference channel shall be in the range of ±1 of the reported median CQI1 more than 90% of the time. b) If the PDSCH BLER using the transport format indicated by median CQI1 is less than or equal to 0.1, then the BLER using the transport format indicated by the (median CQI1+1) shall be greater than 0.1. If the PDSCH BLER using the transport format indicated by the median CQI1 is greater than 0.1, then the BLER using transport format indicated by (median CQI1-1) shall be less than or equal to 0.1. c) The reported CQI2 value according to the reference channel shall be in the range of ±1 of the reported median CQI2 more than 90% of the time. d) If the PDSCH BLER using the transport format indicated by median CQI2 is less than or equal to 0.1, then the BLER using the transport format indicated by the (median CQI2+1) shall be greater than 0.1. If the PDSCH BLER using the transport format indicated by the median CQI2 is greater than 0.1, then the BLER using transport format indicated by (median CQI2-1) shall be less than or equal to 0.1.

Additionally, we propose the following simulation assumption:

Proposal 2: Use following test setup for power domain adaptation test as start point with simulation assumption in Table 2-1.

The general proposed test setup is defined as follows: Set two PDSCH to CSI-RS power offsets, each of which corresponds to one sub-configuration (denoted as offset1 and offset2). For case with N=1, use A-CSI reporting, TE triggers two sub-configurations alternatively with SNR(defined as ratio of CSI-RS power to noise) and collects reported CQI (denoted as CQI1 and CQI2) and calculates median CQI for each sub-configuration respectively.(denoted as median CQI1 and median CQI2)_ For case with N=2, use P-CSI reporting or A-CSI reporting. TE triggers two sub-configurations simultaneously with SNR(defined as ratio of CSI-RS power to noise) and collects reported CQI (denoted as CQI1 and CQI2) and calculate median CQI respectively for sub-configuration. (denoted as median CQI1 and median CQI2)_ TE transmits PDSCH with two different SNR: SNR+offset1 and SNR+offset2, following test metrics are defined: a) The reported CQI1 value according to the reference channel shall be in the range of ±1 of the reported median CQI1 more than 90% of the time. b) If the PDSCH BLER using the transport format indicated by median CQI1 is less than or equal to 0.1, then the BLER using the transport format indicated by the (median CQI1+1) shall be greater than 0.1. If the PDSCH BLER using the transport format indicated by the median CQI1 is greater than 0.1, then the BLER using transport format indicated by (median CQI1-1) shall be less than or equal to 0.1. c) The reported CQI2 value according to the reference channel shall be in the range of ±1 of the reported median CQI2 more than 90% of the time. d) If the PDSCH BLER using the transport format indicated by median CQI2 is less than or equal to 0.1, then the BLER using the transport format indicated by the (median CQI2+1) shall be greater than 0.1. If the PDSCH BLER using the transport format indicated by the median CQI2 is greater than 0.1, then the BLER using transport format indicated by (median CQI2-1) shall be less than or equal to 0.1..

Spatial domain adaption

Considering type1 and type 2 SD adaptation have same test purpose, we can compromise to only introduce PMI requirements for type 1 SD adaption.

Table 2-2: Proposed simulation assumption for Power domain adaption test

| CSI-ReportSubConfig | Sub-config1 |  | PowerOffset: Not configured nzp-CSI-RS-ResourceList-r18: Not configured portSubsetIndicator-r18:0x 11111111 |
| --- | --- | --- | --- |
|  | Sub-config2 |  | PowerOffset: Not configured nzp-CSI-RS-ResourceList-r18: Not configured portSubsetIndicator-r18: 10101010 |
| Others | Reuse Rel-15 8Tx PMI test |  |  |

For test setup of follow PMI case:

For case with N=1, use A-CSI reporting, TE triggers two CSI report sub-configurations and apply the reported CSI alternatively. Please see the following:

Figure 2-1:  Example of test procedure for N=1

For case with N=2, use P-CSI reporting or A-CSI reporting, TE triggers two CSI report two sub-configurations simultaneously and apply the reported CSI alternatively. Please see the following:

Figure 2-2:  Example of test procedure for N=2

For test setup of random PMI case:

TE selects PMI randomly from the union of two codebook corresponding to two spatial patterns.

Then legacy test metric can be reused: γ=TPfollowTPrandom

Proposal 3: Use following test setup for spatial domain adaptation test as start point with simulation assumption in Table 2-2.

For test setup of follow PMI case: For case with N=1, use A-CSI reporting, TE triggers two CSI report sub-configurations and apply the reported CSI alternatively. For case with N=2, use P-CSI reporting or A-CSI reporting, TE triggers two CSI report two sub-configurations simultaneously and apply the reported CSI alternatively. For test setup of random PMI case: Type 1 spatial domain pattern: TE selects PMI randomly from the union of two codebook corresponding to two spatial patterns. Then legacy test metric can be reused: γ=TPfollowTPrandom

Conclusion

In this contribution we provide our views on remaining issues on demodulation and CSI requirements for NES. The observations and proposals are:

Proposal 1: Not to introduce the requirements for SSB less for SCell.

Proposal 2: Use following test setup for power domain adaptation test as start point with simulation assumption in Table 2-1.

The general proposed test setup is defined as follows: Set two PDSCH to CSI-RS power offsets, each of which corresponds to one sub-configuration (denoted as offset1 and offset2). For case with N=1, use A-CSI reporting, TE triggers two sub-configurations alternatively with SNR(defined as ratio of CSI-RS power to noise) and collects reported CQI (denoted as CQI1 and CQI2) and calculates median CQI for each sub-configuration respectively.(denoted as median CQI1 and median CQI2)_ For case with N=2, use P-CSI reporting or A-CSI reporting. TE triggers two sub-configurations simultaneously with SNR(defined as ratio of CSI-RS power to noise) and collects reported CQI (denoted as CQI1 and CQI2) and calculate median CQI respectively for sub-configuration. (denoted as median CQI1 and median CQI2)_ TE transmits PDSCH with two different SNR: SNR+offset1 and SNR+offset2, following test metrics are defined: a) The reported CQI1 value according to the reference channel shall be in the range of ±1 of the reported median CQI1 more than 90% of the time. b) If the PDSCH BLER using the transport format indicated by median CQI1 is less than or equal to 0.1, then the BLER using the transport format indicated by the (median CQI1+1) shall be greater than 0.1. If the PDSCH BLER using the transport format indicated by the median CQI1 is greater than 0.1, then the BLER using transport format indicated by (median CQI1-1) shall be less than or equal to 0.1. c) The reported CQI2 value according to the reference channel shall be in the range of ±1 of the reported median CQI2 more than 90% of the time. d) If the PDSCH BLER using the transport format indicated by median CQI2 is less than or equal to 0.1, then the BLER using the transport format indicated by the (median CQI2+1) shall be greater than 0.1. If the PDSCH BLER using the transport format indicated by the median CQI2 is greater than 0.1, then the BLER using transport format indicated by (median CQI2-1) shall be less than or equal to 0.1..

Proposal 3: Use following test setup for spatial domain adaptation test as start point with simulation assumption in Table 2-2.

For test setup of follow PMI case: For case with N=1, use A-CSI reporting, TE triggers two CSI report sub-configurations and apply the reported CSI alternatively. For case with N=2, use P-CSI reporting or A-CSI reporting, TE triggers two CSI report two sub-configurations simultaneously and apply the reported CSI alternatively. For test setup of random PMI case: Type 1 spatial domain pattern: TE selects PMI randomly from the union of two codebook corresponding to two spatial patterns. Then legacy test metric can be reused: γ=TPfollowTPrandom

Reference

[1]   R1-230073 Power saving techniques in spatial and power domains. Huawei, HiSilicon