3GPP TSG-RAN WG4 Meeting #110	R4-2401750

Athens, Greece, 26 February – 1 March 2024

Agenda Item:	8.3.4.3

Source:	Ericsson

Title:	PMI reporting requirements for FR2 UE multi-Rx reception

Document for:	Discussion

# 1	Introduction

RAN4#109 agreed with the way forward on UE demodulation and CSI requirements for FR2 multi-Rx DL reception [1] and endorsed the draft CRs for PMI reporting requirements [2][3].

This contribution shows our simulation results and discusses the open issues on the PMI reporting requirements with FR2 UE multi-Rx reception scenario.

# 2	Discussion

## 2.1	Test parameters

RAN4 has agreed to define the PMI reporting requirements using the sDCI-based SDM transmission scheme for multi-Rx reception scenario, where PDSCH is transmitted with single layer per TRP, and 2 CSI-RS ports are configured per TRP. Table 1 summarizes the current PMI test assumption according to [1].

Table 1	Test parameters for PMI reporting tests.

| Parameter |  |  |  |  | Unit | Value |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | TRxP #1(Note 1) |  | TRxP #2(Note 1) |
| Transmit TRxP of SSB |  |  |  |  |  | TRxP #1 |  |  |
| PDCCH configuration |  |  | TCI state |  |  | TCI State #1 |  |  |
|  |  |  | CORESETPoolIndex |  |  | 0 |  |  |
| CSI-RS for tracking |  |  | First subcarrier index in the PRB used for CSI-RS |  |  | k0=0 for CSI-RS resources 1,2,3,4 |  | k0=1 for CSI-RS resources 5,6,7,8 |
|  |  |  | First OFDM symbol in the PRB used for CSI-RS |  |  | l0 = 4 for CSI-RS resources 1 and 3 l0 = 8 for CSI-RS resources 2 and 4 |  | l0 = 4 for CSI-RS resources 5 and 7 l0 = 8 for CSI-RS resources 6 and 8 |
|  |  |  | Number of CSI-RS ports (X) |  |  | 1 for CSI-RS resource 1,2,3,4 |  | 1 for CSI-RS resource 5,6,7,8 |
|  |  |  | CDM Type |  |  | ‘No CDM’ for CSI-RS resource 1,2,3,4,5,6,7,8 |  |  |
|  |  |  | Density |  |  | 3 |  |  |
|  |  |  | CSI-RS periodicity |  | Slots | 160 |  |  |
|  |  |  | CSI-RS offset |  | Slots | 80 for CSI-RS resources 1 and 2 81 for CSI-RS resources 3 and 4 |  | 80 for CSI-RS resources 5 and 6 81 for CSI-RS resources 7 and 8 |
|  |  |  | QCL info |  |  | TCI state #0 |  |  |
| Duplex mode |  |  |  |  |  | TDD |  |  |
| Bandwidth |  |  |  |  | MHz | 100 |  |  |
| Subcarrier spacing |  |  |  |  | kHz | 120 |  |  |
| TDD DL-UL configurations |  |  |  |  |  | FR1.120-1 as specified in Annex A.1.3 |  |  |
| Active DL BWP index |  |  |  |  |  | 1 |  |  |
| Propagation channel |  |  |  |  |  | TDLA30-35 |  |  |
| Antenna configuration per TRxP |  |  |  |  |  | High XP 2 x 4 (N1, N2) = (2,1) |  |  |
| Beamforming Model |  |  |  |  |  | As specified in Annex B.4.1 (Note 4) |  |  |
| PDSCH configuration | Mapping type |  |  |  |  | Type A |  |  |
|  | k0 |  |  |  |  | 0 |  |  |
|  | Starting symbol (S) |  |  |  |  | 2 |  |  |
|  | Length (L) |  |  |  |  | 12 |  |  |
|  | PRB bundling type |  |  |  |  | Static |  |  |
|  | PRB bundling size |  |  |  |  | 2 |  |  |
|  | Resource allocation type |  |  |  |  | Type 1 |  |  |
|  | RBG size |  |  |  |  | Config2 |  |  |
|  | VRB-to-PRB mapping type |  |  |  |  | Non-interleaved |  |  |
|  | VRB-to-PRB mapping interleaver bundle size |  |  |  |  | N/A |  |  |
| PDSCH DMRS configuration | Antenna port indexes |  |  |  |  | 1000 |  | 1002 |
|  | TCI state |  |  |  |  | TCI State #1 |  | TCI State #2 |
|  | DMRS Type |  |  |  |  | Type 1 |  |  |
|  | Number of additional DMRS |  |  |  |  | 1 |  |  |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  |  |  |  | 1 |  |  |
| PTRS configuration (Note 5) | Frequency density (KPT-RS) |  |  |  |  | 2 |  | Test 1: N/A Test 2: 2 |
|  | Time density (LPT-RS) |  |  |  |  | 1 |  | Test 1: N/A Test 2: 1 |
|  | Resource Element Offset |  |  |  |  | 2 |  | Test 1: N/A Test 2: 3 |
| TCI State #1 | Type 1 QCL information |  |  | CSI-RS resource |  | CSI-RS resource 1 from 'CSI-RS for tracking’ configuration |  | N/A |
|  |  |  |  | QCL Type |  | Type A |  | N/A |
|  | Type 2 QCL information |  |  | CSI-RS resource |  | CSI-RS resource 1 from 'CSI-RS for tracking’ configuration |  | N/A |
|  |  |  |  | QCL Type |  | Type D |  | N/A |
| TCI State #2 | Type 1 QCL information |  |  | CSI-RS resource |  | N/A |  | CSI-RS resource 5 from 'CSI-RS for tracking’ configuration |
|  |  |  |  | QCL Type |  | N/A |  | Type A |
|  | Type 2 QCL information |  |  | CSI-RS resource |  | N/A |  | CSI-RS resource 5 from 'CSI-RS for tracking’ configuration |
|  |  |  |  | QCL Type |  | N/A |  | Type D |
| Resource allocation |  |  |  |  |  | Full-overlapping |  |  |
| Timing offset of the second TRxP from the first TRxP |  |  |  |  | us | 0 |  |  |
| Frequency offset of the second TRxP from the first TRxP |  |  |  |  | Hz | 0 |  |  |
| Number of HARQ Processes |  |  |  |  |  | 8 |  |  |
| The number of slots between PDSCH and corresponding HARQ-ACK information |  |  |  |  |  | Specific to each TDD UL-DL pattern and as defined in Annex A.1.3 |  |  |
| ZP CSI-RS configuration |  | CSI-RS resource Type |  |  |  | Periodic |  |  |
|  |  | Number of CSI-RS ports (X) |  |  |  | 4 |  |  |
|  |  | CDM Type |  |  |  | FD-CDM2 |  |  |
|  |  | Density (ρ) |  |  |  | 1 |  |  |
|  |  | First subcarrier index in the PRB used for CSI-RS (k0) |  |  |  | Row 4, (8,-) |  |  |
|  |  | First OFDM symbol in the PRB used for CSI-RS (l0) |  |  |  | (13) |  |  |
|  |  | CSI-RS periodicity and offset |  |  | slot | 5/1 |  |  |
| NZP CSI-RS for CSI acquisition |  | CSI-RS resource ID |  |  |  | Resource #9 | Resource #10 |  |
|  |  | CSI-RS resource Type |  |  |  | Aperiodic | Aperiodic |  |
|  |  | Number of CSI-RS ports (X) |  |  |  | 2 | 2 |  |
|  |  | CDM Type |  |  |  | FD-CDM2 | FD-CDM2 |  |
|  |  | Density (ρ) |  |  |  | 1 | 1 |  |
|  |  | First subcarrier index in the PRB used for CSI-RS (k0, k1 ) |  |  |  | Row 3, (8,-) | Row 3, (10,-) |  |
|  |  | First OFDM symbol in the PRB used for CSI-RS (l0) |  |  |  | (12) | (12) |  |
|  |  | CSI-RS periodicity and offset |  |  | slot | Not configured | Not configured |  |
|  |  | aperiodicTriggeringOffset |  |  |  | 0 | 0 |  |
| CSI-IM configuration |  | CSI-IM resource Type |  |  |  | Aperiodic |  |  |
|  |  | CSI-IM RE pattern |  |  |  | Pattern 1 |  |  |
|  |  | CSI-IM Resource Mapping (kCSI-IM,lCSI-IM) |  |  |  | (8,13) |  |  |
|  |  | CSI-IM timeConfig periodicity and offset |  |  | slot | Not configured |  |  |
| ReportConfigType |  |  |  |  |  | Aperiodic |  |  |
| CQI-table |  |  |  |  |  | Table 1 |  |  |
| reportQuantity |  |  |  |  |  | cri-RI-PMI-CQI |  |  |
| csi-ReportMode |  |  |  |  |  | Mode1 |  |  |
| numberOfSingleTRP-CSI-Mode1 |  |  |  |  |  | X=0 |  |  |
| CMR pairing and grouping |  |  |  |  |  | CMR group #1: {NZP CSI-RS resource #9}, with K1=1 CMR group #2: {NZP CSI-RS resource #10}, with K2=1 CMR paring: {NZP CSI-RS resource #9, NZP CSI-RS resource #10} |  |  |
| timeRestrictionForChannelMeasurements |  |  |  |  |  | Not configured |  |  |
| timeRestrictionForInterferenceMeasurements |  |  |  |  |  | Not configured |  |  |
| cqi-FormatIndicator |  |  |  |  |  | Wideband |  |  |
| pmi-FormatIndicator |  |  |  |  |  | Wideband |  |  |
| Sub-band Size |  |  |  |  | RB | 8 |  |  |
| csi-ReportingBand |  |  |  |  |  | 1111111 |  |  |
| CSI-Report periodicity and offset |  |  |  |  | slot | Not configured |  |  |
| Aperiodic Report Slot Offset |  |  |  |  |  | 8 |  |  |
| CSI request |  |  |  |  |  | 1 in slots i, where mod(i, 5) = 1, otherwise it is equal to 0 |  |  |
| reportTriggerSize |  |  |  |  |  | 1 |  |  |
| CSI-AperiodicTriggerStateList |  |  |  |  |  | One State with one Associated Report Configuration Associated Report Configuration contains pointers to NZP CSI-RS and CSI-IM |  |  |
| Codebook configuration |  | CodebookType |  |  |  | typeI-SinglePanel |  |  |
|  |  | CodebookMode |  |  |  | 1 |  |  |
|  |  | (CodebookConfig-N1,CodebookConfig-N2) |  |  |  | (2,1) |  |  |
|  |  | (CodebookConfig-O1,CodebookConfig-O2) |  |  |  | (2,1) |  |  |
|  |  | CodebookSubsetRestriction |  |  |  | 001111 |  |  |
|  |  | RI Restriction |  |  |  | N/A |  |  |
| Physical channel for CSI report |  |  |  |  |  | PUSCH |  |  |
| CQI/RI/PMI delay |  |  |  |  | ms | 1.75 |  |  |
| Maximum number of HARQ transmission |  |  |  |  |  | 4 |  |  |
| Measurement channel |  |  |  |  |  | Test 1: MCS13, One PTRS port across TRxP Test 2: MCS13, One PTRS port per TRxP |  |  |
| PDSCH & PDSCH DMRS Precoding configuration for random Precoding |  |  |  |  |  | Single Panel Type I, Random precoder selection updated per slot, with equal probability of each applicable i1, i2 combination, and with Wideband granularity |  |  |
| Note 1: PDSCH transmission is done from both TRxPs (PDSCH Layer 0 is transmitted from TRxP #1 and PDSCH layer 1 is transmitted from TRxP #2) Note 2: When Throughput is measured using random precoder selection, the precoder shall be updated in each slot (0.125 ms granularity) with equal probability of each applicable i1, i2 combination. Note 3: If the UE reports in an available uplink reporting instance at slot#n based on PMI estimation at a downlink slot not later than slot#(n-4), this reported PMI cannot be applied at the gNB downlink before slot#(n+4). Note 4: Randomization of the principle beam direction per TRxP shall be used as specified in Annex B.2.3.2.3. Note 5: PT-RS configuration in Test 1 uses single port (one TRP) and Test 2 used dual port (both TRPs) |  |  |  |  |  |  |  |  |

## 2.2	Simulation results

Figure 1 shows the simulation results for PMI tests, and Table 2 summarizes the simulation results. The difference between Tests 1-1 and 1-2 is one PTRS port across TRxP or per PTRS. According to the simulation results, we don’t see any significant performance difference between two test cases.

Observation 1: The throughput gains of follow PMI and random PMI at 90% of the peak rate with follow PMI is about 1.27 regardless of PTRS configuration.

Considering the margin, we propose to set gamma to 1.2 for both cases.

Proposal 1: Set gamma=1.2 for both tests 1-1 and 1-2.

| (a) MCS13 1+1, One PTRS port across TRxP | (b) MCS13 1+1, One PTRS port per TRxP |
| --- | --- |

Figure 1	Simulation results for PMI reporting tests.

Table 2	Summary of PMI reporting tests.

| Test number | MCS, rank | FO | TO | rho | PTRS | γ at SNR corresponding to 90% of peak rate with follow PMI |
| --- | --- | --- | --- | --- | --- | --- |
| 1-1 | 13, 1+1 | 0Hz | 0us | -12dB | One PTRS port across TRxP | 8.7dB/1.28 |
| 1-2 | 13, 1+1 | 0Hz | 0us | -12dB | One PTRS port per TRxP | 8.7dB/1.28 |

According to the endorsed draft CR on FRC used for PMI reporting test [3], ‘Overhead for TBS determination’ is set to 6 for both cases one PTRS across TRxP and per TRxP. However, we propose to set the overhead for one PTRS per TRxP to 12 to 1) close the effective code rate for both cases and 2) align with PDSCH demodulation requirements [4].

Proposal 2: Set ‘Overhead for TBS determination’ to 6 for Test 1 (One PTRS port across TRxP) and 12 for Test 2 (One PTRS port per TRxP), respectively.

# 3	Summary

Observation 1: The throughput gains of follow PMI and random PMI at 90% of the peak rate with follow PMI is about 1.27 regardless of PTRS configuration.

Proposal 1: Set gamma=1.2 for both tests 1-1 and 1-2.

Proposal 2: Set ‘Overhead for TBS determination’ to 6 for Test 1 (One PTRS port across TRxP) and 12 for Test 2 (One PTRS port per TRxP), respectively.

# References

- R4-2321140, “WF on [109][318] NR_FR2_multiRX_DL_Demod”, Qualcomm.

- R4-2318554, “Draft CR to 38.101-4: PMI reporting requirements for FR2 multipanel reception”, MediaTek.

- R4-2318555, “Draft CR to 38.101-4: PMI reference measurement channel for FR2 multipanel reception”, MediaTek.

- R4-2321205, “Draft CR on Minimum requirements and FRC definition for sDCI SDM”, Huawei, HiSilicon.