3GPP TSG-RAN WG4 Meeting #110	R4-2401757

Athens, Greece, 26 February – 1 March 2024

Agenda Item:	8.8.3.3

Source:	Ericsson

Title:	UE demodulation requirements for FR2 HST multi-Rx reception

Document for:	Discussion

# 1	Introduction

RAN4#109 agreed with the WF on the UE demodulation requirements for Rel-18 FR2 HST [1]. This contribution shows our simulation results and discusses the remaining open issues on the simultaneous multi-Rx reception scenario.

# 2	Discission

## 2.1	UE demodulation requirements

RAN4#110 agreed with almost all the simulation parameters to define PDSCH demodulation requirements for FR2 HST simultaneous multi-Rx reception scenario [1].

Agreement: - Do not specify baseline UE processing assumption for the FFT window and leave it to UE implementation for FR2 HST performance requirements definition. - RTD introduced - 1.5xCP - MCS pairs - Option1: MCS 13 - Option2: MCS 11 - Test metric - 70% Tput for each PDSCH - Companies are encouraged to provide both ideal and impairments results for MCS pairs (MCS 19, MCS 13) and (MCS 19, MCS 11) based on RTD=1.5xCP in the RAN4#110 meeting, and make the down selection for specifying requirement in the next meeting

Table 1 summarizes the simulation assumption for FR2 HST simultaneous multi-Rx reception scenario [2]. Figure 1 (a) shows our simulation results and Table 2 summarizes the simulation results regarding SNR to achieve 70% of the peak rate. The detailed FRC table is provided in Appendix. Note our simulation results do not apply any received power changes or received time drifts according to the agreements.

Table 1	PDSCH simulation parameters for FR2 HST multi-Rx simultaneous reception.

| Parameters | TRP#1 | TRP#2 |
| --- | --- | --- |
| Carrier frequency | 30 GHz |  |
| SCS/CBW | 120kHz / 200MHz |  |
| TDD Pattern | DDDSU, S=10D2G2U Schedule PDSCH in special slots |  |
| Antenna configuration | 2x2 (Static channel matrix according to TS8.101-4 Annex B.1.) | 2x2 (Static channel matrix according to TS8.101-4 Annex B.1.) |
| TRS periodicity | 10 ms (80 slots) | 10 ms (80 slots) |
| Number of active TCI States | 1 | 1 |
| Number of additional DMRS | 2 | 2 |
| rank | 2 | 2 |
| MCS index (MCS table 1) | 19 | Option 1: 13 Option 2: 11 |
| RTD (us) from TRP#1 | N/A | 1.5 x CP |

| (a) PDSCH relative throughput | (b) RTD and Received power difference at UE with HST-DPS-FR2-BI-B1-MR |
| --- | --- |

Figure 1	Simultaneous multi-Rx panel reception simulation results with 2+2 (per Rx panel).

The remaining open issue for the UE demodulation requirements for this WI is the configured MCS for TRP#2. Figure 1 (b) shows the received time difference (RTD) and power difference according to HST-DPS-FR2-BI-B1-MR. According to the figure, the RTD at UE location of 300m is about 1.5CP, and the power difference between two TRPs is about 6.4dB. According to Table 2, we propose to set MCS11 because the power difference between MCS19 and MCS11 is 6.7dB, which is close to the UE location with RTD=1.5CP.

Proposal 1: Set MCS 11 for TRP#2.

Table 2	Summary of simulation results.

|  | Alignment results | Impairment results |
| --- | --- | --- |
| TRP#1 (MCS19) | 11.1 | 13.1 |
| TRP#2 (MCS13) | 6.2 | 8.2 |
| TRP#2 (MCS11) | 4.4 | 6.4 |

## 2.2	Scheduling

One of the remaining open issues is the MAC-CE scheduling and PDSCH transmission timing after the TCI state switching. Since RAN4 reuses the FR2 HST-DPS bi-directional deployment model specified in Rel-17 HST-DPS, basically we can reuse the scheduling specified in TS38.101-4 Table 7.2.2.2.4 Test 1-1. One difference from Rel-17 HST-DPS is that Rel-18 simultaneous Rx reception receives signals from two RRHs simultaneously. We therefore need to define two sets of scheduling: one for the forward-looking Rx panel and another one for the backward-looking Rx Panel.

Figure 2	Rel-18 FR2 HST multi-Rx simultaneous reception scheduling.

Forward-looking Rx panel

Figure 2 illustrates the multi-Rx simultaneous reception scheduling, where P1 corresponds to the forward-looking Rx panel and P2 corresponds to the backward-looking Rx panel. The forward-looking Rx panel is basically same as Rel-17 HST-DPS as shown in Figure 3.

Figure 3	Detailed scheduling for the forward-looking Rx panel.

Proposal 2: For the forward-looking Rx panel, scheduling TCI state switching command can be slot# 57600 x n, n=0, 1, 2, …

- PDSCH throughput statistics starts: THARQ + TMAC Proc + TfirstSSB + TSSB proc + TfirstTRSafterSSB + TTRS proc

  - THARQ = 4 (slots), TMAC Proc = 24 (slots), TSSB proc = 16 (slots), and TTRS proc = 16 (slots)

  - TfirstSSB =132 (= SSB period (160) - THARQ - TMAC Proc)

  - TfirstTRSafterSSB = 69 (= TRS period (80) - TRS offset from SSB (5) - TTRS proc)

Backward-looking Rx panel

Backward-looking Rx panel start from the position 2 x Ds_offset before the TCI state switching position as illustrated in Figure 2. This means the TCI state switching command is scheduled at slots 57600 x n + 16457 as shown in Figure 4, where the offset 16457 is the number of slots corresponding to the time to move (2 x Ds_offset = 200m) with 350km/h. We should also point that slot numbers (57600 x n + 16457) are DL slot with TDD UL/DL configuration ‘DDDSU’.

Figure 4	Detailed scheduling for the backward-looking Rx panel (Option 1).

Another simplified option is to schedule the MAC CE command in the SSB slot before the slot 2 x Ds_offset, as shown in Figure 5 (Option 2). In this option, the TCI state switching command is scheduled at slots 57600 x n + 16320, and both TfirstSSB and TfirstTRSafterSSB are same as the forward-looking Rx panel case.

Figure 5	Detailed scheduling for the backward-looking Rx panel (Option 2).

We slight prefer Option 2 because of the simplicity.

Proposal 3: For the backward-looking Rx panel, scheduling TCI state switching command can be slot# 57600 x n + 16320, n = 0, 1, 2, …

- PDSCH throughput statistics starts: THARQ + TMAC Proc + TfirstSSB + TSSB proc + TfirstTRSafterSSB + TTRS proc

  - THARQ = 4 (slots), TMAC Proc = 24 (slots), TSSB proc = 16 (slots), and TTRS proc = 16 (slots)

  - TfirstSSB =132 (= SSB period (160) - THARQ - TMAC Proc)

  - TfirstTRSafterSSB = 69 (= TRS period (80) - TRS offset from SSB (5) - TTRS proc)

# 3	Summary

Proposal 1: Set MCS 11 for TRP#2.

Proposal 2: For the forward-looking Rx panel, scheduling TCI state switching command can be slot# 57600 x n, n=0, 1, 2, …

- PDSCH throughput statistics starts: THARQ + TMAC Proc + TfirstSSB + TSSB proc + TfirstTRSafterSSB + TTRS proc

  - THARQ = 4 (slots), TMAC Proc = 24 (slots), TSSB proc = 16 (slots), and TTRS proc = 16 (slots)

  - TfirstSSB =132 (= SSB period (160) - THARQ - TMAC Proc)

  - TfirstTRSafterSSB = 69 (= TRS period (80) - TRS offset from SSB (5) - TTRS proc)

Proposal 3: For the backward-looking Rx panel, scheduling TCI state switching command can be slot# 57600 x n + 16320, n = 0, 1, 2, …

- PDSCH throughput statistics starts: THARQ + TMAC Proc + TfirstSSB + TSSB proc + TfirstTRSafterSSB + TTRS proc

  - THARQ = 4 (slots), TMAC Proc = 24 (slots), TSSB proc = 16 (slots), and TTRS proc = 16 (slots)

  - TfirstSSB =132 (= SSB period (160) - THARQ - TMAC Proc)

  - TfirstTRSafterSSB = 69 (= TRS period (80) - TRS offset from SSB (5) - TTRS proc)

# References

- R4-2321063, “WF on [109][320] NR_HST_FR2_enh_Demod”, Samsung.

- R4-2313920, “Simulation assumption for FR2 HST demodulation”, Ericsson.

# Appendix: Channel model used for FR2 HST multi-Rx simultaneous reception

| Panel 1 | fst=fdcosθ(t) cosθt=Ds_offset+Ds-vtDmin2+Ds_offset+Ds-vt2, 0<t≤Dsv cosθ(t)=cost modDsv, t>Dsv |
| --- | --- |
| Panel 2 | fst=fdcosθ(t) cosθt=-Ds-Ds_offset+vtDmin2+Ds-Ds_offset+vt2, 0<t≤2Ds_offsetv--Ds_offset+vtDmin2+-Ds_offset+vt2, 2Ds_offsetv<t≤Dsv cosθ(t)=cost modDsv, t>Dsv |

| Parameters | Description | HST-DPS-FR2-BI-B1-MR |
| --- | --- | --- |
| Ds | Inter-RRH distance | 700 m |
| Dmin | Distance between rail track and RRH | 150 m |
| v | Train velocity | 350 km/h |
| fd | Maximum Doppler frequency shift | 9722 Hz |
| fc | Carrier frequency | 30 GHz |
| Drx_panel | Distance between two UE Rx panels | 0 m |
| Ds_offset | Switching transmission point between adjacent RRHs | 100 m |

| Parameter |  |  | Unit | Value |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | TRxP #1(Note 1) | TRxP #2(Note 1) |
| Transmit TRxP of SSB |  |  |  | TRxP #1 |  |
| PDCCH configuration |  | TCI state |  | TCI State #1 | TCI State #2 |
|  |  | CORESETPoolIndex |  | 0,1 |  |
| Duplex mode |  |  |  | TDD |  |
| UL-DL configuration |  |  |  | FR2.120-1 |  |
| Active DL BWP index |  |  |  | 1 |  |
| PDSCH configuration | Mapping type |  |  | Type A |  |
|  | k0 |  |  | 0 |  |
|  | Starting symbol (S) |  |  | 1 |  |
|  | Length (L) |  |  | 13 |  |
|  | PRB bundling type |  |  | Static |  |
|  | PRB bundling size |  |  | 2 |  |
|  | Resource allocation type |  |  | Type 1 |  |
|  | RBG size |  |  | Config2 |  |
|  | VRB-to-PRB mapping type |  |  | Non-interleaved |  |
|  | VRB-to-PRB mapping interleaver bundle size |  |  | N/A |  |
| PDSCH DMRS configuration | Antenna port indexes |  |  | {1000,1001} | {1002,1003} |
|  | TCI state |  |  | TCI State #1 | TCI State #2 |
|  | DMRS Type |  |  | Type 1 |  |
|  | Number of additional DMRS |  |  | 2 |  |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  |  | 1 |  |
| Resource allocation |  |  |  | full-overlapping |  |
| Number of HARQ Processes |  |  |  | 8 |  |
| The number of slots between PDSCH and corresponding HARQ-ACK information |  |  |  | As defined in Annex A.1.3 |  |
| Precoding configuration |  |  |  | SP Type I, independent precoding generation is applied for both TRxPs, random per slot with PRB bundling granularity |  |
| Note 1: PDSCH transmission is done from both TRxPs. Transmission from TRxP #1 uses CORESETPoolIndex 0 and transmission from TRxP #2 uses CORESETPoolIndex 1 |  |  |  |  |  |

| Parameter | Unit | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | TRP#1 | TRP#2 (MCS11) | TRP#2 (MCS13) |  |  |
| Channel bandwidth | MHz | 200 | 200 | 200 |  |  |
| Subcarrier spacing | kHz | 120 | 120 | 120 |  |  |
| Allocated resource blocks | PRBs | 132 | 132 | 132 |  |  |
| Number of consecutive PDSCH symbols |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = 4 for i from {0,…,159} |  | N/A | N/A | N/A |  |  |
| For Slot i, if mod(i, 5) = 3 for i from {4,…, 159} |  | 9 | 9 | 9 |  |  |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {5,…,159} |  | 13 | 13 | 13 |  |  |
| For Slot i = 1,2,3 |  | N/A (Note 1) | N/A (Note 1) | N/A (Note 1) |  |  |
| Allocated slots per 2 frames |  | 124 | 124 | 124 |  |  |
| MCS table |  | 64QAM | 64QAM | 64QAM |  |  |
| MCS index |  | 19 | 11 | 13 |  |  |
| Modulation |  | 64QAM | 64QAM | 64QAM |  |  |
| Target Coding Rate |  | 0.50 | 0.37 | 0.48 |  |  |
| Number of MIMO layers |  | 2 | 2 | 2 |  |  |
| Number of DMRS REs |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = 4 for i from {0,…,159} |  | N/A | N/A | N/A |  |  |
| For Slot i, if mod(i, 5) = 3 for i from {4,…, 159} |  | 18 | 18 | 18 |  |  |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {5,…,159} | Bits | 18 | 18 | 18 |  |  |
| For Slot i = 1,2,3 |  | N/A (Note 1) | N/A (Note 1) | N/A (Note 1) |  |  |
| Overhead for TBS determination |  | 6 | 6 | 6 |  |  |
| Information Bit Payload per Slot |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = 4 for i from {0,…,159} | Bits | N/A | N/A | N/A |  |  |
| For Slot i, if mod(i, 5) = 3 for i from {4,…, 159} | Bits | 67584 | 32776 | 42016 |  |  |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {5,…,159} | Bits | 106576 | 51216 | 67584 |  |  |
| For Slot i = 1,2,3 | Bits | N/A (Note 1) | N/A (Note 1) | N/A (Note 1) |  |  |
| Transport block CRC per Slot |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = 4 for i from {0,…,159} | Bits | N/A | N/A | N/A |  |  |
| For Slot i, if mod(i, 5) = 3 for i from {4,…, 159} | Bits | 24 | 24 | 24 |  |  |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {5,…,159} | Bits | 24 | 24 | 24 |  |  |
| For Slot i = 1,2,3 | Bits | N/A (Note 1) | N/A (Note 1) | N/A (Note 1) |  |  |
| Number of Code Blocks per Slot |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = 4 for i from {0,…,159} | CBs | N/A | N/A | N/A |  |  |
| For Slot i, if mod(i, 5) = 3 for i from {4,…, 159} | CBs | 9 | 4 | 5 |  |  |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {5,…,159} | CBs | 13 | 7 | 9 |  |  |
| For Slot i = 1,2,3 | CBs | N/A (Note 1) | N/A (Note 1) | N/A (Note 1) |  |  |
| Binary Channel Bits Per Slot |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = 4 for i from {0,…,159} | Bits | N/A | N/A | N/A |  |  |
| For Slots i = 5 and 85 (Note 3) | Bits | 181968 | 121312 | 121312 |  |  |
| For Slots i = 6 and 86 (Note 3) | Bits | 181968 | 121312 | 121312 |  |  |
| For Slot i, if mod(i, 5) = 3 for i from {8,…,84, 87,…,159} | Bits | 137808 | 91872 | 91872 |  |  |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {7,…,84,87,…,159} | Bits | 210672 | 140448 | 140448 |  |  |
| For Slot i = 1,2,3 | Bits | N/A (Note 1) | N/A (Note 1) | N/A (Note 1) |  |  |
| Max. Throughput averaged over 2 frames | Mbps | 600.334 | 288.957 | 379.39 |  |  |
| Note 1: SS/PBCH block is transmitted in slot #0, slot #1, slot #2 and slot #3 with periodicity 20ms Note 2: Slot i is slot index per 2 frames Note 3: Binary Channel Bits are calculated under assumption of 52 PRBs TRS allocation when the number of allocated resource blocks are more than 52. |  |  |  |  |  |  |