3GPP TSG-RAN WG4 Meeting #110	R4-2401793
Athens, Greece, 26 Feb – 1 Mar 2024

Title:	Discussion on TxD capability for 3Tx

Source:	OPPO

Agenda item:	6.2.4.2

Document for:	Approval

- Introduction

The LS on introduction of per FS TxD capability was sent to RAN2 in [1] as below and this capability was intended to be introduced from Rel-18 where the 3Tx was introduced in the WI.

It was agreed in RAN4 that a new per band per band combination UE capability would be required to indicate support of TxD in the band configured with 2Tx. It was also agreed that the new TxD capability should be enabled from Rel-18.

Later, RAN4 further clarifies in LS [2] that this new per FS TxD capability is used to determine whether UE support TxD for the band in a BC from Rel-18 and onwards.

And the following understanding for CA case is also reached for Rel-18 and onwards: - It is agreed that CA does not inherit the corresponding single band TxD capability. And from Rel-18 and onwards, whether to support TxD for the band in a band combination is determined by [txDiversity2Tx-r18]/[txDiversity4Tx-r18], but not by txDiversity-r16.

However, according to [3], the 3Tx for inter-band CA + TxD is release independent from Rel-17. To make CA+TxD can be applied in Rel-17, the per FS TxD capability should also be enabled in Rel-17 by for example early implementation. RAN2 should be informed with this.

Observation 1:   RAN4 has informed RAN2 that the Per FS TxD capability was introduced from Rel-18 and is used as the only capability to determine whether UE supports TxD of a band in band combination.

Observation 2:   RAN4 has also agreed the inter-band CA+TxD is release independent from Rel-17 which makes there is no available per FS TxD can be used for Rel-17 UE.

Proposal:   	Inform RAN2 that the per FS TxD need to be applied from Rel-17 by for example early implementation.

- References

- R4-2317762, LS on new per band per BC TxD capability, R4->R2, RAN4#108bis 2023-10

- R4-2321983, LS on 2Tx-TxD capability and 4Tx-TxD capability, R4->R2, RAN4#109 2023-11

- R4-2321765, Release independent for 3Tx band combination (38.307), ZTE, RAN4#109 2023-11