3GPP TSG-RAN WG4 Meeting #110	R4-2401805

Athens, GR, 26 Feb – 01 Mar, 2024

Title:	Consideration on FR1 MIMO OTA measurement campaign

Source:	OPPO

Agenda item:	8.12.3

Document for:	Approval

- Introduction

The updated framework for FR1 MIMO OTA was approved in the RAN4 #109 meeting [1]. This contribution provides our considerations on FR1 MIMO OTA measurement campaign and data processing in the data pool.

- Discussion

In the last meeting, it is agreed that the trusted and neutral third party is introduced to collect and post-proceed the device information during measurement campaign [1].

And it is also agreed that the minimum number of commercial devices for defining requirements for each band is 15 [1].

The number of 15 devices is relatively small compared with R18 TRP TRS measurement campaign, which requires [40] devices per band as the minimum threshold [2].

Supposing the situation that some measurement data in the data pool coming from the same model, the CDF curve of all the measurement data may be biased, because of limited measured UE models. To avoid this situation, it is proposed to adding one more threshold of “Minimum number of models for defining requirement for each band”, and the recommended number is 12. The total number of models is gotten from the summary of statistical information provided by RAN4 Secretary.

Proposal 1: Add one more threshold for specifying FR1 MIMO OTA performance requirement, i.e. Minimum number of models for defining requirement for each band: 12.

Furthermore, if the amount of collected measurement data highly exceeds the minimum threshold, e.g. more than 15 models are measured during the campaign, it is proposed to merge those measurement data from the same model. The method of merging data can be further discussed. The intention to merge the data from the same model is that the same model has the same antenna system arrangement, the same antenna pattern design and the same base-band processor in the device, which fundamentally lead to quite similar MIMO OTA performance. Based on this understanding, the measurement data of the same model is propriate to be treated as one data in the data pool.

Proposal 2: If the amount of collected measurement data highly exceeds the minimum threshold, e.g. more than 15 models are measured during the campaign, it is proposed to merge those measurement data from the same model. The method of merging data can be further discussed.

- Conclusions

This contribution provides our considerations on FR1 MIMO OTA measurement campaign and data processing in the data pool. And provide the following proposals.

Proposal 1: Add one more threshold for specifying FR1 MIMO OTA performance requirement, i.e. Minimum number of models for defining requirement for each band: 12.

Proposal 2: If the amount of collected measurement data highly exceeds the minimum threshold, e.g. more than 15 models are measured during the campaign, it is proposed to merge those measurement data from the same model. The method of merging data can be further discussed.

- References

[1] R4-2321137 Updated Framework and time plan for FR1 MIMO OTA performance requirements development (Nov 2023), CAICT

[2] R4-2321099 WF on [109][335] NR_FR1_TRP_TRS_enh, vivo