3GPP TSG-RAN WG4 Meeting #110	R4-2401863

# Athens, Greece, February 26 – March 1, 2024

Source:	Nokia, Nokia Shanghai Bell

Title:	Discussion on Case 1 requirements

Agenda item:	8.5.1.1

Document for:	Discussion

Introduction

This paper presents our view on remaining RRM core aspects related to further enhancements of measurement gaps for Rel-18 [1]. In particular, it presents our view on remaining open issues for Case 1 requirements as discussed at RAN4 #109 [2] [3] i.e. the combination of pre-configured MG and concurrent MG and lists corresponding proposals.

Discussion

Normative work

At RAN4 #109, Case 1 requirements were handled in the normative work. The Big CR [4] was agreed including collision handling for Case 1 requirements. The respective clauses 8.19.5 and 9.1.12 from 38.133 V18.4.0 are duplicated below.

### 8.19.5 Activation/deactivation delay requirements for multiple pre-configured measurement gaps The requirements in this clause apply to a UE configured with multiple pre-configured measurement gaps within concurrent gaps. #### 8.19.5,1 Activation/deactivation delay requirements for non-overlapped activation/deactivation of multiple pre-configured measurement gaps The requirements in this clause only apply when the activation/deactivation procedures of the individual pre-configured measurement gaps do not overlap in time. When multiple pre-configured measurement gaps are activated/deactivated non-overlapped upon DCI/timer-based BWP switch, upon SCell activation/deactivation or upon RRC reconfiguration, for each individual pre-configured measuremeng gap, the requirements defined in clauses 8.19.2, 8.19.3 and 8.19.4 apply. #### 8.19.5.2 Activation/deactivation delay requirements for fully overlapped activation/deactivation of multiple pre-configured measurement gaps The requirements in this clause only apply when the activation/deactivation procedures of the individual pre-configured measurement gaps fully overlap in time. Fully overlapped activation/deactivation of pre-configured measuremeng gaps can occur in the following cases: - Both pre-configured measurement gaps are triggered by the same event. - Two pre-configured measurement gaps are triggered by two events of the same type at the same time. When multiple pre-configured measuremeng gaps are activated/deactivated simultaneously, the activation/deactivation delay equals the delay requirements defined at clause 8.19.2, 8.19.3 and 8.19.4 for DCI/timer-based BWP switch, SCell activation/deactivation or RRC reconfiguration, respectively, plus an additional 2ms post-processing time.

#### 8.19.5.3 Pre-configured measurement gap activation/deactivation delay when colliding with a measurement gap When the pre-configured measurement gap activation procedure is overlapped with one measurement gap occasion and the pre-configured measurement gap has higher priority, the pre-configured gap activation shall be applied 5ms after the overlapping measurement gap. When the pre-configured measurement gap deactivation procedure is overlapped with one measurement gap occasion and the pre-configured measurement gap has higher priority, or when the pre-configured measurement gap activation/deactivation procedure is overlapped with one measurement gap occasion and the measurement gap has higher priority, requirements specified in 8.19.2 apply.

### 9.1.12 Concurrent measurement gaps with Pre-MG #### 9.1.12.1 Introduction When UE supports [ConMGs with Pre-MG] capability, network can provide multiple measurement gap patterms with at least one of the measurement gaps is pre-configured measurement gap (Pre-MG) pattern configured by RRC message(s) as specified in TS 38.331 [2]. Requirements in this section apply when the UE is in SA operation mode. #### 9.1.12.2 Requirements If the UE requires measurement gaps and/or Pre-MGs to identify and measure intra-frequency cells and/or inter-frequency cells and/or inter-RAT E-UTRAN cells, and the UE supports [ConMGs with Pre-MG] but does not support independent measurement gap patterns for different frequency ranges as specified in Table 5.1-1 in [18, 19, 20], in order for the requirements in the following clauses to apply, the network can provide the UE with not more than two per-UE measurement gap patterns for monitoring all the frequency layers. Editor Notes: FFS additional capability for UE to support Pre-MG+Pre-MG Two Pre-MGs may be configured by the network only if the UE supports [Pre-MG+Pre-MG]. If the UE supports both [ConMGs with Pre-MG] and independent measurement gap patterns for different frequency ranges as specified in Table 5.1-1 in [18, 19, 20], in order for the requirements defined for concurrent measurement gaps with Pre-MG to apply, the network can provide the measurement gap patterns’combinations specified in Table 9.1.12.1 for monitoring of all frequency layers. Table 9.1.12-1: The number of Gap Combination Configurations by UE supporting both [ConMGs with Pre-MG] and independent measurement gap patterns Gap Combination Configuration Id; The number of simultaneous configured measurement gap patterns. Per-FR1 measurement gap; Per-FR2 measurement gap; Per-UE measurement gap. 0; 2; 1; 0. 1; 1; 2; 0. 2; 0; 0; 2. 3Note 1; 1; 0; 1. 4Note 1; 0; 1; 1. 5Note 1; 1; 1; 1. 6; 2; 0; 0. 7; 0; 2; 0. Note 1: Gap Combination Configuration Id #3, #4, #5 are only applicable when the per-UE measurement gap is associated to measure PRS for any RSTD, PRS-RSRP, UE Rx-Tx time difference measurement and PRS-RSRPP measurement defined in TS 38.215 [4]. [Note 2]: For UE capable of [Concurrent Pre-MG], up to 2 measurement gap patterns can be configured as Pre-MG in one FR, regardless of whether they are per-UE or per-FR configuration. Otherwise, the gaps can only be configured as Type-1/2 MG. Note 3: In Gap Combination Configuration Id #0, #1, #6, #7, one per-FR measurement gap in an FR can be associated to measure PRS for any RSTD, PRS-RSRP, PRS-RSRPP, RSCP, RSCPD and UE Rx-Tx time difference measurement defined in TS 38.215 [4] provided that UE supports independentGapConfigPRS-r17.. When UE supports [Concurrent Pre-MG], the gap association for a frequency layer is configured by the network via [gapAssociation] in [ToBeMeasureConfig]. In this case the gap association rules in clause 9.1.8.2 shall also apply to either measurement gap or Pre-MG. When autonomous mechanism [1] is used for activation/deactivation of Pre-MG pattern, the UE shall autonomously determine the Pre-MG status only based on the measurement objects associated with the concerned Pre-MG. The related Pre-MG autonomous activation/deactivation mechanism is specified in clause 9.1.7.3.1. When network-controlled mechanism [1] is used for activation/deactivation, the requirements specified in clause 9.1.7.3.2 apply. When UE supports [Concurrent Pre-MG], where at least one of the concurrent gaps is Pre-MG, for a measurement gap pattern supported by the UE is listed in Table 9.1.2-1 based on the applicability specified in table 9.1.2-3. The requirements in clause 9.1.2 are applicable for the UE capable of [Concurrent Pre-MG] and configured with multiple concurrent measurement gap patterns within each activated Pre-MG pattern. #### 9.1.12.3 Collisions involving Pre-MG(s) Collisions between a Pre-MG and a measurement gap may occur only when the Pre-MG is activated. No collisions can occur between a per-FR Pre-MG and a per-FR measurement gap when they are configured in different FRs. Collisions between two Pre-MGs may occur only when both Pre-MGs are activated. No collisions can occur between a per-FR Pre-MGs when they are configured in different FRs. The requirements for [concurrent measurement gaps with Pre-MG] apply provided that the two measurement gaps(at least one of the gaps is activated Pre-MG) colliding with each other are configured with different priorities. #### 9.1.12.4 Collision between Pre-MG activation/deactivation and measurement gap A measurement gap occasion and a Pre-MG activation/deactivation procedure collide when the ending point of the Pre-MG activation/deactivation procedure occurs anywhere within a time period starting 4ms before the starting point of the gap occasion and ending 4ms after the ending point of the gap occasion. The ending point of the Pre-MG activation/deactivation procedure are defined in clause [8.x.y]. When a collision occurs between a measurement gap occasion and a Pre-MG activation procedure, and the Pre-MG is configured with higher priority, the UE shall perform measurements during the measurement gap occasion and the activation of the Pre-MG is delayed until 5ms after the ending point of the measurement gap occasion. When a collision occurs between a measurement gap occasion and a Pre-MG deactivation procedure, and the Pre-MG is configured with higher priority, the measurement gap occasion shall be dropped. The measurement gap occasion shall remain to be dropped until the ending point of the Pre-MG deactivation procedure. When the activated Pre-MG and measurement gap meets the collision rule defined in 9.1.8.3 and the Pre-MG is configured with lower priority, the UE shall perform measurements in the occasion of the measurement gap regardless of whether colliding with the Pre-MG activation procedure. The UE is expected to transmit PUCCH/PUSCH/SRS or receive PDCCH/PDSCH/TRS/CSI-RS for CQI in the corresponding NR serving cells in the slots of the configured Pre-MG that are dropped according to the requirements in clause 9.1.8.4.

Remaining open issues

Collision handling for dynamic collisions

Collision handling was discussed for following 4 scenarios [3]:

- Scenario 1: the pre-configured MG activation procedure is overlapped with one of concurrent gap occasion during the dynamic collision (i.e. Pre-MG has higher priority than the MG)

- Scenario 2: pre-configured MG deactivation procedure is overlapped with one of concurrent gap occasion during the dynamic collision (i.e. Pre-MG has higher priority than the MG)

- Scenario 3: pre-configured MG activation procedure is overlapped with one of concurrent gap occasion where the MG has higher priority than the Pre-MG.

- Scenario 4: One pre-configured MG deactivation procedure is overlapped with another pre-configured MG activation procedure during the dynamic collision (This scenario is for Pre-MG + Pre-MG).

Considering the mapping of above scenarios to requirements in clauses 8.19.5 and 9.1.12, this is listed in Table 1.

| Scenario 1 | Pre-MG activation procedure collides with concurrent gap occasion | Pre-MG has higher priority than concurrent MG | Subclause 9.1.12.4 UE shall perform measurements during the measurement gap occasion and the activation of the Pre-MG is delayed until 5ms after the ending point of the measurement gap occasion. | Subclause 8.19.5.3 the pre-configured gap activation shall be applied 5ms after the overlapping measurement gap. |
| --- | --- | --- | --- | --- |
| Scenario 2 | Pre-MG deactivation procedure collides with concurrent gap occasion | Pre-MG has higher priority than concurrent MG | Subclause 9.1.12.4 the measurement gap occasion shall be dropped. The measurement gap occasion shall remain to be dropped until the ending point of the Pre-MG deactivation procedure. | Subclause 8.19.5.3 requirements specified in 8.19.2 apply: Subclause 8.19.2.1 UE shall be able to finish pre-configured activation or deactivation within 5 ms after the completion of the active BWP switch. If the end of activation/deactivation of Pre-MG is within a gap occasion, the Pre-MG status shall not be changed immediately. Instead, the Pre-MG status shall be changed prior to the next gap occasion. |
| Scenario 3 | Pre-MG activation procedure collides with concurrent gap occasion | Pre-MG has lower priority than concurrent MG | Subclause 9.1.12.4 the UE shall perform measurements in the occasion of the measurement gap regardless of whether colliding with the Pre-MG activation procedure | Subclause 8.19.5.3 Same requirements as for scenario 2. |
| Scenario 3A | Pre-MG deactivation procedure collides with concurrent gap occasion | Pre-MG has lower priority than concurrent MG | Subclause 9.1.12.4 No requirements is specified. | Subclause 8.19.5.3 Same requirements as for scenario 2. |
| Scenario 4A | Pre-MG 1 activation procedure collides with Pre-MG 2 deactivation procedure | Pre-MG1 has higher priority than Pre-MG2 | Subclause 9.1.12.3 / 9.1.12.4 No requirements are specified. | Subclause 8.19.5.2 Requirements for no overlap or full overlap are specified only. Full overlap: When multiple pre-configured measuremeng gaps are activated/deactivated simultaneously, the activation/deactivation delay equals the delay requirements defined at clause 8.19.2, 8.19.3 and 8.19.4 for DCI/timer-based BWP switch, SCell activation/deactivation or RRC reconfiguration, respectively, plus an additional 2ms post-processing time. |
| Scenario 4B | Pre-MG 1 activation procedure collides with Pre-MG 2 deactivation procedure | Pre-MG1 has lower priority than Pre-MG2 | Subclause 9.1.12.3 / 9.1.12.4 No requirements are specified. | Subclause 8.19.5.2 Same requirements as for Scenario 4 |

Table 1: Mapping of identified collision scenarios to subclauses in 38.133.

From Table 1, following observations are made.

For Scenario 1, according to subclause 9.1.12.4, Pre-MG is only activated 5 ms after the ending point of the concurrent MG occasion. This should be aligned in subclause 8.19.5.3: “the pre-configured gap activation shall be applied 5ms after the ending point of the overlapping measurement gap.”

For Scenario 2, according to subclause 9.1.12.4, Pre-MG deactivation is to be performed immediately and concurrent MG is dropped, whilst subclause 8.19.5.3 points to subclause 8.19.2.1 which specifies (only for the case of DCI/timer based BWP switch, hence missing out SCell activation/deactivation and RRC reconfiguration scenarios), if the end of activation/deactivation of Pre-MG is within a gap occasion, the Pre-MG status shall not be changed immediately but shall be changed prior to the next gap occasion. Hence both subclauses are not aligned. If Pre-MG deactivation has higher priority, then Pre-MG deactivation should be prioritized over reception in concurrent MG occasion, as specified in 9.1.12.4. Then concurrent MG occasion is dropped, and the deactivation delay is the same as for single Pre-MG. Thus subclause 8.19.5.3 should state that the Pre-MG deactivation delay requirements are the same as for single Pre-MG, i.e. no overlap of Pre-MG deactivation procedure and gap occasion, in case of DCI/timer-based BWP switch, SCell activation/deactivation or RRC reconfiguration, respectively.

For Scenario 3, the requirements in subclauses 9.1.12.4 and 8.19.5.3 are not clear. In subclause 9.1.12.4 the paragraph starts with “When the activated Pre-MG and measurement gap meets the collision” and ends with “regardless of whether colliding with the Pre-MG activation procedure. Hence it is not clear whether Scenario 3 is referred or not.

The difference between Scenarios 1 and 3 is the different priority of the Pre-MG. Given that in Scenario 1 with higher Pre-MG priority, concurrent MG occasion is not dropped and Pre-MG activation is delayed to 5 ms after the endpoint of the concurrent MG occasion, the same UE behaviour is reasonable to be applied in case of lower priority configured for Pre-MG. Note this only affects the collision between Pre-MG activation procedure and concurrent MG occasion, not the collision between actual Pre-MG occasion and concurrent MG occasion. Thus, requirements for Scenario 3 in subclauses 9.1.12.4 and 8.19.5.3 need to be modified such, that they are same as for Scenario 1.

The scenario of Pre-MG deactivation colliding with concurrent MG occasion with Pre-MG being assigned a lower priority is added in Scenario 3A. In this case reception of concurrent MG should take priority and the deactivation procedure is carried out after concurrent MG occasion similar to Scenario 1 and 3. This needs to be corrected in both subclauses.

For Scenario 4A, when Pre-MG 1 to be activated has higher priority than Pre-MG 2 to be deactivated, no requirements exist in subclauses 9.1.12.3 and 9.1.12.4. Only subclause 8.19.5.2 specifies for the full overlap (simultaneous trigger(s) for both Pre-MGs, independent of configured priorities, a longer activation/deactivation delay by 2ms compared to single Pre-MG activation. Assuming that no requirements will be specified for partial overlap, according to previous RAN4 agreements [5], subclause 9.1.12.3 should refer to subclause 8.19.5.2 in case of full overlap.

For Scenario 4B, when Pre-MG 1 to be activated has lower priority than Pre-MG 2 to be deactivated, the same applies as for Scenario 4A. Thus, the same spec changes should be done as for Scenario 4A.

From the above, the following proposals are made.

For Scenario 1, subclause 8.19.5.3 should state that the pre-configured gap activation shall be applied 5ms after the ending point of the overlapping measurement gap.

For Scenario 2, subclause 8.19.5.3 should state that the Pre-MG deactivation delay requirements are the same as for single Pre-MG in 8.19.2.1 or 8.19.3 or 8.19.4 for DCI/timer-based BWP switch, SCell activation/ deactivation and RRC reconfiguration, respectively, i.e. no overlap of deactivation procedure and gap occasion.

For Scenario 3, requirements in subclauses 9.1.12.4 and 8.19.5.3 should be same as for Scenario 1.

For Scenario 3A, requirements in subclauses 9.1.12.4 and 8.19.5.3 should be added and are aligned to Pre-MG activation procedure in Scenario 1 and 3.

For Scenarios 4A and 4B, subclause 9.1.12.3 should refer to subclause 8.19.5.2 in case of full overlap of Pre-MG activation and other Pre-MG deactivation, assuming that no requirements will be defined for partial overlap.

UE capability for dynamic collisions

RAN4 #109 did not get further progress on the issue whether to define a new UE capability for dynamic collisions [3].

Issue 2-1-4: [Case 1] Whether to define a new UE capability for dynamic collisions? - Proposals - Option 1: Huawei, MTK, QC, vivo - Add a UE capability to indicate whether the UE supports Case 1 gap combinations that cause dynamic collisions. - Option 2: CATT, Nokia, ZTE, CMCC, E///, OPPO, China Telecom - No additional capability is needed to handle the dynamic collision. - Recommended WF - Discuss the options.

Due to the RAN4 #107 agreement to consider only collisions between an activated Pre-MG and a concurrent MG, we do not see a justification to introduce a new UE capability for such dynamic collisions, where Pre-MG status changes between activated and de-activated, as Rel-17 collision rules based on configured priorities are valid in case of Pre-MG instances after Pre-MG activation.

No new UE capability for dynamic collisions is needed for UEs supporting Case 1 requirements, as Rel-17 collision rules based on configured priorities are valid in case of Pre-MG instances after Pre-MG activation.

Pre-MG association

RAN4 #109 did not get further progress on the issue whether the Pre-MG association needs to be clarified [3].

Issue 2-2-1: [Case 1] Pre-MG association clarification - Proposals - Option 1: E///, HW, MTK, vivo, Apple - When NW configures a Pre-MG1 and a Pre-MG2/Type-2 MG in ConMGs, the MO associated with Pre-MG1 will be measured within activated Pre-MG2/Type-2 MG if Pre-MG1 is deactivated and the MO is fully overlapping with activated Pre-MG2/Type-2 MG. - Option 1a: HW - FFS: whether it need to be captured in spec - Option 2: Nokia - For Pre-MG association with deactivated Pre-MG1 and activated Pre-MG2, RAN4 to distinguish the cases: - no overlap or partial overlap of Pre-MG1 and Pre-MG2: UE is required to perform all measurements assigned to Pre-MG1 outside the activated Pre-MG2 (e.g. search for SMTCs outside the MG). - full overlap of Pre-MG1 and Pre-MG2: UE is required to perform all measurements assigned to Pre-MG1 in Pre-MG2 and drop all measurements assigned to Pre-MG2, if Pre-MG1 is configured with higher priority than Pre-MG2, or UE is required to perform all measurements assigned to Pre-MG2 in Pre-MG2 and drop all measurements assigned to Pre-MG1, if Pre-MG1 is configured with lower priority than Pre-MG2. - Option 2a: OPPO - The MO associated with Pre-MG1 is not allowed to be measured with activated Pre-MG2/Type-2 MG without explicated signalling. - Option 3: MTK - No need to introduce implicit association for concurrent gaps with Pre-MG. - Recommended WF - When NW configures a Pre-MG1 and a Pre-MG2/Type-2 MG in ConMGs, the MO associated with Pre-MG1 will be measured within activated Pre-MG2/Type-2 MG if Pre-MG1 is deactivated and the MO is fully overlapping with activated Pre-MG2/Type-2 MG. - Provided that the SSB is within the active BWP of the MO associated with Pre-MG1. - FFS: whether it need to be captured in spec

In our view, the introduction of implicit association for concurrent MG with Pre-MG according to option 1 will yield to additional complexity for managing concurrent gaps both at UE and NW side, since MO’s need to be transferred between gaps, which is done by RRC configuration up to date. Thus, we prefer not to continue discussion on this issue for Rel-18. We support Option 3 above.

There is no need to introduce implicit association for concurrent gaps with Pre-MG.

Pre-MG association collisions related issue

RAN4 #109 further discussed optimizations of the collision handling for concurrent gaps [3].

Issue 2-2-2: [Case 1] Pre-MG association collisions related issue - Proposals - Option 1: Xiaomi - The measurement dropping rules when the concurrent measurement gaps are collided can be optimized, e.g. - Option 1). Only there is overlapping among the [SSBs+Xms] to be measured by these collided concurrent gaps, UE needs to drop the measurement with the lower priority gap. Otherwise, UE can perform these measurements sequentially because UE can return to each of carriers one by one - Recommended WF - Moderator’s understanding: This is the last meeting of core part, hence, unless there is a critical issue with the existing agreement then there is no need to further enhance the collision rules. - Collect views.

In our view, such optimizations in the core part will lead to increased complexity of the core requirements for NR_MG_enh2 and hence will delay the performance requirements work. Thus, this issue should not be further discussed for Rel-18.

There is no need to continue discussion on optimizations of the collision handling for concurrent gaps in Rel-18.

Conclusion

This paper has presented Nokia’s views on Case 1 requirements as discussed at RAN4 #108bis [2], i.e. the combination of pre-configured MG and concurrent MG.

Resulting from this discussion, we make following proposals:

- For Scenario 1, subclause 8.19.5.3 should state that the pre-configured gap activation shall be applied 5ms after the ending point of the overlapping measurement gap.

- For Scenario 2, subclause 8.19.5.3 should state that the Pre-MG deactivation delay requirements are the same as for single Pre-MG in 8.19.2.1 or 8.19.3 or 8.19.4 for DCI/timer-based BWP switch, SCell activation/deactivation and RRC reconfiguration, respectively, i.e. no overlap of deactivation procedure and gap occasion.

- For Scenario 3, requirements in subclauses 9.1.12.4 and 8.19.5.3 should be same as for Scenario 1.

- For Scenario 3A, requirements in subclauses 9.1.12.4 and 8.19.5.3 should be added and are aligned to Pre-MG activation procedure in Scenario 1 and 3.

- For Scenarios 4A and 4B, subclause 9.1.12.3 should refer to subclause 8.19.5.2 in case of full overlap of Pre-MG activation and other Pre-MG deactivation, assuming that no requirements will be defined for partial overlap.

No new UE capability for dynamic collisions is needed for UEs supporting Case 1 requirements, as Rel-17 collision rules based on configured priorities are valid in case of Pre-MG instances after Pre-MG activation.

There is no need to introduce implicit association for concurrent gaps with Pre-MG.

There is no need to continue discussion on optimizations of the collision handling for concurrent gaps in Rel-18.

- References

- RP-232440, “Revised WID: Further Enhancements on NR and MR-DC Measurement Gaps and Measurements without Gaps”, Intel, MediaTek

- R4-2321640, “WF for NR_MGE-2_part1”, Moderator (MediaTek inc.)

- R4-2318166, “Topic summary for [109][210] NR_MG_enh2_part1”, Moderator (MediaTek inc.)

- R4-2321637, “Big CR to TS 38.133 on Further enhancements on NR and MR-DC measurement gaps and measurements without gaps”, MediaTek inc., Intel Corporation

- R4-2317305, “WF on NR_MG_enh2_part1”, MediaTek Inc., RAN4 #108bis