# 3GPP TSG-RAN WG4 Meeting #110                                                              R4-2401880

# Athens, GR, 26 Feb – 01 Mar, 2024

Agenda Item:	7.11.2

Source: 	CMCC

Title: 	TP for TR 38.718-03-01 to add CA_n3-n34-n40

Document for:	Approval

- Introduction

- The request of CA_n3-n34-n40 have been approved in last meeting.

- In this contribution, we will analyze the ∆TIB,c and ∆RIB,c values and the IMD studies for these band combinations.

- Reference

- .

- TP to TR 38.718-03-01

## <<Start of Change for TR 38.718-03-01>>

5.x	CA_n3-n34-n40

5.x.1	Common for 1 band UL and 2 bands UL CA

5.x.1.1	Operating bands for CA

Table 5.x.1.1-1: Inter-band CA operating bands involving FR1 (three bands)

| NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| n3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
| n34 | 2010 MHz | – | 2025 MHz | 2010 MHz | – | 2025 MHz | TDD |
| n40 | 2300 MHz | – | 2400 MHz | 2300 MHz | – | 2400 MHz | TDD |

5.x.1.2	Channel bandwidths per operating band for CA

Table 5.x.1.2-1: Supported bandwidths per CA band combination of band n3+n28+n34

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n3A-n34A-n40A | CA_n3A-n34A CA_n3A-n40A CA_n34A-n40A | n3 | See n3 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n34 | See n34 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n40 | See n40 channel bandwidths in Table 5.3.5-1 |  |

5.x.1.3	∆TIB,c and ∆RIB,c values

For CA_n3-n34-n40, the TIB,c and RIB,c values are reused from CA_n1-n3-n40 and are given in the tables below.

Table 5.x.1.3-1: ΔTIB,c due to NR CA (three bands)

| Inter-band CA combination | ΔTIB,c for NR bands (dB)8 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration9 |  |  |
| CA_n3-n34-n40 | 0.5 | 0.5 | 0.5 |
| NOTE 8: “-” denotes ΔTIB,c = 0. NOTE 9: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3-n5 the band order from left to right is n1, n3 and n5. |  |  |  |

Table 5.1.1.3-2: ΔRIB,c due to NR CA (three bands)

| Inter-band CA combination | ΔRIB,c for NR bands (dB)9 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration10 |  |  |
| CA_n3-n34-n40 | - | - | - |
| NOTE 9: “-” denotes ΔRIB,c = 0. NOTE 10: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3-n8 the band order from left to right is n1, n3 and n8. |  |  |  |

5.x.2	Specific for 2 bands UL CA

5.x.2.1	UE co-existence studies

UE co-existence has been already studied for 2DL/1UL fallback combinations such as CA n3-n34, CA_n3-n40 and n34-n40 and the impact of harmonic interference has been clarified. The own Rx impact of the 3rd band is shown as the followings.

–   3th order IMD generated by dual uplink of Band n3 + Band n34 fall into own Rx of Band n40.

–   there is no additional IMD issue generated by dual uplink of Band n3 + Band n40 fall into own Rx of Band n34.

–   there is no additional IMD issue generated by dual uplink of Band n34 + Band n40 may fall into own Rx of Band n3.

5.x.2.2	REFSENS requirements

Table 5.x.2.2-1 lists the MSD required for the dual connectivity configuration for the cases that IMD interference fall into the own 3rd Rx frequency band.

Table 5.x.2.2-1: MSD for the CA configuration

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n3-n34-n40 | n3 | TBD | TBD | TBD | TBD | TBD | FDD | N/A |
|  | n34 | TBD | TBD | TBD | TBD | TBD | TDD | N/A |
|  | n40 | TBD | TBD | TBD | TBD | TBD | TDD | IMD51 |
| NOTE 1: This band is subject to IMD5 also which MSD is not specified. |  |  |  |  |  |  |  |  |

## <<End Change for TR 38.718-03-01>>