3GPP TSG-RAN WG4 Meeting # 110	R4-2401913

Athens, Greece, Feb. 26 – Mar. 1, 2024

Agenda Item: 8.16.1.3

Source: MediaTek Inc.

Title: Discussion on UE feature list for Objective 7

Document for: Discussion

- Introduction

In this contribution, UE feature list for Objective 7 will be discussed.

- Discussion

According to the exception sheet RP-233969 approved in RAN#103 with following essential part, only solutions based on existing measurement are kept.

| Expected Completion Date: | RAN#103 / March 2024 |
| --- | --- |
| Service(s) impacted: | NR Mobility |
| Specification(s) affected: | TS 38.331 (RAN2) |
| Task(s) within work which are not complete: | WI objective#7, focus on solution based on existing measurement, as below: - RAN2 to define time-based measurement result validation configuration based on RAN4 agreements. - RAN2 signaling to enable reporting of cell reselection measurement or EMR for fast CA/DC setup. - NOTE 1: RAN4 shall not work on any new requirements for this functionality in Rel-18. Only essential corrections are allowed. - NOTE 2: If RAN2 is not able to complete the work, the functionality will be removed from Rel-18. - NOTE 3: Existing measurement means that no additional measurement is performed during RRC Setup/Resume procedure. |
| Consequences if not included in Release 18: | Essential functionality and signalling for SCell/SCG setup delay improvement are not supported. |

As agreed last meeting, the solutions based on existing measurement are applicable to two cases:

- One for UE supporting R16 EMR and EMR is configured

- Another for UE not supporting R16 EMR or early measurement is not configured

According to the exception sheet, RAN2 will design the signalling to enable reporting of cell reselection measurement or EMR for fast CA/DC setup. It is likely the reporting mechanism of cell reselection measurement and EMR are different. We prefer to introduce two UE capabilities, one for each case. UE can have the flexibility to support one of the cases or both of them.

Proposal 1: Introduce two UE capabilities for improvement on SCell/SCG setup delay:

- One for the case that UE supports R16 EMR and EMR is configured

- Another for the cased that UE does not support R16 EMR or early measurement is not configured

- Summary

In this paper, we discussed the UE feature list for objective 7. We have the following proposal.

Proposal 1: Introduce two UE capabilities for improvement on SCell/SCG setup delay:

- One for the case that UE supports R16 EMR and EMR is configured

- Another for the cased that UE does not support R16 EMR or early measurement is not configured