3GPP TSG-RAN WG4 Meeting#110      	R4-2402081

Athens, Greece, February 26 – March 01, 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-1 | CR | < > | rev | - | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | draftCR to 38.101-1 Corrections for aerial NR UEs |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | R4 |  |  |  |  |  |  |  |  |  |
| Source to TSG: | Nokia Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_UAV |  |  |  |  |  | Date: |  |  | 2024-02-14 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Corrections for aerial NR UEs. |  |  |  |  |  |  |  |  |
| Summary of change: |  | As discussed in R4-2402079 some of the definition for aerial NR UEs can be simplified and other corrected as shown in this CR. Also, formatting have been corrected for the clause headers |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The definitions for aerial NR UEs is wrong. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2K.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.521 series |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

***************************** Start of changes ************************************

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

Aerial UE: A UE supporting UAS (Uncrewed Aircraft Systems) as indicated by the capability aerialUE-Capability-r18 [15] and that has an aerial subscription as described in TS 23.256 [17]. The UE is considered to have access to UAS services after the UE ha performed a successful authentication and authorization with the USS as described in TS 23.256 [17].

Aggregated Channel Bandwidth: The RF bandwidth in which a UE transmits and receives multiple contiguously aggregated carriers.

ATG UE: The terminals or user equipments which are mounted in aircraft and support ATG feature as defined in clause x.x.x. from TS38.306.

Carrier aggregation: Aggregation of two or more component carriers in order to support wider transmission bandwidths.

Carrier aggregation band: A set of one or more operating bands across which multiple carriers are aggregated with a specific set of technical requirements.

Carrier aggregation bandwidth class: A class defined by the aggregated transmission bandwidth configuration and maximum number of component carriers supported by a UE.

Carrier aggregation configuration: A combination of CA operating band(s) and CA bandwidth class(es) supported by a UE.

Con-current operation: The simultaneous transmission and reception of sidelink and Uu interfaces while operation is agnostic of the service used on each interface.

Contiguous carriers: A set of two or more carriers configured in a spectrum block where there are no RF requirements based on co-existence for un-coordinated operation within the spectrum block.

Contiguous resource allocation: A resource allocation of consecutive resource blocks within one carrier or across contiguously aggregated carriers. The gap between contiguously aggregated carriers due to the nominal channel spacing is allowed.

Contiguous spectrum: Spectrum Enhanced channel raster: channel raster with a 10 kHz granularity in bands with a 100 kHz channel raster.consisting of a contiguous block of spectrum with no sub-block gaps.

Inter-band carrier aggregation: Carrier aggregation of component carriers in different operating bands.

NOTE:	Carriers aggregated in each band can be contiguous or non-contiguous.

Inter-band con-current operation: Operation of NR Uu carrier and NR Sidelink carrier in different operating bands.

Intra-band contiguous carrier aggregation: Contiguous carriers aggregated in the same operating band.

Intra-band non-contiguous carrier aggregation: Non-contiguous carriers aggregated in the same operating band.

Intra-band SL CA UE: UE that supports NR SL CA operation in a single band

NR SL CA: Aggregation of two or more NR Sidelink component carriers in order to support wider transmission bandwidths

NR SL inter-band con-current operating Band：Band combinations of NR Uu carrier and NR Sidelink carrier in different operating bands.

NR SL-U UE: UE that supports NR Sidelink operation in unlicensed bands (e.g. n46, n96, n102).

RedCap UE: The UE with reduced capabilities as defined in clause 4.2.21.1 from TS38.306 [15].

Sub-band:  For a UE that supports shared spectrum channel access in wideband operation, a sub-band is the set of RBs within an approximately 20 MHz segment of the channel where the wideband channel is uniformly divided into an integer number of 20 MHz sub-bands.  Sub-bands may be separately allocated in uplink and downlink.

Sub-block: This is one contiguous allocated block of spectrum for transmission and reception by the same UE. There may be multiple instances of sub-blocks within an RF bandwidth.

Sub-block bandwidth: The bandwidth of one sub-block.

Sub-block gap: A frequency gap between two consecutive sub-blocks within an RF bandwidth, where the RF requirements in the gap are based on co-existence for un-coordinated operation.

UE transmission bandwidth configuration: Set of resource blocks located within the UE channel bandwidth which may be used for transmitting or receiving by the UE.

Vehicular UE: A UE embedded in a vehicle, permanently connected to an embedded antenna system that radiates externally for NR operating bands.

NOTE:	Vehicular UE does not refer to other UE form factors placed inside the vehicle.

Wideband operation: For a UE that supports shared spectrum channel access, wideband operation refers to operation within a channel larger than 20 MHz in which intra-cell guard bands may be configured to distinguish individual RB-sets

***************************** Unchanged Clauses have been Omitted **********************************

### 6.2K.2	Maximum output power reduction for Aerial UE

### For Aerial UE, the requirements specified in clause 6.2.2 apply.

### 6.2K.3	Additional maximum output power reduction for Aerial UE

#### 6.2K.3.1	General

Additional emission requirements can be signalled by the network. Each additional emission requirement is associated with a unique network signalling (NS) value indicated in RRC signalling by an NR frequency band number of the applicable operating band and an associated value in the field [additionalSpectrumEmissionUAV]. Throughout this specification, the notion of indication or signalling of an NS_UAV value refers to the corresponding indication of an NR frequency band number of the applicable operating band, the IE field frequencyBandListAerial and an associated value of additionalSpectrumEmission in the relevant RRC information elements [7].

To meet the additional requirements, additional maximum power reduction (A-MPR) is allowed for the maximum output power as specified in Table 6.2.1-1. Unless stated otherwise, the total reduction to UE maximum output power is max(MPR, A-MPR) where MPR is defined in clause 6.2.2. Outer and inner allocation notation used in clause 6.2K.3 are defined in clause 6.2.2. Unless stated otherwise, Edge RB allocations get the same AMPR as Outer RB allocations. In absence of modulation and waveform types the A-MPR applies to all modulation and waveform types.

Table 6.2K.3.1-1 specifies the additional requirements with their associated Network Signalling label and the allowed A-MPR and applicable operating band(s). The mapping of NR frequency band numbers and values of the additionalSpectrumEmission to network signalling labels is specified in Table 6.2.3.1-1A.

For almost contiguous allocations in CP-OFDM waveforms in power class 3, the allowed A-MPR defined in clause 6.2K.3 is increased by CEIL{ 10 log10(1 + NRB_gap / NRB_alloc), 0.5 } dB, where CEIL{x, 0.5} means x rounding upwards to closest 0.5dB, NRB_gap is the total number of unallocated RBs between allocated RBs and NRB_alloc is the total number of allocated RBs, and the parameter LCRB is replaced by NRB_alloc + NRB_gap in specifying the RB allocation regions.

Table 6.2K.3.1-1: Additional Maximum Power Reduction (A-MPR) for Uncrewed Aerial UE

| Network signalling label | Requirements (clause) | NR Band | Channel bandwidth (MHz) | Resources blocks (NRB) | A-MPR (dB) |
| --- | --- | --- | --- | --- | --- |
| NS_UAV_44 | 6.5K.3.3.1 | 38 | 25, 30, 40 | Table 6.2K.3.2-1 | Table 6.2K.3.2-2 |
| NS_UAV_46 | 6.5K.3.3.2 | 7 | 5, 10, 15, 20, 25, 30, 35, 40, 50 |  | N/A |
| NS_UAV_70 | 6.5K.3.3.3 | 3 | [5, 10, 15, 20, 25, 30, 35, 40, 45, 50] | Table 6.2K.3.3-1 | Table 6.2K.3.3-2 |

Table 6.2K.3.1-1A: Mapping of network signalling label

| NR band | Value of additionalSpectrumEmission |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| n3 | NS_UAV_01 | NS_UAV_70 |  |  |  |  |  | Reserved |
| n7 | NS_UAV_01 | NS_UAV_46 |  |  |  |  |  | Reserved |
| n38 | NS_UAV_01 | NS_UAV_44 |  |  |  |  |  | Reserved |

#### 6.2K.3.2	A-MPR for NS_UAV_44

Table 6.2K.3.2-1: A-MPR regions for NS_UAV_44

| Channel Bandwidth, MHz | Carrier Center Frequency, Fc, MHz | Regions |  | A-MPR |
| --- | --- | --- | --- | --- |
|  |  | RBend*12*SCS MHz | LCRB*12*SCS MHz |  |
| 25 MHz | 2582.5≤ FC ≤ 2602.5 | <18.0 | > 12*SCS* RBend - 3.6 | A3 |
|  |  | ≥18.0 | <7.2 | A3 |
|  |  | ≥18.0 | ≥7.2 | A6 |
| 30 MHz | 2585 ≤ FC ≤ 2600 | <21.6 | > 12*SCS* RBend - 3.6 | A3 |
|  |  | ≥21.6 | <12.6 | A3 |
|  |  | ≥21.6 | ≥12.6 | A6 |
| 40 MHz | 2590 ≤ FC ≤ 2595 | ≥0, <2.88 | >0 | A1 |
|  |  | ≥2.88, <14.4 | >, 12*SCS*RBend - 3.6 | A2 |
|  |  | ≥14.4, <23.4 | >10.8 | A3 |
|  |  | ≥23.4, <32.4 | >16.2 | A4 |
|  |  | ≥32.4 | >0 | A5 |

Table 6.2K.3.2-2: A-MPR for NS_UAV_44

| Modulation/Waveform |  | A1 | A2 | A3 | A4 | A5 | A6 |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner |
| DFT-s-OFDM | PI/2 BPSK | 5 | 2 | 3 | 7 | 12 | 4 |
|  | QPSK | 5 | 2 | 3 | 7 | 12 | 4 |
|  | 16 QAM | 5 | 2 | 3 | 7 | 12 | 4 |
|  | 64 QAM | 5 |  | 3 | 7 | 12 | 4 |
|  | 256 QAM | 5 |  |  | 7 | 12 |  |
| CP-OFDM | QPSK | 5 | 4 | 5 | 8 | 12 | 6 |
|  | 16 QAM | 5 | 4 | 5 | 8 | 12 | 6 |
|  | 64 QAM | 5 | 4 | 5 | 8 | 12 | 6 |
|  | 256 QAM | 5 |  |  | 8 | 12 |  |

#### 6.2K.3.3	A-MPR for NS_UAV_70

Table 6.2K.3.3-1: A-MPR regions for NS_UAV_70 (Power Class 3)

| Channel | Carrier Center | Regions |  | A-MPR |
| --- | --- | --- | --- | --- |
| Bandwidth, [MHz] | Frequency, Fc, [MHz] | RBstart*12*SCS [MHz] | LCRB*12*SCS [MHz] |  |
| 5 MHz | FC ≤ 1722.5 | ≤ 0.18 | ≤ 2.34 | A1 |
|  |  |  | > 2.34 | A3 |
|  |  | > 0.18, ≤ 0.36 | ≤ 0.36 | A2 |
|  |  |  | > 0.36 | A3 |
|  |  | > 0.36, ≤ 0.72 | < 1.8 | A4 |
|  |  |  | ≥ 1.8 | A3 |
|  |  | > 0.72 | > 1.5*RBstart*12*SCS - 2.5*12*SCS | A4 |
|  |  |  | ≤ 1.5*RBstart*12*SCS – 2.5*12*SCS | A5 |
| 10 MHz | FC ≤ 1735 | ≤ 0.72 | ≤ 1.26 | A1 |
|  |  |  | > 1.26 | A2 |
|  |  | > 0.72, ≤ 3.78 | > 1.5*RBstart*12*SCS - 2.5*12*SCS | A4 |
|  |  |  | ≤ 1.5*RBstart*12*SCS - 2.5*12*SCS | A5 |
|  |  | > 3.78, ≤ 5.04 | > 2.7 | A5 |
|  |  |  | ≤ 2.7 | A6 |
|  |  | > 5.04 | > 0 | A5 |
| 15 MHz | FC ≤ 1747.5 | ≤ 0.72 | ≤ 1.26 | A1 |
|  |  |  | > 1.26 | A2 |
|  |  | > 0.72, ≤ 5.58 | > 1.5*RBstart*12*SCS - 2.5*12*SCS | A4 |
|  |  |  | ≤ 1.5*RBstart*12*SCS - 2.5*12*SCS | A5 |
|  |  | > 5.58, ≤ 9.72 | > 3.78 | A5 |
|  |  |  | ≤ 3.78 | A6 |
|  |  | > 9.72 | > 0 | A5 |
| 20 MHz | FC ≤ 1760 | ≤ 0.72 | ≤ 8.1 | A1 |
|  |  |  | > 8.1 | A4 |
|  |  | > 0.72, ≤ 7.2 | > 1.5*RBstart*12*SCS - 2.5*12*SCS | A4 |
|  |  |  | ≤ 1.5*RBstart*12*SCS - 2.5*12*SCS | A5 |
|  |  | > 7.2, ≤ 12.96 | > 5.58 | A5 |
|  |  |  | ≤ 5.58 | A6 |
|  |  | > 12.96 | > 0 | A5 |
| 25 MHz | FC ≤ 1772.5 | ≤ 0.72 | > 0 | A1 |
|  |  | > 0.72, ≤ 8.64 | > 1.5*RBstart*12*SCS - 2.5*12*SCS | A4 |
|  |  |  | ≤ 1.5*RBstart*12*SCS - 2.5*12*SCS | A5 |
|  |  | > 8.64 | > 6.12 | A5 |
|  |  |  | ≤ 6.12, > 0.72 | A6 |
|  |  |  | ≤ 0.72 | A5 |
| 30 MHz | FC ≤ 1750 | ≤ 24.3 | > 0 | A2 |
|  |  | > 24.3 | > 0 | A5 |
|  | 1750 < FC < 1765 | ≤ 24.3 | > 0 | A5 |
|  |  | > 24.3 | > 0 | A6 |
|  | FC ≥ 1765 | ≥ 0 | > 0 | A5 |
| 35 MHz | FC ≤ 1750 | ≤ 29.16 | > 0 | A2 |
|  |  | > 29.16 | > 0 | A5 |
|  | 1750 < FC < 1765 | ≤ 29.16 | > 0 | A5 |
|  |  | > 29.16 | > 0 | A6 |
|  | FC ≥ 1765 | ≥ 0 | > 0 | A6 |
| 40 MHz | FC ≤ 1740 | ≤ 32.4 | > 0 | A2 |
|  |  | > 32.4 | > 0 | A5 |
|  | FC > 1740 | ≤ 32.4 | > 0 | A5 |
|  |  | > 32.4 | > 0 | A6 |
| 45 MHz | FC ≤ 1740 | ≤ 35.64 | > 0 | A2 |
|  |  | > 35.64 | > 0 | A5 |
|  | FC > 1740 | ≤ 35.64 | > 0 | A5 |
|  |  | > 35.64 | > 0 | A6 |
| 50 MHz | FC ≤ 1740 | ≤ 38.88 | > 0 | A2 |
|  |  | > 38.88 | > 0 | A5 |
|  | FC > 1740 | ≤ 38.88 | > 0 | A5 |
|  |  | > 38.88 | > 0 | A6 |
| NOTE 1: The A-MPR values are listed in Table 6.2K.3.2-2. NOTE 2: For any undefined region, MPR applies. |  |  |  |  |

Table 6.2K.3.3-2: A-MPR for NS_UAV_70 (Power Class 3)

| Modulation/Waveform |  | A1 | A2 | A3 | A4 | A5 | A6 |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner |
| DFT-s-OFDM | PI/2 BPSK | 30 | 25 | 20 | 15 | 10 | 5 |
|  | QPSK | 30 | 25 | 20 | 15 | 10 | 5 |
|  | 16 QAM | 30 | 25 | 20 | 15 | 10 | 5 |
|  | 64 QAM | 30 | 25 | 20 | 15 | 10 | 5 |
|  | 256 QAM | 30 | 25 | 20 | 15 | 10 |  |
| CP-OFDM | QPSK | 30 | 25 | 20 | 15 | 10 | 5 |
|  | 16 QAM | 30 | 25 | 20 | 15 | 10 | 5 |
|  | 64 QAM | 30 | 25 | 20 | 15 | 10 | 5 |
|  | 256 QAM | 30 | 25 | 20 | 15 | 10 |  |
| NOTE 1: The backoff applied is max (MPR, A-MPR) where MPR is defined in Table 6.2.2-1 |  |  |  |  |  |  |  |

***************************** Unchanged Clauses have been Omitted **********************************

## 6.5K	Output RF spectrum emissions for Aerial UE

### 6.5K.1	Occupied bandwidth for Aerial UE

For Aerial UE, the requirements specified in clause 6.5.1 apply.

### 6.5K.2	Out of band emission for Aerial UE

For Aerial UE, the requirements specified in clause 6.5.2 apply.

### 6.5K.3	Spurious emissions for Aerial UE

#### 6.5K.3.0	General

For Aerial UE, the requirements specified in clause 6.5.3.0 apply.

#### 6.5K.3.1	General spurious emissions

For Aerial UE, the requirements specified in clause 6.5.3.1 apply.

#### 6.5K.3.2	Spurious emissions for UE co-existence

For Aerial UE, the requirements specified in clause 6.5.3.2 apply.

#### 6.5K.3.3	Additional spurious emissions

##### 6.5K.3.3.1	Requirement for network signalling value "NS_UAV_44"

When "NS UAV_44" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.5K.3.3.1-1. This requirement also applies for the frequency ranges that are less than FOOB (MHz) in Table 6.5.3.1-1 from the edge of the channel bandwidth.

Table 6.5K.3.3.1-1: Additional requirements for "NS_UAV_44"

| Frequency range (MHz) | Channel bandwidth (MHz) / Spectrum emission limit (dBm) | Measurement bandwidth | Note |
| --- | --- | --- | --- |
|  | 5, 10, 15, 20, 25, 30, 40 MHz |  |  |
| 2620 ≤ f < 2645 | -15.5 | 5 MHz | 1 |
| 2645 ≤ f < 2690 | -40 | 1 MHz |  |
| 2690 ≤ f ≤ 2900 | -50 | 1 MHz |  |
| NOTE 1: For these adjacent bands, the emission limit could imply risk of harmful interference to UE(s) operating in the protected operating band. |  |  |  |

##### 6.5K.3.3.2	Requirement for network signalling value "NS_UAV_46"

When "NS_UAV_46" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.5K.3.3.2-1. This requirement also applies for the frequency ranges that are less than FOOB (MHz) in Table 6.5.3.1-1 from the edge of the channel bandwidth.

Table 6.5K.3.3.2-1: Additional requirements for "NS_UAV_46"

| Frequency range (MHz) | Channel bandwidth (MHz) / Spectrum emission limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
|  | 5, 10, 15, 20, 25, 30, 35, 40, 50 MHz |  |
| 1675 ≤ f ≤ 1710 | -40 | 1 MHz |

##### 6.5K.3.3.3	Requirement for network signalling value “NS_UAV_70"

When "NS UAV_70" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.5K.3.3.3-1. This requirement also applies for the frequency ranges that are less than FOOB (MHz) in Table 6.5.3.1-1 from the edge of the channel bandwidth.

Table 6.5K.3.3.3-1: Additional requirements for "NS_UAV_70"

| Frequency range (MHz) | Channel bandwidth (MHz) / Spectrum emission limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
|  | 5, 10, 15, 20, 25, 30, 35, 40, 45, 50 MHz |  |
| 1675 ≤ f ≤ 1710 | -40 | 1 MHz |

***************************** End of changes ************************************