3GPP TSG-RAN4 Meeting #110	R4-2402177

# Athens, Greece, 26 February – 1 March, 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | xxxx | rev | - | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | draftCR on RRM requirements for SL positioning |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_pos_enh2-Core |  |  |  |  |  | Date: |  |  | 2024-01-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - SL PRS measurement period for multiple Tx UE is FFS - SL PRS-RSRP(P) is assisting measurement for all SL positioning methods. There is no need to define separate requirements for SL PRS-RSRP(P), instead SL PRS-RSRP(P) should follow the requirements defined for the main measurement for the corresponding method. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Update the SL PRS measurement period for multiple Tx UE - Void the clauses for SL PRS-RSRP(P) requirements |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Requirements for SL PRS measurement period are incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 12A |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change 1>

# 12A	NR Sidelink Measurements for Positioning

## 12A.1	Introduction

Clause 12A contains requirements for UE capable of V2X or 5G ProSe operation, which is also capable of performing SL positioning measurements defined in TS 38.215 [4], including SL RSTD, SL PRS-RSRP, SL Rx-Tx time difference, SL PRS-RSRPP measurements, SL AoA, and SL RTOA, provided that:

- 	The SL PRS are received on NR PC5 interface within a single sidelink BWP on a single carrier,

-	The UE is in any cell selection state or the UE is inside NG-RAN coverage while configured for SL positioning operation on a sidelink carrier, which is dedicated to only sidelink operation, and configured with only a PCell on WAN carrier, and

-	The UE is not required to monitor PSCCH, which is associated with SL-PRS in the same slot, outside the SL-DRX active time.

NOTE 1:	Any cell selection state refers to a UE that is out of network coverage and is not associated with a serving cell on any carrier as defined in TS 38.304 [1].

NOTE 2:	When a UE in RRC_CONNECTED state is performing transmissions and/or reception for SL positioning operation, the UE shall meet all the requirements specified in Clause 9 assuming that UE has a dedicated RX/TX chain for the sidelink operation. Otherwise, the UE may interrup the SL positioning measurements in order to meet the measurement requirements specified in Clause 9.

[Prior to performing SL-PRS based measurements, the UE may need to perform the discovery procedure according to TBD].

## 12A.2	SL-RSTD measurements

12A.2.1	Introduction

The requirements in clause 12A.2 shall apply provided the UE has received [SLPP-RequestLocationInformation] message from LMF or another UE via SLPP [37] requesting the UE to measure and report SL RSTD measurements defined in TS 38.215 [4] based on SL-PRS.

12A.2.2	Requirements Applicability

The requirements in clause 12A.2 apply for periodic, aperiodic, and triggered RSTD measurements, provided:

-	SL RSTD related side conditions given in clause TBD for FR1 are fulfilled, for a corresponding Band.

12A.2.3	Measurement Capability

UE SL RSTD measurement capability is as indicated by the UE in [sl-TDOA-ProvideCapabilities], according to TS 38.355 [37].

12A.2.4	Measurement Reporting Requirements

The measurement reporting delay is defined as the time between the moment when the measurement report is triggered and the moment when the UE starts to transmit the measurement report over the air interface.

For UE reporting to LMF, this requirement assumes that the measurement report is not delayed by other SLPP signalling on the DCCH. This measurement reporting delay excludes a delay uncertainty resulted when inserting the measurement report to the TTI of the uplink DCCH. The delay uncertainty is: 2 x TTIDCCH where TTIDCCH is the duration of subframe or slot or subslot when the measurement report is transmitted on the PSSCH with subframe or slot or subslot duration.

For UE reporting to another UE, this requirement assumes that the measurement report is not delayed by other SLPP signalling on the STCH. This measurement reporting delay excludes a delay uncertainty resulted when inserting the measurement report to the TTI of the transmitted STCH. The delay uncertainty is: 2 x TTISTCH where TTISTCH is the duration of subframe or slot or subslot when the measurement report is transmitted on the PSSCH with subframe or slot or subslot duration.

This measurement reporting delay excludes any delay caused by no SL resources for UE to send the measurement report.

The reported SL RSTD measurement values contained in measurement reports shall be based on the measurement report mapping requirements specified in clauses TBD.

The SL RSTD measurements performed and reported according to this section shall meet the SL RSTD measurement accuracy requirements in clause TBD, for each measured SL-PRS resource.

12A.2.5 Measurements Period Requirements

When the physical layer receives last of [sl-TDOA-ProvideAssistanceData] message and [sl-TDOA-RequestLocationInformation] message from LMF or another UE via SLPP [37], the UE shall be able to perform at least [TBD] SL RSTD measurements, with each SL RSTD measurement including measurement on the measured target link and the reference link, defined in TS 38.215 [4], during the measurement period TSL RSTD,Total defined as:

TSL RSTD,Total=s=1STSL RSTD,effect,s  ,

where

S is the number of samples per measured link, define below:

S = 1 for SL-PRS BW>48 PRBs,

S = 4 for 24 PRBs ≤SL-PRS BW≤48 PRBs,

TSL RSTD,effect,s is defined as:

TSL RSTD,effect,s=ts+1-ts , for s<S, where ts and ts+1 are the beginning of the slots where UE receives the s-th and (s+1)-th SCI from Tx UE #j, respectively, triggering SL PRS measurement, while both the number of active slots and number of active resources per slot for the ongoing measurement are below the capabilities [TBD] and [TBD] of the Rx UETSL RSTD,effect,s≥Tdur,s+ΔSLprocts+1ts,

TSL RSTD,effect,s=Tdur,s+ΔSLproc , for s=S,

Tdur,s is the SL-PRS duration for SL-PRS sample s of the SL RSTD measurement,

ΔSLproc=[TBD] is the processing time as indicated via capability [TBD] of the Rx UE.

The start of the measurement period for Tx UE #j is defined as the beginning of the slot where the Rx UE receives the first SCI from Tx UE #j triggering SL PRS measurement, while both the number of active slots and number of active resources per slot for the ongoing measurement are below the capabilities [TBD] and [TBD] of the Rx UE.

If SL PRS-RSRP and/or SL PRS-RSRPP are configured for SL-TDOA, SL RSTD and SL PRS-RSRP and/or SL PRS-RSRPP are performed over the same measurement period.

If the synchronization reference source changes during T… at the measuring UE or [at the UE configured to transmit SL-PRS] for the target measured or reference link [37, clause TBD] for the SL RSTD measurement, while the UE is performing the SL RSTD measurement, then the UE shall restart the SL RSTD measurement after the synchronization reference source change.

[FFS: If the synchronization reference source changes at the measuring UE or at the UE configured to transmit SL-PRS for the target measured or reference link for the SL RSTD measurement, while the measuring UE is performing the SL RSTD measurement, then the measuring UE shall restart the SL RSTD measurement and shall send the measurement report no later than:

TSL RSTD,restart=K+1*TSL RSTD, Total ,

where K is the number of restarts due to the synchronization source changes.]

## 12A.3	Void

TSL PRS-RSRP,Total

TSL PRS-RSRP,Total=s=1STSL PRS-RSRP,effect,s

S

S

TSL PRS-RSRP,effect,s=

ts+1-tsTSL PRS-RSRP,effect,s≥Tdur,s+ΔSLprocts+1ts

TSL PRS-RSRP,effect,s=Tdur,s+ΔSLproc ,

Tdur,s

ΔSLproc=[TBD]

TSL PRS-RSRP,Total

## 12A.4	SL-Rx-Tx measurements

#### 12A.4.1	Introduction

The requirements in clause 12A.4 shall apply provided the UE has received [NR-SL-RxTx-RequestLocationInformation] from LMF or another UE via SLPP [TBD] requesting the UE to measure and report SL Rx-Tx time difference measurements defined in TS 38.215 [4] based on SL-PRS.

#### 12A.4.2	Requirements Applicability

The requirements in clause 12A.4 apply for [periodic,] aperiodic, and triggered SL Rx-Tx time difference measurements, provided:

-	SL Rx-Tx time difference related side conditions given in clause [TBD] for FR1 are met for a corresponding Band.

-	The actual time difference between the corresponding SL-PRS transmission and reception used to derive the measurement is no larger than [160] ms.

#### 12A.4.3	Measurement Capability

SL Rx-Tx time difference measurement capability is as indicated by the UE in [NR-SL-RxTx-ProvideCapabilities] according to TS 38.355 [TBD].

#### 12A.4.4	Measurement Reporting Requirements

The measurement reporting delay is defined as the time between the moment when the measurement report is triggered and the moment when the UE starts to transmit the measurement report over the air interface.

For UE report to LMF, this requirement assumes that the measurement report is not delayed by other SLPP signalling on the DCCH. This measurement reporting delay excludes a delay uncertainty resulted when inserting the measurement report to the TTI of the uplink DCCH. The delay uncertainty is: 2 x TTIDCCH where TTIDCCH is the duration of subframe or slot or subslot when the measurement report is transmitted on the PSSCH with subframe or slot or subslot duration.

For UE report to another UE, this requirement assumes that the measurement report is not delayed by other SLPP signalling on the STCH. This measurement reporting delay excludes a delay uncertainty resulted when inserting the measurement report to the TTI of the sidelink STCH. The delay uncertainty is: 2 x TTISTCH where TTISTCH is the duration of subframe or slot or subslot when the measurement report is transmitted on the PSSCH with subframe or slot or subslot duration.

The measurement reporting delay excludes any delay caused by no SL resources for UE to send the measurement report.

The reported SL Rx-Tx time difference measurement values contained in measurement reports shall be based on the measurement report mapping requirements specified in clause [TBD].

The SL Rx-Tx time difference measurements performed and reported according to this section shall meet the SL Rx-Tx time difference measurement accuracy requirements in clause [TBD], for each measured SL-PRS resource.

#### 12A.4.5	Measurement Period Requirements

When the physical layer receives [NR-SL-RxTx-ProvideAssistanceData] message from [NR-SL-RxTx-RequestLocationInformation] message from LMF or another UE via SLPP [TBD], the UE shall be able to perform multiple at least TBD [FFS: (up to the UE capability specified in Clause [TBD])] SL Rx-Tx time difference measurements, defined in TS 38.215 [4], during TSL RxTx,total defined as:

TSL RxTx,total=s=1STSL RxTx,effect,s+Tuncertain

Where,

S is the number of samples for a single SL Rx-Tx measurement defined below:

S = 1 for SL-PRS BW>48 PRBs,

S = 4 for 24 PRBs ≤SL-PRS BW≤48 PRBs

TSL RxTx,effect,s is defined as:

TSL RxTx,effect,s=ts+1-ts, for s<S, where ts and ts+1 are the beginning of the slots where UE receives the s-th and (s+1)-th SCI from Tx UE #j, respectively, triggering SL PRS measurement, while both the number of active slots and number of active resources per slot for the ongoing measurement are below the capabilities [TBD] and [TBD] of the Rx UE,TSL RxTx,effect,s≥Tdur,s+ΔSLproctsts+1

TSL RxTx,effect,s=Tdur,s+ΔSLproc , for s=S,

Tdur,s is the duration of SL-PRS resources of the s-th sample,

ΔSLproc=[TBD] is the processing time as indicated via capability [TBD] of the Rx UE.

Tuncertain is defined as below:

- If the UE reports the transmission timestamp of a SL PRS as defined in TS 38.215 [4], and the SL PRS transmission occurs after the SL PRS reception used to derive the measurement, Tuncertain is the additional time delay from the SL PRS reception until the actual SL PRS transmission.

- Otherwise, Tuncertain=0.

The start of the measurement period for Tx UE #j is defined as the beginning of the slot where the Rx UE receives the first SCI from Tx UE #j triggering SL PRS measurement, while both the number of active slots and number of active resources per slot for the ongoing measurement are below the capabilities [TBD] and [TBD] of the Rx UE.

If SL PRS-RSRP and/or SL PRS-RSRPP are configured for SL-RTT, SL Rx-Tx time difference and SL PRS-RSRP and/or SL PRS-RSRPP are performed over the same measurement period.

[FFS: If the synchronization reference source changes at the measuring UE or at the UE configured to transmit SL-PRS for the measurement, while the measuring UE is performing the SL Rx-Tx time difference measurement, then the measuring UE shall restart the SL Rx-Tx time difference measurement and shall send the measurement report no later than:

TSL RxTx,restart=K+1*TSL RxTx, Total ,

where K is the number of restarts due to the synchronization source changes.]

Editor’s note: FFS whether to limit the number of restarting.

## 12A.5	Void

## 12A.6	SL-AoA measurements

#### 12A.6.1	Introduction

The requirements in clause 12A.6.5 shall apply provided the UE has received [NR-SL-AOA-RequestLocationInformation] from LMF or another UE via SLPP requesting the UE to measure and report SL AoA measurements defined in TS 38.215 [4].

#### 12A.6.2	Requirements Applicable

The requirements in clause 12A.6 apply for periodic, aperiodic, and triggered SL AoA measurements, provided:

-	SL AoA related side conditions given in clause [TBD] for FR1 are met for a corresponding Band.

#### 12A.6.3	Measurement Capability

SL AoA measurement capability is as indicated by the UE in [NR-SL-AOA-ProvideCapabilities] according to TS 38.355 [37].

#### 12A.6.4	Measurement Reporting Requirements

The measurement reporting delay is defined as the time between the moment when the measurement report is triggered and the moment when the UE starts to transmit the measurement report over the air interface.

For UE reporting to LMF, this requirement assumes that the measurement report is not delayed by other SLPP signalling on the DCCH. This measurement reporting delay excludes a delay uncertainty resulted when inserting the measurement report to the TTI of the uplink DCCH. The delay uncertainty is: 2 x TTIDCCH where TTIDCCH is the duration of subframe or slot or subslot when the measurement report is transmitted on the PSSCH with subframe or slot or subslot duration.

For UE reporting to another UE, this requirement assumes that the measurement report is not delayed by other SLPP signalling on the STCH. This measurement reporting delay excludes a delay uncertainty resulted when inserting the measurement report to the TTI of the sidelink STCH. The delay uncertainty is: 2 x TTISTCH where TTISTCH is the duration of subframe or slot or subslot when the measurement report is transmitted on the PSSCH with subframe or slot or subslot duration.

The measurement reporting delay excludes any delay caused by no SL resources or no SL-PRS resources for UE to send the measurement report.

The reported SL AoA measurement values contained in measurement reports shall be based on the measurement report mapping requirements specified in clauses [TBD].

[The SL AoA measurements performed and reported according to this section shall meet the SL AoA measurement accuracy requirements in clause [TBD], for each measured SL-PRS resource.]

Editor’s note: FFS whether to define accuracy requirements for SLAoA.

#### 12A.6.5	Measurement Period Requirements

When the physical layer receives [NR-SL-AOA-ProvideAssistanceData] message from [NR-SL-AOA-RequestLocationInformation] message from LMF or another UE via SLPP, the UE shall be able to measure multiple [FFS: (up to the UE capability specified in Clause [TBD])] SL AoA measurements, defined in TS 38.215 [4], during TSL AoA,total defined as:

TSL AoA,total=s=1STSL AoA, effect,s ,

where,

S is the number of samples for the SL AoA measurement, defined as below:

  - S = 1 for SL-PRS BW>48 PRBs,

  - S = 4 for 24 PRBs ≤SL-PRS BW≤48 PRBs,

TSL AoA,effect,s is defined as below,

TSL AoA,effect,s=ts+1-ts, for s<S, where ts and ts+1 are the beginning of the slots where UE receives the s-th and (s+1)-th SCI from Tx UE #j, respectively, triggering SL PRS measurement, while both the number of active slots and number of active resources per slot for the ongoing measurement are below the capabilities [TBD] and [TBD] of the Rx UETSL AoA,effect,s≥Tdur,s+TSLproctsts+1,

TSL AoA,effect,s=Tdur,s+ΔSLproc , for s=S,

Tdur,s is the SL-PRS duration for SL-PRS sample s of the SL AoA measurement,

ΔSLproc=[TBD] is the processing time as indicated via capability [TBD] of the Rx UE.

The start of the measurement period for Tx UE #j is defined as the beginning of the slot where the Rx UE receives the first SCI from Tx UE #j triggering SL PRS measurement, while both the number of active slots and number of active resources per slot for the ongoing measurement are below the capabilities [TBD] and [TBD] of the Rx UE.

If SL PRS-RSRP and/or SL PRS-RSRPP are configured for SL-AoA, SL AoA and SL PRS-RSRP and/or SL PRS-RSRPP are performed over the same measurement period.

The requirements in clause 12A.6 do not apply if the SL-PRS configuration given by [NR-SL-PRS-AssistanceData exceeds] exceeds any of the UE measurement capabilities given by [NR-SL-PRS-ResourcesCapability in NR-SL-AOA-ProvideCapabilities].

When a synchronization reference source change occurs at the measuring UE or at the UE configured to transmit SL-PRS for the SL AoA measurement, while the UE is performing the SL AoA measurement, then the UE shall continue and complete the on-going SL AoA measurements while meeting the measurement period requirement in this clause[ and the accuracy requirements in clause [TBD]].

## 12A.7	SL-RTOA measurements

#### 12A.7.1	Introduction

The requirements in clause 12A.7.5 shall apply provided the UE has received [NR-SL-RTOA-RequestLocationInformation] from LMF or another UE via SLPP requesting the UE to measure and report SL RTOA measurements defined in TS 38.215 [4].

#### 12A.7.2	Requirements Applicable

The requirements in clause 12A.7 apply for periodic, aperiodic, and triggered SL RTOA measurements, provided:

-	SL RTOA related side conditions given in clause [TBD] for FR1 are met for a corresponding Band.

#### 12A.7.3	Measurement Capability

SL RTOA measurement capability is as indicated by the UE in [NR-SL-RTOA-ProvideCapabilities] according to TS 38.355 [37].

#### 12A.7.4	Measurement Reporting Requirements

The measurement reporting delay is defined as the time between the moment when the measurement report is triggered and the moment when the UE starts to transmit the measurement report over the air interface.

For UE reporting to LMF, this requirement assumes that the measurement report is not delayed by other SLPP signalling on the DCCH. This measurement reporting delay excludes a delay uncertainty resulted when inserting the measurement report to the TTI of the uplink DCCH. The delay uncertainty is: 2 x TTIDCCH where TTIDCCH is the duration of subframe or slot or subslot when the measurement report is transmitted on the PSSCH with subframe or slot or subslot duration.

For UE reporting to another UE, this requirement assumes that the measurement report is not delayed by other SLPP signalling on the STCH. This measurement reporting delay excludes a delay uncertainty resulted when inserting the measurement report to the TTI of the sidelink STCH. The delay uncertainty is: 2 x TTISTCH where TTISTCH is the duration of subframe or slot or subslot when the measurement report is transmitted on the PSSCH with subframe or slot or subslot duration.

The measurement reporting delay excludes any delay caused by no SL resources or no SL-PRS resources for UE to send the measurement report.

The reported SL RTOA measurement values contained in measurement reports shall be based on the measurement report mapping requirements specified in clauses [TBD].

[The SL RTOA measurements performed and reported according to this section shall meet the SL RTOA measurement accuracy requirements in clause [TBD], for each measured SL-PRS resource.]

Editor’s note: FFS whether to define accuracy requirements for SL RTOA.

#### 12A.7.5	Measurement Period Requirements

When the physical layer receives [NR-SL-RTOA-ProvideAssistanceData] message from [NR-SL-RTOA-RequestLocationInformation] message from LMF or another UE via SLPP, the UE shall be able to measure multiple [FFS: (up to the UE capability specified in Clause [TBD])] SL RTOA measurements, defined in TS 38.215 [4], during TSL RTOA,total defined as:

TSL RTOA,total=s=1STSL RTOA, effect,s ,

where,

S is the number of samples for the SL RTOA measurement, defined as below:

  - S = 1 for SL-PRS BW>48 PRBs,

  - S = 4 for 24 PRBs ≤SL-PRS BW≤48 PRBs,

TSL RTOA,effect,s is defined as below,

TSL RTOA,effect,s=ts+1-ts, for s<S, where ts and ts+1 are the beginning of the slots where UE receives the s-th and (s+1)-th SCI from Tx UE #j, respectively, triggering SL PRS measurement, while both the number of active slots and number of active resources per slot for the ongoing measurement are below the capabilities [TBD] and [TBD] of the Rx UETSL RTOA,effect,s≥Tdur,s+TSLproctsts+1,

TSL RTOA,effect,s=Tdur,s+ΔSLproc , for s=S,

Tdur,s is the SL-PRS duration for SL-PRS sample s of the SL RTOA measurement,

ΔSLproc=[TBD] is the processing time as indicated via capability [TBD] of the Rx UE.

The start of the measurement period for Tx UE #j is defined as the beginning of the slot where the Rx UE receives the first SCI from Tx UE #j triggering SL PRS measurement, while both the number of active slots and number of active resources per slot for the ongoing measurement are below the capabilities [TBD] and [TBD] of the Rx UE.

If SL PRS-RSRP and/or SL PRS-RSRPP are configured for SL-TOA, SL RTOA and SL PRS-RSRP and/or SL PRS-RSRPP are performed over the same measurement period.

The requirements in clause 12A.6 do not apply if the SL-PRS configuration given by [NR-SL-PRS-AssistanceData exceeds] exceeds any of the UE measurement capabilities given by [NR-SL-PRS-ResourcesCapability in NR-SL-RTOA-ProvideCapabilities].

[If the synchronization reference source changes at the measuring UE, while the UE is performing the SL RTOA measurements, the UE shall restart the SL RTOA measurements after the synchronization reference source change.]

Editor’s note: FFS whether to limit the number of restarting.

<End of Change 1>