3GPP TSG-RAN WG4 Meeting #110	R4-2402190

Athens, Greece, 26 February – 1 March, 2024

Title: 	On performance requirements for SRS/SRS CA

Source: 	Huawei, HiSilicon

Agenda item:	8.14.3.4

Document for:	Approval

Introduction

RRM core requirements for SRS/SRS CA are discussed in RAN4#109, and the outcomes are captured in WF [1]. The following issues need to be discussed for the performance part.

- Report mapping

- Measurement accuracy

- RRM test case

In this paper we will provide our views on performance requirements for SRS/SRS CA positioning.

Discussion

Report mapping

RAN1 has agreed to support reporting granularity k = {-3, -4, -5, -6} in addition to {-1, -2} [2]. In last meeting, RAN4 has agreed on the number of reporting entries for k = {-3, -4, -5, -6} [3].

One issue is that the number of entries for differential RSTD and UE Rx-Tx is incorrect in [3]. The maximum reported quantity value and the total number of reported quantity values should decrease by 1 compared to the values in [3]. A draft LS is provided in the Annex.

Proposal 1: RAN4 to correct the report mapping for differential RSTD and UE Rx-Tx.

Another issue discussed last meeting is whether the newly introduced reporting granularity is only applicable to aggregated measurements. Our understanding is that they are applicable for both aggregate and non-aggregate measurements.

Agreement The new ReportingGranularityfactor also supports k = {-3, -4, -5, -6} in addition to {-1, -2} - These k values are applicable for timing measurements for all applicable positioning methods - Support for both DL and UL - Support for both FR1 and FR2 - Reply the RAN4 LS R1-2310797, and CC to RAN2 and RAN3.

On one hand, restricting the usable k value based on aggregate/non-aggregate or the aggregated BW will make the spec complex; on the other hand, such restriction will limit UE/TRP implementation since some UE/TRP may choose to use smaller values in certain scenarios or under certain configurations. In any case, the k value to be used for reporting is up to UE\TRP, and we see no strong motivation to make much restriction in the spec.

Proposal 2: The new reporting granularities with k = {-1, -2, -3, -4, -5, -6} are applicable for both aggregate and non-aggregate measurements.

Measurement accuracy

For PRS CA, up to 3 PFLs can be aggregated, and we suggest to define accuracy requirements for 2-PFL and 3-PFL, respectively. In our view, the same channel, Es/Iot side condition and sample number as requirements for single PFL can be reused. The new aspect is on the BW per PFL.

In existing requirements, some small BW are considered such as 5MHz and 10MHz. We do not think they are typical per-PFL BW for PRS CA. On the other hand, some large BW like 100MHz is not considered but they may be typical for PRS CA. We suggest to at least consider 100MHz for 30kHz SCS, and the BWs in Proposal 4 are suggested as a starting point for defining requirements.

Proposal 3: For PRS CA, RAN4 to define RSTD and UE Rx-Tx accuracy requirements for 2-PFL and 3-PFL, based on same channel, Es/Iot and sample number as requirements for single PFL.

Proposal 4: For PRS CA, RAN4 to use the following per PFL BW as starting point to define accuracy requirements for RSTD and UE Rx-Tx.

- 15kHz SCS: 104 (20M)

- 30kHz SCS: 132 (50M), 272 (100M)

- 60kHz SCS FR1: 64 (50M), 132 (100M)

- 60kHz SCS FR2: 64 (50M), 132 (100M)

- 120kHz SCS: 64 (100M), 128 (200M)

RRM test case

RAN4 should define delay and accuracy TCs for PRS CA. The measurement period requirements are defined considering the case where both aggregate and non-aggregate measurements exist from the same set of assistance data. For the testing, we suggest to focus on the aggregate measurement as it is the new and key aspect for PRS CA.

Proposal 5: For RRM test for PRS CA, only set up the PRS resources for aggregate measurement.

Conclusions

In this paper we provided our views on performance requirements for SRS/SRS CA.

Proposal 1: RAN4 to correct the report mapping for differential RSTD and UE Rx-Tx.

Proposal 2: The new reporting granularities with k = {-1, -2, -3, -4, -5, -6} are applicable for both aggregate and non-aggregate measurements.

Proposal 3: For PRS CA, RAN4 to define RSTD and UE Rx-Tx accuracy requirements for 2-PFL and 3-PFL, based on same channel, Es/Iot and sample number as requirements for single PFL.

Proposal 4: For PRS CA, RAN4 to use the following per PFL BW as starting point to define accuracy requirements for RSTD and UE Rx-Tx.

- 15kHz SCS: 104 (20M)

- 30kHz SCS: 132 (50M), 272 (100M)

- 60kHz SCS FR1: 64 (50M), 132 (100M)

- 60kHz SCS FR2: 64 (50M), 132 (100M)

- 120kHz SCS: 64 (100M), 128 (200M)

Proposal 5: For RRM test for PRS CA, only set up the PRS resources for aggregate measurement.

Reference

- R4-2321543, WF on R18 NR positioning – RedCap positioning and PRS/SRS BW aggregation, Ericsson

- R1-2312395, Reply LS on SRS and PRS bandwidth aggregation for positioning, RAN1

- R4-2321545, Response to reply LS on SRS and PRS bandwidth aggregation for positioning, RAN4

Annex: draft Reply LS on SRS and PRS bandwidth aggregation for positioning

3GPP TSG-RAN WG4 Meeting #110 R4-240xxxx Athens, Greece, 26 February – 1 March, 2024 Title: Reply LS on SRS and PRS bandwidth aggregation for positioning Response to: R1-2312395 Release: Release 18 Work Item: NR_pos_enh2-Core Source: RAN4 To: RAN2 Cc: RAN1, RAN3 Contact Person: #### Name: Li Zhang - E-mail Address: zhangli164@huawei.com Attachments: - 1. Overall Description: RAN4 thanks RAN1 for the LS R1-2312395. In RAN4#109, RAN4 provided the report mapping for k = {-3, -4, -5, -6} for timing related measurements in R4-2321545. In RAN4#110, some errors are spotted in the report mapping for differential report for RSTD and UE Rx-Tx time difference measurement. The corrections are shown in red. # Differential RSTD measurement report mapping For k = -3 Minimum reported quantity value: DIFFRSTD_00000 Maximum reported quantity value: DIFFRSTD_65528 Total number of reported quantity values: 65529 For k = -4 Minimum reported quantity value: DIFFRSTD_000000 Maximum reported quantity value: DIFFRSTD_131056 Total number of reported quantity values: 131057 For k = -5 Minimum reported quantity value: DIFFRSTD _000000 Maximum reported quantity value: DIFFRSTD _262112 Total number of reported quantity values: 262113 For k = -6 Minimum reported quantity value: DIFFRSTD _000000000 Maximum reported quantity value: DIFFRSTD _524224 Total number of reported quantity values: 524225 # Differential UE Rx-Tx measurement report mapping For k = -3 Minimum reported quantity value: DIFF_RX-TX_TIME_DIFFERENCE_00000 Maximum reported quantity value: DIFF_RX-TX_TIME_DIFFERENCE_65528 Total number of reported quantity values: 65529 For k = -4 Minimum reported quantity value: DIFF_RX-TX_TIME_DIFFERENCE_000000 Maximum reported quantity value: DIFF_RX-TX_TIME_DIFFERENCE_131056 Total number of reported quantity values: 131057 For k = -5 Minimum reported quantity value: DIFF_RX-TX_TIME_DIFFERENCE_000000 Maximum reported quantity value: DIFF_RX-TX_TIME_DIFFERENCE_262112 Total number of reported quantity values: 262113 For k = -6 Minimum reported quantity value: DIFF_RX-TX_TIME_DIFFERENCE_000000 Maximum reported quantity value: DIFF_RX-TX_TIME_DIFFERENCE_524224 Total number of reported quantity values: 524225. RAN4 respectfully asks RAN2 to take the above information into account in defining signalling for PRS bandwidth aggregation. 2. Actions: To RAN2: RAN4 respectfully asks RAN2 to take the above information into account in defining signalling for PRS bandwidth aggregation. 3. Date of Next TSG-RAN4 Meetings: RAN WG4 Meeting #110-bis Apr 15 – Apr 19, 2024 TBD, China RAN WG4 Meeting #111 May 20 – May 24, 2024 Fukuoka, Japan