3GPP TSG-RAN WG4 Meeting #110	R4-2402251

Athens, Greece, February 26 – March 1, 2024

Source:	Nokia, Nokia Shanghai Bell

Title:	Discussion on mobile IAB conformance testing

Agenda item:	8.25.3 - RF conformance testing	[NR_mobile_IAB-Perf]

Document for:	Approval

Introduction

During RAN4#109 meeting, the core part for mIAB work item [1] was finalized and CR to IAB core specification was agreed in [2].

RAN4#110 will start the discussion on mobile IAB work item performance part, that should be finalized in RAN4#111 meeting. In this contribution we provide some discussion about it.

Discussion

IAB EMC specification TS 38.175

Current revised WID for Mobile IAB RP-233320 [1] doesn’t include IAB EMC specification TS 38.175 updates in clause 5. However, mIAB should be introduced to IAB EMC specification as well, considering both core and test part. EMC specification should include definitions of mobile IAB types, nodes, abbreviations etc. Missing mobile IAB reference in IAB EMC specification would cause missing of any EMC tests for this type of IAB.  Furthermore, to formally update IAB EMC specification, mIAB WID should be updated to include it.

Observation 1: WID for Mobile IAB doesn’t include IAB EMC specification TS 38.175 update.

We already provided in draft CR [3] with initial modification that are needed for EMC core part into TS 38.175. Thus, it is proposed to revise mobile IAB WID to include IAB EMC specification.

Proposal 1: It is proposed to revise mobile IAB WID to include IAB EMC specification TS 38.175.

IAB test specifications TS 38.176-1 and 38.176-2

Additionally, IAB conformance test specifications are not included in mIAB WID [1]. Already agreed changes for IAB core part in [2] required implementation into IAB conformance test specification for conducted 38.176-1 and radiated 38.176-2.

Observation 2: WID for Mobile IAB doesn’t include IAB conformance test specifications TS 38.176-1 and 38.176-2.

As RAN4 already agreed changes to IAB core specification TS 38.174, these changes have to be reflected in IAB conformance test specifications. However, these test specifications have to be listed in WID as impacted.

Thus, it is proposed to revise mobile IAB WID to include IAB conformance test specifications TS 38.176-1 and TS 38.176-2.

Proposal 2: It is proposed to revise mobile IAB WID to include IAB conformance test specifications TS 38.176-1 and TS 38.176-2.

mIAB impact to IAB test specifications

Introduction of mIAB into IAB core specification TS 38.174 requires also respective changes to IAB conformance test specification. At least following issues should be taken into account for mIAB conformance testing:

- Definitions of mobile IAB

- Output power for mIAB-MT test including testing methodology.

Proposal 3: Following issues should be considered for mIAB conformance testing:

- Definition of mobile IAB

- Output power for mIAB-MT test including testing methodology.

Conclusion

In this contribution we discuss the test and conformance test specifications for IAB. We have made following proposals:

Observation 1: WID for Mobile IAB doesn’t include IAB EMC specification TS 38.175 update.

Proposal 1: It is proposed to revise mobile IAB WID to include IAB EMC specification TS 38.175.

Observation 2: WID for Mobile IAB doesn’t include IAB conformance test specifications TS 38.176-1 and 38.176-2.

Proposal 2: It is proposed to revise mobile IAB WID to include IAB conformance test specifications TS 38.176-1 and TS 38.176-2.

Proposal 3: Following issues should be considered for mIAB conformance testing:

- Definition of mobile IAB

- Output power for mIAB-MT test including testing methodology.

- References

- RP-233320, Mobile IAB (Integrated Access and Backhaul) for NR, Qualcomm

- R4-2319785, CR to TS 38.174 on RF core requirements for NR Mobile IAB, Qualcomm Incorporated, Ericsson, Nokia, Nokia Shanghai Bell, NEC

- R4-2402250, Draft CR to TS 38.175 with mobile IAB introduction to EMC specification, Nokia, Nokia Shanghai Bell