3GPP TSG-RAN WG4 Meeting #110	R4-2402318

Athens, Greece, 26 Feb – 01 Mar, 2024

Agenda item:	5.1.3

Source:            	Google Inc.

Title:                	Discussion on IE intraBandENDC-Support

Document for:	Approval

# Introduction

In RAN4#108, an issue of intra-band EN-DC BCS reporting for inter-band EN-DC band combination with two additional intra-band EN-DC components has been raised for discussion [1]. The WF [2] is approved in RAN4#108 to encourage companies to have further investigation on whether current signaling is sufficient to report the BCS(s) of the intra-band EN-DC components for a higher order inter-band EN-DC band combination with two intra-band EN-DC components. The main issue is that some inter-band EN-DC band combinations with two intra-band EN-DC components are introduced in Rel-17, which may cause the current signaling IE to encounter an ambiguous issue if two intra-band EN-DC/NE-DC components, e.g., DC_3A_n3A and DC_41A_n41A, is within an inter-band EN-DC/NE-DC band combination, e.g., DC_3A-41A_n3A-n41A or DC_1A-3A-41A_n3A-n41A, in the following table.

| EN-DC configuration | Uplink EN-DC configuration (NOTE1) |
| --- | --- |
| DC_3A-41A_n3A-n41A | DC_3A_n3A4 DC_3A_n41A DC_41A_n3A |
| DC_1A-3A-41A_n3A-n41A | DC_1A_n3A DC_1A_n41A DC_3A_n3A4 DC_3A_n41A DC_41A_n3A |

In RAN4#109, one discussion paper [3] and one LS to RAN2 [4] are submitted for the further discussion. The LS to RAN2 is almost agreed but one issue is raised to further check another IE intrabandENDC-support impact which is also related to the signalling for the inter-band EN-DC band combinations including intra-band EN-DC components. Hence, in this paper, we would like to share our view on IE intrabandENDC-support for inter-band EN-DC band combinations with two additional intra-band EN-DC components in the following.

# Discussion

In Rel-15, the IE intraBandENDC-Support in the UE capability signaling is introduced for the network to know whether the UE supports contiguous or non-contiguous spectrum for a pure intra-band EN-DC band combination or an inter-band EN-DC band combination with the additional intra-band EN-DC component. In addition, the discussion for intra-band EN-DC combinations is limited to one LTE carrier and one NR carrier at the beginning in Rel-15. For the pure intra-band EN-DC combinations in TS 38.101-3, there are only four band combinations which includes two contiguous spectrums between LTE carrier and NR carrier, e.g., DC_(n)3AA and DC_(n)41AA, and two non-contiguous spectrums between LTE carrier and NR carrier, e.g., DC_41A_n41A and DC_71A_n71A. Hence, the IE can be easily used to distinguish the intra-band contiguous EN-DC band combination from the non-contiguous EN-DC band combination by indicating intraBandENDC-Support=not-reported for intra-band contiguous EN-DC DL/UL configuration or intraBandENDC-Support=non-contiguous for intra-band non-contiguous EN-DC DL/UL configurations. For example, if LTE (DL:3A,UL:3A) and NR (DL:n3A,UL:n3A) with intraBandENDC-Support=non-contiguous is reported, the network can exactly understand that the UE can support DC_3A_n3A with the same DL configuration DC_3A_n3A and UL configuration DC_3A_n3A configuration.

In addition, the similar rule can be also applied to the inter-band EN-DC band combination with the additional intra-band EN-DC component. For example, if LTE (DL:CA_2A-71A,UL:2A) and NR (DL:n71A,UL:71A) with intraBandENDC-Support=non-contiguous is reported, the network can understand that the UE can support DC_2A-71A_n71A with DL configuration DC_2A-71A_n71A and UL configuration DC_2A_n71A.

[38.306][V17.7.0]

| Definitions for parameters | Per | M | FDD-TDD DIFF | FR1-FR2 DIFF |
| --- | --- | --- | --- | --- |
| intraBandENDC-Support Indicates whether the UE supports intra-band (NG)EN-DC with only non-contiguous spectrum, or with both contiguous and non-contiguous spectrum for the (NG)EN-DC combination as specified in TS 38.101-3 [4]. If the UE does not include this field for an intra-band (NG)EN-DC combination the UE only supports the contiguous spectrum for the intra-band (NG)EN-DC combination. If intrabandENDC-Support-UL is absent and the band combination supports intra-band (NG)EN-DC only in DL, this field indicates the DL capability. If intrabandENDC-Support-UL is absent and the band combination supports intra-band (NG)EN-DC in DL and UL, this field indicates the common capability for both DL and UL. If intrabandENDC-Support-UL is included, intraBandENDC-Support indicates the DL capability. | BC | No | N/A | N/A |
| intrabandENDC-Support-UL Indicates whether the UE supports intra-band (NG)EN-DC in UL with only non-contiguous spectrum, or with both contiguous and non-contiguous spectrum for the intra-band (NG)EN-DC combination as specified in TS 38.101-3 [4]. The UE includes this field only if the UE supports different UL and DL capabilities for the intra-band (NG)EN-DC band combination. When 'both' is indicated in intrabandENDC-Support and in intraBandENDC-Support-UL, the UE supports the following three cases of intra-band (NG)EN-DC: contiguous DL/contiguous UL, non-contiguous DL/non-contiguous UL, contiguous DL/non-contiguous UL. | BC | No | N/A | N/A |

In Rel-16, CBRS band b48+n48 related intra-band EN-DC band combinations are introduced in TS 38.101-3. It is observed that the total carriers in these intra-band EN-DC band combinations are larger than two, so the definition for IE intraBandENDC-Support become ambiguous to differentiate intra-band DL EN-DC and UL EN-DC configuration.

For example, if LTE (DL:CA_48C,UL:48A) and NR (DL:n48A,UL:n48A) with intraBandENDC-Support=not reported(contiguous) is reported, it is clear that the UE can support intra-band contiguous DL configuration DC_(n)48CA with contiguous UL configuration DC_(n)48AA but it is not clear about whether non-contiguous UL configuration DC_48A_n48A can be supported.

In another example, if LTE (DL:CA_48A-48A,UL:48A) and (DL:n48A,UL:n48A) with intraBandENDC-Support=non-contiguous is reported, there are two unclear points which is needed to be clarified. One unclear point is that the network cannot distinguish which band combination between DC_48A_(n)48AA and DC_48A-48A_n48A that the UE can support with the reported DC_48A-48A_n48A in the UE capability signaling. The other unclear point is that the network cannot understand whether intra-band EN-DC UL configuration DC_(n)48AA for DC_48A_(n)48AA  can be supported or not.

These issues are first summarized as Case 3 and Case 4 in the approved WF [5] in RAN4#102-e. After 3-year+ long controversial discussion in RAN4 meeting, the agreement has been reached to introduce the new IE intrabandENDC-Support-UL packaged with the IE intrabandENDC-Support to solve the issue for Case 3 and Case 4, where the details can be shown in the following table in RAN2 reply LS [6]. In addition, in our understanding, this rule can be also applied to inter-band EN-DC band combinations with the additional intra-band EN-DC component, e.g., DC_2A-71A_n71A.

| Scenario# | intraBandENDC-Support | intraBandENDC-Support-UL | UE supports in DL / UL (if applicable) |
| --- | --- | --- | --- |
| 1 | Absent (Contiguous) | Absent | - Contiguous/Contiguous |
| 2 | Absent (Contiguous) | Non-contiguous | - Contiguous/Non-contiguous NOTE: “Case 3” in R2-2300060 (R4-2220837) |
| 3 | Non-contiguous | Absent | - Non-contiguous/Non-contiguous |
| 4 | Both | Absent | - Contiguous/contiguous - Non-contiguous/Non-contiguous |
| 5 | Both | Non-contiguous | - Contiguous/Non-contiguous - Non-contiguous/Non-contiguous |
| 6 | Absent (Contiguous) | Both | - Contiguous/Contiguous - Contiguous/Non-contiguous NOTE: “Case 4” in R2-2300060 (R4-2220837) |
| 7 | Both | Both | - Contiguous/Contiguous - Non-contiguous/Non-contiguous - Contiguous/Non-contiguous |

However, in Rel-17, as the table shown in the previous introduction part, it is observed that DC_3A-41A_n3A-n41A is introduced in TS 38.101-3 sub-clause 5.5B.4.3 and DC_1A-3A-41A_n3A-n41A is introduced in TS 38.101-3 sub-clause 5.5B.4.4, where these band combinations are seen as inter-band EN-DC band combinations with two additional intra-band EN-DC components. Hence, the current definition for IE intrabandENDC-support seems ambiguous because it is only applicable to inter-band EN-DC band combinations with one additional intra-band EN-DC component. The UE does not know how to indicate the spectrum contiguity separately by this IE for the inter-band EN-DC band combination, e.g., DC_3A-41A_n3A-n41A or DC_1A-3A-41A_n3A-n41A, with the first intra-band EN-DC components, e.g., DC_3A_n3A, and the second intra-band EN-DC components, e.g., DC_41A_n41A. Furthermore, the network also cannot differentiate which one of intra-band EN-DC component is associated with the reported IE intrabandENDC-support.

In the following Table 1, we summarize the possible DL/UL configurations and how the IE would be indicated by using the band combinations DC_3A-41A_n3A-n41 and DC_1A-3A-41A_n3A-n41A. Take the inter-band EN-DC band combination, DC_1A-3A-41A_n3A-n41A, with two additional intra-band EN-DC components, e.g., the first intra-band EN-DC component DC_3A_n3A and the second intra-band EN-DC component DC_41A_n41A, as an example.

From UL configuration perspective, it can be observed that the IE intraBandENDC-Support-UL only need to be indicated when the UL EN-DC configuration is DC_3A_n3A, and the IE intraBandENDC-Support-UL can be absent for other UL EN-DC configurations, e.g., DC_1A_n3A, DC_1A_n41A, DC_3A_n41A, and DC_41A_n3A. So there is no ambiguity for the IE intraBandENDC-Support-UL applied to the UL configuration.

However, from DL configuration perspective, the UE can only indicate one IE intraBandENDC-Support according to the current definition in TS 38.306, but there are two intra-band EN-DC components DC_3A_n3A and DC_41A_n41A in the inter band EN-DC band combination DC_1A-3A-41A_n3A-n41. For example, if LTE (DL:CA_1A-3A-41A,UL:1A) and (DL:CA_n3A-n41A,UL:n3A) with intraBandENDC-Support=non-contiguous is reported, the network actually cannot know which intra-band EN-DC components is referred to the IE intraBandENDC-Support. Depending on different implementations, the network may consider the IE intraBandENDC-Support=non-contiguous which is linked to the first intra-band EN-DC component DC_3A_n3A, or to the second intra-band EN-DC component DC_41A_n41A, or to both intra-band EN-DC components, e.g., DC_3A_n3A and DC_41A_n41A. Furthermore, if the band combination such as DC_1A-(n)3AA-41A_n41 is introduced in the future, the only one IE intraBandENDC-Support cannot be used to indicate that the first intra-band EN-DC component DC_(n)3AA is contiguous spectrum and the second intra-band EN-DC component DC_41A_n41A is non-contiguous spectrum.

| Band Combination | UE Capability |  |  |  |  |  | DL/UL EN-DC Configuration |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | LTE-DL | LTE-UL | NR-DL | NR-UL | intraBandENDC-Support | intraBandENDC-Support-UL | DL EN-DC Configuration | UL EN-DC Configuration |
| DC_3A-41A_n3A-n41A | CA_3A-41A | 3A | CA_n3A-n41A | n3A | non-contiguous (? ) | non-contiguous | DC_3A-41A_n3A-n41A | DC_3A_n3A |
|  | CA_3A-41A | 3A | CA_n3A-n41A | n41A | non-contiguous (?) | absent | DC_3A-41A_n3A-n41A | DC_3A_n41A |
|  | CA_3A-41A | 41A | CA_n3A-n41A | n3A | non-contiguous (?) | absent | DC_3A-41A_n3A-n41A | DC_41A_n3A |
| DC_1A-3A-41A_n3A-n41A | CA_1A-3A-41A | 1A | CA_n3A-n41A | n3A | non-contiguous (?) | absent | DC_1A-3A-41A_n3A-n41A | DC_1A_n3A |
|  | CA_1A-3A-41A | 1A | CA_n3A-n41A | n41A | non-contiguous (?) | absent | DC_1A-3A-41A_n3A-n41A | DC_1A_n41A |
|  | CA_1A-3A-41A | 3A | CA_n3A-n41A | n3A | non-contiguous (?) | non-contiguous | DC_1A-3A-41A_n3A-n41A | DC_3A_n3A |
|  | CA_1A-3A-41A | 3A | CA_n3A-n41A | n41A | non-contiguous (?) | absent | DC_1A-3A-41A_n3A-n41A | DC_3A_n41A |
|  | CA_1A-3A-41A | 41A | CA_n3A-n41A | n3A | non-contiguous (?) | absent | DC_1A-3A-41A_n3A-n41A | DC_41A_n3A |

Table 1. intraBandENDC-Support and intraBandENDC-Support-UL indication for the band combinations DC_3A-41A_n3A-n41A and DC_1A-3A-41A_n3A-n41A

Hence, in our view, in order to solve the ambiguous spectrum contiguity issue by indicating only one IE for the inter-band EN-DC band combination with two additional intra-band EN-DC components, it is proposed that RAN4 confirms the ambiguous signaling issue of IE intraBandENDC-Support to separately indicate the spectrum contiguity for the inter-band EN-DC band combinations, e.g., DC_3A-41A_n3A-n41A or DC_1A-3A-41A_n3A-n41A, with two additional intra-band EN-DC components, e.g., the first intra-band EN-DC component DC_3A_n3A and the second intra-band EN-DC component DC_41A_n41A, and RAN4 send LS to RAN2 to find a solution for the ambiguous signaling issue.

Proposal 1: RAN4 confirms the ambiguous signaling issue of IE intraBandENDC-Support to separately indicate the spectrum contiguity for the inter-band EN-DC band combinations, e.g., DC_3A-41A_n3A-n41A or DC_1A-3A-41A_n3A-n41A, with two additional intra-band EN-DC components, e.g., the first intra-band EN-DC component DC_3A_n3A and the second intra-band EN-DC component DC_41A_n41A.

Proposal 2: RAN4 send LS in the Annex to RAN2 for the ambiguous signaling issue.

# Conclusion

The proposals in this contribution are summarized in the following.

Proposal 1: RAN4 confirms the ambiguous signaling issue of IE intraBandENDC-Support to separately indicate the spectrum contiguity for the inter-band EN-DC band combinations, e.g., DC_3A-41A_n3A-n41A or DC_1A-3A-41A_n3A-n41A, with two additional intra-band EN-DC components, e.g., the first intra-band EN-DC component DC_3A_n3A and the second intra-band EN-DC component DC_41A_n41A.

Proposal 2: RAN4 send LS in the Annex to RAN2 for the ambiguous signaling issue.

# Reference

- R4-2313499, “Discussion on IE supportedBandwidthCombinationSetIntraENDC”, RAN4#108, Google

- R4-2314875, “WF on IE supportedBandwidthCombinationSetIntraENDC”, RAN4#108, Google

- R4-2320181, “Discussion on IE supportedBandwidthCombinationSetIntraENDC”, RAN4#109, Google

- R4-2320241, “LS on IE supportedBandwidthCombinationSetIntraENDC”, RAN4#109, Google

- R4-2206344, ”WF on IntrabandENDC-Support”, RAN4#102-e, Xiaomi

- R4-2306665, “Reply LS on intraBandENDC-Support”, RAN4#106bis-e, QualComm

- R4-2402313, “Discussion on IE supportedBandwidthCombinationSetIntraENDC”, RAN4#110, Google

# Annex

3GPP TSG-RAN WG4 Meeting #110	R4-23XXXXX

Athens, Greece, 26 Feb – 01 Mar, 2024

Title:	[Draft] LS on IE intraBandENDC-Support

Response to:

Release:	Rel-17

Work Item:	TEI17

Source:	RAN4

To:	RAN2

Cc:

Contact Person:

Name: Clement Huang

E-mail Address: clementhuang@google.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

1. Overall Description:

RAN4 confirms that the current definition of IE intraBandENDC-Support is ambiguous to separately indicate the spectrum contiguity for the inter-band EN-DC band combinations, e.g., DC_3A-41A_n3A-n41A or DC_1A-3A-41A_n3A-n41A, with two additional intra-band EN-DC components, e.g., the first intra-band EN-DC component DC_3A_n3A and the second intra-band EN-DC component DC_41A_n41A, where these band combinations in the following table are introduced from Rel-17.

| EN-DC configuration | Uplink EN-DC configuration (NOTE1) |
| --- | --- |
| DC_3A-41A_n3A-n41A | DC_3A_n3A4 DC_3A_n41A DC_41A_n3A |
| DC_1A-3A-41A_n3A-n41A | DC_1A_n3A DC_1A_n41A DC_3A_n3A4 DC_3A_n41A DC_41A_n3A |

RAN4 would like to request RAN2 to check how to solve the ambiguous signaling issue for the inter-band EN-DC band combinations with two additional intra-band EN-DC components.

2. Actions:

To RAN2:

ACTION: RAN4 respectfully asks RAN2 to take the above decisions into account to update RAN2 specifications.

3. Date of Next TSG WG RAN4 Meetings:

TSG-RAN WG4 Meeting #110bis             15th April – 19th April 2024              Changsha, CN

TSG-RAN WG4 Meeting #111                  20th May – 24th May 2024               Fukuoka City, JP