3GPP TSG-RAN WG4 Meeting #110	R4-2402466

# Athens, Greece, February 26 – March 1, 2024

Source:	T-Mobile USA

Title:	TP for TR38.850: FDD PC2 for n71 with DL CA_n71(2A)

Agenda item:	7.19.2

Document for:	Approval

- Introduction

This contribution is a text proposal to introduce PC2 n71 for DL CA_n71(2A).

In other PC2 and PC1.5 MSD analysis, T-Mobile USA has been using calculations that increase the Interference by 3dB, but not the noise. This contribution proposes new MSD values based on this approach for single Tx.

- Reference

[1]

- Text Proposal

<Start of text proposal>

## 5.x	CA_ n71(2A)

### 5.x.1	UE maximum output power

Table 5.5A.2-1: NR CA configurations and bandwidth combination sets defined for intra-band non-contiguous CA

| NR CA Configuration | Uplink CA Configurations or single uplink carrier5 | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Maximum Aggregated bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n71(2A) |  | n713 |  | 5, 10, 15, 20 |  | 5,10,15, 20 |  |  |  |  |  | 30 |  | 0 |  |
|  |  |  |  | See n71 channel bandwidths in Table 5.3.5-1 for each carrier up to 25 MHz per carrier |  |  |  |  |  |  |  | 30 |  | 4 and 5 |  |
| NOTE 3: Minimum requirements for Power Class 2 are applicable for this uplink combination or single uplink carrier in this downlink/uplink combination |  |  |  |  |  |  |  |

### 5.x.2	Reference sensitivity requirements

#### 5.x.2.0	General

For PC2, CA_n71(2A) has self-interference for UL n71. This section will examine the existing PC3 MSD and propose MSD for PC2 FDD.

#### 5.x.2.1	Reference sensitivity requirements with PC2 on n71 without TxD

For CA_n71(2A), this is the configuration and MSD for UL n71 with PC3

Table 7.3A.2.2-1: Intra-band non-contiguous CA with one uplink configuration for reference sensitivity in FDD bands.

| CA configuration | SCS (PCC/SCC) (kHz) |  | Aggregated channel bandwidth (PCC+SCC) | Wgap / [MHz] | UL PCC allocation (LCRB) | ΔRIBNC (dB) | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n71(2A) | 15/15 |  | 5MHz + 5MHz | Wgap = 25.0 | 5 | 4.0 | FDD |
|  |  |  |  | Wgap = 5.0 | 20 | 0.0 |  |
|  |  |  | 10MHz + 5MHz | Wgap = 20.0 | 5 (RBstart = 9) | 4.6 |  |
|  |  |  |  | Wgap = 5.0 | 20 (RBstart = 9) | 2.3 |  |
|  |  |  | 15MHz + 10MHz | Wgap = 10.0 | 5 (RBstart = 2) | 22.2 |  |
|  |  |  |  | Wgap = 5.0 | 20 (RBstart = 19) | 5.2 |  |

For PC3 MSD we have N+IPC3. For PC2 MSD we have N+IPC2. So, for PC2 compared to PC3, I increases by 3 dB. For our other PC2 and PC1.5 MSD analysis we have been using the following approach:

MSD due to interference power I is given by:

where N is the noise spectral density and BW is the bandwidth of the carrier. If the initial MSD is known,

then we have:

If I is increased by X dB, then MSD(X) is given by

Using that approach, the following is proposed as a the PC2 MSD which would require a new table in 38.101-1:

Table 7.3A.2.2-1a: Intra-band non-contiguous CA with one PC2 uplink configuration for reference sensitivity in FDD bands.

| CA configuration | SCS (PCC/SCC) (kHz) | Aggregated channel bandwidth (PCC+SCC) | Wgap / [MHz] | UL PCC allocation (LCRB) | ΔRIBNC (dB) | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- |
| CA_n71(2A) | 15/15 | 5MHz + 5MHz | Wgap = 25.0 | 5 | 6.0 | FDD |
|  |  |  | Wgap = 5.0 | 20 | 0.0 |  |
|  |  | 10MHz + 5MHz | Wgap = 20.0 | 5 (RBstart = 9) | 6.8 |  |
|  |  |  | Wgap = 5.0 | 20 (RBstart = 9) | 3.8 |  |
|  |  | 15MHz + 10MHz | Wgap = 10.0 | 5 (RBstart = 2) | 25.1 |  |
|  |  |  | Wgap = 5.0 | 20 (RBstart = 19) | 7.5 |  |

#### 5.x.2.2	Reference sensitivity requirements with PC2 on n71 with TxD

[TBD]