3GPP TSG-RAN WG4 Meeting #110	                                                                            R4-2402611

Athens, Greece, Feb 26th – Mar 1st, 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-1 | CR | 2715 | rev | - | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to TS 38.101-1 for enhanced channel raster |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MediaTek Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_channel_raster_enh-Core |  |  |  |  |  | Date: |  |  | 2024-01-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | According to WF R4-2321752, the enhanced channel raster is not applicable to intra-band contiguous CA, and it is not reflected in specs. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add a Note 2 in Table 5.4.2.3-5 to indicate that enhanced channel raster is not applicable to intra-band contiguous CA. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Ambiguity for applicability of enhanced channel raster to intra-band contiguous CA. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.4.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## << Start of change >>

#### 5.4.2.3	Channel raster entries for each operating band

The RF channel positions on the channel raster in each NR operating band are given through the applicable NR-ARFCN in Table 5.4.2.3-1, using the channel raster to resource element mapping in clause 5.4.2.2.

For NR operating bands with 100 kHz channel raster, ΔFRaster = 20 × ΔFGlobal. In this case every 20th NR-ARFCN within the operating band are applicable for the channel raster within the operating band and the step size for the channel raster in Table 5.4.2.3-1 is given as <20>.

For NR operating bands with 15 kHz channel raster below 3GHz, ΔFRaster = I × ΔFGlobal, where I ϵ {3,6}. Every Ith NR-ARFCN within the operating band are applicable for the channel raster within the operating band and the step size for the channel raster in Table 5.4.2.3-1 is given as < I >.

For NR operating bands with 15 kHz channel raster above 3GHz, ΔFRaster = I × ΔFGlobal, where I ϵ {1,2}. Every Ith  NR-ARFCN within the operating band are applicable for the channel raster within the operating band and the step size for the channel raster in table 5.4.2.3-1 is given as <I>.

In frequency bands with two or more ΔFRaster, the higher ΔFRaster: For 15 kHz and 30 kHz channel raster applies to channels using only the SCS that is equal to or larger than the higher ΔFRaster and SSB SCS is equal to the higher ∆FRaster.

Table 5.4.2.3-1: Applicable NR-ARFCN per operating band

| NR operating band | ΔFRaster (kHz) | Uplink Range of NREF (First – <Step size> – Last) | Downlink Range of NREF (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n1 | 100 | 384000 – <20> – 396000 | 422000 – <20> – 434000 |
| n2 | 100 | 370000 – <20> – 382000 | 386000 – <20> – 398000 |
| n3 | 100 | 342000 – <20> – 357000 | 361000 – <20> – 376000 |
| n5 | 100 | 164800 – <20> – 169800 | 173800 – <20> – 178800 |
| n7 | 100 | 500000 – <20> – 514000 | 524000 – <20> – 538000 |
| n8 | 100 | 176000 – <20> – 183000 | 185000 – <20> – 192000 |
| n12 | 100 | 139800 – <20> – 143200 | 145800 – <20> – 149200 |
| n13 | 100 | 155400 – <20> – 157400 | 149200 – <20> – 151200 |
| n14 | 100 | 157600 – <20> – 159600 | 151600 – <20> – 153600 |
| n18 | 100 | 163000 – <20> – 166000 | 172000 – <20> – 175000 |
| n20 | 100 | 166400 – <20> – 172400 | 158200 – <20> – 164200 |
| n24 | 100 | 325300 – <20> – 332100 | 305000 – <20> – 311800 |
| n25 | 100 | 370000 – <20> – 383000 | 386000 – <20> – 399000 |
| n26 | 100 | 162800 – <20> – 169800 | 171800 – <20> – 178800 |
| n28 | 100 | 140600 – <20> – 149600 | 151600 – <20> – 160600 |
| n29 | 100 | N/A | 143400 – <20> – 145600 |
| n30 | 100 | 461000 – <20> – 463000 | 470000 – <20> – 472000 |
| n31 | 100 | 90500 – <20> – 91500 | 92500 – <20> – 93500 |
| n34 | 100 | 402000 – <20> – 405000 | 402000 – <20> – 405000 |
| n38 | 100 | 514000 – <20> – 524000 | 514000 – <20> – 524000 |
| n39 | 100 | 376000 – <20> – 384000 | 376000 – <20> – 384000 |
| n40 | 100 | 460000 – <20> – 480000 | 460000 – <20> – 480000 |
| n41 | 15 | 499200 – <3> – 537999 | 499200 – <3> – 537999 |
|  | 30 | 499200 – <6> – 537996 | 499200 – <6> – 537996 |
| n462 | 15 | 743334 – <1> – 795000 | 743334 – <1> – 795000 |
| n47 | 15 | 790334 – <1> – 795000 | 790334 – <1> – 795000 |
| n48 | 15 | 636667 – <1> – 646666 | 636667 – <1> – 646666 |
|  | 30 | 636668 – <2> – 646666 | 636668 – <2> – 646666 |
| n50 | 100 | 286400 – <20> – 303400 | 286400 – <20> – 303400 |
| n51 | 100 | 285400 – <20> – 286400 | 285400 – <20> – 286400 |
| n53 | 100 | 496700 – <20> – 499000 | 496700 – <20> – 499000 |
| n54 | 100 | 334000 – <20> – 335000 | 334000 – <20> – 335000 |
| n65 | 100 | 384000 – <20> – 402000 | 422000 – <20> – 440000 |
| n66 | 100 | 342000 – <20> – 356000 | 422000 – <20> – 440000 |
| n67 | 100 | N/A | 147600 – <20> – 151600 |
| n70 | 100 | 339000 – <20> – 342000 | 399000 – <20> – 404000 |
| n71 | 100 | 132600 – <20> – 139600 | 123400 – <20> – 130400 |
| n72 | 100 | 90200 – <20> – 91200 | 92200 – <20> – 93200 |
| n74 | 100 | 285400 – <20> – 294000 | 295000 – <20> – 303600 |
| n75 | 100 | N/A | 286400 – <20> – 303400 |
| n76 | 100 | N/A | 285400 – <20> – 286400 |
| n77 | 15 | 620000 – <1> – 680000 | 620000 – <1> – 680000 |
|  | 30 | 620000 – <2> – 680000 | 620000 – <2> – 680000 |
| n78 | 15 | 620000 – <1> – 653333 | 620000 – <1> – 653333 |
|  | 30 | 620000 – <2> – 653332 | 620000 – <2> – 653332 |
| n79 | 15 | 693334 – <1> – 733333 | 693334 – <1> – 733333 |
|  | 30 | 693334 – <2> – 733332 | 693334 – <2> – 733332 |
| n80 | 100 | 342000 – <20> – 357000 | N/A |
| n81 | 100 | 176000 – <20> – 183000 | N/A |
| n82 | 100 | 166400 – <20> – 172400 | N/A |
| n83 | 100 | 140600 – <20> –149600 | N/A |
| n84 | 100 | 384000 – <20> – 396000 | N/A |
| n85 | 100 | 139600 – <20> – 143200 | 145600 – <20> – 149200 |
| n86 | 100 | 342000 – <20> – 356000 | N/A |
| n89 | 100 | 164800 – <20> – 169800 | N/A |
| n90 | 15 | 499200 – <3> – 537999 | 499200 – <3> – 537999 |
|  | 30 | 499200 – <6> – 537996 | 499200 – <6> – 537996 |
|  | 100 | 499200 – <20> – 538000 | 499200 – <20> – 538000 |
| n91 | 100 | 166400 – <20> – 172400 | 285400 – <20> – 286400 |
| n92 | 100 | 166400 – <20> – 172400 | 286400 – <20> – 303400 |
| n93 | 100 | 176000 – <20> – 183000 | 285400 – <20> – 286400 |
| n94 | 100 | 176000 – <20> – 183000 | 286400 – <20> – 303400 |
| n95 | 100 | 402000 – <20> – 405000 | N/A |
| n963 | 15 | 795000 – <1> – 875000 | 795000 – <1> – 875000 |
| n97 | 100 | 460000 – <20> – 480000 | N/A |
| n98 | 100 | 376000 – <20> – 384000 | N/A |
| n99 | 100 | 325300 – <20> – 332100 | N/A |
| n100 | 100 | 174880 – <20> – 176000 | 183880 – <20> – 185000 |
| n101 | 100 | 380000 – <20> – 382000 | 380000 – <20> – 382000 |
| n1024 | 15 | 795000 – <1> – 828333 | 795000 – <1> – 828333 |
| n104 | 15 | 828334 – <1> – 875000 | 828334 – <1> – 875000 |
|  | 30 | 828334 – <2> – 875000 | 828334 – <2> – 875000 |
| n105 | 100 | 132600 – <20> – 140600 | 122400 – <20> – 130400 |
| n1065 | 100 | 179200 – <20> – 180200 | 187000 – <20> – 188000 |
| n109 | 100 | 140600 – <20> – 149600 | 286400 – <20> – 303400 |
| NOTE 1: The channel numbers that designate carrier frequencies so close to the operating band edges that the carrier extends beyond the operating band edge shall not be used. NOTE 2: The following NREF are allowed for operation in Band n46: see Table 5.4.2.3-2. NOTE 3: The following NREF are allowed for operation in Band n96: see Table 5.4.2.3-3. NOTE 4: The following NREF are allowed for operation in Band n102: see Table 5.4.2.3-4. NOTE 5: In the present version of the specification, only NREF 179800 and 187600 is applicable for 3 MHz channel bandwidth. |  |  |  |

Table 5.4.2.3-2: Allowed NREF (NR-ARFCN) for operation in Band n46

| Channel Bandwidth | Allowed NREF |
| --- | --- |
| 10 MHz | 782000, 788668 |
| 20 MHz | 744000, 745332, 746668, 748000, 749332, 750668, 752000, 753332, 754668, 756000, 765332, 766668, 768000, 769332, 770668, 772000, 773332, 774668, 776000, 777332, 778668, 780000, 781332, 783000, 784332, 785668, 787000, 788332, 789668, 791000, 792332, 793668 |
| 40 MHz | 744668, 746000, 748668, 751332, 754000, 755332, 766000, 767332, 770000, 772668, 775332, 778000, 780668, 783668, 786332, 787668, 790332, 793000 |
| 60 MHz | 745332, 746668, 748000, 752000, 753332, 754668, 766668, 768000, 769332, 773332, 774668, 778668, 780000, 784332, 785668, 791000, 792332 |
| 80 MHz | 746000, 747332, 752668, 754000, 767332, 768668, 774000, 779332, 785000, 791668 |
| 100 MHz | 746668, 753332, 768000, 791000 |
| NOTE: 10 MHz channel bandwidth shall only apply in certain regions where the absence of non 3GPP technologies can be guaranteed on a long-term basis in this version of specification. |  |

Table 5.4.2.3-3: Allowed NREF (NR-ARFCN) for operation in Band n96

| Channel Bandwidth | Allowed NREF |
| --- | --- |
| 20 MHz | 7956681, 797000, 798332, 799668, 801000, 802332, 803668, 805000, 806332, 807668, 809000, 810332, 811668, 813000, 814332, 815668, 817000, 818332, 819668, 821000, 822332, 823668, 825000, 826332, 827668, 829000, 830332, 831668, 833000, 834332, 835668, 837000, 838332, 839668, 841000, 842332, 843668, 845000, 846332, 847668, 849000, 850332, 851668, 853000, 854332, 855668, 857000, 858332, 859668, 861000, 862332, 863668, 865000, 866332, 867668, 869000, 870332, 871668, 873000, 874332 |
| 40 MHz | 797668, 800332, 803000, 805668, 808332, 811000, 813668, 816332, 819000, 821668, 824332, 827000, 829668, 832332, 835000, 837668, 840332, 843000, 845668, 848332, 851000, 853668, 856332, 859000, 861668, 864332, 867000, 869668, 872332 |
| 60 MHz | 798332, 799668, 803668, 805000, 809000, 810332, 814332, 815668, 819668, 821000, 825000, 826332, 830332, 831668, 835668, 837000, 841000, 842332, 846332, 847668, 851668, 853000, 857000, 858332, 862332, 863668, 867668, 869000, 873000 |
| 80 MHz | 799000, 804332, 809668, 815000, 820332, 825668, 831000, 836332, 841668, 847000, 852332, 857668, 863000, 868332 |
| 100 MHz | 799668, 803668, 810332, 814332, 821000, 825000, 831668, 835668, 842332, 846332, 853000, 857000, 863668, 867668, 869000, 870332, 871668 |
| Note 1: NREF is only applicable for DL only operation |  |

Table 5.4.2.3-4: Allowed NREF (NR-ARFCN) for operation in Band n102

| Channel Bandwidth | Allowed NREF |
| --- | --- |
| 20 MHz | 7956681, 797000, 798332, 799668, 801000, 802332, 803668, 805000, 806332, 807668, 809000, 810332, 811668, 813000, 814332, 815668, 817000, 818332, 819668, 821000, 822332, 823668, 825000, 826332, 827668 |
| 40 MHz | 797668, 800332, 803000, 805668, 808332, 811000, 813668, 816332, 819000, 821668, 824332, 827000 |
| 60 MHz | 798332, 799668, 803668, 805000, 809000, 810332, 814332, 815668, 819668, 821000, 825000, 826332 |
| 80 MHz | 799000, 804332, 809668, 815000, 820332, 825668 |
| 100 MHz | 799668, 803668, 810332, 814332, 821000, 825000 |
| Note 1: NREF is only applicable for DL only operation |  |

For NR operating bands with 100 kHz channel raster, Enhanced channel raster is defined with ΔFRaster = 2 × ΔFGlobal. In this case every 2th NR-ARFCN within the operating band are applicable for the channel raster within the operating band and the step size for the channel raster in Table 5.4.2.3-5 is given as <2>.

Table 5.4.2.3-5: Applicable NR-ARFCN per operating band for enhanced channel raster

| NR operating band | ΔFRaster (kHz) | Uplink Range of NREF (First – <Step size> – Last) | Downlink Range of NREF (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n1 | 10 | 384000 – <2> – 396000 | 422000 – <2> – 434000 |
| n2 | 10 | 370000 – <2> – 382000 | 386000 – <2> – 398000 |
| n3 | 10 | 342000 – <2> – 357000 | 361000 – <2> – 376000 |
| n5 | 10 | 164800 – <2> – 169800 | 173800 – <2> – 178800 |
| n7 | 10 | 500000 – <2> – 514000 | 524000 – <2> – 538000 |
| n8 | 10 | 176000 – <2> – 183000 | 185000 – <2> – 192000 |
| n12 | 10 | 139800 – <2> – 143200 | 145800 – <2> – 149200 |
| n13 | 10 | 155400 – <2> – 157400 | 149200 – <2> – 151200 |
| n14 | 10 | 157600 – <2> – 159600 | 151600 – <2> – 153600 |
| n18 | 10 | 163000 – <2> – 166000 | 172000 – <2> – 175000 |
| n20 | 10 | 166400 – <2> – 172400 | 158200 – <2> – 164200 |
| n24 | 10 | 325300 – <2> – 332100 | 305000 – <2> – 311800 |
| n25 | 10 | 370000 – <2> – 383000 | 386000 – <2> – 399000 |
| n26 | 10 | 162800 – <2> – 169800 | 171800 – <2> – 178800 |
| n28 | 10 | 140600 – <2> – 149600 | 151600 – <2> – 160600 |
| n29 | 10 | N/A | 143400 – <2> – 145600 |
| n30 | 10 | 461000 – <2> – 463000 | 470000 – <2> – 472000 |
| n34 | 10 | 402000 – <2> – 405000 | 402000 – <2> – 405000 |
| n38 | 10 | 514000 – <2> – 524000 | 514000 – <2> – 524000 |
| n39 | 10 | 376000 – <2> – 384000 | 376000 – <2> – 384000 |
| n40 | 10 | 460000 – <2> – 480000 | 460000 – <2> – 480000 |
| n50 | 10 | 286400 – <2> – 303400 | 286400 – <2> – 303400 |
| n53 | 10 | 496700 – <2> – 499000 | 496700 – <2> – 499000 |
| n54 | 10 | 334000 – <2> – 335000 | 334000 – <2> – 335000 |
| n65 | 10 | 384000 – <2> – 402000 | 422000 – <2> – 440000 |
| n66 | 10 | 342000 – <2> – 356000 | 422000 – <2> – 440000 |
| n67 | 10 | N/A | 147600 – <2> – 151600 |
| n70 | 10 | 339000 – <2> – 342000 | 399000 – <2> – 404000 |
| n71 | 10 | 132600 – <2> – 139600 | 123400 – <2> – 130400 |
| n74 | 10 | 285400 – <2> – 294000 | 295000 – <2> – 303600 |
| n75 | 10 | N/A | 286400 – <2> – 303400 |
| n76 | 10 | N/A | 285400 – <2> – 286400 |
| n80 | 10 | 342000 – <2> – 357000 | N/A |
| n81 | 10 | 176000 – <2> – 183000 | N/A |
| n82 | 10 | 166400 – <2> – 172400 | N/A |
| n83 | 10 | 140600 – <2> –149600 | N/A |
| n84 | 10 | 384000 – <2> – 396000 | N/A |
| n85 | 10 | 139600 – <2> – 143200 | 145600 – <2> – 149200 |
| n86 | 10 | 342000 – <2> – 356000 | N/A |
| n89 | 10 | 164800 – <2> – 169800 | N/A |
| n90 | 10 | 499200 – <2> – 538000 | 499200 – <2> – 538000 |
| n91 | 10 | 166400 – <2> – 172400 | 285400 – <2> – 286400 |
| n92 | 10 | 166400 – <2> – 172400 | 286400 – <2> – 303400 |
| n93 | 10 | 176000 – <2> – 183000 | 285400 – <2> – 286400 |
| n94 | 10 | 176000 – <2> – 183000 | 286400 – <2> – 303400 |
| n95 | 10 | 402000 – <2> – 405000 | N/A |
| n97 | 10 | 460000 – <2> – 480000 | N/A |
| n98 | 10 | 376000 – <2> – 384000 | N/A |
| n99 | 10 | 325300 – <2> – 332100 | N/A |
| n100 | 10 | 174880 – <2> – 176000 | 183880 – <2> – 185000 |
| n101 | 10 | 380000 – <2> – 382000 | 380000 – <2> – 382000 |
| n105 | 10 | 132600 – <2> – 140600 | 122400 – <2> – 130400 |
| NOTE 1: The channel numbers that designate carrier frequencies so close to the operating band edges that the carrier extends beyond the operating band edge shall not be used. These channel numbers shall also be such that the minimum guard band for each channel bandwidth and SCS specified in Table 5.3.3-1 are met for carriers located at the upper or lower edge of an operating band. NOTE 2: The enhanced channel raster is not applicable to intra-band contiguous CA. |  |  |  |

## << End change>>