3GPP TSG-RAN WG4 Meeting #110	R4-2402626

# Athens, Greece, February 26th – March 1st, 2024

Source:	Nokia, Nokia Shanghai Bell

Title:	On performance requirements for Less Than 5MHz BW

Agenda item:	8.10.5

Document for:	Discussion

Introduction

This contribution discusses the performance requirements related to < 5MHz WI.

Discussion

In the last meeting there was discussion whether the UE supporting < 5MHz is a legacy UE supporting the reduced bandwidth, or if the UE is only supporting < 5MHz BW. In the specifications, it was considered such that the UE is legacy UE, supporting the legacy requirements, but when UE is operating on a cell supporting reduced bandwidth, the new requirements defined in the 38.133 would apply. This is reflected in the specification via following description of the capability: “[for a UE operating on a cell with less than 5MHz BW]”.

Baseline assumption is that the UE supports legacy requirements, and when UE is operating on a cell with less than 5MHz BW, new rel-18 requirements apply.

Assume UE operating on a cell with less than 5MHz BW is also supporting legacy test cases, hence RAN4 can focus test case efforts on those cases with identified specification impact.

- For RLM and BFD, it is important to consider PDCCH bandwidth configuration. For SSB-based cases, the SSB puncturing is the main concern.

Test cases

The following table summarises the specification and corresponding configuration with test case proposals. It can be discussed which functionality has overlap, for example on time index reading, and whether to cover it in one or more test cases.

The test cases in table 1 are proposed to be discussed with the corresponding configuration options.

| Handover Test cases |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| # | Specification section |  | Test case |  |  |  |  |  |  |  |  |  |  | Test cases & Main configuration |  |  |  |  |
| 1 | 6.1.1 NR Handover 6.1.1.2.2 Interruption time |  | Note: text is subject ot CR approval as the latest 3GPP TS 38.133 V18.4.0 (2023-12) did not include the change. - If the target cell is an unknown intra-frequency cell with 12 PRB SSB bandwidth, then Tsearch = [3]*Trs ms. - If the target cell is an unknown inter-frequency cell with 12 PRB SSB bandwidth and, then Tsearch = [5] *Trs ms |  |  |  |  |  |  |  |  |  |  | PBCH: 12 PRB SSB Others, FFS |  |  |  | Intra-frequency handover to unknown cell with 12 PRB SSB BW |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Inter-frequency handover to unknown cell with 12 PRB SSB BW |
| Radio Link Monitoring Test cases |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| # |  | Specification section |  |  | Change |  |  |  |  |  | Test cases & Main configuration |  |  |  |  |  |  |  |
| 2 |  | 8.1 Radio Link Monitoring 8.1.2 Requirements for SSB based radio link monitoring. 8.1.2.1 Introduction |  |  | Table 8.1.2.1-3: PDCCH transmission parameters for out-of-sync evaluation [for a UE operating on a cell with less than 5MHz BW] Table 8.1.2.1-4: PDCCH transmission parameters for in-sync evaluation [for a UE operating on a cell with less than 5MHz BW] |  |  |  |  |  | PDCCH: 3MHz (12 PRBs) 3MHz (15 PRBs) 5MHz (20 PRBs) Others, FFS |  |  |  |  |  | Radio Link Monitoring Out-of-sync Test for FR1 PCell for SSB based RLM RS in non-DRX mode |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Radio Link Monitoring Out-of-sync Test for FR1 PCell for SSB based RLM RS in DRX mode |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Radio Link Monitoring In-sync Test for FR1 PCell configured with SSB-based RLM RS in non-DRX mode |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Radio Link Monitoring In-sync Test for FR1 PCell configured with SSB-based RLM RS in DRX mode |  |
| SSB based beam failure detection |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| # |  | Specification section |  |  |  | Change |  |  | Test cases & Main configuration |  |  |  |  |  |  |  |  |  |
| 3 |  | 8.5.2 Requirements for SSB based beam failure detection 8.5.2.1 Introduction |  |  |  | Table 8.5.2.1-2: PDCCH transmission parameters for beam failure instance [for a UE operating on a cell with less than 5MHz BW] |  |  | PDCCH: 3MHz (12 PRBs) 3MHz (15 PRBs) 5MHz (20 PRBs) Others, FFS |  |  |  |  |  | BFD detection and LR test for FR1 PCell for SSB based RLM RS in non-DRX mode |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | BFD detection and LR test for FR1 PCell for SSB based RLM RS in DRX mode |  |  |  |
| Intra frequency measurements without measurement gaps |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| # | Specification section |  |  | Change |  |  |  |  |  | Test cases & Main configuration |  |  |  |  |  |  |  |  |
| 4 | 9.2.5 Intra frequency measurements without measurement gaps 9.2.5.1 Intra frequency cell identification |  |  | Table 9.2.5.1-23: Time period for time index detection (Frequency range FR1) [for a target cell with 12 or 15 PRB SSB] TSSB_time_index_intra_less_than_5Mhz with [7] samples |  |  |  |  |  | PBCH: 12 PRB SSB Others, FFS |  |  |  |  |  | SA event triggered reporting tests without gap under non-DRX |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | SA event triggered reporting tests without gap under DRX |  |  |
| Intra-frequency measurements with measurement gaps |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| # | Sections |  |  |  |  |  | Summary of the change |  |  |  |  | Test cases & Main configuration |  |  |  |  |  |  |
| 5 | 9.2.6 Intra-frequency measurements with measurement gaps 9.2.6.2 Intra-frequency cell identification |  |  |  |  |  | Table 9.2.6.1-12: Time period for time index detection (Frequency range FR1) [for a target cell with 12 or 15 PRB SSB] TSSB_time_index_intra_less_than_5Mhz with [7] samples |  |  |  |  | PBCH: 12 PRB SSB Others, FFS |  |  |  |  | SA event triggered reporting tests with per-UE gaps under non-DRX |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | SA event triggered reporting tests with per-UE gaps under DRX |  |
| Inter-frequency measurement with measurement gaps |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| # |  | Sections |  |  |  |  |  | Summary of the change |  |  |  |  | Test cases |  |  |  |  |  |
| 6 |  | 9.3.4 Inter-frequency measurement with measurement gaps |  |  |  |  |  | Table 9.3.4-11: Time period for time index detection (Frequency range FR1) [for a target cell with 12 or 15 PRB SSB] TSSB_time_index_inter with [6] samples. |  |  |  |  | PBCH: 12 PRB SSB Others, FFS |  |  |  | SA event triggered reporting tests for FR1 with SSB time index detection when DRX is not used |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | SA event triggered reporting tests for FR1 with SSB time index detection when DRX is used |  |
| Inter frequency measurements without measurement gaps |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| # |  | Sections |  |  |  |  |  | Summary of the change |  |  |  |  | Test cases |  |  |  |  |  |
| 7 |  | 9.3.9 Inter frequency measurements without measurement gaps 9.3.9.1 Inter frequency Cell identification |  |  |  |  |  | Inter frequency measurements without measurement gaps Table 9.3.9.1-5: Time period for time index detection (FR1) [for a target cell with 12 or 15 PRB SSB] TSSB_time_index_inter with [6] samples. |  |  |  |  | PBCH: 12 PRB SSB Others, FFS |  |  |  | SA event triggered reporting tests for FR1 with SSB time index detection when DRX is not used |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | SA event triggered reporting tests for FR1 with SSB time index detection when DRX is used |  |

Table 1: RRM specification impact and test cases for < 5MHz

When considering the configuration impact, the following table, summarising RAN1 design, can be used.

| Channel BW | Transmission BW | Channels | Band n100 | Other bands |
| --- | --- | --- | --- | --- |
| 3MHz | 12 PRB | PBCH (SSB) Note: SSB is punctured from 20 PRB | 12 PRB | Not supported |
|  |  | PDCCH CORESET#0 Note: not punctured, new 12 PRB configuration | 12 PRB |  |
|  |  | PDCCH Dedicated CORESET | 12 PRB Note: only configurations 6 and 12 are supported |  |
|  | 15 PRB | PBCH (SSB) Note: SSB is punctured from 20 PRB | 12 PRB | 12 PRB |
|  |  | PDCCH CORESET#0 Note: 15 PRB CORESET#0 is punctured from 24 PRB | 15 PRB, [12 PRB]note 1 | 15 PRB, [12 PRB] note 1 |
|  |  | PDCCH Dedicated CORESET | 12 PRB, [15 PRB]note 2 Note: only configurations 6 and 12 are supported | 12 PRB, [15 PRB] note 2 |
| 5MHz | 20 PRB | PBCH (SSB) Note: Legacy SSB | 20 PRB | Not supported |
|  |  | PDCCH CORESET#0 Note: CORESET#0 is punctured from 24 PRB | 20 PRB |  |
|  |  | PDCCH Dedicated CORESET | 18 PRB, [20 PRB]note 3 Note: only configurations 6, 12, 18 are supported |  |

- Conclusion

In the paper, the following Observations and Proposals were made:

Observation 1: Baseline assumption is that the UE supports legacy requirements, and when UE is operating on a cell with less than 5MHz BW, new rel-18 requirements apply.

Proposal 1: Assume UE operating on a cell with less than 5MHz BW is also supporting legacy test cases, hence RAN4 can focus test case efforts on those cases with identified specification impact.

For RLM and BFD, it is important to consider PDCCH bandwidth configuration. For SSB-based cases, the SSB puncturing is the main concern.

Proposal 2: The test cases in table 1 are proposed to be discussed with the corresponding configuration options.

| Handover Test cases |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| # | Specification section |  | Test case |  |  |  |  |  |  |  |  |  |  | Test cases & Main configuration |  |  |  |  |
| 1 | 6.1.1 NR Handover 6.1.1.2.2 Interruption time |  | Note: text is subject ot CR approval as the latest 3GPP TS 38.133 V18.4.0 (2023-12) did not include the change. - If the target cell is an unknown intra-frequency cell with 12 PRB SSB bandwidth, then Tsearch = [3]*Trs ms. - If the target cell is an unknown inter-frequency cell with 12 PRB SSB bandwidth and, then Tsearch = [5] *Trs ms |  |  |  |  |  |  |  |  |  |  | PBCH: 12 PRB SSB Others, FFS |  |  |  | Intra-frequency handover to unknown cell with 12 PRB SSB BW |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Inter-frequency handover to unknown cell with 12 PRB SSB BW |
| Radio Link Monitoring Test cases |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| # |  | Specification section |  |  | Change |  |  |  |  |  | Test cases & Main configuration |  |  |  |  |  |  |  |
| 2 |  | 8.1 Radio Link Monitoring 8.1.2 Requirements for SSB based radio link monitoring. 8.1.2.1 Introduction |  |  | Table 8.1.2.1-3: PDCCH transmission parameters for out-of-sync evaluation [for a UE operating on a cell with less than 5MHz BW] Table 8.1.2.1-4: PDCCH transmission parameters for in-sync evaluation [for a UE operating on a cell with less than 5MHz BW] |  |  |  |  |  | PDCCH: 3MHz (12 PRBs) 3MHz (15 PRBs) 5MHz (20 PRBs) Others, FFS |  |  |  |  |  | Radio Link Monitoring Out-of-sync Test for FR1 PCell for SSB based RLM RS in non-DRX mode |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Radio Link Monitoring Out-of-sync Test for FR1 PCell for SSB based RLM RS in DRX mode |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Radio Link Monitoring In-sync Test for FR1 PCell configured with SSB-based RLM RS in non-DRX mode |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Radio Link Monitoring In-sync Test for FR1 PCell configured with SSB-based RLM RS in DRX mode |  |
| SSB based beam failure detection |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| # |  | Specification section |  |  |  | Change |  |  | Test cases & Main configuration |  |  |  |  |  |  |  |  |  |
| 3 |  | 8.5.2 Requirements for SSB based beam failure detection 8.5.2.1 Introduction |  |  |  | Table 8.5.2.1-2: PDCCH transmission parameters for beam failure instance [for a UE operating on a cell with less than 5MHz BW] |  |  | PDCCH: 3MHz (12 PRBs) 3MHz (15 PRBs) 5MHz (20 PRBs) Others, FFS |  |  |  |  |  | BFD detection and LR test for FR1 PCell for SSB based RLM RS in non-DRX mode |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | BFD detection and LR test for FR1 PCell for SSB based RLM RS in DRX mode |  |  |  |
| Intra frequency measurements without measurement gaps |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| # | Specification section |  |  | Change |  |  |  |  |  | Test cases & Main configuration |  |  |  |  |  |  |  |  |
| 4 | 9.2.5 Intra frequency measurements without measurement gaps 9.2.5.1 Intra frequency cell identification |  |  | Table 9.2.5.1-23: Time period for time index detection (Frequency range FR1) [for a target cell with 12 or 15 PRB SSB] TSSB_time_index_intra_less_than_5Mhz with [7] samples |  |  |  |  |  | PBCH: 12 PRB SSB Others, FFS |  |  |  |  |  | SA event triggered reporting tests without gap under non-DRX |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | SA event triggered reporting tests without gap under DRX |  |  |
| Intra-frequency measurements with measurement gaps |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| # | Sections |  |  |  |  |  | Summary of the change |  |  |  |  | Test cases & Main configuration |  |  |  |  |  |  |
| 5 | 9.2.6 Intra-frequency measurements with measurement gaps 9.2.6.2 Intra-frequency cell identification |  |  |  |  |  | Table 9.2.6.1-12: Time period for time index detection (Frequency range FR1) [for a target cell with 12 or 15 PRB SSB] TSSB_time_index_intra_less_than_5Mhz with [7] samples |  |  |  |  | PBCH: 12 PRB SSB Others, FFS |  |  |  |  | SA event triggered reporting tests with per-UE gaps under non-DRX |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | SA event triggered reporting tests with per-UE gaps under DRX |  |
| Inter-frequency measurement with measurement gaps |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| # |  | Sections |  |  |  |  |  | Summary of the change |  |  |  |  | Test cases |  |  |  |  |  |
| 6 |  | 9.3.4 Inter-frequency measurement with measurement gaps |  |  |  |  |  | Table 9.3.4-11: Time period for time index detection (Frequency range FR1) [for a target cell with 12 or 15 PRB SSB] TSSB_time_index_inter with [6] samples. |  |  |  |  | PBCH: 12 PRB SSB Others, FFS |  |  |  | SA event triggered reporting tests for FR1 with SSB time index detection when DRX is not used |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | SA event triggered reporting tests for FR1 with SSB time index detection when DRX is used |  |
| Inter frequency measurements without measurement gaps |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| # |  | Sections |  |  |  |  |  | Summary of the change |  |  |  |  | Test cases |  |  |  |  |  |
| 7 |  | 9.3.9 Inter frequency measurements without measurement gaps 9.3.9.1 Inter frequency Cell identification |  |  |  |  |  | Inter frequency measurements without measurement gaps Table 9.3.9.1-5: Time period for time index detection (FR1) [for a target cell with 12 or 15 PRB SSB] TSSB_time_index_inter with [6] samples. |  |  |  |  | PBCH: 12 PRB SSB Others, FFS |  |  |  | SA event triggered reporting tests for FR1 with SSB time index detection when DRX is not used |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | SA event triggered reporting tests for FR1 with SSB time index detection when DRX is used |  |