3GPP TSG-RAN4 Meeting #110	R4-2402629

# Athens, Greece, 26th Feb 2024 - 1st Mar 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | - | rev | - | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | draftCR for 38.133 on LTM measurements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_Mob_enh2-Core |  |  |  |  |  | Date: |  |  | 2024-02-19 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Changes to make the requirements to define what UE should do, instead of describing what UE should not do. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Editorial corrections to requirement applicability Notes in tables 9.14.5.1-1, 9.14.5.1-3, 9.15.6.1.1-1 and 9.15.6.1.1-2. - Correction to measurement restriction in 9.14.6.1 to consider L1-RSRP measurement, not RLM. - Editorial corrections to requirement applicability 9.15.6.2, 9.15.6.3 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Specification remains unclear |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# <Start of change 1>

Table 9.14.5.1-1: Intra-frequency L1-RSRP measurement period TL1-RSRP_Measurement_Period_SSB_intra in FR1 for UE incapable of [capability of measurement with RTD>CP]

| Configuration | TL1-RSRP_Measurement_Period_SSB_intra (ms) |
| --- | --- |
| non-DRX | max(TReport, ceil(M*P)*TSSB_NBC) |
| DRX cycle ≤ 320ms | max(TReport, ceil(K *M*P)*max(TDRX,TSSB_NBC)) |
| DRX cycle > 320ms | ceil(M*P)*TDRX |
| Note 1: TSSB_NBC is the periodicity of the neighbor cell SSB-Index configured for intra-frequency L1-RSRP measurement. TDRX is the DRX cycle length. TReport is configured periodicity for reporting. Note 2: K = 1.5. Note 3: The requirements in this table apply only if the actual RTD of serving cell and neighbor cell is larger than CP. |  |

Table 9.14.5.1-2: Intra-frequency L1-RSRP measurement period TL1-RSRP_Measurement_Period_SSB_intra in FR1 for UE capable of [capability of measurement with RTD>CP]

| Configuration | TL1-RSRP_Measurement_Period_SSB_intra (ms) |
| --- | --- |
| non-DRX | max(TReport, ceil(M*P)*TSSB_NBC*NLayer) |
| DRX cycle ≤ 320ms | max(TReport, ceil(K *M*P)*max(TDRX,TSSB_NBC) *NLayer) |
| DRX cycle > 320ms | ceil(M*P)*TDRX*NLayer |
| Note 1: TSSB_NBC is the periodicity of the neighbor cell SSB-Index configured for intra-frequency L1-RSRP measurement. TDRX is the DRX cycle length. TReport is configured periodicity for reporting. [NLayer = the number of intra-frequency layers configured for L1-RSRP measurement with [LTM-CSI-ResourceConfig-r18] + the number of inter-frequency layers without measurement gaps which are configured for L1-RSRP measurement with [LTM-CSI-ResourceConfig-r18]]. Note 2: K = 1.5. |  |

Table 9.14.5.1-3: Intra-frequency L1-RSRP measurement period TIntra_ L1-RSRP_Measurement_Period_SSB in FR2 for UE incapable of [capability of measurement with RTD>CP]

| Configuration | TL1-RSRP_Measurement_Period_SSB_intra (ms) |
| --- | --- |
| non-DRX | max(TReport, ceil(M*P*PL1_sharing*N)*TSSB_NBC) |
| DRX cycle ≤ 320ms | max(TReport, ceil(1.5*M*P*PL1_sharing *N)*max(TDRX,TSSB_NBC) ) |
| DRX cycle > 320ms | ceil(1.5*M*P*PL1_sharing*N)*TDRX |
| Note 1: TSSB_NBC is the periodicity of the neighbor cell SSB-Index configured for intra-frequency L1-RSRP measurement. TDRX is the DRX cycle length. TReport is configured periodicity for reporting. Note 2: The requirements in this table apply only if the actual RTD of serving cell and neighbor cell is not larger than CP Note3: When the number of neighbor cells to be measured on intra-frequency and inter-frequency without gap is more than 1, and TCI state(s) of at least one of these neighbor cells is in the active TCI state list, the requirements apply only to the cell(s) that have TCI state(s) in the active TCI state list. |  |

Table 9.14.5.1-4: Intra-frequency L1-RSRP measurement period TL1-RSRP_Measurement_Period_SSB_intra in FR2 for UE capable of [capability of measurement with RTD>CP]

| Configuration | TL1-RSRP_Measurement_Period_SSB_intra (ms) |
| --- | --- |
| non-DRX | max(TReport, ceil(M*P*PL1_sharing*N)*TSSB_NBC) |
| DRX cycle ≤ 320ms | max(TReport, ceil(1.5*M*P*PL1_sharing*N)*max(TDRX,TSSB_NBC)) |
| DRX cycle > 320ms | ceil(1.5*M*P*PL1_sharing*N)*TDRX |
| Note 1: TSSB_NBC is the periodicity of the neighbor cell SSB-Index configured for intra-frequency L1-RSRP measurement. TDRX is the DRX cycle length. TReport is configured periodicity for reporting. Note 2: When the number of neighbor cells to be measured on intra-frequency and inter-frequency without gap is more than 1 and TCI state(s) of at least one of these neighbor cells is in the active TCI state list, the requirements apply only to the cells that have TCI state(s) in the active TCI state list. |  |

<End of change 1>

# <Start of change 2>

#### 9.14.6.1	Measurement restriction for SSB based L1-RSRP

For FR1,

when the SSB for L1-RSRP measurement is in the same OFDM symbol as SSB transmitted from serving cell(s) for RLM, BFD, CBD or L1-RSRP measurement,

-	UE shall be able to measure the SSB for L1-RSRP measurement without any restriction;

when the SSB for L1-RSRP measurement is in the same OFDM symbol as CSI-RS transmitted from serving cell(s) for RLM, BFD, CBD or L1-RSRP measurement,

-	If SSB and CSI-RS have same SCS, UE shall be able to measure the SSB for L1-RSRP measurement without any restriction;

-	If SSB and CSI-RS have different SCS,

-	If UE supports simultaneousRxDataSSB-DiffNumerology, UE shall be able to measure the SSB for L1-RSRP measurement without any restriction;

-	[If UE does not support simultaneousRxDataSSB-DiffNumerology, UE is required to measure one of but not both SSB for L1-RSRP measurement and CSI-RS. Longer measurement period for SSB based L1-RSRP measurement is expected, and no requirements are defined].

For FR2, for UE incapable of [capability of measurement with RTD>CP] and for UE capable of [capability of measurement with RTD>CP],

when the SSB for L1-RSRP measurement on one CC is in the same or adjacent OFDM symbol as SSB transmitted from serving cell(s) for RLM, BFD, or CBD measurement on the same CC or different CCs in the same band, UE is required to measure one of but not both SSBs. Longer measurement period for SSB based L1-RSRP measurement is expected, and no requirements are defined.

<End of change 2>

# <Start of change 3>

##### 9.15.6.1.1	Inter-frequency SSB based L1-RSRP measurement

Table 9.15.6.1.1-1: Measurement delay for inter frequency L1-RSRP measurement without measurement gaps in FR1

| Configuration | TL1-RSRP_Measurement_Period_SSB_inter_withoutGap (ms) |
| --- | --- |
| non-DRX | max(TReport, ceil(M*P)*TSSB_NBC*NLayer) |
| DRX cycle ≤ 320ms | max(TReport, ceil(K *M*P)*max(TDRX,TSSB_NBC)*NLayer) |
| DRX cycle > 320ms | ceil(M*P)*TDRX*NLayer |
| Note 1: TSSB_NBC is the periodicity of the neighbor cell SSB-Index configured for intra-frequency L1-RSRP measurement. TDRX is the DRX cycle length. TReport is configured periodicity for reporting. [NLayer = the number of inter-frequency layers which are configured for L1-RSRP measurement with [LTM-CSI-ResourceConfig-r18] and measured without measurement gaps + the number of intra-frequency layers configured for L1-RSRP measurement with [LTM-CSI-ResourceConfig-r18]]. Note 2: K = 1.5. Note 3: For UE incapable of [capability of measurement with RTD>CP], the requirements apply only if the actual RTD between the serving cell and neighbour cell is not larger than CP. |  |

Editor note: FFS whether to add a new table on measurement delay for UE incapable of [capability of measurement with RTD>CP]

Table 9.15.6.1.1-2: Measurement delay for inter frequency L1-RSRP measurement without measurement gaps in FR2

| Configuration | TL1-RSRP_Measurement_Period_SSB_inter_withoutGap (ms) |
| --- | --- |
| non-DRX | max(TReport, ceil(M*P*PL1_sharing*N)*TSSB_NBC) |
| DRX cycle ≤ 320ms | max(TReport, ceil(1.5*M*P*PL1_sharing *N)*max(TDRX,TSSB_NBC) ) |
| DRX cycle > 320ms | ceil(1.5*M*P*PL1_sharing*N)*TDRX |
| Note 1: TSSB_NBC is the periodicity of the neighbor cell SSB-Index configured for intra-frequency L1-RSRP measurement. TDRX is the DRX cycle length. TReport is configured periodicity for reporting. Note 2: For UE incapable of [capability of measurement with RTD>CP], the requirements apply only if the actual RTD between serving cell and neighbor cell is not larger than CP . Note3: When the number of neighbor cells to be measured on intra-frequency and inter-frequency without gap is more than 1 and TCI state(s) of at least one of the neighbor cells is in the active TCI state list, the requirements apply only to the intra-frequency and inter-frequency without gap neighbor cells that have TCI state(s) in the active TCI state list. |  |

Editor note: FFS whether to add a new table on measurement delay for UE incapable of [capability of measurement with RTD>CP]

<End of change 3>

# <Start of change 4>

#### 9.15.6.2	Measurement restriction for inter frequency L1-RSRP measurement

Measurement restrictions described in the following clauses apply when UE is performing L1-RSRP measurement on neighbor cell(s) without measurement gap.

Unless explicitly stated, the SSB to be measured for L1-RSRP measurement is transmitted from neigbor cell(s).

Editor’s Note: The section may be further updated based any newly achieved agreements.

##### 9.15.6.2.1	Measurement restriction for SSB based L1-RSRP

[For FR1,

when the SSB for L1-RSRP measurement overlaps with the concerned OFDM symbols, where the concern OFDM symbols are:

-	The same OFDM symbol of SSB transmitted from serving cell(s) for RLM, BFD, CBD or L1-RSRP measurement, and

-	If UE support [capability of measurement with RTD>CP], 1 OFDM symbol before or after the OFDM symbol of SSB transmitted from serving cell(s) for RLM, BFD, CBD or L1-RSRP measurement.

- UE shall be able to measure the SSB for L1-RSRP measurement without any restriction;

when the SSB for L1-RSRP measurement overlapps with the concerned OFDM symbols, where the concern OFDM symbols are:

-	the same OFDM symbol of SSB transmitted from serving cell(s) for RLM, BFD, CBD or L1-RSRP measurement, and

-	If UE supports [capability of measurement with RTD>CP] capability, 1 OFDM symbol before or after the OFDM symbol of CSI-RS transmitted from serving cell(s) for RLM, BFD, CBD or L1-RSRP measurement.

-	If SSB and CSI-RS have same SCS, UE shall be able to measure the SSB for L1-RSRP measurement without any restriction;

-	If SSB and CSI-RS have different SCS,

-	If UE supports simultaneousRxDataSSB-DiffNumerology, UE shall be able to measure the SSB for L1-RSRP measurement without any restriction;

-	If UE does not support simultaneousRxDataSSB-DiffNumerology, UE is required to measure one of but not both SSB for L1-RSRP measurement and CSI-RS. Longer measurement period for SSB based L1-RSRP measurement is expected, and no requirements are defined.

For FR2,

when the SSB for L1-RSRP measurement on one CC overlapps with the concerned OFDM symbols, where the concern OFDM symbols are:

-	the same OFDM symbol as SSB transmitted from serving cell(s) for RLM, BFD, or CBD measurement on the same CC or different CCs in the same band, and

-	if UE supports [capability of measurement with RTD>CP] capability, 1 OFDM symbol before or after OFDM symbol of SSB transmitted from serving cell(s) for RLM, BFD, or CBD measurement on the same CC or different CCs in the same band

UE is required to measure one of but not both the SSBs. Longer measurement period for SSB based L1-RSRP measurement is expected, and no requirements are defined.

when the SSB for L1-RSRP measurement on one CC overlapps with the concerned OFDM symbols, where the concern OFDM symbols are

-	the same OFDM symbol as CSI-RS transmitted from serving cell(s) for RLM, BFD, or CBD measurement on the same CC or different CCs in the same band, and

-	If UE supports [capability of measurement with RTD>CP] capability, 1 OFDM symbol before or after OFDM symbol of CSI-RS transmitted from serving cell(s) for RLM, BFD, or CBD measurement on the same CC or different CCs in the same band.

UE is required to measure one of but not both SSB for L1-RSRP measurement and CSI-RS. Longer measurement period for SSB based L1-RSRP measurement is expected, and no requirements are defined.

For FR2, if the network configures same or mixed numerology between SSB for L1-RSRP measurement on one FR2 band and CSI-RS for RLM, BFD, CBD, L1-RSRP or L1-SINR measurement on the other FR2 band, UE shall be able to perform the related SSB based measurements in one band without any measurement restrictions in the other band, provided that UE is capable of independent beam management on this FR2 band pair.]

#### 9.15.6.3	Scheduling availability of UE during inter-frequency L1-RSRP measurements

Scheduling availability restrictions described in the following clauses apply when UE is performing L1-RSRP measurement on inter-frequency cell(s), and UE is receiving PDCCH/PDSCH from serving cell.

Unless explicitly stated, the SSB to be measured for L1-RSRP measurement is transmitted from neighbor cell(s).

##### 9.15.6.3.1	Scheduling availability of UE performing L1-RSRP measurement with a same subcarrier spacing as PDSCH/PDCCH on FR1

There are no scheduling restrictions due to L1-RSRP measurement performed on SSB as RS for L1-RSRP measurement with the same SCS as PDSCH/PDCCH in FR1.

##### 9.15.6.3.2	Scheduling availability of UE performing L1-RSRP measurement with a different subcarrier spacing than PDSCH/PDCCH on FR1

For UEs which support simultaneousRxDataSSB-DiffNumerology [14] there are no restrictions on scheduling availability due to L1-RSRP measurement based on SSB as RS for L1-RSRP measurement. For UEs which do not support simultaneousRxDataSSB-DiffNumerology [14] the following restrictions apply due to L1-RSRP measurement based on SSB configured for L1-RSRP measurement.

-	The UE is not expected to transmit PUCCH/PUSCH/SRS or receive PDCCH/PDSCH/CSI-RS for tracking/CSI-RS for CQI on the concerned OFDM symbols, where the concern OFDM symbols are

-	the same OFDM symbols corresponding to the SSB indexes configured for L1-RSRP measurement, and

-	If UE supports [capability of measurement with RTD>CP] capability, 1 OFDM symbol before or after the OFDM symbols corresponding to the SSB indexes configured for L1-RSRP measurement.

When intra-band carrier aggregation in FR1 is configured, the scheduling restrictions apply to cell(s) in the same band on the symbols that fully or partially overlap with restricted symbols. When inter-band carrier aggregation within FR1 is configured, there are no scheduling restrictions on FR1 cells configured in other bands than the bands in which the cell where L1-RSRP measurement is performed is configured.

##### 9.15.6.3.3	Scheduling availability of UE performing L1-RSRP measurement on FR2

The following scheduling restriction applies due to L1-RSRP measurement.

-	The UE is not expected to transmit PUCCH/PUSCH/SRS or receive PDCCH/PDSCH/CSI-RS for tracking/CSI-RS for CQI on the concerned OFDM symbols, where the concern OFDM symbols are:

-	the same OFDM symbols corresponding to the SSB indexes configured for L1-RSRP measurement, and

-	if UE supports [capability of measurement with RTD>CP] capability, 1 OFDM symbol before or after the OFDM symbols corresponding to the SSB indexes configured for L1-RSRP measurement.

When intra-band carrier aggregation in FR2 is performed, the scheduling restrictions is performed apply to cell(s) in the band on the symbols that fully or partially overlap with restricted symbols.

When inter-band carrier aggregation in FR2 is performed, there are no scheduling restrictions on FR2 cells in the bands due to L1-RSRP measurement performed on FR2 cell(s) in different band(s), provided that UE is capable of independent beam management on this FR2 band pair. Additionally, there is no scheduling restriction if the UE is configured with different numerology between SSB on one FR2 band and data on the other FR2 band provided the UE is configured for IBM operation for the band pair.

If following conditions are met,

-	UE has been notified about system information update through paging,

-	The gap between UE’s reception of PDCCH that UE monitors in the Type 2-PDCCH CSS set and that notifies system information update, and the PDCCH that UE monitors in the Type0-PDCCH CSS set, is greater than 2 slots,

For the SSB and CORESET for RMSI scheduling multiplexing patterns 3, UE is expected to receive the PDCCH that UE monitors in the Type0-PDCCH CSS set, and the corresponding PDSCH, on SSB symbols to be measured for L1-RSRP measurement; and

For the SSB and CORESET for RMSI scheduling multiplexing patterns 2, UE is expected to receive PDSCH that corresponds to the PDCCH that UE monitors in the Type0-PDCCH CSS set, on SSB symbols to be measured for L1-RSRP measurement.

##### 9.15.6.3.4	Scheduling availability of UE performing L1-RSRP measurement on FR1 or FR2 in case of FR1-FR2 inter-band CA

There are no scheduling restrictions on FR1 cell(s) due to L1-RSRP measurement performed on FR2 cell(s).

There are no scheduling restrictions on FR2 cell(s) due to L1-RSRP measurement performed on FR1 cell(s).

##### 9.15.6.3.5	Scheduling availability of UE performing L1-RSRP measurement in TDD bands on FR1

When UE performs L1-RSRP measurement on neighbor cell in a TDD band, the following restrictions apply due to L1-RSRP measurement

-	The UE is not expected to transmit PUCCH/PUSCH/SRS on the concerned OFDM symbols and restricted symbols may partially or fully overlap with UL symbols, where the concern OFDM symbols are:

-	the same OFDM symbols corresponding to the SSB indexes configured for L1-RSRP measurement, and

-	if UE supports [capability of measurement with RTD>CP] capability, 1 OFDM symbol before or after the OFDM symbols corresponding to the SSB indexes configured for L1-RSRP measurement.

<End of change 4>