3GPP TSG-RAN WG4 Meeting #110 	R4-2402710

Athens, Greece, February 26 – March 01, 2024

Source:	Nokia, Nokia Shanghai Bell

Title:	On NCR PDSCH and PDCCH Demodulation Requirements

Agenda item:	8.20.6

Document for:	Discussion

Introduction

The agreements made on NCR (Network-Controlled Repeaters) demodulation performance during RAN4#109 is documented in [1]. They were mainly agreements on the remaining open issue in RAN4#109, which was about Fixed Reference Channels (FRCs). As there are some essential typos in the agreed FRCs in [1], prior to writing the draft CR on the FRCs for Rel-18 NCR-MT, it is needed to first correct them.

In RAN4#109, the simulation results from companies were collected in [2]. As there were some test cases in PDSCH with 1% BLER, which have considerably high spans, offline discussions among companies have been conducted via email. We have updated our simulation results for 1% BLER, and our simulation results on PDSCH and PDCCH of NCR-MT for both ideal and with impairments cases are documented in [3] for information.

Discussion

In [1], the FRC for PDCCH FR2 is agreed as follows:

Issue 1-2-2: FRCs for NCR-MT PDCCH FR2 requirements <Way forward>: Parameter; Unit; Case 1; Case 2; Case 3. Subcarrier spacing; kHz; 30; 30; 30. CORESET frequency domain allocation; 102; 102; 90. CORESET time domain allocation; 1; 1; 1. Aggregation level; 2; 4; 8. DCI Format; 1_0; 1_1; 1_1. Payload (without CRC); Bits; 41; 53; 53.

The FRC for PDCCH FR2 agreed in RAN4#109 is for FR1.

RAN4 to use the following FRC for PDCCH FR2:

| Reference channel | M-FR2-F.2-1 | M-FR2-F.2-2 | M-FR2-F.2-3 |
| --- | --- | --- | --- |
| Subcarrier spacing (kHz) | 120 | 120 | 120 |
| CORESET frequency domain allocation | 60 | 60 | 60 |
| CORESET time domain allocation | 1 | 1 | 1 |
| Aggregation level | 2 | 4 | 8 |
| DCI Format | 1_0 | 1_1 | 1_1 |
| Payload (without CRC) (bits) | 40 | 56 | 56 |

Referring to [1], there were no FRC for CQI FR2. This is because of a typo in the following Issue:

Issue 1-3-2: FRCs for NCR-MT CQI FR1 requirements <Way forward>: Reference channel; Test 1/2. Number of allocated PDSCH resource blocks; 66. Number of consecutive PDSCH symbols; 12. Number of PDSCH MIMO layers; 1. Number of DMRS REs (Note 1); 24. Overhead for TBS determination; 6. Available RE-s for PDSCH; 7590. CQI index; Spectral efficiency; MCS index; Modulation; Information Bit Payload per Slot. 0; OOR; OOR; OOR; N/A. 1; 0.1523; 0; QPSK; 1800. 2; 0.2344; 0; 1800. 3; 0.3770; 2; 2856. 4; 0.6016; 4; 4480. 5; 0.8770; 6; 6528. 6; 1.1758; 8; 8712. 7; 1.4766; 11; 16QAM; 11016. 8; 1.9141; 13; 14343. 9; 2.4063; 15; 17928. 10; 2.7305; 18; 64QAM; 20496. 11; 3.3223; 20; 25104. 12; 3.9023; 22; 29192. 13; 4.5234; 24; 33816. 14; 5.1152; 26; 38936. 15; 5.5547; 28; 42016. NOTE 1: Number of DMRS REs includes the overhead of the DM-RS CDM groups without data NOTE 2: PDSCH is only scheduled on slots which are full DL.

The table in Issue 1-3-2 agreed in RAN4#109 is supposed to be for FR2 instead of FR1.

RAN4 to use the FRC table on Issue 1-3-2 agreed in RAN4#109 for FR2 instead of FR1.

All interested companies have aligned their simulation results via email offline discussions. Our updated simulation results are documented in [3].

The ideal simulation results from interested companies have been aligned, in which the spans are below the agreed threshold.

RAN4 to define demodulation performance requirements for Rel-18 NCR-MT based on the collected PDSCH and PDCCH simulation results from interested companies in RAN4#110.

Conclusion

Interested companies have discussed offline the simulation alignment of Rel-18 NCR-MT PDSCH and PDCCH based on the agreed FRCs. In this contribution, we provide our simulation results on PDSCH and PDCCH for Rel-18 NCR-MT demodulation performance requirements. The collected and aligned simulation results from interested companies shall be used to define demodulation performance requirements for Rel-18 NCR-MT in RAN4#110. The following observation and proposal were made:

Observation 1: The FRC for PDCCH FR2 agreed in RAN4#109 is for FR1.

Proposal 1: RAN4 to use the following FRC for PDCCH FR2:

| Reference channel | M-FR2-F.2-1 | M-FR2-F.2-2 | M-FR2-F.2-3 |
| --- | --- | --- | --- |
| Subcarrier spacing (kHz) | 120 | 120 | 120 |
| CORESET frequency domain allocation | 60 | 60 | 60 |
| CORESET time domain allocation | 1 | 1 | 1 |
| Aggregation level | 2 | 4 | 8 |
| DCI Format | 1_0 | 1_1 | 1_1 |
| Payload (without CRC) (bits) | 40 | 56 | 56 |

Observation 2: The table in Issue 1-3-2 agreed in RAN4#109 is supposed to be for FR2 instead of FR1.

Proposal 2: RAN4 to use the FRC table on Issue 1-3-2 agreed in RAN4#109 for FR2 instead of FR1.

Observation 3: The ideal simulation results from interested companies have been aligned, in which the spans are below the agreed threshold.

Proposal 3: RAN4 to define demodulation performance requirements for Rel-18 NCR-MT based on the collected PDSCH and PDCCH simulation results from interested companies in RAN4#110.

- References

- R4-2321059, WF on NCR_MT demodulation requirements, ZTE Corporation, 3GPP RAN4#109, Chicago, USA, November 13 – 17, 2023

- R4-2319538, Simulation results collection for NCR-MT demodulation requirements, ZTE Corporation, 3GPP RAN4#109, Chicago, USA, November 13 – 17, 2023

- R4-2402711, Simulation Results on NCR PDSCH and PDCCH Demodulation Requirements, Nokia, Nokia Shanghai Bell, 3GPP RAN4#110, Athens, Greece, February 26 – March 01, 2024