3GPP TSG-RAN WG4 Meeting #110	R4-2402732

# Athens, Greece, February 26 – March 1, 2024

Source:	Nokia, Nokia Shanghai Bell

Title:	RRM Core Requirements for LPHAP

Agenda item:	8.14.2.3

Document for:	Discussion

Introduction

The Rel-18 WID [1] for NR positioning includes the enhancements for LPHAP. In RAN4#109 [2], the issues on eDRX cycle in INACTIVE were discussed.

Issue 1-1-1: Start of PRS measurement - Agreement (from AH session) - the PRS measurement reporting periodicity is the configured reportingInterval in RequestLocationInformation. - When periodic PRS measurement reporting is not configured then the start of the PRS measurement is not limited to the PTW - Wayforward - Further discuss during maintenance phase: for Case 1, whether positioning measurement start is limited to PTW when PRS resource are not within PTW Issue 1-1-2: RRM measurement requirements for Case 2 - Agreement (from AH session) - The measurement cycle for defining additional RRM requirements (i.e. cell reselection) for Case 2 with periodic PRS measurement reporting is defined as: Max(Tpos, TDRX) - RRM measurement requirements (i.e. cell reselection requirements) apply when the UE is configured with the SRS transmission for positioning in RRC_INACTIVE. Issue 1-1-3: Impact of eDRX and PRS alignment - Wayforward - Option 1: - RAN4 not to define applicability condition for PRS measurement requirements related to alignment between (e)DRX and PRS configuration. - Option 2: - Wait for RAN2 progress before studying RRM impact of alignment between eDRX and PRS configuration. Issue 1-1-4: Relation between measurement window and PTW - Agreement (from AH session) - RAN4 not to define additional requirements on the PRS measurement window selection in relation to PTW. - Measurement window defined in rel. 17 RRC_INACTIVE state and PPW defined in rel. 17 RRC_CONNECTED state for gap-less measurement has no impact on each other.

Also, RAN4#109 discussed SRS positioning validity area.

Issue 1-2-1: One-shot autonomous TA adjustment - Question 1: what is the UL timing requirements for the subsequent SRS transmission after one-shot autonomous TA adjustment - Agreement (from AH session) - UL timing requirements for the subsequent SRS transmission after one-shot autonomous TA adjustment is ±Te - Question 2: what is the UE behaviour for multiple one-shot autonomous TA adjustment - Option 1 (CMCC): - UL transmit timing after UE autonomous TA adjustment = TDL_new - NTA + 2*(TDL_old - TDL_new), TDL_new is the DL timing from current camping cell. - For the subsequent UE autonomous TA adjustment (except 1st UE autonomous TA adjustment), further discuss how to decide NTA and TDL_old. - Option 1: NTA is the signalled TA from last serving cell (i.e. cell A as shown in Figure 1), TDL_old is the DL timing from lasting serving cell (i.e. T5 from cell A as shown in Figure 1) - Option 2: NTA is the new TA after UE autonomous TA adjustment from last camping cell (i.e. cell B as shown in Figure 1), TDL_old is the DL timing from lasting camping cell (i.e. T3 from cell B as shown in Figure 1) - RAN4 to discuss how to solve the issue: With the number of UE performing autonomous TA adjustment increase, the accumulated TA value decrease. As a result, the accumulated TA used for determining UE UL transmit timing could be zero. - UE only perform up to 1 time autonomous TA adjustment after receiving the signalled TA from last serving cell in Rel-18. - Way forward: - Check RAN1 progress and its impact on RAN4 requirements Issue 1-2-2: Impact of SRS positioning validity area - Agreement (from AH session) - RAN4 to update the applicability condition of UE Rx-Tx requirements considering SRS transmission in SRS positioning validity area. - The requirements apply provided that UE has valid SRS configuration in the current camping cell - RAN4 to update the UE behaviour for UE Rx-Tx requirements considering SRS transmission in SRS positioning validity area. - If cell reselection occurs, and UE reselects to a cell out of the positioning validity area or if UE performs autonomous TA adjustment at reselection, then UE shall restart the measurement Issue 1-2-3: Current RSRP for TA validation (related to RAN2 LS R2-2311568) - Agreement (from AH session) - Reference signal for the current RSRP is the SSB for the currently camped cell Issue 1-2-4: Requirements for TA validation - Agreement (from AH session) - Define TA validation requirements for SRS transmission for positioning in RRC_INACTIVE state for LPHAP with and without validity area configuration in TS 38.133. - Define TA validation requirements for SRS transmission for positioning in RRC_INACTIVE state for RedCap with and without validity area configuration in TS 38.133. - The TA validation requirements for SRS transmission can be based on the existing TA validation requirements defined for CG-SDT in clause 5.5.3 for LPHAP and clause 5.2B.3 for RedCap by adapting the SRS specific aspects e.g. using inactivePosSRS-RSRP-ChangeThreshold, no subsequent SRS transmission per period and SRS-PosRRC-InactiveValidityAreaConfig. - Note: For CR drafting, avoid redundancy of requirements for LPHAP and RedCap since they are the same and instead use reference.

In RAN4#109, PRS measurement in IDLE was discussed.

Issue 1-3-1: PRS measurement requirements - Agreement (from AH session) - Tavailable = LCM (TPRS,i, TDRX), and TDRX is defined as T in Rel-17 TS 38.304 Issue 1-3-2: RRM measurement requirements - Agreement - RRM measurement requirements (i.e. cell reselection requirements) do not apply in RRC_IDLE.

RRM core requirements are discussed in this contribution and few proposals on further proceedings are made.

Discussion

The impacts to RRM core requirements for some of the above enhancements are discussed in this section.

eDRX cycle beyond 10.24s in INACTIVE

Start of PRS measurement

It is agreed that the PRS measurement reporting periodicity is the configured reportingInterval in RequestLocationInformation. Also when periodic PRS measurement reporting is not configured then the start of the PRS measurement is not limited to the PTW.

From the agreement in WF R4-2314462, there are two cases for PRS measurement with RAN eDRX > 10.24s

- Case 1: eDRX cycle is smaller or equal to configured PRS measurement reporting periodicity. In this case, UE starts positioning measurement within PTW.

- Case 2: eDRX cycle is longer than configured PRS measurement reporting periodicity. In this case, positioning measurement start is not limited to PTW.

It needs to be discussed for Case 1, whether positioning measurement start is limited to PTW when PRS resource are not within PTW. In this case, positioning measurement start does not need to be limited to PTW when PRS resource are not within PTW.

Positioning measurement start is not limited to PTW in Case 1.

In case the UE performs the PRS measurement outside of the PTW, RAN4 may need to discuss if the UE measurement behavior is just up to the UE implementations. The eDRX cycle is quite long, so the UE may need synchronization information and/or related QCL RS measurements. While the UE is in a sleep mode, the UE may not be necessarily required to perform SSB measurements, and the specification does not prohibit the measurement behavior. The UE may or may not perform SSB measurement before performing the DL PRS measurement, but it may affect measurement accuracy. It may be necessary to discuss this issue with case 2.

UE may or may not perform SSB measurement before performing the DL PRS measurement, and it may affect accuracy of measurement performed outside of PTW when UE is in a sleep mode with long eDRX cycle.

RAN4 to discuss whether UE measurement behavior before performing the DL PRS measurement, is up to UE implementations when the start of the PRS measurement is not limited to the PTW in Case 2.

Conclusion

The scope of RRM impacts for Rel-18 Expanded and Improved NR positioning is investigated in this contribution.

The following proposals are made.

- Positioning measurement start is not limited to PTW in Case 1.

- UE may or may not perform SSB measurement before performing the DL PRS measurement, and it may affect accuracy of measurement performed outside of PTW when UE is in a sleep mode with long eDRX cycle.

RAN4 to discuss whether UE measurement behavior before performing the DL PRS measurement, is up to UE implementations when the start of the PRS measurement is not limited to the PTW in Case 2.

- References

- RP-230328, “Revised WID on Expanded and Improved NR Positioning,” Intel Corporation, RAN#99.

- R4-2321525, “WF on NR Positioning - LPHAP,” Huawei, HiSilicon, RAN4#109.