3GPP TSG-RAN4 Meeting #110	R4-2402770

# Athens, Greece, 26th Feb 2024 – 1st Mar 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.113 | CR | 0070 | rev | - | Current version: | 16.9.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | (NR_newRAT-Perf) CR to TS 38.113 on introduction of NR with in-band NB-IoT |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Perf |  |  |  |  |  | Date: |  |  | 2024-02-19 |
| Category: | F |  |  |  |  |  | Release: |  |  | 16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - NR with in-band NB-IoT has been introduced in 38.104 and 38.141-1, while not included in 38.113. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - NR with in-band NB-IoT is added. - Fixed reference channel models used in Performance Criteria for continuous and transient phenomena for BS need to be updated. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - NR with in-band NB-IoT is not covered. - Fixed reference channel models used in Performance Criteria for continuous and transient phenomena for BS are not up to date. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 1, 3.1, 3.3, 6.1, 6.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

---------- start of changes----------

# 1	Scope

The present document covers the assessment of NR and NR with NB-IoT in-band operation Base Station (BS) and ancillary equipment in respect of Electromagnetic Compatibility (EMC).

The present document specifies the applicable requirements, procedures, test conditions, performance assessment and performance criteria for NR and NR with NB-IoT in-band operation base stations and associated ancillary equipment in the following categories:

-	BS equipped with antenna connectors or TAB connectors which are possible to be terminated during EMC testing, meeting the BS type 1-C and BS type 1-H RF requirements of TS 38.104 [2], with conformance demonstrated by compliance to TS 38.141-1 [3].

-	BS not equipped with antenna connectors nor TAB connectors, i.e. with antenna elements radiating during the EMC testing, meeting the BS type 1-O and BS type 2-O RF requirements of TS 38.104 [2], with conformance demonstrated by compliance to TS 38.141-2 [4].

Technical requirements related to the antenna and TAB connectors are not included in the present document. These are found in the relevant NR BS product standards TS 38.104 [2], 3GPP TS 38.141-1[3], and 3GPP TS 38.141-2 [4].

The environment classification used in the present document refers to the residential, commercial and light industrial environment classification used in IEC 61000-6-1 [7], IEC 61000-6-3 [8] and IEC 61000-6-8 [33].

The EMC requirements have been selected to ensure an adequate level of compatibility for apparatus at residential, commercial and light industrial environments. The levels, however, do not cover extreme cases which may occur in any location but with low probability of occurrence.

# 2	References

---------- end of changes----------

---------- start of changes----------

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

ancillary equipment: electrical or electronic equipment, that is intended to be used with a receiver or transmitter

NOTE:	It is considered as an ancillary equipment if:

the equipment is intended for use with a receiver or transmitter to provide additional operational and/or control features to the radio equipment, (e.g. to extend control to another position or location); and

the equipment cannot be used on a stand alone basis to provide user functions independently of a receiver or transmitter; and

the receiver or transmitter, to which it is connected, is capable of providing some intended operation such as transmitting and/or receiving without the ancillary equipment (i.e. it is not a sub-unit of the main equipment essential to the main equipment basic functions).

antenna port: for EMC purposes, port for connection of an antenna used for intentional transmission and/or reception of radiated RF energy, equivalent to an RF antenna connector/TAB connector in TS 37.105 [25].

BS type 1-C:	NR base station operating at FR1 with requirements set consisting only of conducted requirements defined at individual antenna connectors.

BS type 1-H:	NR base station operating at FR1 with a requirement set consisting of conducted requirements defined at individual TAB connectors and OTA requirements defined at RIB.

BS type 1-O:	NR base station operating at FR1 with a requirement set consisting only of OTA requirements defined at the RIB.

BS type 2-O:	NR base station operating at FR2 with a requirement set consisting only of OTA requirements defined at the RIB.

channel bandwidth: the RF bandwidth supporting a single NR RF carrier with the transmission bandwidth configured in the uplink or downlink of a cell. The channel bandwidth is measured in MHz and is used as a reference for transmitter and receiver RF requirements.

continuous phenomena: electromagnetic disturbance, the effects of which on a particular device or equipment cannot be resolved into a succession of distinct effects (IEC 60050-161 [9]).

enclosure port: physical boundary of the equipment through which electromagnetic fields may radiate or impinge.

NOTE:	In the case of integral antenna equipment, this port is inseparable from the antenna port.

exclusion band: frequency range(s) not subject to test or assessment.

free-space open area test site: reference test site with precautions to ensure that reflections do not influence the measurement.

NOTE:	The free-space open area test site (FSOATS) is the concept of the test site. A practical approximation is a Fully-Anechoic Room (FAR).

fully-anechoic room: shielded enclosure, the internal surfaces of which are lined with radio-frequency-energy absorbing material (i.e. RF absorber) that absorbs electromagnetic energy in the frequency range of interest.

integral antenna: antenna designed for permanent connection to the equipment and considered part of the enclosure port.

NOTE:	An integral antenna may be fitted internally or externally.

lower RF bandwidth edge: the frequency of the lower edge of the Base Station RF bandwidth, used as a frequency reference point for transmitter and receiver requirements.

NB-IoT operation in NR in-band: NB-IoT is operating in-band when it is located within a NR transmission bandwidth configuration plus 15 kHz at each edge but not within the NR minimum guard band GBChannel.

NB-IoT operation in NR guard band: NB-IoT is operating in guard band when it is located within a NR BS channel bandwidth but is not NB-IoT operation in NR in-band.

operating band: frequency range in which NR operates (paired or unpaired), that is defined with a specific set of technical requirements.

port: particular interface of EUT used for EMC requirements testing purposes.

NOTE:	Any connection point on EUT intended for connection of cables to or from EUT during the EMC testing is considered as a port.

EXAMPLE 1:	Examples of ports for BS type 1-C and BS type 1-H are as presented in figure 3.1-1:

Figure 3.1-1: Examples of ports for BS type 1-C and BS type 1-H

EXAMPLE 2:	Examples of ports for BS type 1-O and BS type 2-O (i.e. with no antenna ports) are as presented in figure 3.1-2:

Figure 3.1-2: Examples of ports for BS type 1-O and BS type 2-O

receiver exclusion band: band of frequencies over which no tests of radiated immunity of a receiver are made, and expressed relative to the BS receive band.

semi-anechoic chamber: shielded enclosure in which all surfaces except the metal floor are covered with material that absorbs electromagnetic energy (i.e. RF absorber) in the frequency range of interest.

signal port: port intended for the interconnection of components of an EUT, or between an EUT and associated equipment and used in accordance with relevant functional specifications (for example for the maximum length of cable connected to it).

spatial exclusion zone: range of angles where no tests of radiated immunity are made for BS type 1-O or BS type 2-O (i.e. half sphere around the EUT's radiating direction).

TAB connector: transceiver array boundary connector

Throughput: number of payload bits successfully received per second for a reference measurement channel in a specified reference condition.

transceiver array boundary: conducted interface between the transceiver unit array and the composite antenna.

transceiver unit: active unit consisting of transmitter and/or receiver which transmits and/or receives radio signals, and which may include passive RF filters.

telecommunication port: ports which are intended to be connected to telecommunication networks (e.g. public switched telecommunication networks, integrated services digital networks), local area networks (e.g. Ethernet, Token Ring) and similar networks.

NOTE:	Telecommunication port is called "wired network port" in CISPR 32 [11] and ETSI EN 301 489-1 [23].

transient phenomena: pertaining to or designating a phenomena or a quantity which varies between two consecutive steady states during a time interval short compared with the time-scale of interest (IEC 60050-161 [9]).

transmitter exclusion band: band of frequencies over which no tests of radiated immunity of a transmitter are made and is expressed relative to the carrier frequencies used (the carrier frequencies of the base stations activated transmitter(s)).

upper RF bandwidth edge: the frequency of the upper edge of the Base Station RF bandwidth, used as a frequency reference point for transmitter and receiver requirements.

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

β	Bandwidth

BWChannel	Channel bandwidth

ΔfOBUE	Maximum offset of the operating band unwanted emissions mask from the downlink operating band edge

ΔfOOB	Maximum offset of the out-of-band boundary from the uplink operating band edge

FDL,low	The lowest frequency of the downlink operating band

FDL,high	The highest frequency of the downlink operating band

FUL,low	The lowest frequency of the uplink operating band

FUL,high	The highest frequency of the uplink operating band

ΔfRIexclusion	Maximum offset of the Radiated Immunity exclusion band from the uplink operating band edge for test without spatial exclusion zone applied

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

AC	Alternating Current

AMN	Artificial Mains Network

BC	Band Category

BS	Base Station

CA	Carrier Aggregation

CDN	Coupling/Decoupling Network

CS	Capability Set

DC	Direct Current

EIRP	Equivalent Isotropic Radiated Power

EMC	Electromagnetic Compatibility

e.r.p.	Effective Radiated Power

ESD	Electrostatic Discharge

EUT	Equipment Under Test

FR	Frequency Range

FAR	Fully-Anechoic Room

FRC	Fixed Reference Channel

FSOATS	Free Space OATS

NC	Non Contiguous

NG	Next Generation

NGC	Next Generation Core

NB-IoT	Narrowband – Internet of Things

NR	New Radio

NR-ARFCN	NR Absolute Radio Frequency Channel Number

NRTC	NR Test Configuration

NTC	Test Configuration for Non-contiguous operation

NSA	Normalized Site Attenuation

OATS	Open Area Test Site

RAT	Radio Access Technology

RF	Radio Frequency

RIB	Radiated Interface Boundary

rms	root mean square

RSM	Reference Site Method

SAC	Semi Anechoic Chamber

SC	Single Carrier

SDL	Supplementary Downlink

TC	Test Configuration

UL	Uplink

# 4	Test conditions

---------- end of changes----------

---------- start of changes----------

## 6.1	Performance criteria for continuous phenomena for BS

The test should, where possible, be performed using a bearer with the characteristics of data rate and throughput defined in table 6.1-1, table 6.1-1a, table 6.1-1b and table 6.1-2. If the test is not performed using one of these bearers (for example, of none of them are supported by the BS), the characteristics of the bearer used shall be recorded in the test report.

The throughput in table 6.1-1, table 6.1-1a, table 6.1-1b and table 6.1-2 is stated relative to the maximum throughput of the FRC.

The BS uplink and downlink paths shall each meet the performance criteria defined in table 6.1-1, table 6.1-1a, table 6.1-1b and table 6.1-2 during the test. If the uplink and downlink paths are evaluated as a one loop then the criteria is two times the throughput reduction shown in table 6.1-1, table 6.1-1a, table 6.1-1b and table 6.1-2 (i.e. throughput > 90 % instead of throughput > 95 %). After each test case BS shall operate as intended with no loss of user control function, stored data and the communication link shall be maintained.

Table 6.1-1: FR1 performance criteria for continuous phenomena for BS

| NR channel bandwidth (MHz) as defined in TS 38.104 section 5.3.2-1 [2] | Sub-carrier spacing (kHz) | Bearer information data rate (Note 3) | Performance criteria (Note 1, Note 2) |
| --- | --- | --- | --- |
| 3 | 15 | G-FR1-A1-7 in annex A.1 in TS 38.104 [2] | Throughput > 95 %, no loss of service |
|  |  | G-FR1-A1-21 in annex A.1 in TS 38.104 [2] for NR with NB-IoT in-band operation |  |
| 5, 10, 15 | 15 | G-FR1-A1-1 in annex A.1 in TS 38.104 [2] |  |
|  |  | G-FR1-A1-10 in annex A.1 in TS 38.104 [2] for NR with NB-IoT in-band operation |  |
| 10, 15 | 30 | G-FR1-A1-2 in annex A.1 in TS 38.104 [2] |  |
| 10, 15 | 60 | G-FR1-A1-3 in annex A.1 in TS 38.104 [2] |  |
| 20 to 50 | 15 | G-FR1-A1-4 in annex A.1 in TS 38.104 [2] |  |
|  |  | G-FR1-A1-11 in annex A.1 in TS 38.104 [2] for NR with NB-IoT in-band operation |  |
| 20 to 100 | 30 | G-FR1-A1-5 in annex A.1 in TS 38.104 [2] |  |
| 20 to 100 | 60 | G-FR1-A1-6 in annex A.1 in TS 38.104 [2] |  |
| NOTE 1: The performance criteria, throughput > 95 %, no loss of service, applies also if a bearer with another characteristics is used in the test. NOTE 2: The performance criteria, throughput > 90 %, no loss of service, applies instead if the uplink and downlink paths are evaluated as a one loop. NOTE 3: These reference measurement channels are not applied for band n46, n96 and n102. |  |  |  |

Table 6.1-1a: FR1 performance criteria for continuous phenomena for band n46

| NR channel bandwidth (MHz) as defined in TS 38.104 section 5.3.2-1 [2] | Sub-carrier spacing (kHz) | Bearer information data rate | Performance criteria (Note 1, Note 2) |
| --- | --- | --- | --- |
| 10 | 15 | G-FR1-A1-12 in clause A.1 in ETSI TS 138 104 [20] | Throughput > 95 % no loss of service |
|  | 30 | G-FR1-A1-13 in clause A.1 in ETSI TS 138 104 [20] |  |
|  | 60 | G-FR1-A1-3 in clause A.1 in ETSI TS 138 104 [20] |  |
| 20 | 15 | G-FR1-A1-14 in clause A.1 in ETSI TS 138 104 [20] |  |
|  | 30 | G-FR1-A1-15 in clause A.1 in ETSI TS 138 104 [20] |  |
|  | 60 | G-FR1-A1-6 in clause A.1 in ETSI TS 138 104 [20] |  |
| 40 | 15 | G-FR1-A1-16 in clause A.1 in ETSI TS 138 104 [20] |  |
|  | 30 | G-FR1-A1-17 in clause A.1 in ETSI TS 138 104 [20] |  |
|  | 60 | G-FR1-A1-6 in clause A.1 in ETSI TS 138 104 [20] |  |
| 60 | 30 | G-FR1-A1-18 in clause A.1 in ETSI TS 138 104 [20] |  |
|  | 60 | G-FR1-A1-6 in clause A.1 in ETSI TS 138 104 [20] |  |
| 80 | 30 | G-FR1-A1-19 in clause A.1 in ETSI TS 138 104 [20] |  |
|  | 60 | G-FR1-A1-6 in clause A.1 in ETSI TS 138 104 [20] |  |
| NOTE 1: The performance criteria, throughput > 95 %, no loss of service, applies also if a bearer with another characteristics is used in the test. NOTE 2: The performance criteria, throughput > 90 %, no loss of service, applies instead if the uplink and downlink paths are evaluated as a one loop. |  |  |  |

Table 6.1-1b: FR1 performance criteria for continuous phenomena for band n96 and n102

| NR channel bandwidth (MHz) as defined in TS 38.104 section 5.3.2-1 [2] | Sub-carrier spacing (kHz) | Bearer information data rate | Performance criteria (Note 1, Note 2) |
| --- | --- | --- | --- |
| 20 | 15 | G-FR1-A1-14 in clause A.1 in ETSI TS 138 104 [20] | Throughput > 95 % no loss of service |
|  | 30 | G-FR1-A1-15 in clause A.1 in ETSI TS 138 104 [20] |  |
|  | 60 | G-FR1-A1-6 in clause A.1 in ETSI TS 138 104 [20] |  |
| 40 | 15 | G-FR1-A1-16 in clause A.1 in ETSI TS 138 104 [20] |  |
|  | 30 | G-FR1-A1-17 in clause A.1 in ETSI TS 138 104 [20] |  |
|  | 60 | G-FR1-A1-6 in clause A.1 in ETSI TS 138 104 [20] |  |
| 60 | 30 | G-FR1-A1-18 in clause A.1 in ETSI TS 138 104 [20] |  |
|  | 60 | G-FR1-A1-6 in clause A.1 in ETSI TS 138 104 [20] |  |
| 80 | 30 | G-FR1-A1-19 in clause A.1 in ETSI TS 138 104 [20] |  |
|  | 60 | G-FR1-A1-6 in clause A.1 in ETSI TS 138 104 [20] |  |
| NOTE 1: The performance criteria, throughput > 95 %, no loss of service, applies also if a bearer with another characteristics is used in the test. NOTE 2: The performance criteria, throughput > 90 %, no loss of service, applies instead if the uplink and downlink paths are evaluated as a one loop. |  |  |  |

Table 6.1-2: FR2 performance criteria for continuous phenomena for BS

| NR channel bandwidth (MHz) | Sub-carrier spacing (kHz) | Bearer information data rate | Performance criteria (Note 1, Note 2) |
| --- | --- | --- | --- |
| 50, 100, 200 | 60 | G-FR2-A1-1 in annex A.1 in TS 38.104 [2] | Throughput > 95 %, no loss of service |
| 50 | 120 | G-FR2-A1-2 in annex A.1 in TS 38.104 [2] |  |
| 100, 200, 400 | 120 | G-FR2-A1-3 in annex A.1 in TS 38.104 [2] |  |
| NOTE 1: The performance criteria, throughput > 95 %, no loss of service, applies also if a bearer with another characteristics is used in the test. NOTE 2: The performance criteria, throughput > 90 %, no loss of service, applies instead if the uplink and downlink paths are evaluated as a one loop. |  |  |  |

## 6.2	Performance criteria for transient phenomena for BS

At the conclusion of the total test (comprising the series of individual exposures to transient phenomena) the EUT shall operate as intended with no user noticeable loss of control functions or stored data. At the conclusion of each exposure the EUT shall operate with no user noticeable loss of the communication link. The channel bandwidth, sub-carrier spacing and bearer information data rate should be chosen based on manufacturers declarations (see declaration D.14 in TS 38.141-1 [3] and/or D.7 in TS 38/141-2 [4]) and defined according to table 6.1-1, table 6.1-1a, table 6.1-1b and table 6.1-2.

## 6.3	Performance criteria for continuous phenomena for Ancillary equipment

---------- end of changes----------