3GPP TSG-RAN4 Meeting #110	R4-2402773

# Athens, Greece, 26th Feb 2024 – 1st Mar 2024

Title: 	[LTE-RF, TEI15] On the open issues regarding EMC TC simplification

Agenda Item:	4.3

Source: 	Ericsson

Document for:	Discussion

# Discussion

In the agreed WF (R4-2321070) [1], there are open issues on maintenance of EMC TC simplification. In this contribution, we would like to provide our feedback.

Companies are encouraged to investigate and provide feedback on the following concepts, and their potential implementation: Concept 1: Do not update EMC Emissions test spec by copy-pasting TC tables content from RF test specification. Instead, remove existing and incomplete TC tables from EMC specifications and replace them by detailed references to related BS RF test specifications, pointing to specific Tx spurious emissions requirements. Concept 2: Do not update EMC Immunity test spec by copy-pasting TC tables content from RF test specification. Instead, remove existing and incomplete TC tables from EMC specifications and replace them by detailed references to related BS RF test specifications, pointing to specific Blocking requirements.

When performing EMC assessment of base station, proper test configurations need to be set to make sure base station work in a typical operating mode.

Although EMC is part of RF work in 3GPP, an EMC specification shall be self-contained, independent of RF specification. An EMC test engineer does not need to be familiar with RF requirements.

Regarding emission, transmitter spurious emission is different from EMC emission. Spurious emissions are emissions which are caused by harmonics emission, intermodulation products, etc., while EMC emission are caused by digital circuits. To some extent, they have resemblance, but they are indeed different requirements, and the mechanism behind is different, in other words, the root cause of failure is different for them. Simply using the word of “reuse” could make people confuse the two concepts.

Regarding immunity, similarly, EMC immunity and blocking are two different requirements. Simply using the word of “reuse” could make people confuse the two concepts. Besides, in some of EMC specs, test configuration of blocking is referred, while in others, where there is no blocking requirement, e.g., NR repeater, test configuration of requirement other than blocking is referred.

To sum up, EMC requirements shall have dedicated table of test configurations for EMC testing purpose. Hence, we propose to keep the current framework of test configurations as it is and do not remove EMC dedicated tables of test configurations.

Proposal 1: Keep the current framework as it is and do not remove EMC dedicated tables of test configurations.

# Conclusion

Proposal 1: Keep the current framework as it is and do not remove EMC dedicated tables of test configurations.

# References

R4-2321070. “WF on maintenance of EMC TC simplification”