3GPP TSG RAN WG4 Meeting #110															R4-2402847

Athens, Greece, 26 Feb. 2024 – 01 Mar. 2024

Agenda Item:	8.4.3

Source: 	Ericsson

Title:	Discussion on performance part of FR2 SCell activation enhancement

Document for:	Discussion

# Introduction

In this contribution, we provide our views on the performance part of the FR2 SCell activation enhancements in R18.

# Discussion

RAN4 agreed on following solutions for FR2 SCell activation delay reduction in Rel-18.

- FG31-1: L3 report-based enhancement

- FG31-2: Beam sweeping factor reduction based SCell activation delay reduction

- FG31-3: Use SSB periodicity instead of SMTC periodicity” and “Performing L1-RSRP measurement in non-DRX mode even whenDRX is configured”

Based on the above enhancement following tests are listed as potential test case list.

Issue 3-2-6: Candidate test case list based on proposals from companies

- Please note the following table doesn’t mean all the test cases in this table will be specified.

- TC1/5/9 will be specified. Other TCs are still FFS.

| Feature | Index | Test case | This TC is needed | This TC is NOT needed | FR1 or FR2 | Volunteer Company to lead CR |
| --- | --- | --- | --- | --- | --- | --- |
| FG31-1: L3 report based enhancement | 1 | FR2 unknown SCell activation with L3 report | Apple, Nokia, CMCC, OPPO, CTC, ZTE, QC, Ericsson, MTK,vivo, Huawei |  | For FR2 | QC (NR-CA) |
|  | 2 | FR2 PUCCH SCell activation delay with L3 report | Nokia, ZTE, Huawei | Apple, MTK,QC,vivo | For FR2 | Huawei |
|  | 3 | Multiple SCell activation delay with FR2 unknown SCell with L3 report | Nokia, CMCC, ZTE, Ericsson, Huawei | Apple, MTK,QC | For FR2 |  |
|  | 4 | FR2 PUCCH SCell activation delay with L3 report with Multiple SCells | CMCC, ZTE, Huawei | Apple, MTK,QC,vivo | For FR2 |  |
|  | 5 | FR1 unknown SCell activation with L3 report | Apple, Nokia, QC, CTC,vivo, MTK |  | For FR1 | vivo |
|  | 6 | FR1 PUCCH SCell activation delay with L3 report | Ericsson | Apple,QC,vivo, MTK | For FR1 |  |
|  | 7 | Multiple SCell activation delay with FR1 unknown SCell with L3 report | Ericsson | Apple,QC,vivo, MTK | For FR1 |  |
|  | 8 | FR1 PUCCH SCell activation delay with L3 report with Multiple Scells |  | Apple,QC,vivo, MTK | For FR1 |  |
| FG31-2: Beam sweeping factors reduction | 9 | FR2 unknown SCell activation with FG31-2 and FG31-3 | Apple, Nokia, CMCC, OPPO, CTC, QC, MTK,vivo, Huawei |  | For FR2 (FG31-3 in this TC is up to issue 3-2-4) | Nokia |
|  | 10 | Multiple SCell activation delay with FR2 unknown SCell with FG31-2 and FG31-3 |  | Apple, MTK,QC,vivo | For FR2(FG31-3 in this TC is up to issue 3-2-4) |  |
|  | 11 | FR2 PUCCH SCell activation delay with FG31-2 and FG31-3 | Nokia, Ericsson, Huawei | Apple, MTK,QC,vivo | For FR2(FG31-3 in this TC is up to issue 3-2-4) |  |
|  | 12 | FR2 PUCCH SCell activation delay with FG31-2 and FG31-3 with Multiple SCells |  | Apple, MTK,QC,vivo | For FR2(FG31-3 in this TC is up to issue 3-2-4) |  |
|  | 13 | FR2 Direct SCell activation delay with FG31-2 and FG31-3 at SCell addition | Nokia, CMCC, Ericsson | Apple, MTK,QC,vivo | For FR2(FG31-3 in this TC is up to issue 3-2-4) |  |
|  | 14 | FR2 Direct SCell activation delay with FG31-2 and FG31-3 at Handover | Nokia, CMCC, Ericsson | Apple, MTK,QC,vivo | For FR2(FG31-3 in this TC is up to issue 3-2-4) |  |
|  | 15 | FR2 Direct SCell activation delay of Multiple Downlink SCells with FG31-2 and FG31-3 at SCell addition |  | Apple, MTK,QC,vivo | For FR2(FG31-3 in this TC is up to issue 3-2-4) |  |
|  | 16 | FR2 Direct SCell activation delay of Multiple Downlink SCells with FG31-2 and FG31-3 at Handover |  | Apple, MTK,QC,vivo | For FR2(FG31-3 in this TC is up to issue 3-2-4) |  |
| FG31-3: (1)Use SSB periodicity instead of SMTC periodicity” (2)“Performing L1-RSRP measurement in non-DRX mode even DRX is configured” | 17 | FR1 unknown SCell activation with FG31-3 | Apple, CMCC, CTC, Ericsson, Nokia,vivo | QC, MTK | For FR1 |  |
|  | 18 | Multiple SCell activation delay with FR1 unknown SCell with FG31-3 |  | Apple,QC,vivo, MTK | For FR1 |  |
|  | 19 | FR1 PUCCH SCell activation delay with FG31-3 |  | Apple,QC,vivo, MTK | For FR1 |  |
|  | 20 | FR1 PUCCH SCell activation delay with FG31-3 with Multiple SCells |  | Apple,QC,vivo, MTK | For FR1 |  |
|  | 21 | FR1 Direct SCell activation delay with FG31-3 at Scell addition |  | Apple,QC,vivo, MTK | For FR1 |  |
|  | 22 | FR1 Direct SCell activation delay with FG31-3 at Handover |  | Apple,QC,vivo, MTK | For FR1 |  |
|  | 23 | FR1 Direct SCell activation delay of Multiple Downlink Scells with FG31-3 at Scell addition |  | Apple,QC,vivo, MTK | For FR1 |  |
|  | 24 | Direct FR2 SCell activation delay of Multiple Downlink Scells with FG31-3 at Handover |  | Apple,QC,vivo, MTK | For FR1 |  |
|  | 25 | FR2 unknown SCell activation with FG31-3 | Nokia, Huawei | Apple,QC, MTK | For FR2(need this TC or not is up to issue 3-2-4) |  |
|  | 26 | Multiple SCell activation delay with FR2 unknown Scell with FG31-3 |  | Apple,QC,vivo, MTK | For FR2(need this TC or not is up to issue 3-2-4) |  |
|  | 27 | FR2 PUCCH SCell activation delay with FG31-3 |  | Apple,QC,vivo, MTK | For FR2(need this TC or not is up to issue 3-2-4) |  |
|  | 28 | FR2 PUCCH SCell activation delay with FG31-3 with Multiple Scells |  | Apple,QC,vivo, MTK | For FR2(need this TC or not is up to issue 3-2-4) |  |
|  | 29 | FR2 Direct SCell activation delay with FG31-3 at Scell addition |  | Apple,QC,vivo, MTK | For FR2(need this TC or not is up to issue 3-2-4) |  |
|  | 30 | FR2 Direct SCell activation delay with FG31-3 at Handover |  | Apple,QC,vivo, MTK | For FR2(need this TC or not is up to issue 3-2-4) |  |
|  | 31 | FR2 Direct SCell activation delay of Multiple Downlink Scells with FG31-3 at Scell addition |  | Apple,QC,vivo, MTK | For FR2(need this TC or not is up to issue 3-2-4) |  |
|  | 32 | Direct FR2 SCell activation delay of Multiple Downlink Scells with FG31-3 at Handover |  | Apple,QC,vivo, MTK | For FR2(need this TC or not is up to issue 3-2-4) |  |

Note: the color code in above table is only for differentiating FR1 and FR2 TCs.

From the above table, we are fine to limit the tests to

- FR2 unknown SCell activation with L3 report

- Multiple SCell activation delay with FR2 unknown SCell with L3 report

- FR1 PUCCH SCell activation delay with L3 report

- FR2 PUCCH SCell activation delay with FG31-2 and FG31-3

- FR2 Direct SCell activation delay with FG31-2 and FG31-3 at SCell addition

- FR1 unknown SCell activation with FG31-3

- RAN4 to define the test cases for at least following scenarios or requirements

- FR2 unknown SCell activation with L3 report

- Multiple SCell activation delay with FR2 unknown SCell with L3 report

- FR1 PUCCH SCell activation delay with L3 report

- FR2 PUCCH SCell activation delay with FG31-2 and FG31-3

- FR2 Direct SCell activation delay with FG31-2 and FG31-3 at SCell addition

- FR1 unknown SCell activation with FG31-3

Since FR2 SCell can be in SA or EN-DC or NR-DC scenario, we think the tests need to be run in all these scenarios.

- RAN4 to perform the tests for EN-DC, NR-DC and SA scenarios.

One issue briefly discussed was how to make sue L3 report is valid in the test case.

We think L3 report is available but not reported can happen when event is configured, and event is not met or reporting configuration is not provided. One scenario where UE may not have reported measurements results are when the event A2 is configured but due to the event is not satisfied, UE may not have reported the results.

  - NW configure MO for the deactivated SCell with short MeascycleScell

  - NW configures event A2 in the reporting configuration of the MO. And maintain SNR above threshold.

  - Time gap between receiving SCell deactivation and SCell activation command is larger than 4s.

  - Under the above conditions UE will have report but not have reported

  - Test whether for L3 report after receiving SCell activation command

- Test configuration for L3 measurement result available but not reported.

  - NW configure MO for the deactivated SCell with short MeascycleScell

  - NW configures event A2 in the reporting configuration of the MO of the MO and maintain the SNR above threshold so that event is not met.

  - Time gap between receiving SCell deactivation and SCell activation command is larger than 4s.

  - Under the above conditions UE will have report but not have reported

  - Test whether for L3 report after receiving SCell activation command

# Summary and Conclusion

In this contribution we have provided our views on the FR2 SCell activation delay reduction performance part and made the following proposals:

- RAN4 to define the test cases for at least following scenarios or requirements

- FR2 unknown SCell activation with L3 report

- Multiple SCell activation delay with FR2 unknown SCell with L3 report

- FR1 PUCCH SCell activation delay with L3 report

- FR2 PUCCH SCell activation delay with FG31-2 and FG31-3

- FR2 Direct SCell activation delay with FG31-2 and FG31-3 at SCell addition

- FR1 unknown SCell activation with FG31-3

- RAN4 to perform the tests for EN-DC, NR-DC and SA scenarios

- Test configuration for L3 measurement result available but not reported.

    - NW configure MO for the deactivated SCell with short MeascycleScell

    - NW configures event A2 in the reporting configuration of the MO and maintain the SNR above threshold so that event is not met.

    - Time gap between receiving SCell deactivation and SCell activation command is larger than 4s.

    - Under the above conditions, UE will have report but not have reported

    - Test whether UE transmits L3 report after receiving SCell activation command

# References

- RP-220977 “WID on Even Further RRM enhancement for NR and MR-DC”, Apple Oppo

- R4-2314446