3GPP TSG-RAN WG4 Meeting #109	                                     R4-2402864

Athens, Greece, 26th Feb 2024 - 1st Mar 2024

Agenda item:			8.13.4

Source:	China Telcom

Title:	WF on the advanced receiver for MU-MIMO scenario

Document for:	Approval

Introduction

This document provides the way forward on advanced receiver for MU-MIMO scenario of Rel-18 NR demodulation requirement evolution WI.

The previous approved WFs are listed as below:

  - R4-2302929 - WF on advanced receiver for MU-MIMO scenario, China Telecom, RAN4#106

  - R4-2305914 - WF on advanced receiver for MU-MIMO scenario, China Telecom, RAN4#106bis-e

  - R4-2309892 - WF on advanced receiver for MU-MIMO scenario, China Telecom, RAN4#107

  - R4-2313993 - WF on advanced receiver for MU-MIMO scenario, China Telecom, RAN4#108

  - R4-2316915 - WF on advanced receiver for MU-MIMO scenario, China Telecom, RAN4#108bis

  - R4-2321114 - WF on advanced receiver for MU-MIMO scenario, China Telecom, Apple, RAN4#109

<Topic #1: Assumptions for the advanced receiver for MU-MIMO>

Sub-topic 1-1 UE Capability Aspect

Capability definition on supported DMRS configuration

Agreement

  - UEs with R-ML for MU-MIMO should support Rel-15 type 1 single symbol DMRS for both target and co-scheduled UE.

  - It is up to UE implementation whether to support other DMRS configurations.

Sub-topic 1-2 Potential required information

The DMRS port information for the co-scheduled UE

Agreement

  - Not to consider additional RRC signaling for DMRS port

Frequency domain resource allocation type for the co-UE and the target UE

Agreement

  - Not to consider additional assumption or signalling on the frequency domain resource allocation type for the co-scheduled UE

New MAC-CE command to assist DMRS port blind detection

Agreement

  - Not to introduce additional MAC-CE based network assistant signalling for DMRS port blind detection.

The modulation order information of the co-scheduled UE (RRC based assistant signaling)

Agreement

  - Do not update the agreed LS to RAN2

<Topic #2: Test parameters and simulation results >

Sub-topic 2-1 Test parameters

Test requirements without modulation order blind detection (DCI index 1-5 is indicated)

- For Rank 1+1 with 2T2R, down select among the following cases:

  - Case#1: Random precoding, TDLC300-100, ULA medium, MCS 13 (Table 1) for Target UE, QPSK for co-UE, full FDRA for the co-UE

  - Case#5: Orthogonal precoding, TDLC300-100, ULA medium, MCS 13 (Table 1) for Target UE, QPSK for co-UE. full FDRA for the co-UE

- For Rank 1+1 with 2T4R:

Agreement

  - Not cover Rank 1+1 with 2T4R for test if test case is defined for rank 2+2

  - IF test requirements are introduced for rank 1+1 with 2T4R, down select among the following cases:

    - Random precoding, TDLC300-100, ULA medium, MCS 13 (Table 1) for Target UE, QPSK for co-UE, full FDRA for the co-UE

    - Orthogonal precoding, TDLC300-100, ULA medium, MCS 13 (Table 1) for Target UE, QPSK for co-UE. full FDRA for the co-UE

- For Rank 2+2 with 4T4R, down select among the following cases:

  - Case#7: Orthogonal precoding, TDLA30-10, ULA Low, MCS 13 (Table 1) for Target UE, QPSK for co-UE, full FDRA for the co-UE

  - Case#8: Orthogonal precoding, TDLA30-10, XP medium, MCS 13 (Table 1) for Target UE, QPSK for co-UE. full FDRA for the co-UE

  - Case#9: Orthogonal precoding, TDLA30-10, ULA Low, MCS 17 (Table 1) for Target UE, 16QAM for co-UE. full FDRA for the co-UE

  - Case#10: Orthogonal precoding, TDLA30-10, XP medium, MCS 17 (Table 1) for Target UE, 16QAM for co-UE. full FDRA for the co-UE

  - Orthogonal precoding, TDLA30-10, XP medium, MCS 17 (Table 1) for Target UE, QPSK for co-UE. full FDRA for the co-UE

Test requirements with modulation order blind detection (DCI index 6 is indicated)

Agreement

- Down select among the Case#21 to Case#34 in R4-2400805:

- Remove Case 35 and 36 in the study given most companies show limited performance gain over the baseline

Whether to tests UE not support BD-MO with R-ML with DCI index 6 is indicated

- Candidate options:

  - Option 1: Introduce test cases only applicable to the UE which can perform E-IRC receiver in that case

  - Option 2: Do not introduce such test for UE not support BD-MO with R-ML

MCS Table

- Candidate options on the RRC assistant information configuration on the MCS table:

  - For UEs not supporting modulation order blind detection:

    - Option 1: No need for the network to inform such information to the UE

    - Option 2: Should be presented regardless of whether the UE supports MO BD

    - Option 2A: ‘64QAM MCS Table’

  - For UEs supporting modulation order blind detection:

    - Option 1: RRC-based assistant signalling on MCS Table should be ‘256QAM MCS Table’

    - Option 2: Align with the MCS Table configuration in the test

Sub-topic 2-2 Test applicability

For UE supporting MO BD, whether to introduce applicability rule to skip test(s) with modulation order indicated

- Candidate options:

  - Option 1: Introduce applicability rule to skip tests with modulation order indicated for UEs capable of BD MO

  - Option 2: Do not introduce applicable rule skip tests with modulation order indicated

Test applicability rule for different test cases

  - Proposal 1:

| UE type | Test applicability | Note |
| --- | --- | --- |
| R-ML for 2 layers across target and co-scheduled UEs with 2 RX with MO signaled | Test 1-1 |  |
| R-ML for up to maxNumberMIMO-LayersPDSCH layers across target and co-scheduled UEs with 4 RX with MO signaled | Test 2-1 Test 3-1 |  |
| R-ML for 2 layers across target and co-scheduled UEs with 2RX with MO Not signaled | Test 1-1 Test 1-2 | Test 1-1 can be skipped if Test 1-2 is passed. |
| R-ML for 2 layers across target and co-scheduled UEs with 4RX with MO Not signaled | Test 2-1 Test 2-2 | Test 2-1 can be skipped if Test 2-2 is passed. |
| R-ML for maxNumberMIMO-LayersPDSCH layers across target and co-scheduled UEs with 4RX with MO Not signaled | Test 2-1 Test 2-2 Test 3-1 Test 3-2 | Test 2-1 can be skipped if Test 2-2 is passed. Test 3-1 can be skipped if Test 3-2 is passed. |
| Test 1-1: 2Tx-2Rx with rank 1+1 with modulation order signaled Test 1-2: 2Tx-2Rx with rank 1+1 with modulation order Not signaled Test 2-1: 2Tx-4Rx with rank 1+1 with modulation order signaled Test 2-2: 2Tx-4Rx with rank 1+1 with modulation order Not signaled Test 3-1: 4Tx-4Rx with rank 2+2 with modulation order signaled Test 3-2: 4Tx-4Rx with rank 2+2 with modulation order Not signaled |  |  |

  - Proposal 2:

| UE feature/capability | Test type |  | Test list | Applicability notes |
| --- | --- | --- | --- | --- |
| MU-MIMO Interference Mitigation advanced receiver with modulation order signalled | FR1 FDD | PDSCH | Clause 5.2.2.1.X Clause 5.2.3.1.X | If UE only support R-ML receivers for MU-MIMO for 2 layers across target and co-scheduled UE with 2Rx, the Test Y1 in clause 5.2.2.1.X should be passed. If UE support R-ML receivers for MU-MIMO for maxNumberMIMO-LayersPDSCH layers across target and co-scheduled UEs with 4RX, the Test Z1 in clause 5.2.3.1.X should be passed. |
|  | FR1 TDD | PDSCH | Clause 5.2.2.2.X Clasue 5.2.3.2.X | If UE only support R-ML receivers for MU-MIMO for 2 layers across target and co-scheduled UE with 2Rx, the Test Y1 in clause 5.2.2.2.X should be passed. If UE support R-ML receivers for MU-MIMO for maxNumberMIMO-LayersPDSCH layers across target and co-scheduled UEs with 4RX, the Test Z1 in clause 5.2.3.2.X should be passed. |
| MU-MIMO Interference Mitigation advanced receiver with modulation order not signalled | FR1 FDD | PDSCH | Clause 5.2.2.1.X Clause 5.2.3.1.X | If UE only support R-ML receivers for MU-MIMO for 2 layers across target and co-scheduled UEs with 2RX, the Test Y2 in clause 5.2.2.1.X should be passed. If UE support R-ML receivers for MU-MIMO for maxNumberMIMO-LayersPDSCH layers across target and co-scheduled UEs with 4RX, the Test Z2 in clause 5.2.3.1.X should be passed. |
|  | FR1 TDD | PDSCH | Clause 5.2.2.2.X Clause 5.2.3.2.X | If UE only support R-ML receivers for MU-MIMO for 2 layers across target and co-scheduled UEs with 2RX, the Test Y2 in clause 5.2.2.2.X should be passed. If UE support R-ML receivers for MU-MIMO for maxNumberMIMO-LayersPDSCH layers across target and co-scheduled UEs with 4RX, the Test Z2 in clause 5.2.3.2.X should be passed. |

<CR work Split >

| Draft CR on | Clause affected | Company |
| --- | --- | --- |
| Applicability Rule | 5.1.1.3 | Samsung |
| FDD 2Rx requirements for advanced receiver for MU-MIMO | 5.2.2.1 | Ericsson |
| FDD 4Rx requirements for advanced receiver for MU-MIMO | 5.2.3.1 | Apple |
| TDD 2Rx requirements for advanced receiver for MU-MIMO | 5.2.2.2 | ZTE |
| TDD 4Rx requirements for advanced receiver for MU-MIMO | 5.2.3.2 | MTK, [Nokia] |
| FRC (Pending decision whether new FRCs need to be introduced or reuse the existing ones) | Annex | Nokia |
| Big CR for 101-4 |  | China Telecom |