3GPP TSG-RAN WG4 Meeting #110	R4-2402865

# Athens, Greece, Feb 26th – Mar 1st, 202

Agenda item:			8.18.9

Source:	Huawei, HiSilicon

Title:	WF on NR_NTN_enh_SAN_UE_demod

Document for:	Approval

- Introduction

This contribution is the way forward for NR NTN enhancement demod for both SAN and UE. The following topics are captured:

- Topic #1: General

- Topic #2: UE demodulation performance requirements

- Topic #3: SAN demodulation performance requirements

Topic #1 General

Sub-topic 1-1: channel model

Issue 1-1-1: Doppler for DL

- Way forward

  - Option 1 (Apple, Huawei): 600Hz

  - Option 2 (Qualcomm): 1200Hz

  - Option 3 (Ericsson): 2000Hz

Issue 1-1-2: Doppler for UL

- Agreements

  - 3000Hz as baseline

Topic #2: UE demodulation performance requirements

Sub-topic 2-1: Test scope

Issue 2-1-1: Test for Disabled HARQ process for above 10 GHz bands

- Agreements:

  - Define performance requirements for Disabled HARQ process

  - Set the number of HARQ Processes as: 4 with feedback disabled, 12 with feedback enabled in 16 HARQ processes with re-Tx disable for all HARQ processes and only transmit initial transmissions with NDI toggling. Throughput shall be measured on processes with HARQ enabled. Which 4 processes to disable are randomly select at test configuration.

| Prop. Channel | MCS | HARQ Config |
| --- | --- | --- |
| NTN-TDLC5-[600] | MCS4 | Disabled HARQ |
|  | MCS4 | 16 HARQ Proc |
| NTN-TDLC5-[600] | MCS4 | 32 HARQ Proc |
|  | MCS13 | 16 HARQ Proc |

Issue 2-1-2: How to define requirements for GSO and NGSO for above 10 GHz bands

- Way forward

  - Option 1: Consider 32 HARQ processes for GSO scenarios and 16 HARQ processes for NGSO scenarios.

  - Option 2: Consider one set of requirements for both NGSO and GSO.

Issue 2-1-3: UE PDCCH demodulation requirements for above 10 GHz bands

- Agreement

  - Define new PDCCH demodulation requirements for above 10GHz bands

    - Sub-topic 2-2: General issues for above 10 GHz bands

Issue 2-2-1: Channel bandwidth

- Agreement

  - 200MHz

Issue 2-2-2: Antenna configuration

- Way forward

  - Option 1: 1Tx1Rx

    - Option 1a: Take 1Tx1Rx for parabolic VSAT antenna configuration for initial demodulation discussion and input from satellite companies is needed

  - Option 2: 1Tx2Rx

    - Option 2a: Need further clarification on impact to demodulation performance with parabolic VSAT antenna configuration

  - Option 3: Both 1Tx1Rx and 1Tx2Rx, with antenna type not limit to parabolic, but also phase antenna array

Issue 2-2-3: Beamforming and beam steering

- Agreements

  - Do not consider beamforming and beam steering for FR2 NTN demodulation requirements.

Issue 2-2-4: Rx phase noise

- Agreements

  - Do not consider PN impact in ideal simulation results alignment. The phase noise impact can be considered in impairment results.

Issue 2-2-5: Applicability rule

- Way forward

  - Option 1: Adding similar applicability rule for FR2 NTN UE optional capabilities as in Rel-17 FR1 NTN UE

| UE-NR-Capability-v1700 |  | Applicability |
| --- | --- | --- |
| nonTerrestrialNetwork-r17 | ntn-ScenarioSupport-r17 |  |
| Supported | GSO only | UE needs to pass the test in Section x of TS38.101-5 |
|  | NGSO only | UE needs to pass the test in Section x of TS38.101-5 |
|  | N/A Note: N/A means UE supports both GSO and NGSO | UE needs to pass the test in Section x of TS38.101-5 |

  - Option 2: Discuss applicability rules once we have concluded the discussion on scenarios and test setup.

    - Sub-topic 2-3: Test setup for above 10 GHz bands

Issue 2-3-1: MCS for PDSCH

- Agreement

  - MCS4 (QPSK, 0.30) and MCS13 (16QAM, 0.48)

Issue 2-3-2: PDCCH aggregation level

- Agreement

  - Option 1: 8 as baseline

  - Option 2: 2 and 4

  - Option 3: 4, 8 and 16

- Topic #3: SAN demodulaton requirements

Sub-topic 3-1: General issues for above 10 GHz bands

Issue 3-1-1: Antenna configuration

- Agreement

  - Do not consider 2Tx

Issue 3-1-2: Phase noise

- Agreement

  - Do not model the phase noise for PUSCH SAN demodulation requirements in FR2-NTN in the ideal simulation assumption. The impact of phase noise can be considered in the impairment results.

Sub-topic 3-2: Test setup for normal PUSCH with CP-OFDM for above 10 GHz bands

Issue 3-2-1: MCS

- Agreement

  - MCS 2/16/20 in Table 1 with downselection based on SNR operating point:

    - MCS: MCS 2, FFS MCS16 and/or MCS20,

    - SNR operating point: should be lower than the link budget suggested SNR value, FFS SNR value

Issue 3-2-2: Additional DM-RS position

- Agreement

  - pos1

Issue 3-2-3: RB assignment

- Agreement

  - Full applicable test bandwidth

Issue 3-2-4: PTRS configuration

- Agreement

  - Not configure PT-RS for all test cases to be defined

  - FFS the configuration of PT-RS for test cases other than MCS2

Issue 3-2-5: Other parameters

- Agreements

| Parameter |  | Value |
| --- | --- | --- |
| Transform precoding |  | Disabled |
| HARQ | Maximum number of HARQ transmissions | 4 |
|  | RV sequence | 0, 2, 3, 1 |
| DM-RS | DM-RS configuration type | 1 |
|  | DM-RS duration | single-symbol DM-RS |
|  | Number of DM-RS CDM group(s) without data | 2 |
|  | Ratio of PUSCH EPRE to DM-RS EPRE | -3 dB |
|  | DM-RS port(s) | {0} |
|  | DM-RS sequence generation | NID=0, nSCID =0 |
| Time domain resource | PUSCH mapping type | B |
|  | Start symbol index | 0 |
|  | Allocation length | 10 |
| Frequency domain resource | Frequency hopping | Disabled |
| Code block group based PUSCH transmission |  | Disabled |

Sub-topic 3-3: Test setup for normal PUSCH with DFT-s-OFDM for above 10 GHz bands

Issue 3-3-1: MCS

- Agreement

  - MCS 2 in Table 1

Issue 3-3-2: Additional DM-RS position

- Agreement

  - pos1

Issue 3-3-3: RB assignment

- Agreement

  - 30 PRBs in the middle of the test bandwidth

Issue 3-3-4: PTRS configuration

- Agreement

  - Do not configure

Issue 3-3-5: Other parameters

- Agreement

| Parameter |  | Value |
| --- | --- | --- |
| Transform precoding |  | Enabled |
| HARQ | Maximum number of HARQ transmissions | 4 |
|  | RV sequence | 0, 2, 3, 1 |
| DM-RS | DM-RS configuration type | 1 |
|  | DM-RS duration | single-symbol DM-RS |
|  | Number of DM-RS CDM group(s) without data | 2 |
|  | Ratio of PUSCH EPRE to DM-RS EPRE | -3 dB |
|  | DM-RS port(s) | {0} |
|  | DM-RS sequence generation | NID=0, nSCID =0 |
| Time domain resource | PUSCH mapping type | B |
|  | Start symbol index | 0 |
|  | Allocation length | 10 |
| Frequency domain resource | Frequency hopping | Disabled |
| Code block group based PUSCH transmission |  | Disabled |

Sub-topic 3-4: Test setup for PUSCH repetition Type A for above 10 GHz bands

Issue 3-4-1: MCS

- Agreement

  - MCS 5 in Table 3

Issue 3-4-2: Other parameters

- Agreement

| Parameter |  | Value |
| --- | --- | --- |
| Transform precoding |  | Disabled |
| HARQ | Maximum number of HARQ transmissions | 4 |
|  | RV sequence | 0, 3, 0, 3 [Note 1] |
| DM-RS | DM-RS configuration type | 1 |
|  | DM-RS duration | single-symbol DM-RS |
|  | Additional DM-RS symbols | Pos1 |
|  | Number of DM-RS CDM group(s) without data | 2 |
|  | Ratio of PUSCH EPRE to DM-RS EPRE | -3 dB |
|  | DM-RS port(s) | 0 |
|  | DM-RS sequence generation | NID=0, nSCID =0 |
| Time domain resource | PUSCH mapping type | B |
|  | Start symbol index | 0 |
|  | Allocation length | 10 |
|  | PUSCH aggregation factor | n2 |
| Frequency domain resource | RB assignment | Full applicable test bandwidth |
|  | Frequency hopping | Disabled |
| Code block group based PUSCH transmission |  | Disabled |
| PT-RS configuration | Frequency density (KPT-RS) | Disabled |
|  | Time density (LPT-RS) | Disabled |
| NOTE 1: The effective RV sequence is {0,2,3,1} with slot aggregation |  |  |

Sub-topic 3-5: Test setup for PUCCH for above 10 GHz bands

Issue 3-5-1: UCI info

- Way forward

  - FFS on the UCI information on PUCCH demodulation requirement.

Issue 3-5-2: Test parameters

- Agreement

    - PUCCH format 0

| Parameter | Test |
| --- | --- |
| Number of UCI information bits | 1 |
| Number of PRBs | 1 |
| First PRB prior to frequency hopping | 0 |
| Intra-slot frequency hopping | N/A for 1 symbol Enabled for 2 symbols |
| First PRB after frequency hopping | The largest PRB index – (Number of PRBs - 1) |
| Group and sequence hopping | neither |
| Hopping ID | 0 |
| Initial cyclic shift | 0 |
| First symbol | 13 for 1 symbol 12 for 2 symbols |
| Antenna configuration | 1x1, 1x2 |
| Propagation condition | NTN-TDLC5-3000 Low |
| Test metric | 1% of DTX to ACK probability 1% of ACK missed detection probability |

    - PUCCH format 1

| Parameter | Test |
| --- | --- |
| Number of information bits | 2 |
| Number of PRBs | 1 |
| Number of symbols | 14 |
| First PRB prior to frequency hopping | 0 |
| Intra-slot frequency hopping | enabled |
| First PRB after frequency hopping | The largest PRB index – (nrofPRBs – 1) |
| Group and sequence hopping | neither |
| Hopping ID | 0 |
| Initial cyclic shift | 0 |
| First symbol | 0 |
| Index of orthogonal cover code (timeDomainOCC) | 0 |
| Antenna configuration | 1x1, 1x2 |
| Propagation condition | NTN-TDLC5-3000 Low |
| Test metric | 1% of DTX to ACK probability 0.1% of NACK to ACK probability 1% of ACK missed detection probability |

    - PUCCH format 2 (ACK missed detection)

| Parameter | Value |
| --- | --- |
| Modulation order | QSPK |
| Starting RB location | 0 |
| Intra-slot frequency hopping | N/A |
| Number of PRBs | 4 |
| Number of symbols | 1 |
| The number of UCI information bits | 4 |
| First symbol | 13 |
| DM-RS sequence generation | NID0=0 |
| Antenna configuration | 1x1, 1x2 |
| Propagation condition | NTN-TDLC5-3000 Low |
| Test metric | 1% of DTX to ACK probability 1% of ACK missed detection probability |

    - PUCCH format 2 (UCI BLER)

| Parameter | Value |
| --- | --- |
| Modulation order | QSPK |
| First PRB prior to frequency hopping | 0 |
| Intra-slot frequency hopping | enabled |
| Frist PRB after frequency hopping | The largest PRB index – (Number of PRBs – 1) |
| Number of PRBs | 9 |
| Number of symbols | 2 |
| The number of UCI information bits | 22 |
| First symbol | 12 |
| DM-RS sequence generation | NID0=0 |
| Antenna configuration | 1x1, 1x2 |
| Propagation condition | NTN-TDLC5-3000 Low |
| Test metric | 1% of BLER |

    - PUCCH format 3

| Parameter | Test 1 | Test 2 |
| --- | --- | --- |
| Modulation order | QPSK |  |
| First PRB prior to frequency hopping | 0 |  |
| Intra-slot frequency hopping | enabled |  |
| First PRB after frequency hopping | The largest PRB index – (Number of PRBs – 1) |  |
| Group and sequence hopping | neither |  |
| Hopping ID | 0 |  |
| Number of PRBs | 1 | 3 |
| Number of symbols | 14 | 4 |
| The number of UCI information bits | 16 | 16 |
| First symbol | 0 | 0 |
| Antenna configuration | 1x1, 1x2 |  |
| Propagation condition | NTN-TDLC5-3000 Low |  |
| Test metric | 1% of BLER |  |

    - PUCCH format 4

| Parameter | Value |
| --- | --- |
| Modulation order | QPSK |
| First PRB prior to frequency hopping | 0 |
| Number of PRBs | 1 |
| Intra-slot frequency hopping | enabled |
| First PRB after frequency hopping | The largest PRB index – (Number of PRBs – 1) |
| Group and sequence hopping | neither |
| Hopping ID | 0 |
| Number of symbols | 14 |
| The number of UCI information bits | 22 |
| First symbol | 0 |
| Length of the orthogonal cover code | n2 |
| Index of the orthogonal cover code | n0 |
| Antenna configuration | 1x1, 1x2 |
| Propagation condition | NTN-TDLC5-3000 Low |
| Test metric | 1% of BLER |

Sub-topic 3-6: Test setup for PRACH for above 10 GHz bands

Issue 3-6-1: Time error tolerance

- Agreement

  - 0.522μ+Tdelay where Tdelay is the largest delay of the propagation channel = 0.06 us

Issue 3-6-2: Requirements under AWGN channel

- Agreement

  - Not define PRACH performance requirements under AWGN channel for above 10GHz bands

Sub-topic 3-7: Test setup for PUSCH with DM-RS bundling for FR1 UL coverage enhancement

Issue 3-7-1: Timing drift

- Agreement

  - Do not model timing drift/frequency drift in the ideal simulation. The impact of timing drift/frequency drift can be considered in the impairment results.

Issue 3-7-2: Antenna configuration

- Agreement

  - Consider both 1Tx1Rx and 1Tx2Rx for PUSCH with DMRS bundling demodulation requirements. Apply the same test applicability for 1T1Rx and 1Tx2Rx performance test defined in TS 38.181.

Issue 3-7-3: Channel model

- Agreement

  - NTN-TDLA100-200

Issue 3-7-4: MCS

- Agreement

  - MCS 4 in Table 1

Issue 3-7-5: Additional DM-RS position

- Agreement

  - pos1

Issue 3-7-6: RB assignment

- Way forward

  - Option 1: Full applicable test bandwidth

  - Option 2: 6RBs for both 15KHz and 30KHz

Issue 3-7-7: PUSCH mapping type

- Agreement

  - A and B. Apply the same test applicability for 1T1Rx and 1Tx2Rx performance test defined in TS 38.181.

Issue 3-7-8: Allocation length

- Agreement

  - 14

Issue 3-7-9: PTRS configuration

- Agreement

  - Do not configure

Issue 3-7-10: RV sequence

- Agreement

  - 0,0,0,0

Issue 3-7-11: PUSCH aggregation factor

- Way forward

  - Option 1: n8

  - Option 2: n4

Issue 3-7-12: pusch-TimeDomainWindowLength

- Way forward

  - Option 1: 8

  - Option 2: 4

Issue 3-7-13: Other parameters

- Agreement

| Parameter |  | Value |
| --- | --- | --- |
| Transform precoding |  | Disabled |
| HARQ | Maximum number of HARQ transmissions | 4 |
| DM-RS | DM-RS configuration type | 1 |
|  | DM-RS duration | single-symbol DM-RS |
|  | Additional DM-RS position | pos1 |
|  | Number of DM-RS CDM group(s) without data | 2 |
|  | Ratio of PUSCH EPRE to DM-RS EPRE | -3 dB |
|  | DM-RS port | 0 |
|  | DM-RS sequence generation | NID0=0, nSCID =0 |
| Time domain resource assignment | Start symbol | 0 |
| Frequency domain resource assignment | Frequency hopping | Disabled |
| Code block group based PUSCH transmission |  | Disabled |
| Note 1: The effective RV sequence is {0, 2, 3, 1} with slot aggregation. |  |  |