3GPP TSG-RAN WG4 Meeting #110	R4-2402870

Athens, Greece, 26 February ‒ 1 March 2024

Title: 	Way forward on [110][325] NR_redcap_enh_demod

Agenda Item:	8.23.5

Source: 	Ericsson

Document for:	Approval

# For agreements

## Work plan

RAN4#110, February 2024 (This meeting) - Agree with the work plan. - Discuss the scope of UE demodulation and CSI reporting requirements. - Agree with the initial simulation assumption for UE demodulation and CSI reporting requirements. RAN4#110-bis, April 2024 - Collect the initial simulation results for alignment. - Finalize the test parameters for UE demodulation and CSI reporting requirements. - Decide the CR work split for TS 38.101-4. RAN4#111, May 2024 - Collect the simulation results. - Agree with the UE demodulation and CSI reporting requirements based on the simulation results with impairments. - Agree with the CR for TS38.101-4.

## UE demodulation and CSI reporting requirements

### UE demodulation requirements

Scope of UE demodulation performance part

- Rel-18 eRedCap UE demodulation performance part focus only on Rel-18 eRedCap UE. Rel-18 eRedCap UE demodulation requirements are specified for UE indicating support of supportOfERedCap-r18.

Whether to define PDSCH demodulation requirements

- Define new PDSCH demodulation requirements for Rel-18 eRedCap UEs in FR1.

Supported UE capability

Define PDSCH demodulation requirements targeting:

- UE not supporting eRedCapNotReducedBB-BW-r18 (FG 48-1)

- UE supporting eRedCapNotReducedBB-BW-r18 (FG 48-2)

Test setup

- Specify PDSCH demodulation requirements for both 1Rx and 2Rx configurations, and for TDD, FDD and HD-FDD configurations.

PDSCH test cases for UE not supporting eRedCapNotReducedBB-BW-r18 (FG 48-1, supportOfERedCap-r18 only)

- RAN4 define the following PDSCH demodulation requirements.

  - FDD/HD-FDD SCS=15kHz

|  | Received antennas | Modulation | Code rate | Rank | PRB size | TBS | Propagation |
| --- | --- | --- | --- | --- | --- | --- | --- |
| QPSK | 1Rx/2Rx | MCS4 in Table 1 | 1/3 | Rank 1 | 25 | 1928 | TDLB100-400 |
| 16QAM | 1Rx/2Rx | MCS13 in Table 1 | 0.47 | Rank 1 | 25 | 6272 | TDLC300-100 |

  - TDD SCS=30kHz

|  | Received antennas | Modulation | Code rate | Rank | PRB size | TBS | Propagation |
| --- | --- | --- | --- | --- | --- | --- | --- |
| QPSK | 1Rx/2Rx | MCS4 in Table 1 | 1/3 | Rank 1 | 12 | 928 | TDLB100-400 |
| 16QAM | 1Rx/2Rx | MCS13 in Table 1 | 0.47 | Rank 1 | 12 | 3104 | TDLC300-100 |

PDSCH test cases for UE supporting eRedCapNotReducedBB-BW-r18 (FG 48-2)

- RAN4 define the following PDSCH demodulation requirements.

  - FDD/HD-FDD SCS=15kHz

|  | Received antennas | Modulation | Code rate | Rank | PRB size | TBS | Propagation |
| --- | --- | --- | --- | --- | --- | --- | --- |
| QPSK | 1Rx/2Rx | MCS4 in Table 1 | 1/3 | Rank 1 | 52 | 3904 | TDLB100-400 |
| 16QAM | 1Rx/2Rx | MCS13 in Table 1 | 0.47 | Rank 1 | 40 | 9992 | TDLC300-100 |

  - TDD SCS=30kHz

|  | Received antennas | Modulation | Code rate | Rank | PRB size | TBS | Propagation |
| --- | --- | --- | --- | --- | --- | --- | --- |
| QPSK | 1Rx/2Rx | MCS4 in Table 1 | 1/3 | Rank 1 | 12 | 928 | TDLB100-400 |
| 16QAM | 1Rx/2Rx | MCS13 in Table 1 | 0.47 | Rank 1 | 25 | 4992 | TDLC300-100 |

Applicability of SDR requirements for eRedCap UEs

- SDR requirements are applicable for Rel-18 eRedCap.

- Existing PRB bundling type and bundling size parameters can be applicable for eRedCap, i.e., PRBs are allocated in contiguous manner.

PDCCH demodulation requirements

- Not define new PDCCH demodulation requirements

  - PDCCH demodulation requirements for Rel-17 RedCap are applicable for Rel-18 eRedCap

PBCH demodulation requirements

- Not define new PBCH demodulation requirements

  - PBCH demodulation requirements for Rel-17 RedCap are applicable for Rel-18 eRedCap

### UE CSI reporting requirements

Whether to define new CSI reporting requirements

- Reuse the same test scenario of Rel-17 RedCap CSI reporting requirements for Rel-18 eRedCap UE.

  - FRC/RMC should adapt to Rel-18 eRedCap UE capabilities (e.g., 10Mbps, baseband BW)

CQI reporting test in static condition

- Define CQI reporting test in static condition for 1Rx/2Rx with TDD/FDD/HD-FDD

  - Reused Rel-17 test scenarios:

    - 1Rx FDD/HD-FDD: TS38.101-4 6.2.1.1.1.1

    - 1Rx TDD: TS38.101-4 6.2.1.2.1.1

    - 2Rx FDD/HD-FDD: TS38.101-4 6.2.2.1.1.4

      - Set to Rank 1

    - 2Rx TDD: TS38.101-4 6.2.2.2.1.5

      - Set to Rank 1

CQI reporting test in fading condition

- Define CQI reporting test in fading condition for 1Rx/2Rx with TDD/FDD/HD-FDD

  - Reused Rel-17 test scenarios:

    - 1Rx FDD/HD-FDD: TS38.101-4 6.2.1.1.2.1

    - 1Rx TDD: TS38.101-4 6.2.1.2.2.1

    - 2Rx FDD/HD-FDD: TS38.101-4 6.2.2.2.1.5

    - 2Rx TDD: TS38.101-4 6.2.2.2.2.4

CQI-TBS table

- Use the following CQI-TBS table to define CQI reporting requirements for Rel-18 eRedCap UE

  - Use rank 1 only for eRedCap CSI reporting requirements.

|  |  |  |  | Rank 1 |  |
| --- | --- | --- | --- | --- | --- |
| TBS Scheme |  |  |  | TBS.1-3y | TBS.1-5y |
| MCS table |  |  |  | 64QAM |  |
| Number of allocated PDSCH resource blocks |  |  |  | 15 | 7 |
| Number of consecutive PDSCH symbols |  |  |  | 12 | 12 |
| Number of PDSCH MIMO layers |  |  |  | 1 | 1 |
| Number of DMRS REs (Note 1) |  |  |  | 24 | 24 |
| Overhead for TBS determination |  |  |  | 0 | 0 |
| Available RE-s |  |  |  | 1800 | 840 |
| CQI index | Spectral efficiency | MCS index | Modulation | Information Bit Payload per Slot |  |
| 0 | OOR | OOR | OOR | N/A | N/A |
| 1 | 0.2344 | 0 | QPSK | 432 | 192 |
| 2 | 0.2344 | 0 |  | 432 | 192 |
| 3 | 0.3770 | 2 |  | 672 | 320 |
| 4 | 0.6016 | 4 |  | 1128 | 504 |
| 5 | 0.8770 | 6 |  | 1608 | 736 |
| 6 | 1.1758 | 8 |  | 2152 | 984 |
| 7 | 1.4766 | 11 | 16QAM | 2664 | 1256 |
| 8 | 1.9141 | 13 |  | 3496 | 1608 |
| 9 | 2.4063 | 15 |  | 4352 | 2024 |
| 10 | 2.7305 | 18 | 64QAM | 4864 | 2280 |
| 11 | 3.3223 | 20 |  | 6016 | 2792 |
| 12 | 3.9023 | 22 |  | 7040 | 3368 |
| 13 | 4.5234 | 24 |  | 8064 | 3824 |
| 14 | 5.1152 | 26 |  | 9224 | 4224 |
| 15 | 5.5547 | 28 |  | 9992 | 4608 |

PMI reporting tests

- Define PMI reporting test for 1Rx/2Rx with TDD/FDD/HD-FDD by reusing the Rel-17 RedCap PMI test setup.

  - For PMI reporting test, the test metric is given as a throughput ratio of follow PMI and random PMI at 90% of peak rate with follow PMI.

  - Reused Rel-17 test scenarios:

    - 1Rx FDD/HD-FDD: TS38.101-4 6.3.1.1.1

    - 1Rx TDD: TS38.101-4 6.3.1.2.1

    - 2Rx FDD/HD-FDD: TS38.101-4 6.3.2.1.1

    - 2Rx TDD: TS38.101-4 6.3.2.2.7

  - Use the following FRC with 25PRBs for SCS=15kHz and 12PRBs for SCS=30kHz

|  | Received antennas | Modulation | Code rate | Rank | PRB size | TBS | Propagation |
| --- | --- | --- | --- | --- | --- | --- | --- |
| FDD/HD-FDD | 1Rx/2Rx | MCS13 in Table 1 | 0.47 | Rank 1 | 25 | 5760 | TDLA30-5 |
| TDD | 1Rx/2Rx | MCS13 in Table 1 | 0.47 | Rank 1 | 12 | 2792 | TDLA30-5 |

RI reporting requirements

- Not define new RI reporting requirements for Rel-18 eRedCap UE

## BS demodulation requirements

### Scope of BS demodulation requirements

- No additional BS demodulation requirements need to be specified for Rel-18 eRedCap.

# Way forward

### UE demodulation requirements

PDSCH demodulation requirements

- Interested companies are encouraged to provide the simulation results in RAN4#110bis.

PDSCH test cases for UE not supporting eRedCapNotReducedBB-BW-r18 (FG 48-1, supportOfERedCap-r18 only)

- FFS whether to define PDSCH demodulation requirements with 64QAM/256QAM.

  - FDD/HD-FDD SCS=15kHz

|  | Received antennas | Modulation | Code rate | Rank | PRB size | TBS | Propagation |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 64QAM 1Rx | 1Rx | MCS19 in Table 1 | 0.50 | Rank 1 | 25 | 9992 | TDLA30-10 |
| 64QAM 2Rx Option 1 | 2Rx | MCS19 in Table 1 | 0.50 | Rank 1 | 25 | 9992 | TDLA30-10 |
| 64QAM 2Rx Option 2 (Same as Rel-17) | 2Rx | MCS19 in Table 1 | 0.50 | Rank 2 | 12 | 9480 | TDLA30-10 |
| 256QAM | 1Rx | MCS20 in Table 2 | 0.67 | Rank 1 | 14 | 9736 | TDLA30-10 |
| 256QAM | 2Rx | MCS24 in Table 2 | 0.82 | Rank 1 | 11 | 9480 | TDLA30-10 |

  - TDD SCS=30kHz

|  | Received antennas | Modulation | Code rate | Rank | PRB size | TBS | Propagation |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 64QAM 1Rx | 1Rx | MCS19 in Table 1 | 0.50 | Rank 1 | 12 | 4736 | TDLA30-10 |
| 64QAM 2Rx Option 1 | 2Rx | MCS19 in Table 1 | 0.50 | Rank 1 | 12 | 4736 | TDLA30-10 |
| 64QAM 2Rx Option 2 (Same as Rel-17) | 2Rx | MCS19 in Table 1 | 0.50 | Rank 2 | 6 | 4736 | TDLA30-10 |
| 256QAM | 1Rx | MCS20 in Table 2 | 0.67 | Rank 1 | 7 | 4864 | TDLA30-10 |
| 256QAM | 2Rx | MCS24 in Table 2 | 0.82 | Rank 1 | 5 | 4352 | TDLA30-10 |

PDSCH test cases for UE supporting eRedCapNotReducedBB-BW-r18 (FG 48-2)

- FFS whether to define PDSCH demodulation requirements with 64QAM/256QAM.

  - FDD/HD-FDD SCS=15kHz

|  | Received antennas | Modulation | Code rate | Rank | PRB size | TBS | Propagation |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 64QAM 1Rx | 1Rx | MCS13 in Table 1 | 0.47 | Rank 1 | 25 | 9992 | TDLC300-100 |
| 64QAM 2Rx Option 1 | 2Rx | MCS19 in Table 1 | 0.50 | Rank 1 | 25 | 9992 | TDLA30-10 |
| 64QAM 2Rx Option 2 (Same as Rel-17) | 2Rx | MCS19 in Table 1 | 0.50 | Rank 2 | 12 | 9480 | TDLA30-10 |
| 256QAM | 1Rx | MCS20 in Table 2 | 0.67 | Rank 1 | 14 | 9736 | TDLA30-10 |
| 256QAM | 2Rx | MCS24 in Table 2 | 0.82 | Rank 1 | 11 | 9480 | TDLA30-10 |

NOTE: 64QAM/256QAM setups are same as the case with not supporting eRedCapNotReducedBB-BW-r18

  - TDD SCS=30kHz

|  | Received antennas | Modulation | Code rate | Rank | PRB size | TBS | Propagation |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 64QAM 1Rx | 1Rx | MCS19 in Table 1 | 0.50 | Rank 1 | 12 | 4736 | TDLA30-10 |
| 64QAM 2Rx Option 1 | 2Rx | MCS19 in Table 1 | 0.50 | Rank 1 | 12 | 4736 | TDLA30-10 |
| 64QAM 2Rx Option 2 (Same as Rel-17) | 2Rx | MCS19 in Table 1 | 0.50 | Rank 2 | 6 | 4736 | TDLA30-10 |
| 256QAM | 1Rx | MCS20 in Table 2 | 0.67 | Rank 1 | 7 | 4864 | TDLA30-10 |
| 256QAM | 2Rx | MCS24 in Table 2 | 0.82 | Rank 1 | 5 | 4352 | TDLA30-10 |

NOTE: 64QAM/256QAM setups are same as the case with not supporting eRedCapNotReducedBB-BW-r18

SDR requirements

- Update TS 38.101-4 5.5.1 as follows:

FR1 single carrier requirements The requirements in this clause are applicable to the FR1 single carrier case. The requirements and procedure defined in Clause 5.5A.1 apply using operating band instead of CA configuration, and bandwidth instead of bandwidth combination. For RedCap and eRedCap, the requirements and procedure are defined in Clause 5.5A.1 except that the MIMO layers are configured to 2 for UE supporting 2 MIMO layers and 1 for UE supporting 1 MIMO layers for all operating band. Antenna configuration is 1x1 for UE supporting 1 layer and 2x2 for UE supporting 2 layers. For eRedCap with reduced peak date rate and reduced baseband bandwidth in FR1, only 25 PRBs for 15kHz SCS and 12RBs for 30kHz SCS are allocated for PDSCH transmission. For RedCap UE with HD-FDD mode, the additional test parameters are specified in Table 5.5.1-1.

Test applicability

- Introduce applicability of demodulation requirements for eRedCap UE as follows in TS 38.101-5 5.1.1.

  - FFS the detailed test applicability

### UE CSI reporting requirements

CQI reporting and PMI reporting requirements

- Interested companies are encouraged to provide the simulation results in RAN4#110bis.

Test applicability

- Introduce applicability of CSI reporting requirements for eRedCap UE as follows in TS 38.101-5 6.1.1

  - FFS the detailed test applicability